September 30, 2022

Matt Walter

Capital Programs Administrator

ODOT-District 3 Office

906 Clark Avenue

Ashland, Ohio 44805

RE: 	Airway/Highway Clearance Analysis

ASD-89-13.36

PID: 114149

Dear Mr. Walter:

We have reviewed the above-mentioned project and have determined that the project is located within 20,000 feet of the Ashland County Airport but will not encroach into the 100:1 notification surface of the Ashland County Airport. This determination is based on an airport elevation of 1,206 feet, a project elevation of 1,213 feet, with a distance of 11,800 feet between the airport and the project locations. The attached diagram will provide a detailed analysis of the study. The study has determined that a height of 111 feet exists between the 100:1 notification surface and the project. For a standard highway intersection improvement project like this, it is not anticipated that any permanent or temporary construction equipment will exceed this 111 foot limit. Therefore, note G118B has been added to the construction plans to require the contractor to complete and file Form 7460‐1 with the FAA at least 30 days prior to the proposed start of construction if they intend to bring out any equipment that exceeds the 111 foot height limit.

The following figures are attached:

Figure 1: Ashland County Airport Notification Triangle

If you have any questions, or need additional information, please contact me at Perry.Morgan@kimley-horn.com.

Sincerely,

Perry Morgan

Kimley-Horn

Z = (C / X) + A - B

Airport Elev, A = 1206 feet

Project Elev, B = 1213 feet

Distance, C = 11,800 feet

Z = 111 ft

100:1 Slope

### Figure 1: Ashland County Airport Notification Triangle

40.920533849, -82.214454033

Note: Notification Necessary