PID 110148, AUG-219-0927: QUANTITY CALCULATIONS

Calculated by:  Lawton Gerlinger, P.E., Date: 04/22/25

Checked by: Dan Grilliot, P.E., Date: x/xx/xx

Revised by:

Roadway, Pavement, Traffic Control (Subsummary on Sheet 9 GP on Sheet 10)

- Item 202 Pavement Removed

  - EPV-1:

    - NW shoulder: 127 SF area

      - Excavation 127 SF x 1.5’ depth = 7 CY

    - SW shoulder: 91 SF area

      - Excavation 91 SF x 1.5’ depth = 5 CY

    - Full depth area excluding approach slab

      - Pavement removed 319 SF/9 = 36 SY

  - EPV-2:

    - NE shoulder: 107 SF area

      - Excavation 107 SF x 1.5’ depth = 6 CY

    - SE shoulder: 101 SF area

      - Excavation 101 SF x 1.5’ depth = 6 CY

    - Full depth area excluding approach slab

      - Pavement removed 307 SF/9 = 35 SY

- Item 202 Wearing course removed

  - EPV-1: Rear approach full depth line to proposed butt joint tie in. approach slabs WC to count = 1328 + 500 SF/9 = 203 SY

  - EPV-2: Forward approach full depth line to proposed butt joint tie in. approach slabs WC to count = 1119 + 500 SF/9 = 180 SY

    - Total: 203 + 180 = 383 SY

  - EPV-3 Bridge: 36 FT x 86.06 FT /9 = 344.24 SY

    - Grand Total= 383 + 344 = 727 SY

- Item 202 Guardrail Removed

  - GR-1: 150 FT

  - GR-2: 150 FT

  - GR-3: 150 FT

  - GR-4: 150 FT

- Item 202 Removal Misc.: Removal of Bridge ID sign and re-erection

  - S-1: 1 ID sign to be removed and replaced.

- Item 204 Subgrade Compaction

  - PV-1: 934 SF from Microstation 934/9 = 104 SY

  - PV-2: 1107 SF from Microstation 1107/9 = 123 SY

    - Total: 104+123 = 227 SY

- Item 209 Linear Grading

  - PV-1: GR-1 and GR-2, Sta. 487+98.50 to Sta. 489+32. Sta. 487+81.98 to Sta. 489+35 : 133.5, 153.02

  - PV-2: GR-3 and GR-4, Sta. 490+30 to Sta. 491+64. Sta. 490+21 to Sta. 491+73 : 134, 152

    - Round both PV-1 and PV-2 to 3 STA. for a total of 6 STA. or 600 ft linear grading.

- Item 301 Asphalt Concrete Base, PG64-22, (449)

  - PV-1: 5 feet of full depth before approach slab. 213 SF from MS, 213 x 8/12 = 142 CF/27 = 6 CY

  - PV-2: 12 feet of full depth after approach slab. 387 SF from MS, 387 x 8/12 = 258 CF/27 = 10 CY

    - Total: 6 + 10 = 16 CY

- Item 304 Aggregate Base

  - PV-1: 6” under approach slab and full depth locations. From MS, 934 SF x 6/12 = 467 CF/27 = 18 CY

  - PV-2: 6” under approach slab and full depth locations. From MS, 1107 SF x 6/12 = 554 CF/27 = 21 CY

    - Total: 18 + 21 = 49 CY

- Item 407 Tack Coat

  - PV-1: 5’ Full depth section all applications will be 0.05 gal/SY. From MS, 214 SF/9 = 24 SY x 0.05 x 3 applications = 3.6 GAL

    - Mill and fill 1.5” surface course will be 0.085 gal/SY. From MS, 1300 SF/9 = 145 SY x 0.085 = 12.33 GAL

  - PV-2: 12’ Full depth section all applications will be 0.05 gal/SY. From MS, 387 SF/9 = 43 SY x 0.05 x 3 applications = 6.45 GAL

    - Mill and fill 1.5” surface course will be 0.085 gal/SY. From MS, 1112 SF/9 = 124 SY x 0.085 = 10.54 GAL

      - Total: 4 + 12 + 7 + 11 = 34 GAL

- Item 441 asphalt concrete surface course, type 1, (449), PG70-22M

  - PV-1: 5’ full depth and wedges area from MS, 214 SF x 1.5/12 = 27 CF/27 = 1 CY

    - Final surface course area from MS, 1515 SF x 1.5/12 = 190 CF/27 = 7 CY

  - PV-2: 12’ full depth and wedges area from MS, 387 SF x 1.5/12 = 49 CF/27 = 2 CY

    - Final surface course area from MS, 1499 SF x 1.5/12 = 188 CF/27 = 7 CY

      - Total: 17 CY

- Item 605 Aggregate Drains

  - Per PDM, every 25’ staggered along roadway. Full depth sections are only 5 and 12 feet long. 4 total 1 in each quadrant. 4 at approx. 8 feet long = 32 Feet

- Item 606 Guardrail, Type MGS

  - GR-5: 50 FT

  - GR-6: 62.5 FT

  - GR-7: 50 FT

  - GR-8: 62.5 FT

    - Transition GR to tie bridge terminal assemblies to end anchors or existing guardrail.

- Item 606 Anchor Assembly, MGS, Type E

  - GR-5: 1 EACH

  - GR-6: 1 EACH

  - GR-7: 1 EACH

  - GR-8: 1 EACH

    - All GR runs on the project need Type E anchor assemblies. Pay length is 57’-1.5” found from Approved Product List for the longest accepted terminal.

- Item 606 MGS Bridge Terminal Assembly, Type TST-2

  - GR-5: 1 EACH

  - GR-6: 1 EACH

  - GR-7: 1 EACH

  - GR-8: 1 EACH

    - Each GR run needs this bridge terminal assembly. Pay length is almost 27 Feet per SCD.

- Item 626 Barrier Reflector, Type 2 Bi-Directional

  - Need reflectors on all new GR. 375 FT along North. 382 FT along South. 100 foot spacing per CMS 626.03. 375/100 = 4 Reflectors 382/100 = 4 Reflectors.

    - 8 total reflectors.

- Item 630 Sign, Flat Sheet, As per Plan

  - 1 SF for new bridge sign. S-2

- Item 630 ground mounted support no. 2 post

  - 2 posts at approx. 5’ each = 10 FT. S-3

- Item 630 Removal of ground mounted sign and reerection

  - 1 sign. 1 Each. S-3

- Item 630 Removal of ground mounted post support and disposal

  - 2 Each. S-3

- Item 646 Edge Line

  - EL-1: 0.05 Miles. CALC: STA. 491+04.85 – STA. 488+58.85 = 246 FT/5280 FT/MILE = 0.04659 MILE

  - EL-2: 0.05 Miles.

- Item 646 CenterLine

  - CL-1: 0.05 Miles.

Erosion Control

- Item 832 Erosion Control

  - 8900 EACH. See BMP estimator spreadsheet

- Item 659 Seeding and Mulching

  - NW Quadrant: 298 SY

  - SW Quadrant: 409 SY

  - SE Quadrant: 292 SY

  - NE Quadrant: 410 SY

    - Total Seeding and Mulching: 1409 SY (See Google Earth File)

- Item 659 Repair Seeding and Mulching

  - 5% of seeding and mulching: 0.05 x 1400 = 70 SY

- Item 659 Commercial Fertilizer

  - (1 TON/7410SY permanent) x 1400 = 0.19 TON

- Item 659 Water

  - (2 x 0.0027 M. GAL/SY) x 1400 = 8 M. GAL

Structures: AUG-219-0927 (Estimated Quantities on Sheet 13)

- Item 202 Portions of Structure Removed, Over 20 foot Span, APP

  - Lump Sum (LS)

- Item 202 Approach Slab Removed

  - 25 FT long x 20 FT wide x 2 Approach slabs = 1000 SF/9 = 112 SY

- Item 202 Bridge Railing Removed

  - First post off the bridge North run: 490+34.30 -6.25 – 489+34.30 = 93.75

  - First post off the bridge South run: 490+43.94 -6.25 – 489+43.94 = 93.75

    - Total bridge rail removed per existing plans = 187.5 FT

- Item 503 Cofferdams and excavation

  - Lump Sum (LS)

- Item 503 Unclassified Excavation

  - Lump Sum (LS)

- Item 505 Pile Driving Equipment Mobilization

  - Lump Sum (LS)

- Item 507 12” cast in place reinforced concrete piles, Driven

  - 16 Total abutment piles x 35 feet = 560 FT

- Item 507 12” cast in place reinforced concrete piles, furnished

  - 16 Total abutment piles x 40 feet = 640 FT

- Item 507 16” cast in place reinforced concrete piles, driven

  - 12 Total pier piles x 50 feet = 600 FT

- Item 507 16” cast in place reinforced concrete piles, furnished

  - 12 Total pier piles x 55 feet = 660 FT

- Item 509 Epoxy coated Steel Reinforcement

  - Abutment/Footing, pier and Approach slab rebar: See sheet 23.

- Item 509 Galvanized Steel Reinforcement

  - Diaphragm and Bridge deck steel: See sheet 26.

- Item 511 Class QC2 Concrete with QC/QA, Superstructure

  - Diaphragms: 3.55 SQ FT from section view, 40.38 feet is the length of Diaphragm. 3.55 SF x 40.38 x 2 diaphragms = 287 CF/27 = 11 CY

  - Transverse Section area, MS is 53.14 SF x 86.01’ bridge limits = 4571 CF/27 = 170 CY

  - Pier cap section area, MS: 5.77 SF x 35.78 FT – [6 x 1.33FT x pi x 1.5 FT] x 2 piers = 338 CF/27 = 13 CY

  - Pier shear key section area, MS is 0.92 SF x 35.78’ x 2 piers = 66 CF/27 = 3 CY

    - Grand Total: 11 + 170 + 13 + 3 = 197 CY

- Item 511 Class QC1 Concrete, Abutment including footing

  - Abutment and footing section area from MS, 14.28 SF x 54.15’ length x 2 abut. = 1547 CF/27 = 58 CY

  - Wingwalls: Left elevation area from MS – 14.79 SF, Right elevation area from MS – 16.65 SF. (14.79 + 16.65) x 2 abutments x 2.25’ thickness = 142 CF/27 = 6 CY

    - Total: 58 + 6 = 64 CY

- Item 516 ½” preformed expansion joint filler

  - Abutment section back side joint filler from MS, 9”/12 wide x 40.55’ long x 2 abutments = 60 SF

- Item 516 1” Preformed Expansion Joint Filler

  - Approach slab filler: 6” length, 1.57’ section view of approach slab x 4 quads = 3 SF

  - Wingwall filler: 3.083’ height x 2.25’ width x 4 quads = 28 SF

  - Abutment filler: 8”/12 width x 40.55’ long x 2 abutments = 54 SF

    - Grand Total: 3 + 28 + 54 = 85 SF

- Item 516 Integral abutment expansion joint seal

  - Deck and approach slab interface: 36’ wide x 1/cos(15) = 37.27 FT x 2 approaches = 75 FT

- Item 516 2” Deep Joint Sealer, APP

  - At interface between concrete and asphalt: 36’ wide x 1/cos(15) = 37.27 FT x 2 approaches = 75 FT

- Item 517 Railing (Three Steel Tube Bridge Railing)

  - 8’ + 72’ + 5’ + 8’ = 93 FT each side x 2 sides = 186 FT

- Item 518 Porous Backfill with Geotextile Fabric

  - 350 SF face of abutment elevation, from MS x 2’ deep x 2 abutments = 1400 CF/27 = 52 CY.

- Item 518 Steel Drip Strip

  - [5.43’ + 5 + 72’ FT per side + (11 posts x 2.0 FT)] x 2 sides = 208.86 or 209 FT

- Item 518 6” Perforated Corrugated Plastic Pipe

  - Perforated is buried pipe behind abutments: 2 abutments x 55 FT = 110 FT

- Item 518 6” Non-Perforated Corrugated Plastic Pipe, Including Specials

  - Non-Perforated is 90-degree bend and out towards the creek: 15 FT each segment x 4 segments = 60 FT

- Item 523 Dynamic load testing

  - 2 EACH

- Item 526 Reinforced Concrete Approach Slabs with QC/QA (T=13”), APP

  - 36 FT wide x 20 FT long x 2 = 1440 SF/9 = 160 SY

- Item 601 Tied concrete block mat with type 1 underlayment

  - 4 total at 2 SY each = 8 SY

- Item 601 Rock Channel Protection, Type C With Filter

  - From MS area 1 of RCP = 1149 SF ; area 2 = 1140 SF  x 2 FT deep = 4578 CF/27 = 170 CY

- Item 611 Precast reinforced concrete outlet

  - 4 EACH, 1 per quad

- Item 625 Structure Grounding system

  - 1 EACH

Maintenance of Traffic

- Item 614 Detour signing

  - Lump Sum (LS)

Incidentals

- Item 614-Maintaining Traffic (LS)

- Item 619-Field Office, Type B 3 (MNTH)

- Item 623-Construction Layout Stakes and Surveying (LS)

- Item 624-Mobilization (LS)

END OF CALCULATIONS

Misc.

BDM 306.2.4 Wingwalls. Max slope behind the wingwall is 1 vertical to 2 horizontal.

909.57’ rear abutment deck edge elevation. Top of rear bench elevation is 906.50’. 3.08 feet. 3.08 x 2 = 6.16 feet or 6’-2” is min. wingwall length to get from roadway grade to bench elevation.

909.36’ forward abutment deck edge elevation. Top of fwd bench elevation is 906.30’. 3.06 feet. 3.06 x 2 = 6.12 feet is min. wingwall length to get from roadway grade to bench elevation.

Each wingwall per plan is just over 8 feet long from roadway grade to the end. Plan wingwall length will accommodate 2:1 slope from roadway grade to bench elevation. 6’-6” is non-skew dimension. Still fits the requirement.