PID 115502, AUG-274-4.31: QUANTITY CALCULATIONS

Calculated by:  Dan Grilliot, P.E., Date: 10/11/23

Checked by: Lawton Gerlinger, P.E., Date: 11/20/23

Revised by: Dan Grilliot, P.E., Date: 4/11/24

Plan Split #1 =  01/BRO/06 = ODOT Field Pave Section; 02/BRO/06 = New Bremen Field Pave Section

Roadway

- Item 201-Clearing and Grubbing (LS)

  - Plan Split #1 = LUMP SUM

  - Plan Split #2 = LUMP SUM

  - Total = LUMP SUM

Erosion Control

- Item 659-Seeding and Mulching (SY)

  - Estimated area north side of road = 2215 sq. ft. (measured around work limits)

  - Estimated area south side of road = 13919 sq. ft. (measured around work limits)

  - Total = 16134 sq. ft. = 1794 sq. yd.

  - Plan Split #1 = 897 sq. yd.

  - Plan Split #2 = 897 sq. yd.

- Item 659-Commerical Fertilizer (TON)

  - 1 ton per 7410 sq. yd. of permanent seeded area

  - 1794 sq. yd x (1 ton/7410 sq. yd.) = 0.24 ton = Total

  - Plan Split #1 = 0.12 ton

  - Plan Split #2 = 0.12 ton

- Item 659-Water (MGAL)

  - 1794 sq. yd x 2 applications x 0.0027 MGAL/sq. yd. = 9.7 MGAL = 10 MGAL = Total

  - Plan Split #1 = 5 MGAL

  - Plan Split #2 = 5 MGAL

- Item 832- Erosion Control

  - Work considered maintenance with $1,000 provided for one location.

  - Plan Split #1 = $500

  - Plan Split #2 = $500

  - Total = $1,000

Drainage

- Item 202-Pipe Cleanout Over 48” (FT)

  - Plan Split #1 = 192 ft.

  - Plan Split #2 = 458 ft.

  - Total = 650 ft.

- Item 503-Cofferdams and Excavation Bracing (LS)

  - Plan Split #1 = LUMP SUM

  - Plan Split #2 = LUMP SUM

  - Total = LUMP SUM (LS)

- Item 611-Field Paving of Existing Pipe, 6’-1” x 4’-7” Pipe Arch (FT)

  - Plan Split #1 = 96 ft. x 2 pipes = 192 ft.

  - Plan Split #2 = 229 ft. x 2 pipes = 458 ft.

  - Total = 650 ft.

Maintenance of Traffic

- Item 614-Detour Signing (LS)

  - Plan Split #1 = Lump Sum

  - Plan Split #2 = Lump Sum

  - Total = LUMP SUM (LS)

Incidentals

- Item 614-Maintaining Traffic (LS)

  - Plan Split #1 = Lump Sum

  - Plan Split #2 = Lump Sum

  - Total = LUMP SUM (LS)

- Item 623-Construction Layout Stakes and Surveying (LS)

  - Plan Split #1 = Lump Sum

  - Plan Split #2 = Lump Sum

  - Total = LUMP SUM (LS)

- Item 624-Mobilization (LS)

  - Plan Split #1 = Lump Sum

  - Plan Split #2 = Lump Sum

  - Total = LUMP SUM (LS)

END OF CALCULATIONS