BEL-379-2.26 (114189)

Wall Design Notes

2/28/2022

Exploration

Three borings were taken here, two in the roadway and one in the driveway down below the proposed retaining wall. Bedrock varies 21-27’ in the borings, but the top of rock isn’t well defined. For B-001-1, at 17’ it’s characterized as rock where coring starts, but it’s described as clay and “TR” isn’t until 27’ deep. It looks like water is concentrated around 14-15’ deep which is directly above the very weak rock, which is most likely where the failure plane is.

Field Drive

I stopped out at the site on 2/17/22 and concluded that moving the driveway to the south is a good option. It’s flatter here than I thought it would be. With a little fill, I think we can match the grade that the existing driveway was at, plus the sight distance will be better. Centering the driveway 100’ south of the curve warning sign would be optimal and should put us clear of the proposed wall. This will allow more flexibility with the wall type knowing that we don’t have to bury the wall to go under the drive.

Design Section

My typical design section will be along the center of the slide, using B-001-0 and B-001-1. Project survey is combined with Lidar survey to get a full picture of the slope. The head scarp is well defined and extends across both lanes of the roadway. I’m not sure if there is any rock exposed behind the ditch, but the back slope is extremely steep which leads me to believe rock is very shallow under the ditch and drops off quick. There’s an obvious toe bulge pretty shallow down the slope, just above the area that connects the two small fields. There is water pumping out here as well which most likely matches the high water contents in the logs.

The proposed guardrail is offset 15’ and the preliminary wall offset is 19’. I’ll check guardrail clearance once the wall is designed. Based on the amount of wall “stickup”, were most likely looking at a soldier pile wall, not a plug pile.

Gstabl Stability Analysis

With my initial run, F.S. = 2.377 and the failure extends all the way to the toe of slope. I modeled the “questionable” rock as a very weak rock which will prevent the failure from extending into it, which I believe is correct based on water content. I modeled the layer just above this rock as A-7-6 which is what it’s characterized as in the logs. I’m probably going to change this layer to an artificial “soft rock” and drop the cohesive strength until we get a F.S. = 1.00. Dropping the artificial soft rock layer down to c’=0 and φ’=6 and lowering the strength of the overburden A-6a layers gives a F.S. – 1.00. The critical shear surface stays in the upper part of the slope which matches the toe bulge pretty well. The head scarp starts up the backslope a little further than needed, but it’s also pretty close to what I was aiming for. The critical failure surface is X=198.529, Y=142.597, R=101.788.

I’m satisfied with this and will move on the downslope analysis.

Downslope Analysis

With no adjustments to the lower slope, just restricting the failure to below the wall (assumed 19’ offset), the F.S. = 1.156 which makes sense with the thick artificial soft rock layer. The critical failure surface is X=107.721, Y=224.404, R=198.829.

Adjusting the lower slope to fail and running again…I dropped the slope just below the wall 4’ vertically, failing along the failure surface. Since there is a stream at the toe, I did not leave the “raised” toe bulge since it could get eroded away. The adjusted F.S. = 1.306 with a 4’ drop in the existing lower slope. For the rest of my analysis, I’ll model the lower slope at this height and ignore and passive resistance here.

UAslope Analysis

Assuming a 19’ offset

| Spacing (ft) | Load (lbs) |
| --- | --- |
| 3.5 | 61,524 |
| 4.0 | 66,290 |
| 4.5 | 70,164 |
| 5.0 | 73,180 |
| 5.5 | 75,356 |
| 6.0 | 76,742 |

Lpile Analysis

Soldier pile – beam width for top 8’. Assuming 31.3’ beam length for estimate which may need refined. Assuming 10’ rock socket into “questionable” rock.

| Offset | King Diam. | Spacing | Rock Socket | Beam | M Cap. | V Cap. | Defl (in) | Estimate (assuming xx’) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 19’ | 24” | 4.0’ | 10’ | HP-12x53 | 34% | 54% | N/A |  |
| 19’ | 30” | 5.0’ | 10’ | W-18x76 | 144% | 213% | 0.83” |  |
| 19’ | 30” | 6.0’ | 12’ | W-18x76 | 104% | 152% | 0.87” |  |

Based on my Lpile analysis, my preliminary design is going to be a traditional solider pile wall, 30” shafts w/W-18x76 beams. I’ll use 12’ rock sockets to be conservative with this very weak rock.