COS-751-0386

Below are my calculations for fill below OHWM, linear impact etc.

Temporary

TAF:

Length of Channel Impact, width of the TAF footprint  South/North Banks = 100’

South Bank: Length of TAF at OHWM and below = 86.8’

North Bank: Length of TAF at OHWM and below = 82.0’

(86.8’ + 82.0’) x 100’ / 43560 sq. ft./acre = 0.388 acres

Note Cofferdam Areas included in TAF:

15’-0” x 53’-0” / 43560 sq.ft./acre = 0.018 acres x 2 piers 0.036 acres

Area impacted = 0.388 acres

Fill Volume within OHWM limits:

South Bank:

Ave. Depth = 8.32’

Ave. Width of TAF = 85.0’

Area: 7184.9 sq.ft

7185 sq. ft. x 8.32’ / 27 cu.ft./cu. yd. = 2,214 cu.yd.

Pier 1 Cofferdam Ave. Depth = 11.34’

Cofferdam Volume for Pier 1

15’ x 53’ x 11.34’ / 27 cu.ft./cu.yd. = 334 cu. yd.

2,214 cu.yd. + 334 cu.yd. = 2,548 cu.yd.

TAF, fill below OHWM South Bank = 2,548 cu. yd.

North Bank:

Ave. Depth = 7.5’

Ave. Width of TAF = 85.0’

Area: 7184.9 sq.ft

6467 sq. ft. x 7.5’ / 27 cu.ft./cu. yd. = 1,796 cu.yd.

Pier 1 Cofferdam Ave. Depth = 10.0’

Cofferdam Volume for Pier 2

15’ x 53’ x 10.0’ / 27 cu.ft./cu.yd. = 295 cu. yd.

1,796 cu.yd. + 295 cu.yd. = 2,091 cu.yd.

TAF, fill below OHWM North Bank = 2,091 cu. yd.

Permanent

Total Linear feet of impact to the Tuscarawas River = 100’

Dumped Rock Fill, Type C (2’ thick):

South Bank Pier 1 Dumped Rock Fill, Type C:

15’-0” x 53’-0” / 43560 sq.ft./acre = 0.018 acres

North Bank Pier 2 Dumped Rock Fill, Type C:

15’-0” x 53’-0” / 43560 sq.ft./acre = 0.018 acres

Total: 0.018 ac. + 0.018 ac. = 0.036 ac.

Total Permanent Area impacted = 0.036 acres

Volume:

South Bank Pier 1: 15’-0” x 53’-0” x 2’-0” / 27 cu.ft./cu.yd. = 59 cu. yd.

South Bank Pier 1 Dumped Rock below OHWM = 59 cu. yd

North Bank Pier 2: 15’-0” x 53’-0” x 2’-0” / 27 cu.ft./cu.yd. = 59 cu. yd.

North Bank Pier 2 Dumped Rock below OHWM = 59 cu. yd