CUY-14 Utility Bridge: General Notes

- BDM Note [601.2-1]

- BDM Note [601.3-1]

- BDM Note [602.1-4]

- BDM Note [602.2-2]

- DESIGN DATA:

STRUCTURAL STEEL - ASTM A709 GRADE  50  - YIELD STRENGTH  50  KSI

STEEL H-PILES AND PILE CAP - ASTM A572 – YIELD STRENGTH 50 KSI

SHEET PILE – ASTM A328 – YIELD STRENGTH 36 KSI (MINIMUM)

- BDM Note [606.1-1]

- BDM Note [606.4-1]

- BDM Note [610.3-1]

THE FOLLOWING SECTIONS OF THE C&MS APPLY:

Holes for High Strength and Bearing Bolts - C&MS 513.19

High Strength Steel Bolts, Nuts and Washers - C&MS 513.20

Welding - C&MS 513.21

Nondestructive Testing - C&MS 513.25

Shipping, Storage and Erection - C&MS 513.26

- BDM Note [611.3-1]

- ITEM 513 – STRUCTURAL STEEL, MISC.: ERECTION OF TRUSS BRIDGE

THE TEMPORARY TRUSS BRIDGE SHALL BE DELIVERED TO THE PROJECT SITE BY STATE FORCES UPON NOTIFICATION FROM THE CONTRACTOR THAT THE TEMPORARY STEEL ABUTMENTS ARE IN-PLACE AND SUITABLE TO RECEIVE THE BRIDGE. THE CONTRACTOR SHALL BE RESPONSIBLE FOR UNLOADING AND ERECTING THE TRUSS BRIDGE, AND FOR MAINTAINING THE BRIDGE DURING CONSTRUCTION OPERATIONS. UPON COMPLETION OF CONSTRUCTION ACTIVITIES, THE CONTRACTOR SHALL BE RESPONSIBLE FOR REMOVING THE TEMPORARY TRUSS BRIDGE AND LOADING IT ONTO TRANSPORTATION SUPPLIED BY THE STATE FOR RELOCATION OFF-SITE.

COORDINATION WITH ODOT: THE CONTRACTOR SHALL COORDINATE WITH THE DEPARTMENT FOR DELIVERY AND REMOVAL OF THE TEMPORARY TRUSS BRIDGE USING THE FOLLOWING CONTACT INFORMATION:

Patrick A. Toman, P.E.

District Bridge Design Engineer
ODOT District 12: Cuyahoga, Geauga & Lake Counties
5500 Transportation Blvd., Garfield Heights, Ohio 44125
(p) 216.584.2147

COORDINATION WITH ACROW: THE CONTRACTOR SHALL COORDINATE WITH THE FABRICATOR OF THE TEMPORARY TRUSS BRIDGE FOR SITE-SPECIFIC ERECTION AND REMOVAL REQUIREMENTS USING THE FOLLOWING CONTACT INFORMATION:

Abbey Smith

Business Development Manager, Great Lakes Region, Ohio office
181 New Road — Suite 202, Parsippany, New Jersey 07054
(M) 419.322.9705

(O) 973.244.0080

MAINTENANCE:  Maintain all portions of the temporary structure in good condition with regard to strength AND safety. Maintain the waterway opening shown on the plans at all times.  If debris accumulates within the waterway opening or on any part of the structure promptly remove the debris.  The Department will compensate for debris removal according to C&MS 109.05.

CLOSING OF THE TEMPORARY STRUCTURE:  If for any reason or at any time the

temporary structure’s ability to safely carry THE DESIGN LOADING is in question, immediately take the actions necessary to ENSURE THE SAFETY OF THE PUBLIC AND CONSTRUCTION WORKERS, repair and reopen the temporary structure. When closing a temporary structure for this purpose, immediately notify the Engineer. Water elevations exceeding the design year highwater elevation or an excessive accumulation of debris within the waterway opening shall be sufficient reasons to close the temporary structure. Mark the design year highwater elevation with fluorescent paint on the temporary structure, at a visible location.  The Department will consider the costs associated with closing the temporary structure to be incidental to ITEM 513 – STRUCTURAL STEEL, MISC.: ERECTION OF TRUSS BRIDGE.