1401 South Main Street, Suite 203

North Canton, OH 44720

Phone:  330-794-5490

Fax:  330-794-5694

www.stahlsheaffer.com

September 25, 2020

Re:	CUY-087-15.12

Village of Hunting Valley

Village of Moreland Hills

Cuyahoga County, OH

Hydrologic and Hydraulic Analysis

Stahl Sheaffer Engineering (SSE) performed an analysis to reflect the existing, proposed, and temporary hydraulic conditions for the replacement of the culvert that carries SR 87 over Griswald Creek.

The existing structure is a reinforced concrete box culvert with a 10’ span and 4’ rise.  The existing culvert slope is 0.458% with approximately 3 feet of cover. .  The existing culvert has two bends or three sectionsand is skewed under the roadway.  There is also an existing arch over the existing box culvert that is proposed to be removed as part of this project.  The proposed structure is not expected to impact any upstream or downstream structures near the site.  Field observations and survey data show that the stream alignment to the existing structure is good, with the upstream channel meeting the existing inlet perpendicular.  The proposed structure will be a partial replacement of the existing structure.   The existing upstream section will remain, the center and last sections will be replaced with a 10’ x 7’ box culvert with a portion of the last section shortened in length vy approximately 50 feet.

# Hydraulic Criteria

According to ODOT’s Location & Design Manual Volume 2, Section 1004.2, the design-year frequency for other highways with over 2,000 ADT is the 25-year storm.   Allowable headwater design controls for the design storm per 1006.2-A for drainage areas 1,000 acres or less shall not exceed 1’ below the near low edge of pavement. Other criteria per 1006.2.2 may also apply but the culvert inlet condition however, will remain unchanged and controls the hydraulics at this location therefore the inlet headwater elevations will remain unchanged.  The proposed structure condition matches existing conditions, except that the length has been reduced.  The proposed structure will be replaced within ODOT ROW at a slightly larger opening, 10’ x 7’, to allow for ease of inspection.

For temporary construction conditions, the water surface elevation from the calculated standard temporary dischargeSTD in the channel cross-section was determined in the design hydraulic model.  Culverts to provide A cross-sectional area allowing an unimpeded discharge equal to twice the highest monthly flow without producing a rise in backwater above the ordinary highwater mark should be provided for construction phase conditions according to ODOT’s Location & Design Manual Volume 2, Section 1012,.

# Hydrologic Analysis

Griswald Creek drains 0.47 sq. mi. upstream of the structure site at SR87.  The drainage area is rural residential with forestation and brush.  The drainage area is designated as Zone X (area of minimal flood hazard) within the FIS Panel 39035C0233E, according to FEMA FIRMette Map prepared for this study. According to ODOT’s Location & Design Manual Volume 2, Section 1012, the standard temporary discharge (STD) is determined by multiplying the peak average monthly discharge form USGS Streamstats by 2.  The USGS’ StreamStats v4.4.0 hydrologic web application was used to delineate the drainage area and estimate the following peak storm flows:

| Standard Temporary | Design | Check, & Scour Design | Design Check & Scour Check |
| --- | --- | --- | --- |
| QSTD = 3 cfs | Q25 = 198 cfs | Q50 = 243 cfs | Q100 = 292 cfs |

# Hydraulic Analysis

| Existing | Inlet Invert El. | Outlet Invert El. |
| --- | --- | --- |
| 10’ x 4’ box | 964.22 | 962.75 |
| Proposed | Inlet Invert El. | Outlet Invert El. |
| 10’ x 7’ box | 964.22 | 962.99 |

Hydraulic analysis of existing, proposed, and construction phase conditions was performed using the Army Corps of Engineers HEC-RAS, version 5.0.7 river analysis system.  The Manning’s n value used for the channel, 0.040, are normal for clean, winding with some pools and shoals.  The overbank Manning’s n value, 0.06 are for light brush and trees upstream and 0.05 for scattered brush and heavy weeds downstream.  Since the stream is not located in an FIS study area, there are no recorded water surface elevations for the design events, therefore boundary conditions for the models were limited to existing upstream and downstream slopes.  Local testimony was not available for flooding conditions, however, based on the topography and features upstream, it would seem that the stream flood elevation does not reach the adjacent pond or pump station which is also indicated in the model.

Results of the hydraulic analysis confirm that the proposed culvert modifications do not alter the hydraulics or freeboard at the inlet of the culvert.  The proposed model converges to existing conditions upstream and downstream, proving that the proposed construction will have no effect on the culvert crossing.

| Existing Model | 25-yr HW El | 100-yr HW El |
| --- | --- | --- |
| 10’ x 4’ box | 968.11 | 969.29 |
| Proposed Structure | 25-yr HW El | 100-yr HW El |
| 10’ x 7’ box | 968.11 | 969.29 |

For temporary conditions, it will be necessary to divert stream flow by upstream diversion dam, and either bypass pipes or a pump.  Five (5) 1.0’ diameter smooth pipes were evaluated to divert flow through a dam and work area for approximately 230’ which resulted in minor increase above existing water surface elevations of 0.28’.   This increase is still beneath the dam device and does not adversely impact the stream or adjacent properties.  A pump scenario is thought to be the most feasible for construction with the top of dam device set at 1.0’ above the STD elevation at the culvert inlet, or 965.36.

# Hydraulic Conclusions and Recommendations

Results of the hydraulic analysis confirm that the proposed conditions conform to the hydraulic study guidance given in L&D Vol. 2.  The proposed model converges to existing conditions upstream and downstream, proving that the proposed construction will have no effect on the modification of the proposed culvert length. It is recommended that a dam and pump system be proposed for temporary diversion of the stream channel. Coordination with the District Environmental Coordinator is recommended to determine the preferred method for temporary conditions and the waterway permit conditions.

SUPPORTING DATA

FEMA FIRMette

USGS Streamstats Analysis

HECRAS Summary Tables