R0 11/13/18

R1 01/03/19

R2 01/10/19

HYDRAULIC REPORT

GEA-166-1.08

SFN 2801027, PID 104099

S.R. 166 (Rock Creek Road) Over Northwest Tributary of Bates Creek    Hambden Township, Geauga County, Ohio

Submitted to:
 ODOT District 12

Revised, January, 2019

| Prepared by: Stahl Sheaffer Engineering, LLC 1401 North Main Street, Suite 203 North Canton, Ohio 44702 (330) 794-5490 |  |
| --- | --- |

Prepared by

Name of the entity producing the document 
(possibly logo, but no larger than ours)

TABLE OF CONTENTS

1 INTRODUCTION	3

2 EXISTING HYDRAULIC CONDITIONS	3

3 HYRAULIC DESIGN CRITERIA	3

4 HYDROLOGY	4

5 PROPOSED STRUCTURE	4

6 HYDRAULIC ANALYSIS	5

7 SCOUR	7

8 HYDRAULIC CONCLUSIONS	7

Appendix A - FEMA-Firms Map

Appendix B – StreamStats Report

Appendix C – Boundary Condition Regression Analysis

Appendix D – HEC-RAS Figures and Tables (Revised 1-10-19)

Appendix E – RCP Design

Appendix F – Site Photos

## 1 INTRODUCTION

The purpose of this report is to summarize the study performed to determine the required hydraulic opening to convey the design year event for replacement of the existing twin pipe structure under SR166.

## 2 EXISTING HYDRAULIC CONDITIONS

The existing structure is composed of twin 72 in. pipe arches (72”x48”) sloping at 1.6% with 2 ft. of cover and concrete endwalls at each end.  The pipes are 9 ft. o/c, are 50 ft. long and cross SR166 at 90o (0o skew).  There is also an existing concrete box culvert (12’x5’) under Williams Rd. (CR68) approximately 230 ft. upstream of the SR166 crossing.  The hydraulic conditions of the Williams Road structure could be affected by the proposed structure replacement at SR 166. Also, approximately 500’ downstream of SR166 the stream crosses a private residential drive through twin 30” smooth lined corrugated plastic pipes.  Both the upstream and downstream structures were incorporated in the hydraulic analysis.

Field observations and survey data show that the stream alignment to the existing structure is fair to poor, with the upstream channel meandering to the east to meet the existing inlet.  Under normal flow conditions, flow is favored to the southwest pipe.

## 3 HYRAULIC DESIGN CRITERIA

According to ODOT’s Location & Design Manual Volume 2, Section 1004.2, the design-year frequency for other highways with over 2,000 ADT is the 25-year storm.   Allowable headwater design controls for the design storm per 1006.2-A for drainage areas 1,000 acres or less shall not exceed 1’ below the near low edge of pavement.

Applicable check storm controls per 1006.2.2 include the following:

- 2 feet below the lowest ground elevation adjacent to an occupied building for a 50-year storm (it is not intended, however, to lower existing high-water elevations around buildings).

[Stahl Sheaffer has identified the controlling low elevation of 1246.8± for an adjacent building located at 14406 Rock Creek Road, north of the structure location]

- The designer should generally limit the maximum 100-year headwater depth to twice the diameter or rise of the culvert.

- A replacement structure should be sized to prevent overtopping by the 100-year flood where such overtopping would not occur with the existing structure.

- A replacement structure should be sized such that flooding of upstream productive land is not increased for the 100-year flood when compared to the existing structure. Judgment shall be used in implementing this criteria, considering the type of upstream property and sensitivity to the accuracy of the computed flood stages.

## 4 HYDROLOGY

The northwest tributary to Bates Creek drains 110 acres (0.64 sq. mi.) upstream of the structure site at SR166.  The drainage area is primarily rural residential with some forestation and brush.  The drainage area is designated as Zone X (area of minimal flood hazard) within the FIS Panel 39055C0135D, according to FEMA FIRMette Map prepared for this study. The USGS’ StreamStats v4.2.0 hydrologic web application was used to delineate the drainage area and estimate the following peak storm flows:

| Design | Check, & Scour Design | Design Check & Scour Check |
| --- | --- | --- |
| Q25 = 179 cfs | Q50 = 209 cfs | Q100 = 240 cfs |

## 5 PROPOSED STRUCTURES

## The proposed structures analyzed in this hydraulic report were developed after studying six original alternatives (a-f) that were described in Stahl Sheaffer’s Alternatives Evaluation Report (AER).  In that study, we analyzed three 10’ x 4’ and three 9’ x 5’ options of various lengths.  Originally, we recommended a 9’ x 5’ box with a 36’ – 8” length.  The AER’s review comments from ODOT resulted in a 42 ft. length so Stahl Sheaffer further refined the hydraulic analysis using this culvert length.  It was advanced and submitted at Stage 1 with the hydraulic results shown in an amended AER memo, dated 6/12/18.  It was not named as a separate alternate at that time.  This hydraulic report was submitted in response to the Stage 2 Review Comments.  There are two 42 ft. long culverts studied in this report, the first being Alternate ‘g’ (9’ x 5’ x 42’-0”), the same alternate that was refined in the AER Amendment Memo.  Alternate ‘h’ (9’ x 4’ x 42’-0”) was developed in response to Stage 2 Review comments.

## The proposed structure sizes were determined in conjunction with the proposed roadway pavement alternative and maintenance of traffic scheme. Due to the poor structure alignment the existing pipes have with the stream, Stahl Sheaffer proposed the alignment of the new structure be moved approximately 4’ downstation (south west) from the centerline of the existing southwest pipe.  This would require channel work to redirect the upstream meander caused by the poor existing structure alignment. Proposed tie in points to existing stream alignment would be about 70’ upstream and 50’ downstream of the proposed culvert.

The primary difference between Alternates ‘g’ and ‘h’ are that Alternate ‘g’ was recessed below the natural flow line and provides a natural channel bottom, once silted, and Alternate ‘h’ does not, and that the box size for Alternate ‘g’ is a standard ASTM C1577 size and Alternate ‘h’ is not.  Proposed invert elevations were lowered from existing invert elevations by 1.75’ for alternate ‘g’ and 1.00’ for alternate ‘h’.  The purpose of this was to limit impacts to the roadway vertical profile, and to return the flowline of the structure to the current stream grade.  Alternate ‘g’ was modeled with 9” (0.75’) of flow blocked to simulate a silted future condition.  Alternate ‘h’ was modeled with no blockage. Existing and proposed invert elevations (at the initial condition) are shown in the table below.

|  |  | Existing | Inlet Invert El. | Outlet Invert El. |
| --- | --- | --- | --- | --- |
|  |  | 2 - 72”x48” pipe arches | 1230.70± | 1229.91± |
|  |  | Proposed | Inlet Invert El. | Outlet Invert El. |
|  | g | 9’x5’ x 42’-0” box | 1228.95 | 1228.25 |
|  | h | 9’x4’ x 42’-0” box | 1229.70 | 1229.03 |

## 6 HYDRAULIC ANALYSIS

Results of the hydraulic analysis confirm that the proposed model conforms to the hydraulic study guidance given in L&D Vol. 2.  The proposed model converges to existing conditions upstream at STA. 12+50 and downstream at STA. 8+75, proving that the proposed construction will have no effect on the culvert crossing the private residential drive downstream at STA. 4+39. Furthermore, water surface elevations are reduced marginally in some locations.  This is important for the hydraulic performance of the upstream structure at Williams Road.

Hydraulic analysis of existing and proposed models were performed using the Army Corps of Engineers HEC-RAS, version 5.0.3 river analysis system.  In compliance with the scope of services, only 4-sided precast concrete box shapes were evaluated as proposed alternatives. Several sizes and invert elevations were modeled and compared to the existing model.  The proposed alternative selected meets the hydraulic design criteria while minimizing impacts to the existing vertical profile of SR166 and the project footprint.

The Manning’s n value used for the channel, 0.040, are normal for clean, winding with some pools and shoals.  The overbank Manning’s n value, 0.055 are for scattered to light brush and heavy weeds.  Since the stream is not located in an FIS study area, there are no recorded water surface elevations for the design events, therefore boundary conditions for the models were limited to existing upstream and downstream slopes.  The stream slopes were determined by regression analysis of the surveyed channel bottom. Regression analysis was performed in excel.

|  | Hydraulic Results for 25-yr Design Criteria |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Existing Model | Near LE Pavt. El. | 25-yr HW El | Freeboard |
|  |  | 2 - 72”x48” pipe arches | 1236.50 | 1233.63 | 2.87’>1’ |
|  |  | Selected Structure |  | Near LE Pavt. El. |  | 25-yr HW El |  | Freeboard |  |
|  | g | 9’x5’ x 42’-0” box |  | 1236.50 |  | 1233.19 |  | 3.31’>1’ ok |  |
|  | h | 9’x4’ x 42’-0” box | 1236.50 | 1233.21 | 3.29’>1’ ok |

|  | Effect on Williams Road Structure: 25-yr Design Criteria |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Existing Model | Low Chord. El. | 25-yr HW El | Freeboard |
|  |  | 2 - 72”x48” pipe arches | 1235.65 | 1234.42 | 1.23’ |
|  |  | Selected Structure |  | Low Chord. El. |  | 25-yr HW El |  | Freeboard |  |
|  | g | 9’x5’ x 42’-0” box |  | 1235.65 |  | 1234.40 |  | 1.25’ |  |
|  | h | 9’x4’ x 42’-0” box | 1235.65 | 1234.40 | 1.25’ |

|  | Effect on Private Drive Twin 30” Culvert: 25-yr Design Criteria |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Existing Model | Low Roadway El. | 25-yr HW El | Overtopping |
|  |  | 2 - 72”x48” pipe arches | 1230.04 | 1230.70 | 0.66’ |
|  |  | Selected Structure |  | Low Roadway. El. |  | 25-yr HW El |  | Overtopping |  |
|  | g | 9’x5’ x 42’-0” box |  | 1230.04 |  | 1230.70 |  | 0.66’ |  |
|  | h | 9’x4’ x 42’-0” box | 1230.04 | 1230.70 | 0.66’ |

|  | Hydraulic Results for 100-yr Design Criteria |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Existing Model | Near LE Pavt. El. | 100-yr HW El | Freeboard |
|  |  | 2 - 72”x48” pipe arches | 1236.50 | 1234.39 | 2.11’>0’ |
|  |  | Selected Structure |  | Near LE Pavt. El. |  | 100-yr HW El |  | Freeboard |  |
|  | g | 9’x5’ x 42’-0” box |  | 1236.50 |  | 1234.32 |  | 2.18’>0’ ok |  |
|  | h | 9’x4’ x 42’-0” box | 1236.50 | 1234.32 | 2.18’>0’ ok |

|  | Effect on Williams Road Structure: 100-yr Design Criteria |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Existing Model | Low Chord. El. | 100-yr HW El | Freeboard |
|  |  | 2 - 72”x48” pipe arches | 1235.65 | 1235.09 | 0.56’ |
|  |  | Selected Structure |  | Low Chord. El. |  | 100-yr HW El |  | Freeboard |  |
|  | g | 9’x5’ x 42’-0” box |  | 1235.65 |  | 1235.04 |  | 0.61’ |  |
|  | h | 9’x4’ x 42’-0” box | 1235.65 | 1235.04 | 0.61’ |

|  | Effect on Private Drive Twin 30” Culvert: 100-yr Design Criteria |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Existing Model | Low Roadway El. | 100-yr HW El | Overtopping |
|  |  | 2 - 72”x48” pipe arches | 1230.04 | 1230.81 | 0.77’ |
|  |  | Selected Structure |  | Low Roadway. El. |  | 100-yr HW El |  | Overtopping |  |
|  | g | 9’x5’ x 42’-0” box |  | 1230.04 |  | 1230.81 |  | 0.77’ |  |
|  | h | 9’x4’ x 42’-0” box | 1230.04 | 1230.81 | 0.77’ |

|  | Hydraulic Results for 50-yr Design Criteria |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Existing Model | Adjacent Building Low El. | Adjacent 50-yr Water Surface El. | Freeboard |
|  |  | 2 - 72”x48” pipe arches | 1246.8± | 1234.00 | 12.80’>2’ |
|  |  | Selected Structure |  | Adjacent Building Low El. |  | Adjacent 50-yr Water Surface El. |  | Freeboard |  |
|  | g | 9’x5’ x 42’-0” box |  | 1246.8± |  | 1233.19 |  | 13.61’>2’ ok |  |
|  | h | 9’x4’ x 42’-0” box | 1246.8± | 1233.19 | 13.61’>2’ ok |

## 7 SCOUR

Scour countermeasures for the proposed culvert outlet are determined by the proposed outlet velocity and structure rise per L&D Volume 2 section 1107.2 and Figure 1107-7 found in appendix E.  Design year for scour is the 50-yr storm with a scour check for the 100-yr storm per L&D Volume 2 Section 1008.10.1.  Additionally, inlet protection for wingwall footings of full height headwalls shall be armored with Type B rock channel protection with filter per section 1106.2.2.  Design and check values for outlet velocity are as follows:

|  | Outlet Velocity |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Existing Model | 25-yr outlet velocity | 50-yr outlet velocity | 100-yr outlet velocity |
|  |  | 2 - 72”x48” pipe arches | 6.24 fps | 6.98 fps | 7.68 fps |
|  |  | Selected Structure | 25-yr outlet velocity | 50-yr outlet velocity | 100-yr outlet velocity |
|  | g | 9’x5’ x 42’-0” box | 5.86 fps | 6.58 fps | 12.97 fps |
|  | h | 9’x4’ x 42’-0” box | 5.91 fps | 6.63 fps | 12.86 fps |

## 8 HYDRAULIC CONCLUSIONS

Results of the hydraulic analysis confirm that either of the selected culvert options conform to the hydraulic study guidance given in L&D Vol. 2.  The proposed model converges to existing conditions upstream at STA. 12+50 and downstream at STA. 8+75, proving that the proposed construction will have no effect on the culvert crossing the private residential drive downstream at STA. 4+39. Furthermore, water surface elevations are reduced marginally in some locations.  This is important for the hydraulic performance of the upstream structure at Williams Road.  Alternate ‘g’ will provide a recessed initial flow line and an eventual natural channel bottom.  Alternate ‘h’ will match the culvert’s bare concrete invert to the natural stream grade.  Alternate ‘g’ will be made from an ASTM standard 9’ x 5’ box section.  Alternate ‘h’ will require a non-standard section and accompanying special pay items and plan notes; and custom supplier pricing.