September 25, 2020

Re:	SR 3022 over Maranda Run

Center Township, Greene County, Pennsylvania

Stahl Sheaffer Engineering (SSE) performed an analysis to reflect the existing, proposed, and temporary hydraulic conditions for the replacement of the culvert that carries SR 87 over Griswald Creek.

The existing structure is a reinforced concrete box culvert with a 10’ span and 4’ rise.  The existing culvert slope is 0.458% with over 10 ft. of cover and concrete endwalls at each end.  The existing culvert has two bends in the overall length and is skewed under the roadway.  There is also an existing arch over the existing box culvert that is proposed to be removed as part of this project.  The proposed structure is not expected to impact any upstream or downstream structures near the site.

Field observations and survey data show that the stream alignment to the existing structure is good, with the upstream channel meandering to the east to meet the existing inlet.  Under normal flow conditions, flow is favored to the southwest pipe.

# Hydraulic Criteria

According to ODOT’s Location & Design Manual Volume 2, Section 1004.2, the design-year frequency for other highways with over 2,000 ADT is the 25-year storm.   Allowable headwater design controls for the design storm per 1006.2-A for drainage areas 1,000 acres or less shall not exceed 1’ below the near low edge of pavement.

Applicable check storm controls per 1006.2.2 include the following:

- 2 feet below the lowest ground elevation adjacent to an occupied building for a 50-year storm (it is not intended, however, to lower existing high-water elevations around buildings).

[Stahl Sheaffer has identified the controlling low elevation of 1246.8± for an adjacent building located at 14406 Rock Creek Road, north of the structure location]

- The designer should generally limit the maximum 100-year headwater depth to twice the diameter or rise of the culvert.

- A replacement structure should be sized to prevent overtopping by the 100-year flood where such overtopping would not occur with the existing structure.

- A replacement structure should be sized such that flooding of upstream productive land is not increased for the 100-year flood when compared to the existing structure. Judgment shall be used in implementing this criteria, considering the type of upstream property and sensitivity to the accuracy of the computed flood stages.

# Hydrologic Analysis

Griswald Creek drains 0.47 sq. mi. upstream of the structure site at SR87.  The drainage area is rural residential with forestation and brush.  The drainage area is designated as Zone X (area of minimal flood hazard) within the FIS Panel 39055C0135D, according to FEMA FIRMette Map prepared for this study. According to ODOT’s Location & Design Manual Volume 2, Section 1012, the standard temporary discharge (STD) is determined by multiplying the peak average monthly discharge form USGS Streamstats by 2.  The USGS’ StreamStats v4.4.0 hydrologic web application was used to delineate the drainage area and estimate the following peak storm flows:

| Standard Temporary | Design | Check, & Scour Design | Design Check & Scour Check |
| --- | --- | --- | --- |
| QSTD = 3 cfs | Q25 = 198 cfs | Q50 = 243 cfs | Q100 = 240 cfs |

# Hydraulic Analysis

## The proposed structure analyzed matches existing conditions, except that the length has been reduced.  The proposed structure will be replaced within ODOT ROW at a slightly larger opening, 10’ x 7’, in order to allow for ease of inspection.  The culvert inlet condition, however, will remain unchanged and controls the hydraulics at this location.

|  |  | Existing | Inlet Invert El. | Outlet Invert El. |
| --- | --- | --- | --- | --- |
|  |  | 10’ x 4’ box | 964.22 | 962.75 |
|  |  | Proposed | Inlet Invert El. | Outlet Invert El. |
|  |  | 10’ x 7’ box | 964.22 | 962.99 |

Results of the hydraulic analysis confirm that the proposed model conforms to the hydraulic study guidance given in L&D Vol. 2.  The proposed model converges to existing conditions upstream and downstream, proving that the proposed construction will have no effect on the culvert crossing.

Hydraulic analysis of existing and proposed models were performed using the Army Corps of Engineers HEC-RAS, version 5.0.7 river analysis system.  The Manning’s n value used for the channel, 0.040, are normal for clean, winding with some pools and shoals.  The overbank Manning’s n value, 0.06 are for light brush and trees upstream and 0.05 for scattered brush and heavy weeds downstream.  Since the stream is not located in an FIS study area, there are no recorded water surface elevations for the design events, therefore boundary conditions for the models were limited to existing upstream and downstream slopes.  Local testimony was not available for flooding conditions, however, based on the topography and features upstream, it would seem that the stream flood elevation does not reach the adjacent pond or pump station.

|  | Hydraulic Results for 25-yr Design Criteria |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Existing Model | 25-yr HW El | 100-yr HW El | Freeboard |
|  |  | 10’ x 4’ box | 1233.63 | 1234.39 | 2.87’>1’ |
|  |  | Selected Structure | 25-yr HW El | 100-yr HW El | Freeboard |
|  |  | 10’ x 7’ box | 1233.19 | 1234.32 | 3.31’>1’ ok |

|  | Hydraulic Results for Temporary Design Criteria |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Existing Model |  | Adjacent 50-yr Water Surface El. | Freeboard |
|  |  | 10’ x 4’ box culvert |  | 1234.00 | 12.80’>2’ |
|  |  | Selected Structure |  | Adjacent 50-yr Water Surface El. | Freeboard |
|  |  | 10’ x 7’ box culvert |  | 1233.19 | 13.61’>2’ ok |

# Hydraulic Conclusions

Results of the hydraulic analysis confirm that the proposed conditions conform to the hydraulic study guidance given in L&D Vol. 2.  The proposed model converges to existing conditions upstream at STA. 12+50 and downstream at STA. 8+75, proving that the proposed construction will have no effect on the culvert crossing the private residential drive downstream at STA. 4+39. Furthermore, water surface elevations are reduced marginally in some locations.  This is important for the hydraulic performance of the upstream structure at Williams Road.  Alternate ‘g’ will provide a recessed initial flow line and an eventual natural channel bottom.  Alternate ‘h’ will match the culvert’s bare concrete invert to the natural stream grade.  Alternate ‘g’ will be made from an ASTM standard 9’ x 5’ box section.  Alternate ‘h’ will require a non-standard section and accompanying special pay items and plan notes; and custom supplier pricing.