Drainage Design Criteria

# 1010 Maintenance of Traffic Drainage

1010.1 General

Positive drainage during Maintenance of Traffic (MOT) operations is furnished under items 614 and 615 of the CMS for most projects. Evaluate MOT drainage for projects on Interstates and Expressways that have one or more of the following or as directed by the District:

- Multi-phased MOT operations

- Profile changes in the roadway that temporarily create a sag point different than the final design

- Traffic maintained adjacent to concrete barrier with less than 2 feet clear distance from the edge of lane to the edge of barrier

Furnish a minimum dry lane width of 10 feet for each travelled lane. Determine the spread of water on the pavement using a 2 year design frequency unless a different frequency is specified by the District.

Provide MOT drainage by utilizing permanent drainage items for final design and temporary drainage items. Temporary drainage items may include items such as inlets, storm sewers, culverts, ditches, perforated conduits, catch basins, conduits jacked and bored, opening cuts in concrete barrier, French drains, pavement saw cut openings, etcetera. These drainage items may conflict with future MOT phases and may require removal quantities in subsequent MOT phases.

Use permanent drainage items for final design where feasible. Furnish a minimum diameter of 12 inches

for temporary storm sewer and 18 inches for temporary culverts.

Provide temporary drainage items on the MOT plan per plan note D124.

# 1011 Temporary Structures

The design year and other hydraulic requirements for temporary structures are defined in CMS 502.02. Ensure scour depth is accounted for in the in the design of a temporary bridge and foundation.

Show the water surface elevation (“high water”) and velocity of the design year discharge on the temporary structure plans. Ensure the design year discharge does not contact the lowest portion of the superstructure of a temporary bridge.

Culvert pipes may be used in lieu of a bridge structure provided controls specified in Section 1006 are not exceeded for the design year discharge.

Refer to Section 500 of the Bridge Design Manual for other details regarding temporary structures.

1012 Waterway Permit Hydraulic Analysis

A hydraulic analysis is required when Temporary Access Fill is proposed in a waterway. This information is intended for permit application purposes and determining if TAF standard conditions contained within the Waterway Permit Special Provisions are obtainable. This information should not be included in the project plan set. Perform the following:

- Determine the highest average monthly flow from USGS StreamStats (ie: largest of Q1, Q2, Q3...Q12). If a delineation is not feasible in USGS StreamStats, notify the District Environmental Coordinator. Provide technical assistance to determine an appropriate means for obtaining the appropriate design flow.

- Multiply the highest monthly flow by 2 to determine the Standard Temporary Discharge (STD).

Drainage Design Criteria

- Determine the water surface elevation from the STD in the channel cross-section using Manning’s open channel equation. Use the design hydraulic model if a model was required for hydraulic design.

- Provide hydraulic analysis of the permitted Temporary Access Fill constructed to 1-foot above OHWM elevation. Coordinate this analysis with the District Environmental Coordinator (DEC). Determine if culverts and/or partial width construction are feasible to maintain the required Hydraulic Opening. The Hydraulic Opening as defined in the Waterway Special Provisions is “The cross-sectional area allowing an unimpeded discharge equal to twice the highest monthly flow without producing a rise in backwater above the OHWM."

  - If it is determined that the TAF can be constructed with the required Hydraulic Opening, record this discharge on the temporary access fill checklist, which is a component of the Permit Determination Request (PDR). Indicate that the project meets the flow requirements on the checklist.

  - If it is determined that the TAF cannot be constructed with the required Hydraulic Opening, record the STD on the temporary access fill checklist, which is a component of the PDR. Indicate that the project does not meet the flow requirements on the checklist. For projects that cannot provide a Temporary Access Fill with the required Hydraulic Opening, perform the following:

    - Notify the DEC.

    - Perform hydraulic analyses to establish alternative parameters for setting the required STD and TAF as necessary for inclusion into the Waterway Special Provisions. Coordinate the hydraulic analyses with the District Environmental Coordinator, Office of Construction Administration and Office of Environmental Services. The purpose of this analysis is to establish a preliminary design of the TAF for use by the Department when setting contract requirements associated with waterway permitted fills. Provide technical assistance to the Department as necessary for re-evaluating the OHWM, lowering the TAF required elevation, raising the allowable backwater or a combination of these options.