PID 114979, D07-BH-FY25(D): QUANTITY CALCULATIONS

Calculated by: Jun Xie, Date: 8/22/24

Checked by:

Revised by:

Plan Splits:

- 01/STR/13

  - AUG-66-1819

  - DAR-705-0681

  - LOG-47-0369

  - SHE-705-0031

- 02/IMS/47

  - SHE-75-0528

- 03/STR/04

  - SHE-29-1350

Structure: SHE-029-1350

(03/STR/04)

- Item 601E26000 DUMPED ROCK FILL, TYPE B

  - Vol: 68.43sf x 2.5’ = 171cf / 27 = 7cy

(ORD: STA. 713+71 to STA. 713+86)

- Item 659E10000 SEEDING AND MULCHING

- (ORD) 291.09sf / 9 = 33sy

- Item 659E10000 REPAIR SEEDING AND MULCHING

- 33sy x 5% = 2sy

- Item 659E20000 COMMERCIAL FERTILIZER

- 33sy / (7410 sy/ton) = 0.01 TON

- Item 659E35000 WATER

  - 2 applications x 33sy x 0.0027MGal/sy = round up to 1 MGal

- Item 832E30000 EROSION CONTROL

  - SHE-029-1350:

    - 1,000 EACH (minimum quantity for erosion control is sufficient)

- Item 202E11201 PORTIONS OF STRUCTURE REMOVED, AS PER PLAN

  - Lump Sum

- Item 503E21300 UNCLASSIFIED EXCAVATION

  - Lump Sum

- Item 509E25000 UNCOATED STEEL REINFORCEMENT

- 223 lbs (See reinforcement schedule on sheet. 36/54)

- Item 510E10000 DOWEL HOLES WITH NONSHRINK, NONMETALLIC GROUT

  - 16 Dowel holes wing wall/headwall repair.

- Item 511E46610 CLASS QC1 CONCRETE, HEADWALL

- Headwall: 1’ x 2’ x (7’-11”) = 15.83cf /27 = 1cy

- Wingwall repair + extension volume: 2 wing walls x (1.25’ x 0.5sf + 3.18’ x 0.85sf + 1’ x 5.28sf + 4.78’ x 1’ x 3.25’) = 49cf / 27 = 2cy

- Total: 1cy + 2cy = 3cy

Structure: AUG-066-1819

(01/STR/13)

- Item 202E23500 WEARING COURSE REMOVED

  - AUG-66-1819:

    - On Bridge: (STA. 986+17.25 to 987+22.75) (40’ x 105.5’) = 4220sf/9 = 469sy

    - Off Bridge: (STA. 983+50.00 to 986+17.25) 9237.47sf/9 = 1,027sy

    - Off Bridge: (STA. 987+22.75 to 990+00.00) 8984.82sf/9 = 999sy

    - Total = 469sy + 1027sy + 999sy = 2,495sy

- Item 209E60200 LINEAR GRADING

  - (from ORD) (STA. 983+50.00 to 986+17.25) 3 STA + (STA. 987+22.75 to 990+00.00) 3 STA = 6 STA

- Item 407E10000 TACK COAT

  - AUG-66-1819:

    - Off Bridge: (STA. 983+50.00 to 986+17.25) 1027sy x 0.085gal/sy = 88gal

    - Off Bridge: (STA. 987+22.75 to 990+00.00) 999sy x 0.085gal/sy = 85gal

(Application Rate C&MS Table 407.06-1, 0.085gal/sy)

    - Total = 88gal + 85gal = 173gal

- Item 441E50100 ASPHALT CONCRETE SURFACE COURSE, TYPE 1, (449), PG70-22M

    - Off Bridge: (ORD: STA. 983+50.00 to STA. 986+17.25) 9237.47sf (from ORD) x (1.5”/12) = 1155cf/27 = 43cy

    - Off Bridge: (ORD: STA. 987+22.75 to STA. 990+00.00) 8984.82sf (from ORD) x (1.5”/12) = 1124cf/27 = 42cy

      - 43cy + 42cy = 85cy

- Item 621E54000 RAISED PAVEMENT MARKER REMOVED

  - 6 EA

- Item 646E10010 EDGE LINE, 6"

  - 2 edge lines x (650’ + 2 x 5’) / 5280 = 0.25mi

- Item 646E10200 CENTER LINE

  - (650’+ 2 x 5’)/5280 = 0.13mi

- Item 659E10000 SEEDING AND MULCHING

  - AUG-066-1819: (ORD) 1879sf + 1281sf = 3160sf / 9 = 352sy

- Item 659E14000 REPAIR SEEDING AND MULCHING

  - AUG-066-1819: 352sy x 5% = 18sy

- Item 659E20000 COMMERCIAL FERTILIZER

  - AUG-066-1819: 352sy/ (7410 sy/ton) = 0.05 TON

- Item 659E35000 WATER

  - AUG-066-1819:

    - 2 applications x 352sy x 0.0027MGal/Sy = 2 MGal

- Item 832E30000 EROSION CONTROL

  - AUG-066-1819:

    - 1,000 EACH (minimum quantity for erosion control is sufficient)

- Item 509E20001 CONCRETE REINFORCEMENT, REPLACEMENT OF EXISTING CONCRETE REINFORCEMENT, AS PER PLAN

  - Estimate: 50 lbs.

- Item 516E31011 2" DEEP JOINT SEALER, AS PER PLAN

- 2 x 40' = 80'

- Item 518E22300 SPECIAL - STEEL DRIP STRIP

- 2 sides x 17 Posts on bridge per side x 1.5’ + 105.5’ x 2 sides = 262ft

- Item 519E11101 PATCHING CONCRETE STRUCTURE, AS PER PLAN

- Total abutment patch area: 5sf + 7.5sf = 13sf

- Item 848E10201 SUPERPLASTICIZED DENSE CONCRETE OVERLAY USING HYDRODEMOLITION, AS PER PLAN, 1¾”

- (40’ x 105.5’) = 4220sf/9 = 469sy

- Item 848E20001 SURFACE PREPARATION USING HYDRODEMOLITION, AS PER PLAN

- (40’ x 105.5’) = 4220sf/9 = 469sy

- Item 848E30200 SUPERPLASTICIZED DENSE CONCRETE OVERLAY (VARIABLE THICKNESS), MATERIAL ONLY

- Assume 50% measured unsound area.

    - Var. Area = 4220 x 0.5 = 2110sf/9 = 235sy

- Use BDM 403.4.1 average depth of variable thickness = 2".

- Var. Vol = 2110sf x 2"/12 = 352cf/27 = 13cy

- Item 848E50000 HAND CHIPPING

- BDM 403.4.1: 10% of deck area (D7 Pref): 0.10 x 469sy = 47sy. Round up to 50sy.

- Item 848E50101 TEST SLAB, AS PER PLAN

- Lump Sum

Structure: DAR-705-0681

(01/STR/13)

- Item 202E23500 WEARING COURSE REMOVED

  - DAR-705-0681:

    - On Bridge: (32’ x 79.5’) = 2544sf / 9 = 283sy

    - Off Bridge: (ORD: STA. 359+50.25 to STA. 361+15.25) 4187.01sf / 9 = 466sy

    - Off Bridge: (ORD: STA. 361+94.75 to STA. 363+59.75) 4187.01sf / 9 = 466sy

    - Total = 283sy + 466sy + 466sy = 1215sy

- Item 202E38001 GUARDRAIL REMOVED, AS PER PLAN

  - (from ORD) (STA. 359+73.28 to 361+15.25) 142’ + (STA. 361+94.75 to 363+47.01) 152’ + (STA. 359+64.57 to 361+15.25) 151’ + (STA. 361+94.75 to 363+37.93) 143’ = 588’

- Item 209E60200 LINEAR GRADING

  - (from ORD) (STA. 360+13.16 to 361+14.92) 1 STA + (STA. 361+95.08 to 363+21.84) 1.25 STA + (STA. 359+88.16 to 361+14.92) 1.25 STA + (STA. 361+95.08 to 362+96.84) 1 STA = 4.5 STA

- Item 407E10000 TACK COAT

  - DAR-705-0681:

    - Off Bridge: (ORD: STA. 359+50.25 to STA. 361+15.25) 4187.01sf / 9 = 466sy x 0.085gal/sy = 40gal

    - Off Bridge: (ORD: STA. 361+94.75 to STA. 363+59.75) 4187.01sf / 9 = 466sy x 0.085gal/sy = 40gal

(Application Rate C&MS Table 407.06-1, 0.085gal/sy)

    - Total = 80gal

- Item 441E50100 ASPHALT CONCRETE SURFACE COURSE, TYPE 1, (449), PG70-22M

  - DAR-705-0681:

    - Off Bridge: (ORD: STA. 359+50.25 to STA. 361+15.25) 4187.01sf (from ORD) x (1.5”/12) = 524cf / 27 = 20cy

    - Off Bridge: (ORD: STA. 361+94.75 to STA. 363+59.75) 4187.01sf (from ORD) x (1.5”/12) = 524cf /27 = 20cy

      - 20cy + 20cy = 40cy

- Item 606E15050 GUARDRAIL, TYPE MGS

  - (STA. 360+13.16 to 361+14.92) 25’ + (STA. 361+95.08 to 363+21.84) 50’ + (STA. 359+88.16 to 361+14.92) 50’ + (STA. 361+95.08 to 362+96.84) 25’ = 150’

- Item 606E26150 ANCHOR ASSEMBLY, MGS TYPE E (MASH 2016)

  - 4 EA

- Item 606E34600 MGS BRIDGE TERMINAL ASSEMBLY, TYPE TST-2

  - 4 EA

- Item 626E00110 BARRIER REFLECTOR, TYPE 2 (BIDIRECTIONAL)

  - 8 EA

- Item 621E54000 RAISED PAVEMENT MARKER REMOVED

  - 5 EA

- Item 646E10010 EDGE LINE, 6"

  - 2 edge lines x (409.5’+2 x 5’) = 839' / 5280 = 0.16mi

- Item 646E10200 CENTER LINE (DASHED)

- (409.5’ + 2 x 5’) / 5280 = 0.08mi

- Item 659E10000 SEEDING AND MULCHING

  - DAR-705-0681: (ORD) 1700sf + 1700sf + 1700sf + 1700sf + 427sf + 427sf + 131sf = 7785sf / 9 = 865sy

- Item 659E14000 REPAIR SEEDING AND MULCHING

  - DAR-705-0681: 865sy x 5% = 44sy

- Item 659E20000 COMMERCIAL FERTILIZER

  - DAR-705-0681: 865sy/ (7410 sy/ton) = 0.12 TON

- Item 659E35000 WATER

  - DAR-705-0681:

    - 2 applications x 865sy x 0.0027MGal/Sy = 5 MGal

- Item 832E30000 EROSION CONTROL

  - DAR-705-0681:

    - 1,000 EACH (minimum quantity for erosion control is sufficient)

- Item 202E11203 PORTIONS OF STRUCTURE REMOVED, OVER 20 FOOT SPAN, AS PER PLAN

- Lump Sum

- Item 202E38500 BRIDGE RAILING REMOVED

- 2ea x 79.5’ (bridge limits) = 159’

- Item 503E11100 COFFERDAMS AND EXCAVATION BRACING

  - Lump Sum

- Item 503E21300 UNCLASSIFIED EXCAVATION

  - Lump Sum

- Item 507E71200 SPECIAL - PILE ENCASEMENT

  - Total length: 162’ (See sheet 40/54).

- Item 509E20001 CONCRETE REINFORCEMENT, REPLACEMENT OF EXISTING CONCRETE REINFORCEMENT, AS PER PLAN

  - Estimate: 100 lbs.

- Item 509E25000 UNCOATED STEEL REINFORCEMENT

  - Deck edge replacement: 5594 lbs. (See reinforcement schedule on sheet 42/54)

  - Pile encasement: 1554 lbs. (See reinforcement schedule on sheet 40/54)

  - Total: 5594 + 1554 = 7148 lbs.

- Item 511E34410 CLASS QC2 CONCRETE, SUPERSTRUCTURE

- Vol: 2 deck edge x 4.73sf x 79.5' = 752cf / 27 = 28cy

- Item 511E81300 CONCRETE, MISC.: EMBEDDED GALVANIC ANODE

- Deck edge replacement: 2ea x 40 = 80 anodes.

- Item 516E31011 2" DEEP JOINT SEALER, AS PER PLAN

  - 2 x 32' = 64'

- Item 517E70100 RAILING (THREE STEEL TUBE BRIDGE RAILING)

  - 2 edges x (8’ + 72’ + 8’) = 176’

- Item 518E22300 SPECIAL - STEEL DRIP STRIP

  - 10 Posts on bridge per side x 2’ x 2 sides + 79.5’ x 2 sides = 199’

- Item 519E11101 PATCHING CONCRETE STRUCTURE, AS PER PLAN

  - (FWD abut) 12sf + (pier 1 cap) 8sf = 20sf.

- Item 848E10200 SUPERPLASTICIZED DENSE CONCRETE OVERLAY USING HYDRODEMOLITION, 1¾”

  - (32’ x 79.5’) = 2544sf/9 = 283sy

- Item 848E20001 SURFACE PREPARATION USING HYDRODEMOLITION, AS PER PLAN

  - (26’ x 79.5’) = 2067sf/9 = 230sy

- Item 848E30200 SUPERPLASTICIZED DENSE CONCRETE OVERLAY (VARIABLE THICKNESS), MATERIAL ONLY

- Assume 50% measured unsound area.

    - Var. Area = 2067 x 0.5 = 1034sf / 9 = 115sy

- Use BDM 403.4.1 average depth of variable thickness = 2".

- Var. Vol = 1034sf x 2"/12 = 173cf/27 = 7 cy

- Item 848E50000 HAND CHIPPING

- BDM 403.4.1: 10% of deck area (D7 pref.): 0.10 x 230sy = 23sy

Structure: LOG-047-0369

(01/STR/13)

- Item 202E23500 WEARING COURSE REMOVED

  - LOG-047-0369:

    - Off Bridge: (STA. 194+32 to 194+82) 1810.25sf / 9 = 202sy

    - Off Bridge: (STA. 197+05 to 197+55) 1863.91sf / 9 = 208sy

    - Total: 202sy + 208sy = 410sy

- Item 202E38000 GUARDRAIL REMOVED

  - (from ORD) (STA. 193+42.15 to 194+82, LT.) 140’ + (STA. 197+05 to 198+44.84, LT.) 137.5’ + (STA. 193+42.15 to 194+82, RT.) 140’ + (STA. 197+05 to 198+44.84, RT.) 137.5’ = 555’

- Item 407E10000 TACK COAT

  - LOG-047-0369:

    - Off Bridge: (ORD: STA. 194+32 to 194+82) 1810.25sf / 9 = 202sy x 0.085gal/sy = 18gal

    - Off Bridge: (ORD: STA. 197+05 to 197+55) 1863.91sf / 9 = 208sy x 0.085gal/sy = 18gal

(Application Rate C&MS Table 407.06-1, 0.085gal/sy)

    - Total = 18gal + 18gal = 36gal

- Item 441E50100 ASPHALT CONCRETE SURFACE COURSE, TYPE 1, (449), PG70-22M

  - LOG-047-0369:

    - Off Bridge: (ORD: STA. 194+32 to 194+82) 1810.25sf (from ORD) x (1.5”/12) = 227cf / 27 = 9cy

    - Off Bridge: (ORD: STA. 197+05 to 197+55) 1863.91sf (from ORD) x (1.5”/12) = 246cf /27 = 10cy

    - Total: 9cy + 10cy = 19cy

- Item 601E28001 DUMPED ROCK FILL, TYPE D, AS PER PLAN

  - Estimated area 3’ x 3’ x approach slab depth: 1.5’ = 13.5 CU. FT./27 = Round up to 1cy

- Item 606E15050 GUARDRAIL, TYPE MGS

  - (STA. 193+42.15 to 194+82) 112.5’ + (STA. 197+05 to 198+44.84) 62.5’ + (STA. 193+42.15 to 194+82) 112.5’ + (STA. 197+05 to 198+44.84) 62.5’ = 350’

- Item 606E26050 ANCHOR ASSEMBLY, MGS TYPE B

  - 2 EA (NE Quad & SE Quad)

- Item 606E34600 MGS BRIDGE TERMINAL ASSEMBLY, TYPE TST-2

  - 4 EA

- Item 626E00110 BARRIER REFLECTOR, TYPE 2 (BIDIRECTIONAL)

  - 8 EA

- Item 630E85100 REMOVAL OF GROUND MOUNTED SIGN AND REERECTION

  - 2 EA

- Item 630E86010 REMOVAL OF GROUND MOUNTED POST SUPPORT AND REERECTION

  - 2 EA

- Item 621E54000 RAISED PAVEMENT MARKER REMOVED

  - 4 EA

- Item 646E10010 EDGE LINE, 6"

  - 2 edge lines x (323’+2 x 5’) = 666' / 5280 = 0.13mi

- Item 646E10200 CENTER LINE (SOLID/DASHED)

- (323’ + 2 x 5’) / 5280 = 0.07mi

- Item 659E10000 SEEDING AND MULCHING

  - LOG-047-0369: (ORD) 1449sf + 1449sf + 1449sf + 1449sf = 5796sf / 9 = 644sy

- Item 659E14000 REPAIR SEEDING AND MULCHING

  - LOG-047-0369: 644sy x 5% = 33sy

- Item 659E20000 COMMERCIAL FERTILIZER

  - LOG-047-0369: 644sy/ (7410 sy/ton) = 0.09 TON

- Item 659E35000 WATER

  - LOG-047-0369:

    - 2 applications x 644sy x 0.0027MGal/Sy = 4 MGal

- Item 832E30000 EROSION CONTROL

  - LOG-047-0369:

    - 1,000 EACH (minimum quantity for erosion control is sufficient)

- Item 202E11203 PORTIONS OF STRUCTURE REMOVED, OVER 20 FOOT SPAN, AS PER PLAN

- Lump Sum

- Item 202E38500 BRIDGE RAILING REMOVED

  - 2ea x 223’ (bridge limits) = 446’

- Item 509E10000 EPOXY COATED STEEL REINFORCEMENT

  - Deck edge replacement: 6882 lbs. (See reinforcement schedule on sheet 45/54)

- Item 509E20001 CONCRETE REINFORCEMENT, REPLACEMENT OF EXISTING CONCRETE REINFORCEMENT, AS PER PLAN

  - Estimate: 100 lbs.

- Item 511E34444 CLASS QC2 CONCRETE, BRIDGE DECK

  - Vol = 2ea x 2.5' x (1’-8”) x 223' = 1858.4cf / 27 = 69cy

- Item 511E81300 CONCRETE, MISC.: EMBEDDED GALVANIC ANODE

  - 2 deck edges x 110 anodes = 220 anodes.

- Item 516E31011 2" DEEP JOINT SEALER, AS PER PLAN

  - 2 x 40' = 80'

- Item 517E70101 RAILING (THREE STEEL TUBE BRIDGE RAILING), AS PER PLAN

  - 2 edges X (8’ + 216’ + 8’) = 464’

- Item 518E22300 SPECIAL - STEEL DRIP STRIP

  - 28 Posts on bridge per side x 2’ x 2 sides + [223’(bridge limits) – 2 x 3.25’(bridge limit to abutment face)] x 2 sides = 545ft. Round up to 550ft.

- Item 848E10200 SUPERPLASTICIZED DENSE CONCRETE OVERLAY USING HYDRODEMOLITION, 1¾”

  - (40’ x 223’) = 8920sf/9 = 992sy

- Item 848E20001 SURFACE PREPARATION USING HYDRODEMOLITION, AS PER PLAN.

  - (40’ - 2 x 2.5’) x 223’ = 7805sf/9 = 868sy

- Item 848E30200 SUPERPLASTICIZED DENSE CONCRETE OVERLAY (VARIABLE THICKNESS), MATERIAL ONLY

  - 4385sf measured unsound. 4385sf / 8920sf x 100% = 49.2%

  - Based on BDM C403.4.1 table 403-3

    - Measured unsound area 50%, estimating factor 1.50, variable depth thick area of 75%. 1 winter =1.1

    - Var. Area = 7805 x 0.75 = 5854sf / 9 =651sy

  - Use BDM 403.4.1 average depth of variable thickness = 2".

  - Var. Vol = 5854sf x 2"/12 x 1.1= 1074/27 = 40cy

- Item 848E50000 HAND CHIPPING

  - BDM 403.4.1: 10% of deck area (D7 pref.): 0.10 x 868sy = 87sy

Structure: SHE-075-0528

(02/IMS/47)

- Item 202E23500 WEARING COURSE REMOVED

- SHE-075-0528:

    - Off Bridge: (ORD: STA. 18+12.91 to STA. 18+62.91) 3200.7sf / 9 = 356sy;

    - Off Bridge: (ORD: STA. 21+37.09 to STA. 21+87.09) 3200.6sf / 9 = 356sy.

    - Total = 712sy

- Item 407E10000 TACK COAT

- Off Bridge: (ORD: STA. 18+12.91 to STA. 18+62.91) 3200.7sf = 356sy x 0.085gal/sy = 31gal;

- Off Bridge: (ORD: STA. 21+37.09 to STA. 21+87.09) 3200.6sf = 356sy x 0.085gal/sy = 31gal

(Application Rate C&MS Table 407.06-1, 0.085gal/sy)

- Total = 62gal

- Item 441E70100 ASPHALT CONCRETE SURFACE COURSE, TYPE 1, (449), PG70-22M

- Off Bridge: (ORD: STA. 18+12.91 to STA. 18+62.91) 3200.7sf x 1.5”/12 = 400cf /27 = 15cy

- Off Bridge: (ORD: STA. 21+37.09 to STA. 21+87.09) 3200.6sf x 1.5”/12 = 400cf /27 = 15cy

- Total = 30cy

- Unit price benchmark PID 105410. Award price: $575/cy

- Item 646E10110 LANE LINE, 6”

  - 2 x (400’ + 2 x 5’) / 5280 = 0.16mi

- Item 646E10200 CENTER LINE (DOUBLE SOLID)

  - (401’+ 2 x 5’) / 5280 = 0.08mi

- Item 646E10300 FT CHANNELIZING LINE, 8"

  - (84.5’ + 5’) + (146.5’ + 5’) = 241’

- Item 646E20300 LANE ARROW

  - 1 Each

- Item 832E30000 EROSION CONTROL

  - SHE-075-0528:

    - 1,000 EACH (minimum quantity for erosion control is sufficient)

- Item 509E20001 CONCRETE REINFORCEMENT, REPLACEMENT OF EXISTING CONCRETE REINFORCEMENT, AS PER PLAN

  - Estimate: 50 lbs.

- Item 516E31011 2" DEEP JOINT SEALER, AS PER PLAN

  - 2 x 70' = 140'

- Item 519E12300 PATCHING CONCRETE BRIDGE DECK - TYPE B

- 801sf x 1.05 / 9= 94sy

- Item 848E10201 SUPERPLASTICIZED DENSE CONCRETE OVERLAY USING HYDRODEMOLITION, 1¾”

- (64’ x 274.18’) = 17548sf/9 = 1950sy

- Item 848E20001 SURFACE PREPRATION USING HYDRODEMOLITION, AS PER PLAN

  - (64’ x 274.18’) = 17548sf/9 = 1950sy

- Item 848E30200 SUPERPLASTICIZED DENSE CONCRETE OVERLAY (VARIABLE THICKNESS), MATERIAL ONLY

  - 3685sf unsound area.

    - 3685sf/ (274’ x 64’) = 20.86%

  - Based on BDM C403.4.1 table 403-3, estimating factor 2.00 (20% measured unsound), variable thickness area of 40%. 1 winter =1.1

    - Var. Area = 17548sf x 0.4 x 1.1 = 7722sf/9 = 858sy

  - Use BDM 403.4.1 average depth of variable thickness = 2".

    - Var. Vol = 7722sf x 0.17’ = 1287cf / 27 = 49cy

- Item 848E50000 HAND CHIPPING

- (BDM 403.4.1) 10% of variable area: 0.10 x 1950sy x 0.4 = 78sy

- (D07 Overlay Memo) 10% of deck area: 0.1 x 1950sy = 195sy

- Adjusted Hand Chipping Area = (195sy + 78sy) / 2 = 137sy

- Item 848E50200 FULL-DEPTH REPAIR

  - Potential full depth area: 2sf + 2sf + 1sf + 1sf = 6sf

  - Full depth repair volume: 6sf x 8.5”/12 (ex. deck thickness) = 4.25cf/27 = Round up to 1cy.

Structure: SHE-705-0031

(01/STR/13)

- Item 202E23500 WEARING COURSE REMOVED

- SHE-705-0031:

    - On Bridge: 1923.2sf / 9= 214sy

    - Off Bridge: (ORD: STA. 14+00.00 to STA. 16+19.96) 6214.61sf (from ORD) / 9 = 691sy

    - Off Bridge: (ORD: STA. 16+80.06 to STA. 19+00.00) 6153sf (from ORD) / 9 = 684sy

    - Total = 1589sy

- Item 202E38001 GUARDRAIL REMOVED, AS PER PLAN

  - (from ORD) (STA. 15+76.26 to 16+25.48) 62.5’ + (STA. 16+86.08 to 18+44.64) 155’ + (STA. 14+60.12 to 16+14.34) 155’ + (STA. 16+72.02 to 16+96.60) 25’ = 398’

- Item 209E60200 LINEAR GRADING

  - (from ORD) (STA. 14+00.00 to 16+20.93, LT.) 2.25 STA + (STA. 16+90.35 to 19+00.00, LT.) 2.25 STA + (STA. 14+00.00 to 16+10.03, RT.) 2.25 STA + (STA. 16+78.23 to 19+00.00, RT.) 2.25 STA = 9 STA

- Item 407E10000 TACK COAT

  - SHE-705-0031:

    - Off Bridge: (ORD: STA. 14+00.00 to STA. 16+19.96) 691sy x 0.085gal/sy = 59gal

    - Off Bridge: (ORD: STA. 16+80.06 to STA. 19+00.00) 684sy x 0.085gal/sy = 59gal

(Application Rate C&MS Table 407.06-1, 0.085gal/sy)

    - Total = 118gal

- Item 441E50100 ASPHALT CONCRETE SURFACE COURSE, TYPE 1, (449), PG70-22M

    - Off Bridge: (ORD: STA. 14+00.00 to STA. 16+19.96) 6214.61sf (from ORD) x (1.5”/12) = 777cf / 27 = 29cy

    - Off Bridge: (ORD: STA. 16+80.06 to STA. 19+00.00) 6153sf (from ORD) x (1.5”/12) = 770cf /27 = 29cy

      - 29cy + 29cy = 58cy

- Item 617E10100 COMPACTED AGGREGATE

  - SHE-705-0031 SE Quad driveway blend

    - (from ORD) 459.38sf x 2”/12(depth) = 77cf /27 = 3cy

- Item 606E13000 GUARDRAIL, TYPE 5

  - (from ORD) (LT) 37.5’ + (LT) 100’ + (RT) 100’ + (RT) 25’ = 263’

- Item 606E26100 ANCHOR ASSEMBLY, TYPE E

  - 2 EA

- Item 606E26500 ANCHOR ASSEMBLY, TYPE T

  - 2 EA

- Item 606E35140 BRIDGE TERMINAL ASSEMBLY, TYPE 4

  - 4 EA

- Item 626E00110 BARRIER REFLECTOR, TYPE 2 (BIDIRECTIONAL)

  - (STA. 15+76.04 to 16+19.96, LT) 2EA + (STA. 16+19.96 to 16+80.06, LT) 1EA + (STA. 16+80.06 to 18+41.37, LT) 2EA + (STA. 14+61.48 to 16+19.96, RT) 2EA + (STA. 16+19.96 to 16+80.06, RT) 1EA + (STA. 16+80.06 to 16+97.55, LT) 2EA = 10EA

- Item 630E85100 REMOVAL OF GROUND MOUNTED SIGN AND REERECTION

  - (STA. 16+96.60, sign ”322”) 1 EA

- Item 630E86010 REMOVAL OF GROUND MOUNTED POST SUPPORT AND REERECTION

  - (STA. 16+96.60, sign ”322”) 1 EA

- Item 621E54000 RAISED PAVEMENT MARKER REMOVED

  - 8 EA

- Item 646E10010 EDGE LINE, 6"

  - [(497.49’ + 2 x 5’) + (502.52’ + 2 x 5’)]/5280 = 0.20mi

- Item 646E10200 CENTER LINE

  - (500’+ 2 x 5’)/5280 = 0.10mi

- Item 659E10000 SEEDING AND MULCHING

  - SHE-705-0031: (ORD) 3155.60sf + 3789.07sf = 6944.67sf / 9 = 772sy

- Item 659E14000 REPAIR SEEDING AND MULCHING

  - SHE-705-0031: 772sy x 5% = 39sy

- Item 659E20000 COMMERCIAL FERTILIZER

  - SHE-705-0031: 772sy/ (7410 sy/ton) = 0.11 TON

- Item 659E35000 WATER

  - SHE-705-0031:

    - 2 applications x 772sy x 0.0027MGal/Sy = 5 MGal

- Item 832E30000 EROSION CONTROL

  - SHE-705-0031:

    - 1,000 EACH (minimum quantity for erosion control is sufficient)

- Item 202E11203 PORTIONS OF STRUCTURE REMOVED, OVER 20 FOOT SPAN, AS PER PLAN

- Lump Sum

- Item 202E38603 BRIDGE RAILING REMOVED FOR REUSE, AS PER PLAN

  - 2 side x 60.1’ (bridge limits) = 121’

[Bridge railing panel to be replaced on both side of bridge (only one edge to be replaced)]

- Item 503E11100 COFFERDAMS AND EXCAVATION BRACING

  - Lump Sum

- Item 503E21300 UNCLASSIFIED EXCAVATION

  - Lump Sum

- Item 507E71200 SPECIAL - PILE ENCASEMENT

  - Total length: 162’ (See sheet 52/54).

- Item 509E20001 CONCRETE REINFORCEMENT, REPLACEMENT OF EXISTING CONCRETE REINFORCEMENT, AS PER PLAN

  - Estimate: 100 lbs.

- Item 509E25000 UNCOATED STEEL REINFORCEMENT

- Deck edge replacement: 2096 lbs. (See reinforcement schedule on sheet 54/54)

- Wing wall repair + abutment reface: 906 lbs. (See reinforcement schedule on sheet 51/54)

- Pile encasement: 1520 lbs. (See reinforcement schedule on sheet 52/54)

- Total: 2096 + 906 + 1520 = 4522 lbs.

- Item 510E10000 DOWEL HOLES WITH NONSHRINK, NONMETALLIC GROUT

  - 122 (See reinforcement schedule on sheet 51/54)

- Item 511E34410 CLASS QC2 CONCRETE, SUPERSTRUCTURE

  - Vol = 2.59sf x 60.1' = 155.7cf / 27 = 6cy

- Item 511E50210 CLASS QC1 CONCRETE, SUBSTRUCTURE

  - Wingwall repair volume: 42cf + 7cf + 9cf + 35cf = 93cf / 27 = 4cy

- Item 511E81300 CONCRETE, MISC.: EMBEDDED GALVANIC ANODE

  - North deck edge replacement: 26 anodes.

- Item 516E13200 ½” PREFORMED EXPANSION JOINT FILLER

  - 2 abutments x 33’ x 0.5’ = 33sf

- Item 516E31011 2" DEEP JOINT SEALER, AS PER PLAN

  - 2 x 34.05' = 69'

- Item 517E75601 DEEP BEAM BRIDGE RETROFIT RAILING, AS PER PLAN

  - 2 sides x (6.13’ + 6.25’ x 9 panels + 6.13’) = 137’

- Item 517E76302 RAILING, MISC: TYPE B ANCHORS

  - 10 railing posts x 4 anchors (each post) = 40 EA

- Item 518E22300 SPECIAL - STEEL DRIP STRIP

  - 2 sides x (10 Posts on bridge left side x 1.5’ +60.1’) = 151ft

- Item 530E00600 SPECIAL – STRUCTURES, MISC: ABUTMENT REFACING WITH GALVANIC ANODE PROTECTION

  - Total abutment reface area: (R.A.)108sf + (F.A.)109sf = 217sf

- Item 848E10201 SUPERPLASTICIZED DENSE CONCRETE OVERLAY USING HYDRODEMOLITION, 1¾”, AS PER PLAN

  - 1923.2sf (from ORD)/9 = 214sy

- Item 848E20001 SURFACE PREPARATION USING HYDRODEMOLITION, AS PER PLAN

  - 1803.0sf (from ORD, exclude deck edge replacement area)/9 = 201sy

- Item 848E30200 SUPERPLASTICIZED DENSE CONCRETE OVERLAY (VARIABLE THICKNESS), MATERIAL ONLY

  - Assume 50% measured unsound area.

    - Var. Area = 1803sf x 0.5 = 902sf / 9 = 100sy

  - Use BDM 403.4.1 average depth of variable thickness = 2".

  - Var. Vol = 902sf x 2"/12 x 1.1 = 165cf/27 = 6cy

- Item 848E50000 HAND CHIPPING

  - BDM 403.4.1: 10% of deck area (D7 pref.): 0.10 x 1803sf = 180sf/9 = 20sy

END OF CALCULATIONS