PID 116923, DAR-47-0316 and DAR-127-3053: QUANTITY CALCULATIONS

Calculated by:  Lawton Gerlinger, P.E., Date: 3/4/25

Checked by: Dan Grilliot, P.E., Date: 4/3/25

Revised by:

Roadway, Pavement, Traffic Control DAR-47

- Item 202 Pavement Removed

  - PV-1: 2337.50 SF from Microstation = 2205 SF/9 = 245 SY

  - PV-2: 2310.40 SF from Microstation = 2163 SF/9 = 241 SY

    - Total: 245 + 241 = 486 SY

- Item 202 Guardrail Removed, APP

  - GR-1: 150 FT

  - GR-2: 150 FT

  - GR-3: 150 FT

  - GR-4: 150 FT

- Item 204 Subgrade Compaction

  - PV-1: 30’ x 75’ = 2250 SF/9 = 250 SY

  - PV-2: 30’ x 75’ = 2250 SF/9 = 250 SY

    - Total: 250 + 250 = 500 SY

- Item 209 Linear grading

  - 485’ + 100’ BTA round up to 600 = 6 Total Stations

- Item 301 Asphalt Concrete Base, PG64-22, (449)

  - PV-1: 25’ x 75’ x .5 = 937.5 CF/27 = 35 CY

  - PV-2: 25’ x 75’ x .5 = 937.5 CF/27 = 35 CY

    - Total: 35 + 35 = 70 CY

- Item 304 Aggregate Base

  - PV-1: 26’ x 75’ x .5 = 975 CF/27 = 36 CY

  - PV-2: 26’ x 75’ x .5 = 975 CF/27 = 36 CY

    - Total: 36 + 36 = 72 CY

- Item 407 Tack Coat (0.055 GAL/SY)

  - PV-1: (75’ x 24’) + (75’ x 25’) = 3675 SF/9 = 409 SY x 0.055 GAL/SY = 23 GAL

  - PV-2: (75’ x 24’) + (75’ x 25’) = 3675 SF/9 = 409 SY x 0.055 GAL/SY = 23 GAL

    - Total: 23 + 23 = 46 GAL

- Item 441 Asphalt Concrete Surface Course, Type 1, (449), PG70-22M

  - PV-1: 75’ x 30’ x 3/12 = 562.5 CF/27 = 21 CY

  - PV-2: 75’ x 30’ x 3/12 = 562.5 CF/27 = 21 CY

    - Total = 42 CY

- Item 606 Guardrail, Type MGS

  - GR-1: 75 FT

  - GR-2: 75 FT

  - GR-3: 75 FT

  - GR-4: 75 FT

    - Transition GR to tie bridge terminal assemblies to end anchors or existing guardrail.

- Item 606 Anchor Assembly, MGS, Type E

  - GR-1: 1 EACH

  - GR-2: 1 EACH

  - GR-3: 1 EACH

  - GR-4: 1 EACH

- Item 606 MGS Bridge Terminal Assembly, Type TST-2

  - GR-1: 1 EACH

  - GR-2: 1 EACH

  - GR-3: 1 EACH

  - GR-4: 1 EACH

    - Each GR run needs this bridge terminal assembly. Pay length is almost 27 Feet per SCD.

- Item 626 Barrier Reflector, Type 2 Bi-Directional

  - Need reflectors on all new GR. 300’ GR runs. 100 foot spacing per CMS 626.03. 300/100 = 3 Reflectors per side. 6 total.

- Item 630 Sign, flat sheet, as per plan

  - Each sign is 24” x 4” round to 1 SF per sign. 2 signs so 2 SF.

- Item 646 Edge Line

  - EL-1: 0.04 Miles. CALC: STA. 166+34.54 – STA. 164+44.54 = 190 FT/5280 FT/MILE = 0.03598 MILE

  - EL-2: 0.04 Miles.

- Item 646 CenterLine

  - CL-1: 0.08 Miles.

Roadway, Pavement, Traffic Control DAR-127

- Item 202 Pavement Removed

  - PV-1: 4251.35 SF from Microstation = 4147 SF/9 = 461 SY

  - PV-2: 5473.90 SF from Microstation = 5265 SF/9 = 585 SY

    - Total: 461 + 585 = 1046 SY

- Item 202 Guardrail Removed, APP

- GR-1: 125 FT

- GR-2: 125 FT

- GR-3: 125 FT

- GR-4: 125 FT

- Item 202 Removal Misc.: Removal of Bridge ID sign and reerection

  - 2 Signs. Location on Site plan

- Item 204 Subgrade Compaction

- PV-1: 34’ x 112’ + 15’ x 44’= 4468 SF/9 = 497 SY

- PV-2: 34’ x 136’ + 15’ x 44’ = 5284 SF/9 = 587 SY

    - Total: 497 + 587 = 1084 SY

- Item 209 Linear grading

- 290’ round up to 300 = 3 STA per side = 6 Total Stations

- Item 301 Asphalt Concrete Base, PG64-22, (449)

- PV-1: 37’ x 112’ x .5 = 2072 CF/27 = 77 CY

- PV-2: 37’ x 136’ x .5 = 2516 CF/27 = 93 CY

    - Total: 77 + 93 = 170 CY

- Item 304 Aggregate Base

- PV-1: 38’ x 112’ x .5 + 15’ x 44’ x .5’ = 2458 CF/27 = 91 CY

- PV-2: 38’ x 136’ x .5 + 15’ x 44’ x .5’ = 2914 CF/27 = 108 CY

    - Total: 91 + 108 = 199 CY

- Item 407 Tack Coat (0.055 GAL/SY)

- PV-1: (112’ x 36’ x 2 applications) = 8064 SF/9 = 896 SY x 0.055 GAL/SY = 50 GAL

- PV-2: (136’ x 36’ x 2 applications) = 9792 SF/9 = 1088 SY x 0.055 GAL/SY = 60 GAL

    - Total: 50 + 60 = 110 GAL

- Item 441 Asphalt Concrete Surface Course, Type 1, (449), PG70-22M

- PV-1: 112’ x 36’ x 3/12 = 1008 CF/27 = 37 CY

- PV-2: 136’ x 36’ x 3/12 = 1224 CF/27 = 45 CY

    - Total = 82 CY

- Item 601 Rock channel protection Type c

  - Plan area of rear abutment from Microstation, 254.25 SF x 2’ depth = 508.50 CF/27 = 19 CY

  - Plan area of forward abutment from Microstation, 256.50 SF x 2' depth = 513 CF/27 = 19 CY

    - Total = 38 CY

- Item 606 Guardrail, Type MGS

- GR-1: 62.5 FT

- GR-2: 75 FT

- GR-3: 50 FT

- GR-4: 37.5 FT

    - Transition GR to tie bridge terminal assemblies to end anchors or existing guardrail.

- Item 606 Anchor Assembly, MGS, Type E

- GR-1: 1 EACH

- GR-2: 1 EACH

- GR-3: 1 EACH

- GR-4: 1 EACH

- Item 606 MGS Bridge Terminal Assembly, Type TST-2

- GR-1: 1 EACH

- GR-2: 1 EACH

- GR-3: 1 EACH

- GR-4: 1 EACH

    - Each GR run needs this bridge terminal assembly. Pay length is almost 27 Feet per SCD.

- Item 626 Barrier Reflector, Type 2 Bi-Directional

- Need reflectors on all new GR. 300’ GR runs. 100 foot spacing per CMS 626.03. 300/100 = 3 Reflectors per side. 6 total.

- Item 646 Edge Line

- EL-1: 0.06 Miles. CALC: STA. 846+87.21 – STA. 843+77.21 = 310 FT/5280 FT/MILE = 0.0587 MILE

- EL-2: 0.06 Miles.

- Item 646 CenterLine

- CL-1: 0.06 Miles.

Erosion Control

- Item 832 Erosion Control

  - 5000 EACH

- Item 659 Seeding and Mulching

  - DAR-47

    - NW Quadrant: 102 SY

    - SW Quadrant: 188 SY

    - NE Quadrant: 110 SY

    - SE Quadrant: 128 SY

      - Total Seeding and Mulching: 102+188+110+128 = 528 SY

- Item 659 Repair Seeding and Mulching

  - 5% of seeding and mulching: 0.05 x 528 = 26 SY

- Item 659 Commercial Fertilizer

  - (1 TON/7410SY permanent) x 528 + (1 TON/11,111 SY interseeding) x 26 = 0.07 TON

- Item 659 Water

  - (2 x 0.0027 M. GAL/SY) x 528 + (0.0027 M. GAL/SY) x 26 = 3 M. GAL

- Item 659 Seeding and Mulching

- DAR-127

    - NW Quadrant: 196 SY

    - SW Quadrant: 200 SY

    - NE Quadrant: 163 SY

    - SE Quadrant: 182 SY

      - Total Seeding and Mulching: 196+200+163+182 = 741 SY

- Item 659 Repair Seeding and Mulching

  - 5% of seeding and mulching: 0.05 x 528 = 37 SY

- Item 659 Commercial Fertilizer

  - (1 TON/7410SY permanent) x 528 + (1 TON/11,111 SY interseeding) x 26 = 0.10 TON

- Item 659 Water

  - (2 x 0.0027 M. GAL/SY) x 528 + (0.0027 M. GAL/SY) x 26 = 4 M. GAL

Structures: DAR-47-0316

- Item 202 Portions of Structure Removed, Over 20 foot Span, APP

  - Lump Sum (LS)

- Item 202 Wearing course removed

  - Existing asphalt on bridge deck: 32’ x 30’ + 15’ x  = 960 SF/9 = 107 SY

- Item 202 Bridge Railing Removed

  - 37.5 FT each side (Existing Plans) = 75 FT

- Item 503 Unclassified Excavation

  - Lump Sum (LS)

- Item 509 Concrete Reinforcement, replacement of existing concrete reinforcement, as per plan

  - 50 lbs for contingency.

- Item 509 Uncoated Steel Reinforcement

  - Deck Edge rebar: See sheet 16. 1688 lbs

- Item 511 Class QC2 Concrete, Superstructure, Reconstruction

  - 1 Deck edge: 5.5 sf From Microstation, 5.5’ x 30’ x 2 deck edges = 330 CF/27 = 13 CY

- Item 516 2” Deep Joint Sealer, APP

  - At interface between concrete and asphalt: 32’ wide x 1/cos(18.50) = 33.74 FT x 2 approaches = 68 FT

- Item 517 Railing (Three Steel Tube Bridge Railing)

  - See sheet 13: 40 FT each side x 2 sides = 80 FT

- Item 518 Steel Drip Strip

  - [30 FT per side + (4 posts x 1.5 FT)] x 2 sides = 72 FT

- Item 519 Patching concrete structure

  - 16 SF at 4SF per wingwall top

- Item 625 structure grounding system

  - 1 EACH

- Item 844 Galvanic Anode Protection, as per plan

  - 39 anodes per deck edge. 78 EACH

- Item 848 Superplasticized dense concrete overlay using hydrodemolition, 2 ¾”

  - 32’ x 30’ = 960 SF/9 = 107 SY

- Item 848 Surface preparation using hydrodemolition, as per plan

  - 32’ x 30’ – (2 x 3’ x 30’) = 780 SF/9 = 87 SY

- Item 848 Superplasticized Dense Concrete overlay (variable thickness), Material only

  - Asphalt on top of existing deck, have to assume a percentage delaminated. 20%

    - 20% per BDM 403-3 table use 40%. 780 SF x 2/12” x 40% = 52 CF/27 = 2 CY

- Item 848 Hand chipping

  - 10% of estimated variable thickness area per BDM 403.4.1. 0.4 x 780 SF x 0.1 = 31.20 SF/9 = 4 SY

- Item 848 Test slab

  - Lump Sum (LS)

Structures: DAR-127-3053

- Item 202 Portions of Structure Removed, Over 20 foot Span, APP

  - Lump Sum (LS)

- Item 202 Approach slab removed

  - 24’ x 15’ x 2 approach slab = 720 SF/9 = 80 SY

- Item 202 Wearing course removed

  - Existing asphalt on bridge deck: 44’ x 22.10’ = 972.4 SF/9 = 108 SY

  - Existing asphalt on approach slabs: 2 x 24’ x 15’ = 720 SF/9 = 80 SY

    - Total = 188 SY

- Item 202 Bridge Railing Removed

  - 25 FT each side (Existing Plans) = 50 FT

- Item 202 Cofferdams and Excavation Bracing

  - Lump Sum (LS)

- Item 503 Unclassified Excavation

  - Lump Sum (LS)

- Item 509 Epoxy coated Steel Reinforcement

  - Abutment and approach slab: See sheet 24. 284 + 7856 = 8140 lbs

- Item 509 Galvanized Steel Reinforcement

  - Bridge Slab: See sheet 24. 14943 lbs

- Item 510 Dowel Holes with Nonshrink, Nonmetallic Grout

  - See sheet 24. 48 abutments, 94 bridge slab = 142 Total

- Item 511 Class QC2 Concrete, Superstructure

  - Abutment tops: 3.58 SQ FT from section view, 47 feet is the length of each abutment top. 3.58 SF x 47 x 2 abutments = 336.5 CF

  - Bridge slab: 63.65 SF from transverse section view, 22.10’ bridge limit. 63.65 SF x 22.10’ = 1407 CF

    - Total = 336.5 CF + 1407 CF = 1743.50 CF/ 27 = 65 CY

- Item 511 Class QC1 concrete, retaining/wingwall not including footing

  - Wingwalls: 13.25 SF from wingwall section x 1.5’ x 4 wingwalls = 79.5 CF/27 = 3 CY

- Item 516 1” Preformed Expansion Joint Filler

  - 47’-2” x 8/12” x 2 abutment seats = 62.89 SF

  - 1’-6” x 2.81’ x 4 wingwall/superstructure = 16.86 SF

  - 6” x 12” x 4 approach slab and superstructure interface = 2 SF

    - Total = 63 + 17 + 2 = 82 SF

- Item 516 1/2” Preformed Expansion Joint Filler

  - 6/12” x 47’-2” x 2 abutment backs = 47.17 SF, 48 SF

- Item 516 2” Deep Joint Sealer, APP

  - At interface between concrete and asphalt: 44’ wide x 1/cos(18) = 46.26 FT x 2 approaches = 92.5 FT

- Item 516 Integral Abutment Expansion Joint Seal

  - [1’+ 4.5/12 + 46.30’ + 4.5/12 + 1’] x 2 abutments = 98.1 FT

- Item 517 Railing (Three Steel Tube Bridge Railing)

  - 30 FT each side x 2 sides = 60 FT

- Item 518 Porous Backfill with Geotextile Fabric

  - 150 SF section area (1 foot below cutline) x 2’ deep x 2 abutments = 600 CF/27 = 23 CY

- Item 518 Steel Drip Strip

  - [20 FT per side + (3 posts x 1.5 FT)] x 2 sides = 50 FT

- Item 526 Reinforced Concrete Approach Slabs (T=12”), APP

  - 44 FT wide x 15 FT long x 2 = 1320 SF/9 = 147 SY

Maintenance of Traffic

- Item 614 Detour signing

  - Lump Sum (LS)

Incidentals

- Item 614-Maintaining Traffic (LS)

- Item 619-Field Office, Type B 5 (MNTH)

- Item 623-Construction Layout Stakes and Surveying (LS)

- Item 624-Mobilization (LS)

END OF CALCULATIONS

Misc.

Wingwall length. BDM 306.2.4 Wingwalls. Max slope behind the wingwall is 1 vertical to 2 horizontal. Wingwall elevations are approx. 909.5 feet. End of wingwalls existing ground is 906 feet. 909.5-906 = 3.5 feet vertical. Wingwalls need to be at least 7 feet in length.