G105 NOTE

PRIMARY PROJECT CONTROL MONUMENTS GOVERN ALL POSITIONING ON ODOT PROJECTS.

SEE SHEET _______ OF THE PLANS FOR A TABLE CONTAINING PROJECT CONTROL INFORMATION.

USE THE FOLLOWING PROJECT CONTROL, VERTICAL POSITIONING, AND HORIZONTAL POSITIONING PARAMETERS FOR ALL SURVEYING:

PROJECT CONTROL

POSITIONING METHOD:			ODOT VRS

MONUMENT TYPE:			TYPE “B”

VERTICAL POSITIONING

ORTHOMETRIC HEIGHT DATUM:	NAVD88

GEOID:					GEOID12A

HORIZONTAL POSITIONING

REFERENCE FRAME:			NAD83(2011) (EPOCH 2010.0)

ELLIPSOID:				GRS80

MAP PROJECTION:			LAMBERT CONFORMAL CONIC 2 STANDARD PARALLEL

COORDINATE SYSTEM:			OHIO STATE PLANE, SOUTH ZONE

PROJECT ADJUSTMENT FACTOR:	1.00004204

ORIGIN OF COORDINATE SYSTEM:	0,0

USE THE POSITIONING METHODS AND MONUMENT TYPE USED IN THE ORIGINAL SURVEY TO RESTORE ALL MONUMENTS RELATED TO PRIMARY PROJECT CONTROL THAT ARE DAMAGED OR DESTROYED BY CONSTRUCTION ACTIVITIES. RESTORE THE DAMAGED OR DESTROYED MONUMENTS IN ACCORDANCE WITH CMS 623.