June 1, 2022

Mr. Richard Ortman, PE

Project Manager

Ohio Department of Transportation, District 6

400 E William St.

Delaware, Ohio 43015

Re:		Airway/Highway Clearance Analysis

FRA-670-0.03

PID 111641

Dear Mr. Ortman:

We have reviewed the above-mentioned project and have determined that there are no public airports within 20,000’ of the project site. However, there are several heliports nearby, the closest being the Columbus Police Aviation Section heliport at 2130 West Broad Street. Since this heliport is 1,390’ from the project site, and considering a 25:1 notification surface, we have determined that any construction equipment on the project site that exceeds 61’ will require FAA Notification. As a result of this we have included note G118B, limiting contractor equipment to a height of 61’ or less, which should be in excess of the height of any necessary equipment or materials.

Respectfully,

MICHAEL BAKER INTERNATIONAL, INC.

Kevin Dickens, P.E.

Project Manager

6’

55.6’

Heliport

Elev. 773

25:1

Distance = 1390’

I-670 ELEV. 767