Hydraulic Analysis Report

Project Data

Project Title: GRE-68-13.56

Designer: AZF

Project Date: Wednesday, August 25, 2021

Project Units: U.S. Customary Units

Notes:

Curb and Gutter Analysis: Curb and Gutter Analysis Notes: Spread at north side of roundabout

Gutter Input Parameters

Longitudinal Slope of Road: 0.0050 ft/ft

Cross-Slope of Pavement: 0.0200 ft/ft

Depressed Gutter Geometry

Cross-Slope of Gutter: 0.0833 ft/ft

Manning's n: 0.0150 
Gutter Width: 2.0000 ft 
Design Flow: 0.8100 cfs

Gutter Result Parameters

Width of Spread: 5.7155 ft

Gutter Depression: 1.5192 in

Area of Flow: 0.4533 ft^2

Eo (Gutter Flow to Total Flow): 0.8415

Gutter Depth at Curb: 2.8909 in

Inlet Input Parameters

Inlet Location: Inlet on Grade 
Inlet Type: Curb Opening 
Length of Inlet: 0.0010 ft 
Local Depression: 0.0000 in

Inlet Result Parameters

Intercepted Flow: 0.0002 cfs 
Bypass Flow: 0.8098 cfs 
Efficiency: 0.0003