GRE-68-13.56 Drainage Design

The drainage within the GRE-68-13.56 project limits will be conveyed primarily by ditches and sheet flows off the roadway. All roadway drainage ultimately outlets into the Little Miami River. Some drainage areas outlet into an existing channel that flows into the Little Miami River approximately 1,150 feet west of the project site. Since the roadways have a high traffic volume (over 6,000 ADT) and are high speed (design speed greater than 45 mph), the pavement spread was calculated using a 5-year storm. The ditches were designed using a 10-year storm for depth and a 5-year storm for ditch protection. The drainage areas for the curbed part of the roadway are small and made up primarily of pavement, therefore, a 10-minute time of concentration was used for all spread calculations. All ditch design drainage areas have a 15-minute time of concentration.

This project has five distinct watersheds. The first watershed is along the north right leg of US 68. The runoff starts at the end of the project south to a curb cut at the low point which discharges into a proposed ditch. The second watershed is along the north left leg of US 68 and north right leg of SR 235. The runoff starts at the end of the project on US 68 and is contained in the gutter line running south along US 68, through the nose near the roundabout, and north on SR 235. The runoff then flows into an existing channel east of the north right leg of SR 235. The third watershed is along the north left leg of SR 235. This runoff sheet flows of the roadway and into an existing channel. The fourth watershed is along the south right leg of US 68. This runoff sheet flows off the road and into the existing channel nearby. The fifth watershed is along the south right leg of US 68. This runoff sheet flows off the road and into the existing channel.

The Project EDA for this project is 3.07 acres, therefore, BMPs are needed. The calculated treatment percentage is 22.77% for a total treatment requirement of 0.70 acres. There are two vegetated filter strip locations, one is on the left side of the north leg of US 68 and the other is on the right side of the south leg of US 68. These vegetated filter strips combine for a total treatment area of 0.38 acres. There is also one vegetated biofilter for this project. It is on the right side of the north leg of US 68 and has a total treatment area of 0.33 acres. The total treatment area provided for the project is 0.71 acres which is more than the required 0.70 acres. Refer to the project site plan in the plan set and BMP calculations sheet in this drainage calculations book for BMP locations and calculations.