Flat Sign Instructions

Files needed:

Sign Post Excel (115713)

Subsummary Excel (115713)

GIS Flat Sign Excel for County

- Rename Sheets and place in Eng Folder in Projectwise.

- Grab Sign Post excel from 115713

Rename sheets based on Routes.  These are in pairs.  (Yellow is GIS sign data and

Blue is set with calcs for Post heights)

Open sign workbook (colored yellow) and replace all data from each Road workbook.

Larger roads with Ramps have a second yellow sheet for those.

Recommend adding another Column at A for ramps numbered 1-xxx to make

Subsummary easier to program.

- Use Pathweb to run each road and updated GIS data.  Do not delete number column (A).  These are for references and lookup tables.

The numbers are coded for road type per contractor:

Green – Single Lane (Only 1 through lane for each side)

Yellow – Multi Lane (with at least 1 direction having multiple through lanes)

Red – Expressway (multi Lane with Separation and ramps at same elevation)

Blue – Freeway (Multi Lane with Separation and ramps at differing elevations)

- Add/Delete signs missing from GIS data base.  Add/Delete reference numbers (Column A of Yellow sheet as needed).

Adjust sign dimensions based on Coloring and number of signs and posts.

On groups with One Way Signs (R6-1), rearrange so one ways are on top.

On groups that are Front/Back on Post, rearrange to place taller sign first.

- Open Route workbook (Blue) “RR-###” for Road.  Copy a single sign set from Columns B-AB and paster to Ref 1 (should be Row 3).  (Recommend one from SR-700s or 800s.)

Check Columns B,C,D,U,V,AA and AB to ensure in ‘  ‘ is the name of the correct road.

Highlight and drag for all reference numbers.

Checks need to be done.  B3-D3 should read Yellow reference #1 data.

E3 should read ft² =(width*height)/144

F3 should read centroid height.  This changes with the combinations of signs but general is .5 height + x’ (depending on road function).  This is built already.

- There are two sets of double signs (back-to-back like Chevrons with only account for 1 height or a stack which accounts for the signs stacked with the required 1” spacing between.

- There are also various combinations for 3-7 signs stacked in various configurations.  Please pay attention to these.  Dual Posts account for the height of the heights stack.

G3-L3 are set to read F3 and run off the chart starting at AD for the various sign configurations.  These only show 2.1 (Type 2 single post) through 4.2 (Type 4 double post).  Chart has type 6s added, but not addressed in these.

M3-R3 Are set to read True/False based on the total sign ft² on the post.  This is important to consider for stacks of signs.

S3 will read the max true from above.  In sets this will read the min type and number for the sign.

T3 is the chosen option.  It is set to default to equal “best option” but can be adjusted.  This is what the post height is based on.

U3 and V3 read function class and number of signs from Yellow workbook

W3 reads the chosen option as a function for the post height.  It is 42” + 5/6/7(based on function Class).  Ceiling rounds up and in some cases round up an extra foot.

Recommend changing ‘Ceiling’ to ‘RoundUp’

There are bold lines separation groups throughout the posts (blue) worksheet.

Chevrons are double sided on 1 post.

Doubles stacked are already written to add 1” between sign.

X3 is a function reading what is going into the ground as a function of Chosen Option

Y-AA are checks

AB is a reference for sign type

DO NOT MERGE ANY OF THESE.

Go through based on number of signs and update with correct formulas.

Copy F through T for each corresponding type and paste into row.

Then copy a V-AB for each type and paste into row.

Recommend starting with a Front back Set, then a double stack and

increase from there for the entire road.

IF clarification is needed talk to Tony Collins.

- Run through road on all checks and see why they are not True or Good.

- Complete these steps for each Road

- Open Subsummary sheets.

Rename in order of Roads for County

Place first page number in AG7

Page numbers for sheets should autofill.  If you need to add sheets Copy from page number cell to Total cell and paste below blank space.  Select header and add 1 to the number after the plus at the end

Highlight all cells between reference number 1 and comment for line 60 and unmerge.

Do this for each sheet

YOU CANNOT COPY FORMULAS INTO MERGED CELLS.

Sign Reference number starts at 1 and reads +1 for each sheet below.  At page changes

they read last item +1.  Adjust as needed.

County and Route are fill ins.  Place in and copy to fill sheets.

Nearly ALL of this data is reading as a lookup table

Road (Yellow) is columns I-L, AA and AB

Road Post (Blue) is columns G, M-T,X

U is set to read the sign code to match the required reflectors signs and set to

multiple by number of posts.

If the Sign Text does not fit in the box, add a row below it and delete the bottom row from the sheet.  This will allow you to not adjust formulas.  To fit on a sheet only 60 items can be on a sheet.

Copy a row and paste it to the next sheet. Items will need to be adjusted.

- Sign Text – adjust the number with F to the sign reference number +1

=IF('[115713_SIGN POSTS.xlsb]US-22'!$F56="","",VLOOKUP($D103,'[115713_SIGN

POSTS.xlsb]US-22'!$A$2:$M$252,6,TRUE))

- The items up to reflectors -Adjust the number with T and W to sign reference number +2

=IF(TRUNC('[115713_SIGN POSTS.xlsb]US-22 posts'!$T57)=2,'[115713_SIGN POSTS.xlsb]US-22 posts'!$W57,"")

You will need to do this for all Sheets.

From Column O-T, AA and AB, if done correctly all sets will have numbers in the top cell and 0 for the rest (See Yellow highlights below).  The formulas are set to be blank for non-values.  Merge together cells in Subsummaries (See Green highlights below).

Should end up like this

- Set totals on subsummaries to be carried to to equal each sheet.

- Copy single item and start next road.

You will have to go through each formula and change the route name to match the appropriate road.

Here you would change US-22 to proper type and number.

SR- State Route

US- US Route

IR- Interstate Route

Once this is done, highlight cells and drag for sheet.  Repeat steps 8-10 for each road.