April 16, 2024

Project/File: HAM-COLUMBIA CONNECTOR

Stephanie Otten

505 South S.R. 741

Lebanon, Ohio 45036

Reference: Airway/Highway Clearance Analysis

Project/File: HAM-COLUMBIA CONNECTOR

Dear Stephanie,

Reference:	Airway/Highway Clearance Analysis

We have reviewed the referenced project and have determined that the project and the temporary construction equipment will not encroach into the (100:1) notification surface of the Lunken Airport (LUK-CINCINNATI). This determination is based on an airport elevation of 483.3 feet at the northwest corner of runway 25, a project elevation of 499.61 feet, a traverse way adjustment height of 7 feet for bicyclists (based on Multimodal Design Guide Section 3.6.1), a temporary construction equipment height of 25 feet for construction trucks on the shared-use path (based on Location & Design Manual Volume 3 Figure 1407-3), with a distance of 18,375 feet between the airport and the project location.

The attached diagram and calculation will provide a detailed analysis of the study. The study has reflected that a height of 160.4 feet exists between the (100:1) notification surface and the adjusted elevation of the project location, and a height of 142.4 feet exists between the (100:1) notification surface and the elevation of the temporary construction equipment. Therefore, based on these spare heights, only plan note G118B will be required.

Regards,

Steve

Steven N. Shadix PE, PS

Senior Principal

Direct: 513 619-6470
Mobile: 513 218-4895
Steve.Shadix@stantec.com

Stantec
10200 Alliance Road, Suite 300
Cincinnati OH 45242