2022-03-25

Meeting with Ben Helmes of Great Parks to discuss property lines

It is our belief that the property line has always followed the northerly line of the former PCCC & ST L Railroad (66.00 in width).  Although we do not have a copy of the Railroad Valuation map, we are fairly certain there was not a jog or angle point in this line, within the limits of our project. The challenge was to re-establish the location of the former Railroad since little physical evidence of the railroad remains.

We examined various conflicting records, including old tax maps, road right-of-way plans, and existing surveys. This meeting was setup to discuss our findings, and the property lines that will be shown and labeled on the accompanying exhibit, based on the following circumstantial evidence.

- Re-established the existing southerly right-of-way line of Wooster Pike based on the HAM-50-31.95 RW Plan done in 2004.

- Examined 1947 and 1970 Tax Map (road scales 60 feet). It is believed that the southerly tangent right-of-way line remained unchanged and future widening occurred on the north side of Wooster Pike.

- HAM-50-(3.61) – (32.70) Part I (date unknown) southerly RW line scales 40.00 shown with curve that matches current RW plan. Additional RW was acquired at the southwest corner of US 50 & Newtown Road – stations & offset from curved centerline

- Found existing monuments shown on Nordloh and Leesman surveys of Perimeter Technoligies property (42) which was the first time that “a possible gap” was identified.

- Used the Newtown Road -CR 374 (1983) for what we deemed to be the most reliable record location of the northerly line of the former Railroad.  Depending on what is held to orient this plan, yields the two different lines shown on the current Hamilton County Auditor’s Map.

- It is believed that the Nordloh survey oriented the Newtown Road RW Plan with the current HAM-50-31.95 Plan (2004), matching the angle point in the westerly right-of-way line being 65.08 feet right of US 50 station 1716+89.19.

- We oriented the Newtown Road RW Plan with the easterly extension of the “southerly RW line, being 40.00 feet southerly of the tangent centerline extended” shown at station 15+61.76.  This re-establishes the former northerly RR line at 20 feet northerly of, and parallel to, the existing concrete wall (matches scaled distance). Also, the bridge abutment scales good, and the south line of parcel 13-WD lines up with the north edge of the Little Miami River. This line agrees, more or less, with the location shown on the following documents.

- 1947 Tax Map – Lot Line 12-13 scaled 146’ (calc 144.4’)

- 1947 Tax Map – p/l 10’ east of Lot Line 9-10 total dist. = 209’ (calc 208.8’)

- Hamilton County Park District deed (O.R. 6995 Pg 547 – highlighted blue courses shown on 1970 Tax Map) generally agrees with monuments found to the west on the former northerly RR line at the SE corner of parcel 29

- Agrees with PB 465-81_ by Leesman_2018 (147.78’ plat vs 147.29’ calculated) and shown in yellow on the 1970 Tax Map

- Hamilton County Sewer Easement PB 331-46  (west side of easement 1.1’ north)

- Very close to the existing chain link fence