Meeting with Ben Helmes of Great Parks to discuss property lines (2022-03-25)

It is our belief that the property line has always followed the northerly line of the former PCC & ST L Railroad (66.00 in width), and there is not a jog or angle point in this line within the limits of our project. With little physical evidence of the actual railroad that remains, we examined various conflicting records, including old tax maps, road right-of-way plans, and existing surveys.  Stantec’s findings are based on the following, and delineated upon the attached exhibit;

(A), (B), (C) – all deeds describe the line as being along the northerly line of the former Railroad

(D) – We re-established the centerline and southerly right-of-way line of US 50 based on the HAM-50-31.95 RW Plan and monument boxes found.

(E) – The Newtown Plan was oriented to the HAM-50-31.95 Plan at holding station 15+83.41 at the intersection of the easterly extension of the southerly tangent RW line of US 50.  Plan information was held to determine the location of the original railroad RW, which matches the scaled distance to the concrete retaining wall of the path.

(F) – Location of the southerly line of the 0.8693 acre tract as Surveyed by Nordloh & Associates (2007). This line is described as “the northerly line of the Hamilton County Park District tract, formerly the P.C.C. & St. L. Railroad” in the legal description by Nordloh (O.R. 10549, Pg. 947).

(G) – Consolidation Plat prepared by Leesman dated 2/20/16, course information matches said Nordloh survey.  A note on the plat states there may be a gap between RR right-of-way. The legal description by Leesman (O.R. 13113 Pg. 1691) describes the line the same as Nordloh.

(H) – Consolidation Plat prepared by Leesman dated 4/19/2018.  The southerly line of the 2.949 acre tract is described as being “the north line of the Hamilton County Park District” (O.R. 11489 Pg. 1597 ).  The distance along the east line of the 2.949 acre tract is shown as 147.78 feet, our calculated distance is 148.29 feet. The distance along the westerly line of said 0.8692 acre tract (G) is 139.98 feet.

(I) – Course information shown on the Hamilton County Sewer Easement Plat generally agrees with the line established from item (E).

(J) –1947 Tax Map comparisons.  Lot Line 12/13 scaled distance of 146’ (calc. 144.4’). The old property line shown 10 feet east of Lot Line 9/10 total distance = 209’ (calc. 208.8’).