| To: | File | From: | James Mason Cincinnati |
| --- | --- | --- | --- |
| Project/File: | 173620147 Spring Hill Hydraulics | Date: | August 25, 2022 |

Reference:	Spring Hill No-Rise Analysis

Reference: file

Hydraulic Analysis

Stantec obtained the effective FEMA hydraulic model of the Little Miami River. The 1-dimensional, unsteady state hydraulic model was developed in the Hydraulic Engineering Center – River Analysis System (HEC-RAS) format. The proposed site is located between HEC-RAS model river stations 39180 and 41238.

Pre-Project Model

A pre-project conditions geometry was developed in HEC-RAS to update the effective model geometry and reflect current conditions of the site. The datum of the HEC-RAS model was shifted from NGVD 29 to NAVD 88. New model cross sections were added between the effective cross sections 39180 and 41238 to capture detailed topography of the project site. Figure 1 shows the placement of the new model cross sections.

River channel geometry for the new cross sections was interpolated using the geometry editor tools in HEC-RAS based on the bounding effective model cross sections. Overbank geometry was updated using an existing surface digital elevation model (DEM) of the project area.

Within the project area, overbank Manning’s ‘n’ values were horizontally varied based on land use observed in aerial imagery of the site taken in 2021. A table of Manning’s ‘n’ values and corresponding land use values is shown in Table 1 below.

Table 1 Manning’s N values

| Manning’s N vs Land Cover |  |
| --- | --- |
| Trees / Forest | 0.12 |
| Developed, Open Space | 0.04 |
| Developed, Medium Intensity | 0.12 |
| Developed, High Intensity | 0.16 |
| Asphalt / Concrete Pavement | 0.016 |

Figure 1. Hydraulic Model Cross Sections

Post Project Model

Post-project conditions geometry was developed in HEC-RAS to reflect proposed Little Miami Scenic Trail Connection (LMST) location during flood conditions. The purpose of the model is to determine impacts to the Base Flood water surface elevations due to fill placement in the floodplain during the occurrence of the 100-year discharge on the Little Miami River.

Channel geometry is identical to the pre-project model. Overbank geometry in the project area was updated to reflect proposed grading of the LMST. Manning’s ‘n’ values were updated to reflect the changes to the land-use in proposed conditions. The trail was set to a consistent ‘n’ value = 0.016. Interpolated model cross sections were revised to reflect the addition of the LMST and are shown in Figure 2 below. A comparison of pre- and post-project HEC-RAS model cross section geometry is shown in Figure 3.

Figure 2. Proposed Location of LMST

Figure 3. Pre-and Post HEC-RAS Cross Section Geometry Comparison

Model Results

A comparison of pre-project and post-project hydraulic model results within the project area is presented in Table 2 below. Model results indicate no rise in post-project water surface elevations within the project area or immediately upstream and downstream of the project site.

Table 2. Pre- and Post-Project Base Flood Water Surface Elevation (WSE) Comparison

| Hydraulic Model Results |  |  |  |  |
| --- | --- | --- | --- | --- |
| River Station | Pre-Project 100-yr WSE (Maximum, Feet NAVD88) | Post-Project 100-yr WSE (Maximum, Feet NAVD88) | Change in Base Flood Elev. (Feet) | Notes |
| 42968 |  |  |  | Newtown Pedestrian bridge |
| 42929 | 495.38 | 495.38 | 0.00 | Upstream of Project Site |
| 42449 | 495.16 | 495.16 | 0.00 | Upstream of Project Site |
| 41238 | 494.38 | 494.38 | 0.00 | Upstream Limit of Project |
| 41072 | 494.30 | 494.30 | 0.00 | Project Area |
| 40641 | 494.05 | 494.05 | 0.00 | Project Area |
| 40005 | 493.61 | 493.61 | 0.00 | Project Area |
| 39699 | 493.42 | 493.42 | 0.00 | Project Area |
| 39180 | 493.13 | 493.13 | 0.00 | Downstream Limit of project |
| 38268 | 492.71 | 492.71 | 0.00 | Downstream of Project Site |
| 37178 | 492.44 | 492.44 | 0.00 | Downstream of Project Site |
| 36422 | 492.22 | 492.22 | 0.00 | Downstream of Project Site |
| 36197 | 492.14 | 492.14 | 0.00 | Downstream of Project Site |
| 36153 |  |  |  | Mariemont Railroad |

Regards,

Stantec Consulting Services Inc.

James Mason

Civil Engineer in Training

Phone: (513) 619-6461

Mobile:

james.mason@stantec.com

stantec.com

Attachment:	[Attachment]