PROJECT DESCRIPTION

THIS PROJECT INVOLVES THE CONSTRUCTION OF A TANGENT DRILLED SHAFT WALL APPROXIMATELY 350 TO 375 FEET LONG TO PROTECT IR 90 EASTBOUND FROM EROSION AT AN OUTBOARD BEND IN THE CHAGRIN RIVER IN WAITE HILL, LAKE COUNTY, OHIO.

HISTORIC RECORDS

HISTORIC BORINGS PERFORMED IN 1957 AS PART OF THE LAK-1-4.02 PROJECT ALONG THE IR 90 ALIGNMENT PRIOR TO THE CONSTRUCTION OF IR 90. ONE (1) BORING WAS PERFORMED NEAR STA. 249+00, OR NEARLY ADJACENT TO THE PROJECT SITE. THE BORING ENCOUNTERED SANDY SILT (A-4a) UNDERLAIN BY GRAVEL (A-1-a) AND GRAVEL WITH SAND AND SILT (A-2-4) BEFORE ENCOUNTERING REFUSAL NEAR EL. 603. THE THEN CURRENT GROUND SURFACE WAS NEAR EL. 612.

HISTORIC PLANS FOR THE LAK-1-4.02 PROJECT INDICATE THAT AT THIS PROJECT LOCATION, THE EXISTING GROUND SURFACE WAS NEAR EL. 612 TO EL. 613. PROPOSED GRADE RANGED FROM APPROXIMATELY EL. 630 TO EL. 626, INDICATING APPROXIMATELY 14 TO 18 FEET OF NEW FILL WAS PLACED TO ACHIEVE THE ROADWAY GRADE OF IR 90. SURVEYED ELEVATIONS OF OUR BORINGS PERFORMED ALONG THE SHOULDER OF IR 90 EASTBOUND RANGED FROM EL. 625.6 TO 627.

GEOLOGY

THE SITE LIES WITHIN THE ERIE LAKE PLAIN PHYSIOGRAPHIC REGION. THIS REGION TYPICALLY CONSISTS OF PLEISTOCENE-AGE LACUSTRINE SAND, SILT, CLAY, AND WAVE-PLANED GLACIAL TILL OVER DEVONIAN-AND MISSISSIPPIAN-AGE SHALES AND SANDSTONES. AVAILABLE GEOTECHNICAL INFORMATION WITHIN THE VICINITY OF THE PROJECT SUGGESTS THAT NATURAL SOILS ARE PREDOMINANTLY COMPOSED OF HIGH SILT CONTENT SOILS WITH OCCASIONAL LAYERS OF MORE CLAYEY OR SANDY SOILS. THE SOILS ENCOUNTERED WITHIN OUR BORINGS GENERALLY AGREED WITH THE GEOLOGIC MAPPING. TOPOGRAPHIC MAPPING INDICATES THE GROUND SURFACE ALONG IR 90 EASTBOUND WITHIN THE PROJECT LIMITS RANGES FROM APPROXIMATELY EL. 625 TO EL. 627. BEDROCK TOPOGRAPHY MAPPING SUGGESTS THAT BEDROCK MAY BE ENCOUNTERED BETWEEN APPROXIMATELY EL. 595 TO EL. 605. BEDROCK WAS ENCOUNTERED AT THE PROPOSED DEPTH IN ALL BORINGS DURING THE FIELD INVESTIGATION.

THE SITE IS LOCATED WITHIN AN ANCIENT OXBOW OF THE CHAGRIN RIVER WHICH BECAME BLOCKED, AND THE RIVER WAS REROUTED DURING THE CONSTRUCTION OF IR 90. SEASONAL HIGH FLOWS WITHIN THE CHAGRIN RIVER COMBINED WITH THE IR 90 EMBANKMENT ON AN OUTSIDE BEND IN THE RIVER SUBJECT THE ROADWAY EMBANKMENT TO SATURATION AND EROSIVE FORCES.

RECONNAISSANCE

S&ME VISITED THE SITE ON MULTIPLE OCCASIONS ON JULY 20, 2021 AND BETWEEN APRIL 20, 2022, TO APRIL 28, 2022, TO OBSERVE THE SITE CONDITIONS, MARK BORING LOCATIONS AND PERFORM THE FIELD INVESTIGATION. IT WAS OBSERVED THAT MUCH OF THE HILLSIDE/EMBANKMENT IS COVERED WITH VEGETATION WITH SIGNS OF EROSION BY THE BEND DUE TO THE ACTION OF THE CHAGRIN RIVER.

SUBSURFACE EXPLORATION

S&ME SELECTED AND MARKED THE BORING AND CORE LOCATIONS IN THE FIELD AT THE APPROXIMATE LOCATIONS APPROVED BY ODOT DISTRICT 12. A HANDHELD GPS WITH HORIZONTAL SUB-METER ACCURACY WAS USED TO RECORD THESE LOCATIONS. THE COORDINATES OBTAINED BY S&ME AND THE GROUND SURFACE ELEVATIONS WERE PROVIDED BY MS CONSULTANTS.

THE BORINGS AND OTHER EXPLORATIONS WERE COMPLETED IN TWO PHASES. THE FIRST PHASE OF EXPLORATION INCLUDED TWO (2) BORINGS, FIVE (5) WILDCAT DYNAMIC CONE PENETROMETER (DCP) SOUNDINGS AND THREE (3) HAND SAMPLING BORINGS. THESE EXPLORATIONS WERE PERFORMED BETWEEN AUGUST 5 AND 6, 2021. THE TWO BORINGS, B-001-0-21 AND B-002-0-21, WERE PERFORMED IN THE OUTSIDE SHOULDER OF EASTBOUND IR 90 USING AN ATV-MOUNTED DRILLING RIG USING A 2¼-INCH I.D. HOLLOW-STEM AUGER. DISTURBED (BUT REPRESENTATIVE) SOIL SAMPLES WERE OBTAINED BY LOWERING A 2-INCH O.D. SPLIT-BARREL SAMPLER TO THE BOTTOM OF THE BORING AND THEN DRIVING THE SAMPLER INTO THE SOIL WITH BLOWS FROM A 140-POUND HAMMER FREELY FALLING 30 INCHES (AASHTO T206 - STANDARD PENETRATION TEST, SPT). IN ACCORDANCE WITH THE ODOT SPECIFICATIONS FOR GEOTECHNICAL EXPLORATIONS (SGE), THE HAMMER SYSTEM ON THE DRILL RIG HAD BEEN CALIBRATED ON NOVEMBER 25, 2020, IN ACCORDANCE WITH ASTM D4633 TO DETERMINE THE DRILL ROD ENERGY RATIO (91.0%). THE ENERGY RATIO, AS USED TO DETERMINE CORRECTED BLOW COUNTS, HAS BEEN LIMITED TO 90% AS REQUIRED BY THE SGE.

THE SECOND PHASE OF EXPLORATION INCLUDED THREE (3) BORINGS, TERMED B-001-6-22, B-001-7-22 AND B-002-4-22, WERE PERFORMED BETWEEN APRIL 26 AND 28, 2022. THE SECOND PHASE BORINGS WERE ADVANCED IN THE OUTSIDE SHOULDER OF EASTBOUND IR 90 BY A TRUCK-MOUNTED DRILL RIG USING A 3¼-INCH HOLLOW-STEM AUGER. DISTURBED, BUT REPRESENTATIVE, SOIL SAMPLES WERE ATTEMPTED (AASHTO T206 – STANDARD PENETRATION TEST, SPT). IN ACCORDANCE WITH ODOT SPECIFICATIONS, THE HAMMER SYSTEM ON THE DRILLING RIG WAS CALIBRATED (ASTM D4633) ON MARCH 22, 2022, TO DETERMINE THE DRILL ROD ENERGY RATIO (87.0%).

EXPLORATION FINDINGS

ASPHALT PAVEMENT WAS ENCOUNTERED AND MEASURED TO BE 8 TO 13 INCHES THICK IN THE ROADWAY SHOULDER BORINGS UNDERLAIN BY 8 INCHES OF AGGREGATE BASE IN BORING B-001-0-21 AND 2 INCHES OF AGGREGATE BASE IN BORING B-002-0-21. NO AGGREGATE BASE WAS NOTED IN THE 2022 BORINGS.

BELOW THE PAVEMENT SECTION, EXISTING FILL WAS VISUALLY IDENTIFIED IN ALL BORINGS TO DEPTHS RANGING FROM 14.5 TO 18 FEET BELOW THE EXISTING GRADE. THE FILL WAS COMPOSED OF DISCONTINUOUS ZONES OF STIFF TO HARD SANDY SILT (A-4a), VERY-STIFF SILT (A-4b), STIFF TO HARD SILT AND CLAY (A-6a), MEDIUM-DENSE TO DENSE GRAVEL WITH SAND AND SILT (A-2-4), AND MEDIUM-DENSE GRAVEL WITH SAND, SILT AND CLAY (A-2-6).

NATURAL SOIL WAS ENCOUNTERED TO DEPTHS RANGING FROM 23.5 TO 28.5 FEET WHERE THE SHALE BEDROCK WAS ENCOUNTERED. THE NATURAL SOILS CONSISTED OF STIFF TO HARD SANDY SILT (A-4a), STIFF TO VERY-STIFF SILT AND CLAY (A-6a), AND LOOSE TO DENSE GRAVEL WITH SAND AND SILT (A-2-4).

THE RECOVERED BEDROCK WAS PREDOMINANTLY COMPOSED OF SLIGHTLY TO HIGHLY WEATHERED, VERY WEAK TO WEAK, DARK-GRAY SHALE AND WAS ENCOUNTERED IN THE BORINGS BETWEEN EL. 597.7 AND EL. 603.2. AN OBSTRUCTION IN THE CORE BARREL DURING THE INITIAL CORING IN BORING B-001-7-22 RESULTED IN A SIGNIFICANT CORE LOSS FROM 31 TO 37 FEET.

GROUNDWATER WAS ENCOUNTERED IN THREE BORINGS (B-001-0-21, B-001-6-22 AND B-001-7-22) BETWEEN THE DEPTHS OF 23.5 TO 26.5 FEET. THE DEPTHS WHERE WATER WAS ENCOUNTERED IN THESE BORINGS IS GENERALLY AT OR JUST ABOVE THE SURFACE OF THE BEDROCK AND IS ROUGHLY THE SAME ELEVATION AS THE WATER LEVEL IN THE CHAGRIN RIVER.

SPECIFICATIONS

THIS GEOTECHNICAL EXPLORATION WAS PERFORMED IN ACCORDANCE WITH THE STATE OF OHIO, DEPARTMENT OF TRANSPORTATION, OFFICE OF GEOTECHNICAL ENGINEERING, SPECIFICATIONS FOR GEOTECHNICAL EXPLORATIONS, DATED JULY 2021 AND JANUARY 2022.

AVAILABLE INFORMATION

THE SOIL, BEDROCK, AND GROUNDWATER INFORMATION COLLECTED FOR THIS SUBSURFACE EXPLORATION THAT CAN BE CONVENIENTLY DISPLAYED ON THE SOIL PROFILE SHEETS HAS BEEN PRESENTED. GEOTECHNICAL REPORTS, IF PREPARED, ARE AVAILABLE FOR REVIEW ON THE OFFICE OF CONTRACT SALES WEBSITE.

RECON – S&ME 7/20/21; 4/20/22

DRILLING – S&ME 8/5-6/21; 4/26-28/22

DRAWN – DM 10/18-19/22

REVIEWED – BKS 10/19/22

Soil Counts Table

| Classification | Mechanical | Visual |
| --- | --- | --- |
| A-2-4 | 2 | 2 |
| A-2-6 | - | 3 |
| A-4a | 8 | 24 |
| A-4b | 1 | - |
| A-6a | 2 | 14 |
| Total | 13 | 43 |

Scour Sampling Table

| Boring ID | Sample ID | Sample Depth (ft) | Sample Elevation | D50 (mm) |
| --- | --- | --- | --- | --- |
| B-001-0-21 | SS-6 | 13.5 – 15.0 | 613.5 – 612.0 | 0.0148 |
| B-001-0-21 | SS-7 | 16.0 – 17.5 | 611.0 – 609.5 | 0.0363 |
| B-001-0-21 | SS-9 | 21.0 – 22.5 | 606.0 – 604.5 | 0.2249 |
| B-002-0-21 | SS-5 | 11.0 – 12.5 | 614.6 – 613.1 | 0.3959 |
| B-002-0-21 | SS-8 | 18.5 – 20.0 | 607.1 – 605.6 | 0.294 |
| B-002-0-21 | SS-9 | 21.0 – 22.5 | 604.6 – 603.1 | 0.2822 |
| H-001-4-21 | HA-1 | 0.0 – 1.5 | 612.9 – 611.4 | 0.7016 |
| H-001-5-21 | HA-1A | 0.0 – 1.0 | 605.2 – 604.2 | 0.4085 |
| H-002-3-21 | HA-1A | 0.0 – 0.7 | 606.1 – 605.4 | 0.1757 |

Rock Unconfined Compressive Strength Test Results Table

| Boring ID | Sample ID | Sample Depth (ft) | Sample Elevation | Qu (psi) |
| --- | --- | --- | --- | --- |
| B-001-6-22 | NQ-13 | 31.6 – 31.9 | 595.8 – 595.5 | 645 |
| B-001-6-22 | NQ-15 | 41.4 – 41.8 | 586.0 – 585.6 | 1,296 |
| B-001-7-22 | NQ-18 | 43.9 – 44.3 | 582.3 – 581.9 | 1,432 |
| B-002-4-22 | NQ-13 | 30.4 – 30.8 | 594.7 – 594.3 | 1,041 |

Rock Slake Durability Test Results Table

| Boring ID | Sample ID | Sample Depth (ft) | Sample Elevation | SDI (%) |
| --- | --- | --- | --- | --- |
| B-001-6-22 | NQ-12 | 27.5 – 30.5 | 599.9 – 596.9 | 42.8 |
| B-002-4-22 | NQ-13 | 27.0 – 31.0 | 598.1 – 594.1 | 81.1 |