# UTILITIES

THERE ARE NO UNDERGROUND UTILITIES SHOWN ON THIS PLAN. THE

NATURE OF THE WORK REQUIRED BY THIS PROJECT WILL NOT AFFECT

ANY KNOWN UNDERGROUND UTILITIES THAT EXIST UNDER OR

ADJACENT TO THE WORK AREA.

# CONTINGENCY QUANTITIES

THE CONTRACTOR SHALL NOT ORDER MATERIALS OR PERFORM WORK FOR

ITEMS DESIGNATED BY PLAN NOTE TO BE USED "AS DIRECTED BY THE ENGINEER" UNLESS AUTHORIZED BY THE ENGINEER.  THE ACTUAL WORK LOCATIONS AND QUANTITIES USED FOR SUCH ITEMS SHALL BE INCORPORATED INTO THE FINAL CHANGE ORDER GOVERNING THE COMPLETION OF THIS PROJECT.

# PAVEMENT MARKINGS

ALL LONG LINE AND AUXILIARY PAVEMENT MARKINGS (STOP LINES, CROSSWALK LINES, CHANNELIZING LINES, ETC.) SHOWN IN THE PLANS ARE TAKEN FROM EXISTING LOCATIONS. THE CONTRACTOR SHALL DOCUMENT ALL PAVEMENT MARKING LOCATIONS THAT WILL BE REMOVED/ OBLITERATED DURING THIS PROJECT AND PLACE MARKINGS AT THE LOCATION OF THE EXISTING MARKINGS, UNLESS SHOWN OR STATED DIFFERENTLY IN THE PLANS AND/OR DIRECTED OTHERWISE BY THE ENGINEER.

CENTER LINE MARKINGS SHALL BE PLACED PER THE NO PASSING ZONE LOGS FOUND ON THE WEBSITE BELOW. ANY DISCREPANCIES BETWEEN THE EXISTING MARKINGS ON THE PAVEMENT AND THE LOGS SHALL BE BROUGHT TO THE ATTENTION OF THE ENGINEER PRIOR TO PLACEMENT.

https://www.transportation.ohio.gov/wps/portal/gov/odot/programs/traffic-regulations/no-passing-zones

THE CONTRACTOR SHALL NOTIFY THE PROJECT ENGINEER A MINIMUM

OF 24 HOURS PRIOR TO APPLYING PAVEMENT MARKING MATERIALS ON

ANY ROUTES SO THAT ODOT PERSONNEL MAY BE PRESENT DURING

PAVEMENT MARKING OPERATIONS.

PER CMS 641.04, THE CONTRACTOR SHALL PROVIDE ODOT PERSONNEL A COPY OF THE DLS SHORT REPORT AT THE END OF EVERY WORKDAY THROUGH CLOUD BASED REPORTING. THE CONTRACTOR SHALL NOT RECEIVE PAYMENT FOR ANY WORK DONE WITHOUT NOTIFICATION AS STATED ABOVE OR IF DSL SHORT REPORTS ARE NOT PROVIDED DAILY. SEPARATE DLS REPORTS SHALL BE PROVIDED FOR EVERY RUN OF LONG LINE MARKING PLACED. COMBINED DLS REPORTS (I.E. CENTER LINE AND EDGE LINE) WILL NOT BE ACCEPTED OR PAID BY THE DEPARTMENT.

# ENVIRONMENTAL NOTES

TWO STRUCTURES IN THE PROJECT AREA INVOLVE WORK OVER A STREAM,

SLM 4.28 OVER MUDDY FORK, AND SLM 5.34 OVER SOUTH FORK LICKING

RIVER. ALL WORK IS PROHIBITED TO OCCUR BELOW THE OHWM IN STREAMS THAT FLOW UNDER THE STRUCTURES, AND NO MATERIAL MAY ENTER ANY STREAM DURING CONSTRUCTION. ALL BRIDGE SCUPPERS SHALL BE COVERED,

AT THE APPROVAL OF THE ENGINEER, FOR ANY WORK TAKING PLACE ON AND/

OR ADJACENT TO THE STRUCTURES MENTIONED ABOVE.

ITEM 209, LINEAR GRADING

IN ORDER TO PROVIDE POSITIVE DRAINAGE FROM THE ROADWAY SURFACE

TO THE SHOULDER BREAK, THE EXISTING ROADWAY SHOULDERS SHALL BE

GRADED AND SHAPED USING A GRADER OF ADEQUATE SIZE TO PERFORM

THE WORK TO THE SATISFACTION OF THE ENGINEER.

ALL LINEAR GRADING WORK SHALL BE COMPLETED PRIOR TO PLACEMENT

OF THE ASPHALT SURFACE COURSE. NO LINEAR GRADING SHALL OCCUR

IN CURBED SECTIONS.

IF LINEAR GRADING IS NOT REQUIRED, AS DETERMINED BY THE ENGINEER,

THIS ITEM SHALL BE NON-PERFORMED.

ALL EXCESS MATERIAL REMAINING AROUND GUARDRAIL AND OTHER AREAS

AFTER THE GRADER WORK IS COMPLETED SHALL BE REMOVED AND DISPOSED

OF OFF-SITE BY THE CONTRACTOR.  ALL EQUIPMENT, LABOR, OR INCIDENTALS REQUIRED TO COMPLETE THIS ITEM SHALL BE INCLUDED FOR PAYMENT IN

THE UNIT PRICE BID FOR ITEM 209, LINEAR GRADING.

ITEM 209, LINEAR GRADING

LOCATION 1A:  13.61 MILE

LOCATION 1B:  13.42 MILE

ITEM 209, PREPARING SUBGRADE FOR SHOULDER PAVING, AS PER PLAN

BEFORE PLACEMENT OF THE SURFACE COURSE, THE CONTRACTOR SHALL

EXCAVATE AN AREA 10 INCHES WIDE OUTSIDE THE EXISTING PAVED SHOULDER

TO PROVIDE A LEVEL SURFACE FREE OF VEGETATION FOR PLACEMENT OF

ITEM 617, COMPACTED AGGREGATE, AS PER PLAN.

ALL EXCAVATED MATERIAL SHALL BE REMOVED AND DISPOSED OFF-SITE BY

THE CONTRACTOR PRIOR TO PLACEMENT OF THE COMPACTED AGGREGATE.

ALL EQUIPMENT, LABOR, AND INCIDENTALS REQUIRED TO PERFORM

EXCAVATION OF SHOULDER SHALL BE INCLUDED FOR PAYMENT IN THE UNIT

PRICE BID FOR ITEM 209, PREPARING SUBGRADE FOR SHOULDER PAVING,

AS PER PLAN.

ITEM 254, PAVEMENT PLANING, ASPHALT CONCRETE, BY DEPTH

DEPTH OF PLANING SHALL BE AS SHOWN ON THE PAVEMENT AND SHOULDER DATA TABLES. PLANING SHALL BE THE FULL WIDTH OF THE EXISTING PAVEMENT, INCLUDING PAVED SHOULDERS. THE ROADWAY SHALL BE PLANED SUCH THAT POSITIVE DRAINAGE IS CREATED FROM THE CENTER LINE TO THE EDGE OF PAVEMENT IN TANGENT SECTIONS AND SHALL FOLLOW EXISTING SUPERELEVATIONS WHERE APPLICABLE. ALL REQUIREMENTS OF ITEM 254

SHALL APPLY.

IF DURING PLANING OPERATIONS EXCESSIVE RIDGES OR OTHER IRREGULARITIES ARE FOUND, PLANING DEPTH ADJUSTMENTS SHALL BE MADE UP TO 3/8 INCH, AS DIRECTED BY THE ENGINEER. PAYMENT SHALL BE INCLUDED IN THE UNIT PRICE BID PER CMS 254.07.

ITEM 253, PAVEMENT REPAIR

AN ESTIMATED QUANTITY FOR PAVEMENT REPAIR HAS BEEN INCLUDED

IN THE PLAN TO BE USED AS DIRECTED BY THE ENGINEER. REPAIRS

SHALL TAKE PLACE PRIOR TO ANY PLANING OPERATIONS.

THE INTENT OF THIS OPERATION IS TO REPAIR THE ASPHALT OVER

DETERIORATED TRANSVERSE JOINTS SHOWING SPALLING AND EXTENSIVE

CRACKING. THE EXISTING TRANSVERSE JOINT SPACING IS 60 FT.

DEPTH OF EXCAVATION SHALL BE 7" (OR TOP OF CONCRETE BASE, WHICHEVER COMES FIRST). THE MINIMUM LENGTH SHALL BE 6 FT, CENTERED OVER JOINT

BUT MAY BE EXTENDED TO COVER ADJACENT JOINTS IF NECESSARY. THE

MINIMUM WIDTH SHALL BE 12’ OR ENTIRE TRAVEL LANE (SHOULDERS NOT INCLUDED).

AFTER EXCAVATION HAS BEEN COMPLETED, THE FACE OF THE REPAIR SHALL BE COATED WITH 407 TACK COAT. REPLACEMENT MATERIAL WILL BE 7" (OR DEPTH

OF EXCAVATION) OF ITEM 301 ASPHALT CONCRETE BASE, PG64-22 (PLACED, COMPACTED, AND TACKED IN TWO LIFTS).

ALL EXCAVATION, MATERIALS, LABOR, EQUIPMENT, TOOLS, TRAFFIC CONTROL

AND INCIDENTALS NEEDED TO COMPLETE THE WORK DESCRIBED ABOVE SHALL

BE PAID UNDER ITEM 253, PAVEMENT REPAIR.

ITEM 253, PAVEMENT REPAIR

LOCATION 1A:  500 CU.YD. (SLM 5.44-7.55)

LOCATION 1B:  1,500 CU.YD.  (SLM 7.55-14.10, 0.00-0.19)

# ITEM 407, NON-TRACKING TACK COAT

THE RATE OF APPLICATION OF THE ITEM 407, NON-TRACKING TACK COAT

SHALL BE PER CMS TABLE 407.06-1 AND SUBJECT TO ADJUSTMENT AS

DIRECTED BY THE ENGINEER.  PLAN QUANTITIES INDICATE AN AVERAGE APPLICATION RATE OF 0.08 GAL/SY FOR TACK COAT UNDER THE SURFACE

COURSE AND 0.05 GAL/SY UNDER INTERMEDIATE COURSE. (FOR ESTIMATING PURPOSES ONLY).

# ITEM 408, PRIME COAT, AS PER PLAN

THE CONTRACTOR SHALL APPLY ONE COAT OF MC-70 (AS PER CMS 702) AT

A RATE OF 0.40 GAL/SY TO THE COMPLETED AGGREGATE SHOULDER. TO

REDUCE AGGREGATE LOSS, THE PRIME COAT SHALL BE APPLIED WITHIN

SEVEN (7) DAYS AFTER PLACEMENT OF THE AGGREGATE SHOULDER OR LIQUATED DAMAGES PER CMS 108.07 WILL BE ASSESSED. THE CONTRACTOR SHALL PROVIDE A SHIELD TO PREVENT THE SPRAYING OR DRIFTING OF LIQUID BITUMINOUS MATERIAL ONTO THE EDGE OF PAVEMENT OR EDGE LINE. THE ATTENTION OF THE CONTRACTOR IS DIRECTED TO 107.10 OF SPECIFICATIONS.

# ITEM 409, SAWING AND SEALING ASPHALT CONCRETE PAVEMENT JOINTS

# THIS ITEM SHALL BE USED AT BRIDGE DECK END TRANSVERSE JOINTS RECEIVING

# AN ASPHALT OVERLAY AND CONSTRUCTED PER CMS 409.03. THE SAW CUT

# LINE SHOULD MATCH THE BRIDGE DECK END AND MAY BE SKEWED. THE

# CONTRACTOR SHALL PRE-MARK THE SAW CUT LINE AND OBTAIN APPROVAL

# FROM THE ENGINEER BEFORE PROCEEDING WITH THE SAW CUT.

# ITEM 516, 2" DEEP JOINT SEALER, AS PER PLAN (A)

THE CONTRACTOR SHALL PLACE A 1" X 2" DEEP BEAD OF JOINT SEALER (AS

PER 705.04) AT THE LOCATIONS SHOWN IN PLANS. THE CONTRACTOR SHALL

SAW CUT A CHANNEL FOR THE JOINT SEALER. THE COST FOR SAW CUTTING

THE CHANNEL FOR THE JOINT SEALER SHALL BE INCLUDED FOR PAYMENT

WITH ITEM 516, 2" DEEP JOINT SEALER, AS PER PLAN (A).

ITEM 516, 2" DEEP JOINT SEALER, AS PER PLAN (B)

REMOVE ALL FOREIGN MATERIAL AND DEBRIS FROM THE EXISTING JOINT BETWEEN THE APPROACH SLAB AND DECK OR BACKWALL. ANY SPALLS ADJACENT TO THE JOINT LESS THAN OR EQUAL TO 3" SHALL BE CLEANED AND SEALED

WITH THIS ITEM. FOR SPALLS GREATER THAN 3", USE ITEM SPECIAL, PATCHING CONCRETE BRIDGE DECKS AS DESCRIBED ON THIS SHEET.

AFTER REMOVAL, PERFORM A NEW SAWCUT TO ESTABLISH A 1/2" WIDE BY

2-1/4” DEEP JOINT ALONG THIS INTERFACE. ONCE THE JOINT HAS BEEN OPENED OR CREATED, AIRBLAST THOROUGHLY PRIOR TO PLACEMENT OF

HOT APPLIED JOINT SEALER PER 705.04, AS DIRECTED BY THE ENGINEER.

DO NOT DISTURB EXPANSION JOINT INSIDE JOINT ARMOR IF PRESENT.

PAYMENT FOR ALL LABOR, EQUIPMENT, AND MATERIALS SHALL BE INCLUDED

IN THE UNIT PRICE BID FOR ITEM 516, 2" DEEP JOINT SEALER, AS PER PLAN (B).

ITEM 516, 2" DEEP JOINT SEALER, AS PER PLAN (C)

THE CONTRACTOR SHALL CLEAN THE JOINTS SHOWN IN THE DETAIL BELOW 
AS DESCRIBED IN CMS 516.06 PRIOR TO PLACEMENT OF THE JOINT SEALER. 
THE JOINT/VOID SHALL BE FILLED WITH EXPANDING FOAM AND CURED FOR 
24 HOURS. AFTER CURING THE CONTRACTOR SHALL CUT OUTEXCESS FOAM 
TO A DEPTH OF 1/2" MEASURED FROM THE EXISTING CONCRETE SURFACE. 
AFTER THE FOAM HAS BEEN REMOVED THE JOINTS SHALL BE SEALED WITH 
DECK-O-SEAL GUN GRADE JOINT SEALANT OR AN APPROVED EQUAL. THE 
COLOR SHALL BE STONE GRAY. APPROVED MANUFACTURE'S APPLICATION 
METHODS SHALL BE FOLLOWED DURING SURFACE PREPARATION AND 
APPLICATION FOR MAXIMUM EFFECTIVENESS.

DECK-O-SEAL
P.O. BOX 397
HAPSHIRE, IL 60140
PHONE: 800-542-7665

PAYMENT FOR THE ABOVE WORK SHALL BE MADE AT THE UNIT PRICE BID

FOR ITEM 516, 2" DEEP JOINT SEALER, AS PER PLAN (C) AND SHALL INCLUDE ALL LABOR, EQUIPMENT, AND INCIDENTALS REQUIRED TO COMPLETE THE WORK DESCRIBED.

LIC-16-0327

R.A.S. (L/R) PARAPET/CURB = 52 FT

F.A.S. (L/R) PARAPET/CURB = 52 FT

QUANTITIES CARRIED TO BRIDGE TREATMENT DATA SHEET.

ITEM SPECIAL, PATCHING CONCRETE BRIDGE DECKS, TYPE B

THIS ITEM SHALL CONFORM TO PROPOSAL NOTE 512 ALONG WITH THE 
FOLLOWING REQUIREMENTS:

ALL BRIDGE DECK PATCHING SHALL USE COMMERCIAL GRADE QUIKRETE 
FAST SET DOT MIX EXTENDED OR APPROVED EQUAL. THE MATERIAL SHALL
BE PLACED AT A MINIMUM DEPTH OF 4” AND CURED PER 
MANUFACTURER RECOMMENDATIONS. 

ALL LOCATIONS AND DIMENSIONS WILL BE DETERMINED BY THE ENGINEER.
THE INTENT OF THE WORK IS TO CORRECT HEAVILY DETERIORATED

SURFACE ZONES (I.E. FAILED CONCRETE/ASPHALT PATCHES, MAP-CRACKED DETERIORATED AREAS, SPALLED/OPEN POTHOLES, ETC.) AND NOT TO

CORRECT ALL UNSOUND AREAS OF THE BRIDGE DECK/OVERLAY.

TRAFFIC WILL NOT BE PERMITED ON THE FINISHED CONCRETE SURFACE 
UNTIL AFTER TWO TEST BEAMS HAVE ATTAINED AN AVERAGE MODULUS 
OF RUPTURE OF 400 PSI.

PAYMENT FOR ALL OF THE ABOVE-DESCRIBED LABOR, EQUIPMENT, AND 
MATERIALS WILL BE MADE AT THE CONTRACT PRICE BID FOR ITEM SPECIAL,
PATCHING CONCRETE BRIDGE DECKS, TYPE B.

# ITEM 617, COMPACTED AGGREGATE, AS PER PLAN

ALL AGGREGATE SHALL BE 100% CRUSHED LIMESTONE OR RECYCLED ASPHALT CONCRETE PAVEMENT (RAP) MEETING THE REQUIREMENTS OF 703.18.

ALL AREAS SHALL BE LOOSENED AND FREE OF VEGETATION PER 617.04 PRIOR

TO PLACEMENT OF COMPACTED AGGREGATE. AGGREGATE SHOULDERS SHALL

BE SLOPED TO PROVIDE POSITIVE DRAINAGE AWAY FROM THE ROADWAY. AGGREGATE SHOULDERS MAY BE REDUCED TO ONE (1) FOOT WIDE WHERE NECESSARY AND MAY BE OMITTED ON SLOPES STEEPER THAN 6:1 AT THE

APPROVAL OF THE ENGINEER.

IN AREAS WHERE TOPSOIL IS ENCOUNTERED, THE WIDTH OF THE COMPACTED AGGREGATE SHALL BE REDUCED TO THE WIDTH OF THE EXISTING BERM. IF

NO EXISTING BERM EXISTS THAN THE COMPACTED AGGREGATE SHALL BE

NON-PERFORMED.

SHOULDER PREPARATION SHALL BE INCLUDED FOR PAYMENT IN THE

UNIT PRICE BID FOR ITEM 617, COMPACTED AGGREGATE, AS PER PLAN.

# ITEM 621, RAISED PAVEMENT MARKER REMOVED

AN ESTIMATED QUANTITY HAS BEEN INCLUDED IN THE PLANS TO REMOVE

RAISED PAVEMENT MARKERS FOR DISPOSAL BY THE CONTRACTOR. RPM

REMOVAL SHALL NOT OCCUR SOONER THAN 10 DAYS PRIOR TO RESURFACING

OF THE ROADWAY. ALL RPM'S REMOVED SHALL BECOME THE PROPERTY OF

THE CONTRACTOR.

ITEM 611, CATCH BASIN ADJUSTED TO GRADE, AS PER PLAN

ITEM 611, CATCH BASIN RECONSTRUCTED TO GRADE, AS PER PLAN

ITEM 611, MANHOLE ADJUSTED TO GRADE

ITEM 638, VALVE BOX ADJUSTED TO GRADE

THESE ITEMS SHALL BE USED TO ADJUST AND/OR RECONSTRUCT CATCH

BASINS, MANHOLES, AND WATER VALVE BOXES TO GRADE LOCATED THROUGHOUT THE PROJECT LIMITS AS DESCRIBED BELOW:

EXISTING CONCRETE COLLARS SHOULD ONLY BE ADJUSTED IF BROKEN, DAMAGED, OR MISALIGNED AS DIRECTED BY THE ENGINEER. ALL

ADJUSTMENTS SHALL BE AGREED ON BY THE PROJECT ENGINEER AND

THE CITY OF PATASKALA BEFORE WORK MAY BEGIN.

CATCH BASIN ADJUSTED TO GRADE INCLUDES ALL NECESSARY WORK

TO RESET FRAME PER CMS 611.11D AND SHALL ALSO INCLUDE THE REBUILDING

OF STRUCTURE WALLS DOWN TO 12 INCHES BELOW THE BASIN FRAME, AS

DIRECTED BY THE ENGINEER.

CATCH BASIN RECONSTRUCTED TO GRADE INCLUDES ALL WORK PER CMS 611.11C THAT REQUIRES THE REBUILDING OF STRUCTURE WALLS GREATER

THEN 12 INCHES BELOW THE BASIN FRAME AND EXTENDS TO THE BOTTOM

OF THE BASIN, AS DIRECTED BY THE ENGINEER.

PRIOR TO THE START OF THE PROJECT, THE CITY WILL BE MADE AWARE TO CLEAN OUT ALL CATCH BASIN DEBRIS WITHIN THEIR CORPORATION LIMITS

AND WILL NOTIFY THE PROJECT ENGINEER WHEN THE WORK IS COMPLETED. ONCE VERIFIED BY THE ENGINEER, THE CONTRACTOR WILL BE RESPONSIBLE

FOR REMOVING ALL CATCH BASIN DEBRIS AND DISPOSING OFF SITE AT THE COMPLETION OF THE PROJECT. THIS WORK SHALL BE INCLUDED IN THE

CATCH BASIN ADJUSTMENT AND RECONSTRUCTION COSTS.

ANY GAS VALVE BOXES AND TELEPHONE COMPANY MANHOLES ON THIS

PROJECT SHALL NOT BE DISTURBED.

ALL MATERIALS, LABOR, EQUIPMENT, TOOLS, AND INCIDENTALS NECESSARY

TO COMPLETE THE WORK AS DESCRIBED ABOVE AND SHALL BE INCLUDED

FOR PAYMENT WITH THE ITEMS LISTED BELOW.

ITEM 611, CATCH BASIN ADJUSTED TO GRADE, AS PER PLAN

LOCATION 1A:  2 EACH

LOCATION 1b:  1 EACH

ITEM 611, CATCH BASIN RECONSTRUCTED TO GRADE, AS PER PLAN

LOCATION 1A:  2 EACH

LOCATION 1b:  1 EACH

ITEM 611, MANHOLE ADJUSTED TO GRADE

LOCATION 1A:  3 EACH

LOCATION 1b:  1 EACH

ITEM 638, VALVE BOX ADJUSTED TO GRADE

LOCATION 1A:  3 EACH

LOCATION 1b:  1 EACH

RESIDENTIAL AND COMMERCIAL DRIVES

AN ESTIMATED QUANTITY OF ITEM 441, ASPHALT CONCRETE, HAS BEEN INCLUDED IN THE PLANS TO BE USED AS DIRECTED BY THE ENGINEER TO PAVE APPROACH AREAS TO EXISTING DRIVEWAYS. PAVING SHALL EXTEND AN AVERAGE OF 4' INTO THE DRIVEWAY (MEASURED FROM THE EDGE OF PAVEMENT OR PAVED SHOULDER IF PRESENT). THE ENGINEER MAY EXTEND PAVING DISTANCE FOR ASPHALT DRIVEWAYS IN ORDER TO PROVIDE SMOOTH TRANSITION AND/OR ELIMINATE SHORT DISTANCES OF UNDESIRABLE PROFILE. ABRUPT CHANGES IN DRIVEWAY PROFILE ARE NOT PERMITTED.

EXISTING BUTT JOINTS OF RESIDENTIAL AND COMMERCIAL DRIVES

SHOULD BE MATCHED IF PRESENT. SEE SHEETS 20-47 FOR DETAILS.

GRAVEL DRIVES SHALL ALSO BE PAVED AS DESCRIBED ABOVE. FIELD

DRIVES AND OIL-WELL DRIVES SHALL NOT BE PAVED.

IF AN EXISTING APRON CANNOT BE PAVED OVER (FOR EXAMPLE, BROKEN

INTO SMALL PIECES) AS DETERMINED BY THE ENGINEER, IT SHALL BE REMOVED BEFORE BEING PAVED BACK 4' INTO THE DRIVEWAY. ALL GRADING, PRIME OR TACK COAT, MATERIALS, LABOR, EQUIPMENT TOOLS AND INCIDENTALS NECESSARY TO COMPLETE THE DRIVES SHALL BE INCLUDED IN THE UNIT PRICE BID FOR THE ITEMS LISTED BELOW.

BUTT JOINTS AT THE END OF ALL DRIVEWAYS SHALL BE 1.25” IN DEPTH TO ACCOMMODATE THE SURFACE COURSE.  NO WORK SHALL BE PERFORMED

ON DRIVEWAYS LOCATED IN CURB SECTIONS UNLESS OTHERWISE DIRECTED

BY THE ENGINEER.

THE FOLLOWING QUANTITIES HAVE BEEN CARRIED TO THE LOCATION

SUB-SUMMARIES FOR THE ABOVE-DESCRIBED PURPOSE:

# ITEM 254, PAVEMENT PLANING, ASPHALT CONCRETE, 1.50”

# LOCATION 1A:   840 SQ.YD.

# LOCATION 1B:   840 SQ.YD.

# ITEM 441, ASPHALT CONCRETE SURFACE COURSE, TYPE 1, (448),

# PG70-22M, 1.50”

# LOCATION 1A:   35 CU.YD.

# LOCATION 1B:   35 CU.YD.

MAILBOX TURN OUTS

AN ESTIMATED QUANTITY OF ITEM 441, ASPHALT CONCRETE, HAS BEEN

INCLUDED IN THE PLAN TO BE USED AS DIRECTED BY THE ENGINEER TO

PAVE MAILBOX TURN OUTS. TURN OUTS SHALL BE PAVED AS SHOWN IN

STANDARD DRAWING BP-4.1. THE ENGINEER MAY MODIFY TO MEET EXISTING CONDITIONS IF NECESSARY.

ALL GRADING, MATERIALS, LABOR, EQUIPMENT, AND INCIDENTALS NECESSARY

TO COMPLETE THE MAILBOX, TURN OUTS SHALL BE INCLUDED IN THE UNIT

PRICE BID FOR THE ITEMS LISTED BELOW.

# ITEM 254, PAVEMENT PLANING, ASPHALT CONCRETE, 1.50”

# LOCATION 1A:   530 SQ.YD.

# LOCATION 1B:   440 SQ.YD.

# ITEM 441, ASPHALT CONCRETE SURFACE COURSE, TYPE 1, (448),

# PG70-22M, 1.50”

# LOCATION 1A:   22 CU.YD.

# LOCATION 1B:   19 CU.YD.

ITEM 632, DETECTOR LOOP, AS PER PLAN

ALL STOP LINE INDUCTANCE DETECTOR LOOPS SHALL BE THE POWER

HEAD CONFIGURATION PER TC-82.10. THE MINIMUM LENGTH SHALL

BE 20’. THE STOP LINE DETECTOR LOOPS SHALL NOT BE WIRED TO ANY

OTHER LOOPS AND SHALL HAVE ITS OWN DETECTOR CHANNEL. ALL

STOP LINE DETECTION SHALL BE TESTED FOR A BICYCLE TARGET.

ALL DILEMMA ZONE INDUCTANCE DETECTOR LOOPS SHALL BE ANGULAR

DESIGN DETECTION (ADD) PER TC-82.10. ALL DILEMMA DETECTION

ZONES SHALL BE TESTED FOR A MOTORCYCLE TARGET.

ALL DETECTOR LOOPS SHALL BE CUT INTO THE PLANED SURFACE OR

THE PROPOSED INTERMEDIATE COURSE AT A DEPTH OF 4" FROM THE

PROPOSED SURFACE ELEVATION. THE CONTRACTOR SHALL TEST ALL

LEAD-IN CABLES PRIOR TO MAKING THE FINAL SPLICE.

ALL MATERIALS, LABOR, TOOLS, EQUIPMENT, TRAFFIC CONTROL AND

INCIDENTALS NECESSARY TO PERFORM THE WORK DESCRIBED ABOVE

SHALL BE INCLUDED IN THE UNIT PRICE BID FOR ITEM 632, DETECTOR LOOP,

AS PER PLAN. THE FOLLOWING QUANTITY HAS BEEN CARRIED TO THE

LOCATION SUB-SUMMARIES TO BE USED AS DIRECTED BY THE ENGINEER.

SEE SHEETS 20-47 FOR LOCATIONS. DETECTOR LOOPS SHOULD ONLY BE

INSTALLED IF DAMAGED DURING PAVING OPERATIONS, AS DIRECTED BY

THE ENGINEER.

# S.R. 16/ ETNA PARKWAY

# STOP BAR (POWERHEAD):  3 EACH

# DILEMMA ZONE (ANGULAR):  2 EACH

# S.R. 16/ OAK MEADOW DR./ VINE ST.

# STOP BAR (POWERHEAD):  4 EACH

# S.R. 16/ S.R. 310 (NORTH/SOUTH)

# STOP BAR (POWERHEAD):  9 EACH

# S.R. 16/ OXFORD DR.

# STOP BAR (POWERHEAD):  6 EACH

# S.R. 16/ CORYLUS DR.

# STOP BAR (POWERHEAD):  7 EACH

# DILEMMA ZONE (ANGULAR):  2 EACH

# S.R. 16/ ONE HEALTHLY PL.

# STOP BAR (POWERHEAD):  3 EACH

# S.R. 16/ WATKINS RD.

# STOP BAR (POWERHEAD):  4 EACH

# ITEM 632, DETECTOR LOOP, AS PER PLAN

# LOCATION 1A:   40 EACH

# ITEM 441, ASPHALT CONCRETE SURFACE COURSE, TYPE 1, (448),

AS PER PLAN, PG70-22M

Follow 403, except as follows:

- Offset the AC gauge for each JMF for the project prior to the project’s start using 403.06.A. and the MODIFIED Supplement 1043 procedure below.

- During S-1043.07 process, a RAP sample obtained from the JMF-designated RAP pile will be extracted in the Asphalt Level 3 lab to verify the RAP AC %. The RAP AC % will be within 0.3% of the average RAP AC % from the JMF. If RAP AC % is outside of the 0.3%, the verification pan process will stop, and District Testing will allow one opportunity to rework the RAP pile at the mix plant and resample. Resampling requires District Testing to be present. If the resample is still outside of the 0.3%, the JMF will be rescinded and need to be redesigned.

Follow 403.06 except as follows:

- Ensure asphalt binder content does not exceed Table 403.06.G-1. Adjustments to mix plant control settings must be submitted to and approved by District Testing prior to making the adjustment.  The adjustment cannot exceed +/- 0.2% from Design AC % from JMF. Do not lower THE VIRGIN binder content or increase RAP percent. Ensure plant ticket shows the adjustment and is set to the adjusted total AC % at all times afterwards.

- Record the daily verification pan results in a separate worksheet and make sure it’s posted in the plant facility and available to the monitors. Include the date ran, verification pan result, and initials of who ran it. Ensure a printout of the daily verification pan is also included with the TE-199.

Follow Supplement 1043 for AC Gauge offset, except as FOLLOWS:

Follow 1043.07 except as followS:

  - Notify District Testing a minimum of one week prior to making verification pans.

  - District Testing will witness a solvent extraction from a sample from the RAP pile that is to be used in the JMF to verify the RAP AC %. RAP AC % will be within 0.3% of RAP AC % determined in JMF. If outside of 0.3%, do not proceed and the JMF will need to be redesigned.

  - District Testing will witness the verification pans being blended, mixed, and compacted.

  - Make a minimum of three verification pans for the JMF that are at the JMF asphalt binder content. Make one additional verification pan for each additional district the JMF will be used in.

  - In addition, turn possession over of the calibration AC gauge pans used to determine the fit coefficient to District Testing.

For AC content pay acceptance, replace 1043.08 with the following:

Calculate an AC gauge offset amount for each JMF and mix plant in accordance with the following procedure prior to start of any production for the JMF. Notify District Testing 24 hours prior to offsetting gauge.

- Ensure printer is on and place the first verification pan in the AC gauge and run.

- After the 16-minute test, take the verification pan out and turn 180 degrees and place back in AC gauge and run.

- Repeat Steps 1 and 2 with second and third verification pans.

- For each run, take the JMF asphalt binder content minus the AC gauge AC % to obtain the offset for that run.

- Average all offsets for a final offset.

- Retain all of the verification pans. After the final offset is determined, District Testing will choose two of the verification pans and send one of these two to OMM to extract and reflux.

- District Testing will use the two verification pans to offset their AC gauge.

Before the beginning of a production day, run the verification pan in the AC gauge and ensure the offset AC gauge amount is within 0.14% of the JMF asphalt binder content. During the start of production for the JMF, solvent extract the first two QC samples and compare to the offset AC gauge. Ensure solvent extraction is within 0.3% of offset AC gauge. If more than 0.3% off, immediately resample and run AC gauge and solvent extract immediately. If two consecutive samples are more than 0.3% off, immediately stop production, contact THE MONITORING team, and investigate the reason for the problem. Once two consecutive QC samples are within 0.3% of offset AC gauge, the final offset gauge is confirmed.

After confirming the AC Gauge offset amount proceed with determining AC contents of production samples by the AC gauge according to 1043.09.

Only determine one AC Gauge offset amount per JMF. If more than

30 days has lapsed since the JMF was last tested, re-do the offset procedure above with two verification pans (one from the contractor and one from the district). If an AC Gauge offset amount is later determined, by an investigation of both the Contractor and the District, to be incorrect re-do the offset procedure.

In addition, also determine the AC gauge offset following the current procedure as outlined in Supplement 1043 Dated January 18, 2019 and provide the information to the Department. This AC gauge offset number will not be used during QC testing.

PN 420 SMOOTHNESS SPECIFICATION

PROPOSAL NOTE 420 SHALL ONLY APPLY IN THE FOLLOWING SECTIONS OF

S.R. 16 OUTSIDE THE CITY OF PATASKALA CORPORATION LIMITS, UNLESS DIRECTED OTHERWISE BY THE ENGINEER:

LIC-16-(7.55-14.10)

LIC-16I-(0.00-0.19)

# PAYMENT FOR PN 420 SMOOTHNESS SHALL BE INCLUDED IN THE UNIT PRICE

# BID FOR ITEM 441, ASPHALT CONCRETE SURFACE COURSE, TYPE 1, (448),

AS PER PLAN, PG70-22M, AND INCLUDE ALL LABOR, EQUIPMENT, AND INCIDENTALS NECESSARY TO COMPLETE THE WORK.

LONGITUDINAL COLD JOINT PLACEMENT

THE CONTRACTOR SHALL LIMIT THE AMOUNT OF LONGITUDINAL COLD JOINTS PLACED ON THE PROJECT. DUE TO VARYING PAVEMENT WIDTHS ON THE PROJECT, ECHELON PAVING TECHNIQUES MAY BE REQUIRED. ALL COLD JOINTS PLACED OUTSIDE OF THE PAVEMENT CROWN SHALL BE APPROVED BY THE ENGINEER.

IF ADDITIONAL COLD JOINTS ARE REQUIRED IN ORDER TO SAFELY MAINTAIN

TRAFFIC, THEY SHALL BE PLACED OUTSIDE OF THE WHEEL PATH. THIS WORK

SHALL BE CONSIDERED INCIDENTAL TO PLACEMENT OF THE SURFACE COURSE.

AIRWAY/HIGHWAY CLEARANCE FOR AIRPORTS AND HELIPORTS

THIS PROJECT HAS BEEN IDENTIFIED AS BEING WITHIN THE INFLUENCE AREA OF

A PUBLIC USE AIRPORT OR HELIPORT. NO TEMPORARY STRUCTURES OR CONSTRUCTION EQUIPMENT AT MAXIMUM OPERATING HEIGHT SHALL EXCEED

A HEIGHT OF 25 FT. IF ANY TEMPORARY STRUCTURES OR CONSTRUCTION EQUIPMENT WILL EXCEED THIS HEIGHT, FURTHER COORDINATION WITH THE FEDERAL AVIATION ADMINISTRATION (FAA), AND ODOT OFFICE OF AVIATION, WILL BE NECESSARY PRIOR TO ERECTING SUCH TEMPORARY STRUCTURES OR OPERATING SUCH EQUIPMENT ON THE PROJECT. THE CONTRACTOR

WILL BE REQUIRED TO FILE A NEW FAA FORM 7460-1, ADVISING THE

FAA THAT AERONAUTICAL STUDY NO. 2023-AGL-22271-OE, 2023-AGL-22272-OE AND 2023-AGL-22273-OE IS BEING RESUBMITTED AND THAT AN ALTERATION

TO THE ORIGINAL SUBMISSION IS REQUESTED.

NOTIFY THE ODOT OFFICE OF AVIATION WHEN RESUBMITTING AN

FAA FORM 7460-1. NO TEMPORARY STRUCTURES OR CONSTRUCTION EQUIPMENT SHALL EXCEED THE PERMISSIBLE HEIGHT, UNTIL A COPY

OF THE FAA APPROVAL AND THE ODOT OFFICE OF AVIATION PERMIT

HAS BEEN FURNISHED TO THE PROJECT ENGINEER.

FAA APPROVAL MAY TAKE UP TO 45 DAYS. ALL SUBMISSIONS SHALL

BE DIRECTED TO THESE OFFICES:

EXPRESS PROCESSING CENTER

THE FEDERAL AVIATION ADMINISTRATION

SOUTHWEST REGIONAL OFFICE

AIR TRAFFIC AIRSPACE BRANCH ASW-520

2601 MEACHAN BLVD.

FORT WORTH, TX 76137-4298

OHIO DEPARTMENT OF TRANSPORTATION

OFFICE OF AVIATION

2829 WEST DUBLIN-GRANVILLE ROAD

COLUMBUS, OHIO 43235

614-387-2346