ITEM 614, MAINTAINING TRAFFIC

A MINIMUM OF 1-LANE OF TRAFFIC IN EACH DIRECTION SHALL BE MAINTAINED AT ALL TIMES ON S.R. 16 BY USE OF THE EXISTING

PAVEMENT AND STANDARD DRAWINGS MT-97.12.

TWO-WAY TRAFFIC SHALL BE MAINTAINED AT ALL TIMES EXCEPT THAT

ONE-WAY TRAFFIC WILL BE PERMITTED FOR MINIMUM PERIODS OF TIME CONSISTENT WITH THE REQUIREMENTS OF THE SPECIFICATIONS FOR PROTECTION OF COMPLETED ASPHALT CONCRETE COURSES INCLUDING REPAIRS.

AT NO TIME SHALL TRAFFIC BE MAINTAINED ON THE PLANED SURFACE,

AT LEAST ONE COURSE OF ASPHALT CONCRETE SHALL BE IN PLACE BEFORE OPENING TO TRAFFIC. THIS RULE DOES NOT APPLY TO PLANING AT BRIDGES OR ACROSS BRIDGES UNLESS THE BRIDGE IS BEING TREATED THE SAME AS THE ADJACENT ROADWAY.

LENGTH AND DURATION OF LANE CLOSURES AND RESTRICTIONS SHALL BE AT THE APPROVAL OF THE ENGINEER. THE MAXIMUM LANE CLOSURE LENGTH SHALL BE PER MT-97.12. IT IS THE INTENT TO MINIMIZE THE IMPACT TO THE TRAVELING PUBLIC. LANE CLOSURES OR RESTRICTIONS OVER SEGMENTS OF THE PROJECT, IN WHICH NO WORK IS ANTICIPATED WITHIN A REASONABLE TIME FRAME, AS DETERMINED BY THE ENGINEER, SHALL NOT BE PERMITTED.  THE LEVEL OF UTILIZATION OF MAINTENANCE OF TRAFFIC DEVICES SHALL BE COMMENSURATE WITH THE WORK IN PROGRESS.

TEMPORARY TRAFFIC SIGNALS MAY USED FOR INSTALLATION OF BRIDGE DECK SEALING AND PATCHING. THEY SHALL BE REMOVED ONCE MATERIAL HAS CURED, AS DIRECTED BY THE ENGINEER.

A LANE CLOSURE ON S.R. 37 (PER MT-95.30) UNDER BRIDGE LIC-16-1415 IS PERMITTED BETWEEN THE HOURS OF 9AM-4PM FOR PIER PATCHING.

ALL WORK AND TRAFFIC CONTROL DEVICES SHALL BE IN ACCORDANCE WITH CMS 614 AND OTHER APPLICABLE PORTIONS OF THE SPECIFICATIONS, AS WELL AS THE OHIO MANUAL OF UNIFORM TRAFFIC CONTROL DEVICES.  PAYMENT FOR ALL LABOR, EQUIPMENT AND MATERIALS SHALL BE INCLUDED IN THE LUMP SUM CONTRACT PRICE FOR ITEM 614, MAINTAINING TRAFFIC, UNLESS SEPARATELY ITEMIZED IN THE PLAN.

WORK RESTRICTIONS

ALL WORK ON THIS PROJECT SHALL OCCUR FROM THE HOURS OF 7PM-7AM (I.E. NIGHT PAVING). ANY WORK OUTSIDE THESE HOURS SHALL BE APPROVED IN WRITING FROM THE CITY OF PATASKALA, AND AT THE APPROVAL OF THE ENGINEER. ALL EXISTING LANES SHALL BE OPEN TO TRAFFIC DURING DAY TIME HOURS WITH THE EXCEPTION OF MARKING PAVEMENT REPAIRS.

NO TRAFFIC SIGNAL WORK SHALL BEGIN AT THE INTERSECTION OF BROAD ST./SUMMIT RD. UNTIL COMPLETION OF THE PERMIT PROJECT (ESTIMATED COMPLETION DATE OF 12/1/24).

INTERIM COMPLETION DATE

ALL WORK ON THIS PROJECT SHALL BE COMPLETED BY 10/15/24, WITH THE EXCEPTION OF TRAFFIC SIGNAL WORK AT BROAD ST. AND SUMMIT RD. WHICH SHALL BE COMPLETED BY THE PROJECT COMPLETION DATE.

A DISINCENTiVE of $1,500 per day shall be assessed if the interim completion date stated above IS NOT MET.

BUTT JOINT

# A BUTT JOINT WILL BE REQUIRED AT THE LOCATIONS SPECIFIED BELOW

# AND PER STANDARD DRAWING BP-3.1 UNLESS OTHERWISE SHOWN IN

# THE PLANS.

# THE MINIMUM ASPHALT WEDGE LENGTH AT BUTT JOINTS SHALL BE 10’.

# THE GRINDING FOR BUTT JOINTS SHALL BE INCLUDED WITH ITEM 254, PAVEMENT PLANING, ASPHALT CONCRETE.

| LOC. | ROUTE | DESCRIPTION | SLM | ITEM 614 ASPHALT CONCRETE FOR MAINTAINING TRAFFIC CU. YD. |
| --- | --- | --- | --- | --- |
| 1A | S.R. 16 | BEGIN WORK | 0.00 | 2.0 |
|  |  | SUSPEND WORK | 0.99 | 1.0 |
|  |  | RESUME WORK | 1.20 | 1.0 |
|  |  | BRIDGE: LIC-16-0327 | 3.27 | 2.0 |
|  |  | TOTAL |  | 6.0 |
| 1B | S.R. 16 | BRIDGE: LIC-16-1415 | 14.15 | 2.0 |
|  | S.R. 16I | END WORK | 0.19 | 1.0 |
|  |  | TOTAL |  | 3.0 |

NOTIFICATION OF ROAD CLOSURE OR RESTRICTIONS

THE CONTRACTOR WILL ADVISE THE PROJECT ENGINEER A MINIMUM OF TWENTY-ONE (21) DAYS PRIOR TO THE FOLLOWING:  THE START OF CONSTRUCTION ACTIVITIES, LANE RESTRICTIONS, LANE CLOSURES, AND/OR ROAD CLOSURES.  THE PROJECT ENGINEER WILL FORWARD THIS INFORMATION TO THE FOLLOWING:

DISTRICT PUBLIC INFORMATION OFFICER (PIO) BY FAX AT (614) 887-4510 OR EMAIL AT D05.pio@dot.ohio.gov

DISTRICT PERMIT SECTION BY FAX AT (614) 887-4525 OR EMAIL AT brian.bosch@dot.ohio.gov

CENTRAL OFFICE SPECIAL HAUL PERMITS SECTION BY FAX AT (614) 728-4099 OR EMAIL AT hauling.permits@dot.ohio.gov

THE PIO WILL, IN TURN, NOTIFY THE PUBLIC, THE LOCAL EMERGENCY SERVICES, AFFECTED SCHOOLS AND BUSINESSES, AND ANY OTHER IMPACTED LOCAL PUBLIC AGENCY OF ANY OF THE ABOVE-MENTIONED ITEMS, VIA MEDIA SOURCES.

DAYTIME LANE CLOSURES FOR MARKING PAVEMENT REPAIRS

THE CONTRACTOR SHALL PROVIDE TRAFFIC CONTROL FOR DAYTIME LANE CLOSURES TO PROVIDE A SAFE AREA FOR ODOT PERSONAL TO MARK PAVEMENT REPAIRS AHEAD OF RESURFACING OPERATIONS. THE CLOSURE WILL BE A MOVING OPERATION AND SHALL ADHERE TO MT-99.20, AT THE APPROVAL OF THE ENGINEER. THE ESTIMATED COMPLETION TIME TO MARK THE PAVEMENT REPAIRS IS FIVE (5) 8-HOUR WORKDAYS. PAYMENT SHALL BE INCLUDED IN THE LUMP SUM BID FOR ITEM 614, MAINTAINING TRAFFIC.

ITEM 614, WORK ZONE PAVEMENT MARKINGS

THE CONTRACTOR SHALL PLACE ALL WORK ZONE PAVEMENT MARKINGS

IN ACCORDANCE WITH CMS 614.11 AND STANDARD DRAWING MT-99.20 UNLESS OTHERWISE DIRECTED BY THE ENGINEER. THE QUANTITIES BELOW ARE FOR PLACEMENT OF TEMPORARY MARKINGS ON THE SURFACE COURSE FOR BOTH THE CONCRETE AND ASPHALT SECTIONS OF ROADWAY.

ITEM 614, WORK ZONE CENTER LINE, CLASS III, 642 PAINT

Location 1A:  8.19 MILE

LOCATION 1B:  8.44 MILE

ITEM 614, WORK ZONE CHANNELIZING LINE, CLASS III, 642 PAINT

LOCATION 1A:  9,545 FT

Location 1B:  3,121 FT

ITEM 614, WORK ZONE STOP LINE, CLASS III, 642 PAINT

(S.R. 16 ONLY UNLESS SIGNALIZED INTERSECTION)

LOCATION 1A:  877 FT

Location 1B:  191 FT

# ITEM 614, WORK ZONE MARKING SIGN

THE CONTRACTOR SHALL PLACE ALL WORK ZONE MARKING SIGNS

IN ACCORDANCE WITH CMS SECTION 614.04, UNLESS OTHERWISE

DIRECTED BY THE ENGINEER.

# W8-H12a (NO EDGE LINES):

# LOCATION 1A:  15 EACH

# LOCATION 1B:  20 EACH

# R4-1 (DO NOT PASS):

# LOCATION 1A:  24 EACH

# LOCATION 1B:  23 EACH

# R4-2 (PASS WITH CARE):

# LOCATION 1A:  19 EACH

# LOCATION 1B:  12 EACH

ALL “NO EDGE LINES” SIGN LOCATIONS SHALL BE APPROVED BY

THE ENGINEER PRIOR TO INSTALLATION.

ANY revisions TO THE CENTER LINE PASSING/NO PASSING LOGS

SHALL BE REFLECTED IN THE WORK ZONE SIGNING.

IN ADDITION, THE CONTRACTOR SHALL ERECT A “GROOVED PAVEMENT” SIGN 250 FEET IN ADVANCE OF ANY SECTION OF ROADWAY WHERE TRAFFIC MUST TRAVEL ON A PLANED SURFACE. “GROOVED PAVEMENT” SIGNS SHALL BE INCLUDED FOR PAYMENT WITH THE LUMP SUM BID FOR ITEM 614 MAINTAINING TRAFFIC AS PER CMS SECTION 614.055.

# ITEM 614, WORK ZONE MARKING SIGN

Location 1A:  58 EACH

Location 1B:  55 EACH

ITEM 614, LAW ENFORCEMENT OFFICER WITH PATROL CAR FOR ASSISTANCE

Use of law enforcement officers (LEOs) by contractors other than the uses specified in this note will not be permitted at project cost.   LEOs should not be used where the Ohio Manual of Uniform Traffic Control Devices (OMUTCD) intends that flaggers be used. In addition to the requirements of C&MS 614 and the OMUTCD, a uniformed LEO with an official patrol car (car with top-mounted emergency flashing lights and complete markings of the appropriate law enforcement agency) should be provided for controlling traffic for the following tasks, as directed BY THE ENGINEER:

- For lane closures: during initial set-up periods, tear down periods, substantial shifts of a closure point or when new lane closure arrangements are Initiated.

- WHEN TRAFFIC NEEDS TO BE DIRECTED THROUGH AN ENERGIZED TRAFFIC SIGNAL CONTRARY TO THE SIGNAL DISPLAY (E.G., DIRECTING MOTORISTS THROUGH A RED LIGHT)

- night work, at approval of the engineer

In general, LEOs should be positioned at the point of lane restriction or road closure and to manually control traffic movements through intersections in work zones. LEOs should not forgo their traffic control responsibilities to apprehend motorists for routine traffic violations. However, if a motorist’s actions are considered to be reckless, then THE pursuit of the motorist is appropriate. The LEOs work at the direction of the Contractor.  The Contractor is responsible for securing the services of the LEOs and communicating the intentions of the plans with respect to duties of the LEOs. The Engineer shall have final control over the LEOs’ duties and PLACEMENT AND will resolve any issues that may arise between the two parties.

The LEO shall report in to the Contractor prior to the start of the shift to receive instructions regarding specific work assignments during his/her shift.  The LEO is expected to stay at the project site for the entire duration of his/her shift.  the leo shall report to the contractor at the end of his/her shift.  once the leo has completed the duties described above and still has time remaining on his/her shift, the leo may be asked to patrol through the work zone (with flasihg lights off) or be placed at a location to deter motorists from speeding.  should it be necessary to leave the project site, the leo shall notify the engineer.  The Contractor shall provide the LEO with a two-way communication device which shall be returned to the Contractor at the end of his/her shift.

Law enforcement officers (with patrol car) required by the traffic maintenance tasks above shall be paid for on a unit price (hourly) basis under Item 614, Law Enforcement Officer (With Patrol Car) for assistance. The hours paid shall include THE MINIMUM show-up time required by the law enforcement agency involved. Any additional costs (administrative or otherwise) incurred by the Contractor to obtain the services of a LEO are included with the  bid unit price for Item 614, Law enforcement Officer With Patrol Car FOR ASSISTANCE.

Item 614, Law Enforcement Officer With Patrol CaR FOR ASSISTANCE

location 1a:  500 hour

LOCATION 1B:  500 HOUR

ITEM 614, PORTABLE CHANGEABLE MESSAGE SIGN, AS PER PLAN

The Contractor shall furnish, install, maintain and remove, when no longer needed, A changeable message sign, on site, for the duration of the project. The sign shall be of a type shown on a list of approved PCMS units AVAILABLE ON THE Office of Materials Management WEB PAGE.  The list contains Class A and B units with minimum legibility distances of 650 FEET and 475 FEET, respectively.

Each sign shall be trailer-mounted and equipped with a functional dimming mechanism, to dim the sign during darkness, and a tamper and vandal proof enclosure.  Each sign shall be provided with appropriate training and operation instructions to enable on-site personnel to operate and troubleshoot the unit.  The sign shall also be capable of being powered by an electrical service drop from a local utility company. THE PCMS SHALL BE DELINEATED

IN ACCORDANCE WITH C&MS 614.03.

The probable PCMS locations and work limits for those locations are shown on sheet(s) of the plan. Placement, operation, maintenance and all activation of the signs by the Contractor shall be as directed by the Engineer. the PCMS shall be located in a highly visible position yet protected from traffic. The Contractor shall, at the direction of the Engineer, relocate the PCMS to improve visibility or accommodate changed conditions. When not in use, the PCMS shall be turned off. Additionally, when not in use for extended periods of time, the PCMS shall be turned, facing away from all traffic.

The Engineer shall be PROVIDED WITH access to each sign unit and shall be provided with appropriate training and operation instructions to enable ODOT personnel to operate and troubleshoot the unit, and to revise sign messages, if necessary.

The Contractor shall implement a system whereby changeable messages will be implemented within 2 hours following telephone notification from the Project Engineer to a designated phone.

All messages to be displayed on the sign will be provided by the Engineer. A list of all required pre-programmed messages will be given to the Contractor at the project preconstruction conference. The sign shall have the capability to store up to 99 messages. Message memory or pre-programmed displays shall not be lost as a result of power failures to the on-board computer.  The sign legend shall be capable of being changed in the field.  three-line presentation formats with up to six message phases shall be supported. PCMS format shall permit the complete message for each phase to be read at least twice.

The PCMS shall contain an accurate clock and programming

logic which will allow the sign to be activated, deactivated or messages changed automatically at different times of the day

for different days of the week.

COOPERATION BETWEEN CONTRACTORS

A PERMIT PROJECT AT THE INTERSECTION OF BROAD ST. AND SUMMIT RD. WILL BE CONSTRUCTED CONCURRENTLY WITH THIS PROJECT. IT IS IMPERATIVE THAT THE CONTRACTORS COOPERATE FULLY WITH EACH OTHER AS OUTLINED IN SECTION 105.08 OF THE CMS MANUAL. ALL MAINTENANCE OF TRAFFIC SHALL BE COORDINATED BETWEEN PROJECTS AND NOT CONFLICT WITH ONE ANOTHER.

ITEM 614, PORTABLE CHANGEABLE MESSAGE SIGN, AS PER PLAN (Cont’d.)

The PCMS unit shall be maintained in good working order by the Contractor in accordance with the provisions of C&MS 614.07.

The Contractor shall, prior to activating the unit, make arrangements, with an authorized service agent for the PCMS,

to assure prompt service in the event of failure.  Any failure shall not result in the sign being out of service for more than 12 hours, including weekends.  Failure to comply may result in an order to stop work and open all traffic lanes and/or in the Department taking appropriate action to safely control traffic.  The entire cost to control traffic, accrued by the Department due to the Contractor’s noncompliance, will be deducted from moneys due, or to become due the Contractor on his contract.

The Contractor shall be responsible for 24-hour-per-day operation and maintenance of these signs on the project for

the duration of the phases when the plan requires their use.

Payment for the ABOVE-DESCRIBED item shall be at the contract unit price. Payment shall include all labor, materials, equipment, fuels, lubricating oils, software, HARDWARE, and incidentals to perform the ABOVE-DESCRIBED work.

a total of 2 pcms MAY be required for this project.

2 PCMS X 6 mONTH = 12 SNMT

Item 614, Portable Changeable Message Sign, AS PER PLAN

LOCATION 1A:  6 SNMT

LOCATION 1B:  6 SNMT

FLOODLIGHTING

FLOODLIGHTING OF THE WORK SITE FOR OPERATIONS CONDUCTED DURING NIGHTTIME PERIODS SHALL BE ACCOMPLISHED SO THAT THE LIGHTS DO NOT CAUSE GLARE TO THE DRIVERS ON THE ROADWAY. TO ENSURE THE ADEQUACY OF THE FLOODLIGHT PLACEMENT, THE CONTRACTOR AND THE ENGINEER SHALL DRIVE THROUGH THE WORK SITE EACH NIGHT WHEN THE LIGHTING IS IN PLACE AND OPERATIVE PRIOR TO COMMENCING ANY WORK. IF GLARE IS DETECTED, THE LIGHT PLACEMENT AND SHIELDING SHALL BE ADJUSTED TO THE SATISFACTION OF THE ENGINEER BEFORE WORK PROCEEDS. PAYMENT FOR ALL LABOR, EQUIPMENT AND MATERIALS SHALL BE INCLUDED IN THE LUMP SUM BID FOR ITEM 614, MAINTAINING TRAFFIC.

DROP-OFFS IN WORK ZONES

DROP-OFFS THAT DEVELOP DURING CONSTRUCTION OPERATIONS AND THAT ARE NOT OTHERWISE PROVIDED FOR IN THE PLANS SHALL BE TREATED AS SHOWN ON STANDARD DRAWING MT-101.90. WHERE THE PLANS DO NOT PROVIDE SPECIFIC ITEMS FOR LABOR, EQUIPMENT, OR MATERIALS TO IMPLEMENT THE DROP-OFF TREATMENTS SPECIFIED, THEY SHALL BE INCLUDED FOR PAYMENT IN THE LUMP SUM BID FOR ITEM 614, MAINTAINING TRAFFIC.