PID 114956, MER-BH-FY25: QUANTITY CALCULATIONS

Calculated by:  Lawton Gerlinger, P.E., Date: 10/10/23

Checked by: Dan Grilliot, P.E., Date: x/xx/xx

Revised by:

Roadway, Pavement, Traffic Control (All 3 structures itemized per item)

- Item 202 Guardrail Removed

  - MER-219-0983:

  - MER-716-0015: NW quad 75 FT, NE quad 56.25 FT, SE quad 87.5 FT, SW quad 62.50’ = 282 FT

- Item 202 Removal Misc.: Removal of Bridge ID sign and re-erection

  - MER-716-0015: 2 signs total NW and SE corners.

- Item 254 Pavement Planing, Asphalt Concrete, Variable.

  - MER-49-2289: Areas from ORD (Rear and forward)

    - 866 SF + 877 SF = 1743 SF / 9 = 194 SY

  - MER-716-0015: areas from ORD (Rear and forward)

    - 3003 SF + 3005 SF = 6008 SF / 9 = 668 SY

- Item 407 Tack Coat (0.08 GAL/SY and 0.06 GAL/SY)

  - MER-49-2289

    - Total area: 5480 SF. 1st layer application rate of 0.08. 2nd layer application rate of 0.06.

    - 5480 SF/9= 609 SY x 0.08 + 609 SY x 0.06 = 85.26 or 86 GAL.

  - MER-219-0983

    - Total area: 3947 SF. 1st layer application rate of 0.08. 2nd layer application rate of 0.06.

    - 3947 SF/9 = 439 SY x 0.08 + 439 SY x 0.06 x 2 applications = 88 GAL

  - MER-716-0015

    - Total area: Rear- 3738 SF + 3740 SF = 7478 SF / 9 = 831 SY x 0.08 GAL/SY = 67 GAL

- Item 441 Asphalt Concrete Surface Course, Type 1, (448), PG70-22M, 1½”

  - MER-49-2289

    - 5480 SF x 1.5”/12 = 685 CF/27 = 25.37 or 26 CY

  - MER-219-0983

    - 1898 SF + 2049 SF x 2”/12 = 658 CF/27 = 25 CY

  - MER-716-0015

    - 7478 SF x (1.5/12) = 935 CF / 27 = 35 CY

- Item 441 Asphalt Concrete Intermediate Course, Type 2, (449), Variable

  - MER-49-2289

    - Approx. length of variable is 35 feet, x 2 sides. 70’ x 40 feet x Varies (1¾” to 0”) = 233.33 CF/27 = 8.64 or 9 CY

  - MER-219-0983

    - Approx. length of variable is 35 feet, x 2 sides. 70’ x 40 feet x Varies (4” to 0”) = 467 CF/27 = 18 CY

- Item 441 Asphalt Concrete Intermediate Course, Type 1, (448), 1½”

  - MER-49-2289

    - [43.42’ + 30’ + 30’] x 40’ = 4136 SF x 1.5/12 = 517 CF/27 = 19 CY

- Item 606 Guardrail, Type MGS

  - MER-716

    - GR-1: 12.5 FT

    - GR-2: 0 FT

    - GR-3: 12.5 FT

    - GR-4: 31.25 FT

      - Transition GR to tie bridge terminal assemblies to end anchors or existing guardrail.

  - MER-219

    - GR-1: 120 FT total – 25 ft bta – 50 foot type e = 45 ft/6.25 = 7 x 6.25 = 43.75 FT

    - GR-2: 56.25 FT total – 25 ft bta = 31.25 FT

    - GR-3: 106.25 FT total – 25 ft bta – 50 foot type e = 31.25 FT

    - GR-4: 37.5 FT total – 25 ft bta = 12.5 FT

- Item 606 Anchor Assembly, MGS, Type T

  - MER-716

    - GR-1: 1 EACH

    - GR-2: 0

    - GR-3: 1 EACH

    - GR-4: 1 EACH

  - MER-219

    - GR-1: 1 EACH

    - GR-2: 1 EACH

    - GR-3: 0

    - GR-4: 1 EACH

- Item 606 Anchor Assembly, MGS Type E, mash 2016

  - MER-716

    - GR-2: 1 EACH

  - MER-219

    - GR-3: 1 EACH

- Item 606 MGS Bridge Terminal Assembly, Type TST-2

  - GR-1: 1 EACH

  - GR-2: 1 EACH

  - GR-3: 1 EACH

  - GR-4: 1 EACH

    - Each GR run needs this bridge terminal assembly. Pay length is almost 27 Feet per SCD.

- Item 606 Type 4 bridge terminal assembly

  - MER-219-0983

    - Type 4 needed because of keeping DBR rail and retrofitting it. 4 EACH

- Item 621 RPM Removed

  - MER-49-2289: Remove 2 RPM within paving limits. Do not reinstall?

- Item 626 Barrier Reflector, Type 2 Bi-Directional

  - MER-716

    - Need reflectors on all new GR. 215 feet North run and 225 feet South. Use 4 reflectors each run for 8 Total. This includes bridge TST rail.

  - MER-219-

    - Need reflectors on all new GR. 300 feet North GR run and 170 feet South. 5 North run and 3 South run. CMS 626.03 Layout requires up to 100 foot spacing.

- Item 630 – Sign, Flat sheet, As per plan

  - MER-49-2289: NW corner GR needs a new sign. 1 EACH

- Item 644 Edge Line

  - MER-49-2289: 150’ x 2 edge lines = 300’/5280 = 0.06 MILE

- Item 644 Centerline dashed

  - MER-49-2289: 150’/5280 = 0.03 MILE

- Item 646 Edge Line

  - MER-716-0015: 386’ x 2 edge lines = 772’/5280 = .15 MILE

  - MER-219-0983: 176.51 x 2 edge lines = 353.02’/5280 = 0.07 MILE

- Item 646 Centerline dashed

  - MER-716-0015: 386’ / 5280 = 0.08 MILE

  - MER-219-0983: 176.51’/5280 = 0.04 MILE (solid, dashed)

- Item 646 Stop Line

  - MER-219-0983: 12 feet

Erosion Control

- Item 832 Erosion Control

  - Plan split 1: 2000 (EA)

  - Plan split 2: 4000 (EA)

- Item 659 Seeding and Mulching

  - Plan split 1: MER-49

    - Not expecting any disturbance

  - Plan split 2: MER-219 and MER-716

    - MER-219 not expecting disturbance

    - MER-716: 8’ wide x 80’ long x 4 quads = 2560/9 = 285 (SY)

- Item 659 Commercial Fertilizer

  - Plan split 1: MER-49

    - 0 (TON)

  - Plan split 2: MER-219 and MER-716

    - 0.04 (TON)

- Item 659 Water

  - Plan split 1: MER-49

    - 0 (MGAL)

  - Plan split 2: MER-219 and MER-716

    - 2 (MGAL)

Structures: MER-49-2289

- Item 202 Wearing course removed

  - Bridge limits and width

    - 43.42’ x 40’ = 1736.8 SF/9 = 193 SY

- Item 512 Type 3 Waterproofing, APP

  - Bridge limits + 4 feet overlapping on approach slabs

    - [43.42’ + 4’] x 40’ width = 1897 SF/9 =210.76 or 211 SY

- Item 518 Steel Drip Strip

  - [43.42 FT per side + (7 posts x 1.5 FT)] x 2 sides = 107.84 or 108 FT

Structures: MER-219-0983

- Item 202 Portions of Structure removed, Over 20-foot span, as per plan

  - Lump Sum (LS)

- Item 202 Bridge Railing Removed for Reuse, APP

  - South rail [68.75’ + 12.5’] + [75’ + 12.5’] = 168.75’ or 169 (FT) total.

- Item 509 Reinforcing steel

  - 53 long bars #5 bars need 3 bars @ 26.75’ :

  - 2 dowels per beam See sheet xx for details. xxxx (LB) for box beam dowel bars.

  - 114 bars @ 8” spa. X 2 bars @ 20.33/cos(28) each :

- Item 510 Dowel holes with nonshrink, nonmetallic grout

  - See sheet xx for details. 1440 (EA) dowel holes in existing concrete box beams. Backwall dowels are

- Item 511 Class QC1 concrete, abutment

  - Concrete is for approach slab side pour: 1.25’ x 2.0’ x 40/cos(28 deg) x 2 ends = 227 (CF)/27 = 9 (CY)

- Item 511 Class QC2 Concrete, Bridge Deck

  - 6” thickness(0.5’)x 40’ wide x [76.51’-3.5’] + [5/12 x 40/cos(28 deg) x 2 sides] = 1498 (CF)/27 = 56 (CY)

- Item 512 Concrete Repair by Epoxy Injection

  - Looking at photos: Rear Abutment- 3 FT, Pier (West side)- 9 FT + 6 FT +12 FT + 12 FT, Pier (East side)- 20 FT + 20 FT + 3 FT + 3 Ft + 4 FT, Forward Abutment- 6 FT + 6 FT + 20 FT + 5 FT + 4 FT + 5 FT + 3 FT + 4 FT.

    - Grand Total: 145 (FT)

- Item 516 Structural Expansion Joint Including Elastomeric Compression Seal

  - 40/cos(28 deg) width x 2 sides = 90 (FT)

- Item 516 2” deep joint sealer, APP

  - 40/cos(28 deg) width x 2 sides = 90 (FT)

- Item 517 DBR railing retrofit

  - 87.5’ North side and 81.25’ South side = 169 (FT) total.

- Item 518 Steel Drip Strip

  - [Bridge limits: 76.51’ + 1.5’x12 posts for post covering] + [Bridge limits: 76.51’ + 1.5’x11 posts for post covering] = 188 (FT)

- Item 519 Concrete patching

  - Pier diaphragm: 2’ x 1’ = 2 SF total

Structures: MER-716-0015

- Item 202 Portions of Structure Removed, Over 20 foot Span, APP

  - Lump Sum (LS)

- Item 202 Wearing course removed

  - Approach slabs and wedges from ORD, rear: 739 SF, forward: 737 SF. Deck area 86.10’ x 36’ = 3100 SF

    - Total = 739+737+3100 = 4576 SF/9 = 509 SY

- Item 202 Bridge Railing Removed

  - [81.25’ per side x 2] + 4[2.46’ pay limits past last post] = 173 (FT) total.

- Item 503 Unclassified Excavation

  - Lump Sum (LS)

- Item 503 Cofferdams and Excavation Bracing

  - Lump Sum (LS)

- Item 507 Pile Encasements

  - 12 piles, approx. 14’ tall = 168(FT)

- Item 509 Reinforcing steel, replacement of existing reinforcing steel

  - Quantity of 50 (LB) in the event of existing steel that needs replaced.

- Item 509 Reinforcing steel

  - See sheet xx for details. 5544 (LB)

- Item 511 Class QC2 concrete, bridge deck

  - Deck edges: 4.32 SF from ORD x bridge limits of 86.10’ x 2 sides = 744 CF/27 = 28 (CY)

- Item 511 Concrete, MISC.: Embedded Galvanic Anode (EGA)

  - Deck edges: 86.10’/1.5’ Max spa. = 57.4. Use 56 anodes per side x 2 sides = 112 EACH

- Item 514 Zinc rich primer

  - Span 1: 20’ + 8’ + 4’ 1 foot width, Centerspan: 20’x2’, Span 3: 10’x1’ and 7’x3’ = 103 (SF)

- Item 516 2” deep joint sealer, APP

  - 36/cos(20 deg) deck width x 2 sides = 77 (FT)

- Item 517 TST (Three Steel Tube) railing

  - [81.25’ per side x 2] + 4[2.46’ pay limits past last post] = 173 (FT) total.

- Item 518 Steel Drip Strip

  - [Bridge limits: 86.10’+ 1.5’x14 posts for post covering] x 2 sides = 215 (FT)

- Item 601 Crushed aggregate slope protection

  - 2 SY around each wingwall (4) = 8 SY

- Item 848 Superplasticized Dense Concrete overlay using hydrodemolition, 2”

  - 86.10’x36’ = 3100(SF)/9 = 345 (SY)

- Item 848 Surface preparation using hydrodemolition, APP

  - 86.10’x30’ = 2583(SF)/9 =287 (SY)

- Item 848 Superplasticized Dense Concrete overlay (Variable thickness), material only

  - From pictures deck edges are bad, asphalt quality looks ok… and bottom of slab looks rough in spots. Estimate 40% for variable area.

    - 2755*.40 =1081 (SF) Assumed 50% measured unsound

    - BDM T403-3: %variable thickness 50% to 67.5%

    - BDM Variable Area = 0.675*2755 = 1860 (SF)

    - BDM factors (none applied) First gen overlay and no winter factor added.

    - District factors (none applied) leaves variable area at 1860 (SF)

    - BDM variable depth, 2” Plan variable depth = 1.25” +9/8” +6/8” +.75” = 3.875” Use 3” for calc.

    - Variable volume: 3/12*1860 (SF) = 465(CF)/27 = 18 (CY)

- Item 848 Hand Chipping

  - BDM 403.4.1 HC area = 10% of Variable area

    - 0.1*1860 (SF) = 186(SF)/9 = 21 (SY)

- Item 848 Full Depth: Not anticipating full depth repairs on a slab.

- Item 848 Test Slab

  - Lump Sum (LS)

Maintenance of Traffic

- Item 614 Detour signing

  - Lump Sum (LS)

Incidentals

- Item 614-Maintaining Traffic (LS)

- Item 619-Field Office, Type B 3 (MNTH) Ask construction?

- Item 623-Construction Layout Stakes and Surveying (LS)

- Item 624-Mobilization (LS)

END OF CALCULATIONS