Bridge Load Rating Report

Bridge: MOT-70-0603

Brookville-Salem Pike over IR-70

SFN: 5704987

03/11/16

Chris Jackman, P.E.			 Date

Senior Bridge Engineer

8415 Pulsar Place

Suite 300

Columbus, OH 43240

##### TABLE OF CONTENTS

1.	BRIDGE LOAD RATING REPORT SUMMARY	1

2.	INPUT FILE	2

3.	OUTPUT FILE	27

4.	BRR SCHEMATICS	29

5.	DESIGN DRAWINGS	31

# INPUT FILE

Username:	brr

Date:	Tuesday, March 08, 2016 10:30:18

Bridge ID   5704987   MOT-70-0603

NBI Structure ID (8): 5704987

Description: FOUR SPAN (76', 108'-6", 108'-6", 76') EXISTING STEEL PLATE GIRDER WITH NEW COMPOSITE DECK (8.5").  OUT TO OUT WIDTH 35'-4", SBR-1-13 PARAPETS (32'-0" TOE TO TOE).  50.125 DEGREE RIGHT FORWARD SKEW WITH TANGENT ALIGNMENT.  PROPOSED REHABILITATION 2018.

Description

Location:	0.51 MI. W OF JCT. SB-SR-

Total Length:	376.02  (ft)

Facility Carried:	BROOKVILLE-SALEM P

Route Number:	070

Feature Intersected:	IR-70

Mi Post:	6.03  (mi)

Units:	US Customary

Year Built:	1964

Recent ADTT:	204

District:

County:

Owner:	State Highway Agency

National Highway System:	0 Not on NHS

Functional Class:	07 Rural Mjr Collector

Global Reference Point

X Coordinate:	0.000  (ft)

Y Coordinate:	0.000  (ft)

Elevation:	  (ft)

Longitude:	  (Degrees)

Latitude:	  (Degrees)

Materials

Structural Steel

Name:	Grade 50

Description:	AASHTO M270 Grade 50

Specified minimum yield strength (Fy):	50.000  (ksi)

Specified minimum tensile strength (Fu):	65.000  (ksi)

Coefficient of thermal expansion:	0.0000065000  (1/F)

Density:	0.4900  (kcf)

Modulus of elasticity (E):	29000.00  (ksi)

Name:	1936  to 1963

Description:	Built 1936 to 1963 - steel unknown

Specified minimum yield strength (Fy):	33.000  (ksi)

Specified minimum tensile strength (Fu):	  (ksi)

Coefficient of thermal expansion:	0.0000065000  (1/F)

Density:	0.4900  (kcf)

Modulus of elasticity (E):	29000.00  (ksi)

Concrete

Name:	CLASS QC2

Description:	CLASS QC2

Specified compressive strength at 28 days (f'c):	4.500  (ksi)

Initial specified compressive strength (f'ci):	  (ksi)

Coefficient of thermal expansion:	0.0000060000  (1/F)

Density (for dead loads):	0.150  (kcf)

Density (for modulus of elasticity):	0.145  (kcf)

Modulus of elasticity (Ec):	3865.20  (ksi)

Poisson's ratio:	0.200

Modulus of rupture:	0.509  (ksi)

Shear factor:	1.000

Composition of concrete:	Normal

Initial modulus of elasticity (Eci):	  (ksi)

Reinforcing Steel

Name:	Grade 60

Description:	60 ksi reinforcing steel

Specified yield strength (Fy):	60.000  (ksi)

Modulus of elasticity (Es):	29000.00  (ksi)

Ultimate strength (Fu):	90.000  (ksi)

Type:	Epoxy

No prestressing strand materials.

No timber materials.

Beam Shapes

Steel Shapes

Steel Angles

Name:	L 3x3x0.3125

Description:	L 3x3x0.3125 Imported from AISC Tables (1994)

Angle size 1:	3.0000  (in)

Angle size 2:	3.0000  (in)

Thickness:	0.3125  (in)

k:	  (in)

Xyy:	0.8650  (in)

Yxx:	0.8650  (in)

tan(tetha):

Cross sectional Area (A):	1.780  (in^2)

Nominal load:	6.100  (lb/ft)

Moment of Inertia Ixx:	1.510  (in^4)

Moment of Inertia Iyy:	1.510  (in^4)

Rzz:	0.5890  (in)

No steel channels.

No steel I shapes.

No steel structural tee shapes.

Prestressed Shapes

No prestressed shapes.

Timber Shapes

No timber shapes.

Appurtenances

Parapets

Name:	SBR-1-13

Description:	Single Slope 42" Parapet

X1:	10.0000  (in)

X2:	8.0000  (in)

X3:	0.0000  (in)

Y1:	0.0000  (in)

Y2:	42.0000  (in)

Y3:	0.0000  (in)

Y4:	0.0000  (in)

Distance to Center of Gravity of Load:	  (in)

Additional Load:	  (kip/ft)

Concrete Density:	0.1500  (kcf)

No concrete medians.

No concrete generics.

No steel railings.

Impact

Standard Impact Factor

Type:	Standard - AASHTO

LRFD Dynamic Load Allowance

Fatigue and fracture limit states:	15.0  (%)

All other limit states:	33.0  (%)

Factors

No LFD Factors specified.

No LRFD Factors specified.

Bridge Alternatives   MOT-70-0603

Reference Line

Reference Line Length:	  (ft)

Starting Station:	  (ft)

Bearing:	N 90^ 0' 0.00" E

Global Positioning

Distance:	0.000  (ft)

Offset:	0.000  (ft)

Elevation:	  (ft)

Structures

Name:	MOT-70-0603

Description:	BROOKVILLE-SALEM PIKE SPANS 1 THOUGH 4

Structure Alternatives

Name:	MOT-70-0603

Description:

Superstructure Definition:	BROOKVILLE-SALEM SPANS 1 THRU 4

Superstructure Definition   BROOKVILLE-SALEM SPANS 1 THRU 4

Definition

Units:	US Customary

Number of spans:	4

Number of girders:	5

Length

Span	(ft)

1	76.0000

2	108.5000

3	108.5000

4	76.0000

Frame Structure Simplified Definition:

Support	Frame Connection

1

2

3

4

5

Girder Spacing Display Type:	Perpendicular

Average Humidity:	  (%)

Analysis

Default Library Factors

Factor Override

Analysis Module

Analysis Method:	ASD

Analysis Module:

Analysis Module Component:

Properties:

Analysis Method:	LFD

Analysis Module:

Analysis Module Component:

Properties:

Analysis Method:	LRFD

Analysis Module:

Analysis Module Component:

Properties:

Analysis Method:	LRFR

Analysis Module:

Analysis Module Component:

Properties:

Analysis Method:	Distribution Factors

Analysis Module:

Analysis Module Component:

Properties:

Default rating method:	LFD

Impact

Standard Impact Factor

Type:	Standard - AASHTO

LRFD Dynamic Load Allowance

Fatigue and fracture limit states:	15.0  (%)

All other limit states:	33.0  (%)

Structure Framing Plan Details

Layout

Skew

Support	(Degrees)

1	-50.1250

2	-50.1250

3	-50.1250

4	-50.1250

5	-50.1250

Girder Spacing Orientation:	Perpendicular

Girder	Girder Spacing

Bay	Start	End

(ft)	(ft)

1	7.5000	7.5000

2	7.5000	7.5000

3	7.5000	7.5000

4	7.5000	7.5000

Diaphragms

Girder Bay 1

Distance	Distance	Diaphragm	Number of	Diaphragm

Left Girder	Right Girder	Spacing	Spaces	Weight

(ft)	(ft)	(ft)		(kip)

0.00	0.00	0.00	1	0.2000

13.08	4.11	13.08	14	0.1500

13.08	4.11	0.00	1	0.1500

196.25	187.27	12.42	1	0.1500

208.67	199.69	13.08	12	0.1500

369.00	369.00	0.00	1	0.2000

Girder Bay 2

Distance	Distance	Diaphragm	Number of	Diaphragm

Left Girder	Right Girder	Spacing	Spaces	Weight

(ft)	(ft)	(ft)		(kip)

0.00	0.00	0.00	1	0.2000

17.19	8.21	0.00	1	0.1500

17.19	8.21	13.08	13	0.1500

187.27	178.29	12.42	1	0.1500

199.69	190.71	13.08	12	0.1500

369.00	369.00	0.00	1	0.2000

Girder Bay 3

Distance	Distance	Diaphragm	Number of	Diaphragm

Left Girder	Right Girder	Spacing	Spaces	Weight

(ft)	(ft)	(ft)		(kip)

0.00	0.00	0.00	1	0.2000

21.29	12.31	0.00	1	0.1500

21.29	12.31	13.08	12	0.1500

178.29	169.31	12.42	1	0.1500

190.71	181.73	13.08	13	0.1500

369.00	369.00	0.00	1	0.2000

Girder Bay 4

Distance	Distance	Diaphragm	Number of	Diaphragm

Left Girder	Right Girder	Spacing	Spaces	Weight

(ft)	(ft)	(ft)		(kip)

0.00	0.00	0.00	1	0.2000

12.31	3.37	13.08	12	0.1500

12.31	3.37	0.00	1	0.1500

169.31	160.37	12.42	1	0.1500

181.73	172.79	13.08	14	0.1500

369.00	369.00	0.00	1	0.2000

Structure Typical Section

Deck

Left start width:	17.67  (ft)

Left end width:	17.67  (ft)

Right start width:	17.67  (ft)

Right end width:	17.67  (ft)

Left start overhang:	2.67  (ft)

Left end overhang:	2.67  (ft)

Deck (Cont'd)

Deck concrete:	CLASS QC2

Total deck thickness:	8.5000  (in)

Deck crack control parameter:	  (kip/in)

Sustained modular ratio factor:

Parapet

Measure	Measured	Distance	Distance	Front Face

Name	Load Case	To	From	At Start	At End	Orientation

SBR-1-13	DC2	Front	Left Ed...	1.67	1.67	Right

SBR-1-13	DC2	Front	Right E...	1.67	1.67	Left

Lane Position

Offset Left Start:	-16.00  (ft)

Offset Left End:	-16.00  (ft)

Offset Right Start:	16.00  (ft)

Offset Right End:	16.00  (ft)

Wearing Surface

Wearing surface material:

Description:

Wearing surface thickness:	  (in)

Wearing surface density:	  (pcf)

Load case:

Load Case Description

Load Case Name	Description	Stage	Type	Time

(Days)

DC1		Non-composite (Sta...	D,DC

DC2		Composite (long te...	D,DC

DW		Composite (long te...	D,DW

Superstructure Loads

Uniform Temperature

Load Case:

Temperature rise:	  (F )

Temperature fall:	  (F )

Gradient Temperature

Load Case:

Temperature value T1:	  (F )

Temperature value T2:	  (F )

Temperature value T3:	  (F )

Wind

Load Case:

Design Pressure:	  (psf)

Wind Load Path:	Truss action

DL Distribution

Stage 1 Dead Load Distribution:	Tributary Area

Stage 2 Dead Load Distribution:	Uniformly to All Girders

Bolt Definitions

Name:	A325

Description:

Library Designation:	ASTM A 307

Blot threads excluded form shear plane:	FALSE

Bolt diameter:	0.8750  (in)

Hole diameter:	0.9375  (in)

Connection Type:	Bearing

Hole Size:	Standard

Load Direction:	Any Direction

Surface Class:	Class A

ASD Allowable shear stress:	11.000  (ksi)

ASD Nominal Slip Resistance:	  (ksi)

LFD Allowable shear stress:	18.000  (ksi)

LFD Design Slip Resistance:	  (ksi)

LRFD Minimum tensile strength, Fub:	60.000  (ksi)

LRFD Kh:	1.000

LRFD Ks:	0.330

Shear Connector Definitions

Name:	SHEAR STUD

Connector Type:	Stud

Stud Diameter:	0.8750  (in)

Stud Height:	7.0000  (in)

LRFD Steel Minimum Tensile Strength:	  (ksi)

Stiffener Definitions

Transverse Stiffeners

Name:	INTERMEDIATE STIFFENERS

Stiffener number:	Pair

Plate Width:	6.0000  (in)

Plate Thickness:	0.3750  (in)

Material:	1936  to 1963

Top Gap:	0.0000  (in)

Bottom Gap:	0.0000  (in)

Top Weld:

Web Weld:

Bottom Weld:

Bearing Stiffeners

Name:	ABUTMENT STIFFENER

Plate Width:	6.5000  (in)

Plate Thickness:	1.0000  (in)

Material:	Grade 50

Top outside clip length:	0.0000  (in)

Top inside horizontal clip length:	0.2500  (in)

Top inside vertical clip length:	0.2500  (in)

Bottom outside clip length:	0.2500  (in)

Bottom inside horizontal clip length:	0.2500  (in)

Bottom inside vertical clip length:	0.0000  (in)

Top Weld:

Web Weld:

Bottom Weld:

Name:	PIER BEARING STIFFENER

Plate Width:	6.0000  (in)

Plate Thickness:	0.7500  (in)

Material:	Grade 50

Top outside clip length:	0.0000  (in)

Top inside horizontal clip length:	0.2500  (in)

Top inside vertical clip length:	0.2500  (in)

Bottom outside clip length:	0.2500  (in)

Bottom inside horizontal clip length:	0.2500  (in)

Bottom inside vertical clip length:	0.0000  (in)

Top Weld:

Web Weld:

Bottom Weld:

No prestress stress limits.

No prestress properties.

No vertical shear reinforcement definitions.

No horizontal shear reinforcement definitions.

Member   Girder 1

Link with: None

Description: Girder member generated using the design wizard

Existing:	Girder 1 - Member alternative generated using the design wizard

Current:	Girder 1 - Member alternative generated using the design wizard

Number of Spans:	4

Span	Span Length

Number	(ft)

1	76.000000

2	108.500000

3	108.500000

4	76.000000

Support	Frame Connection

1

2

3

4

5

Pedestrian load:	  (lb/ft)

Member Loads

Member Loads - Settlement

Support	Horizontal	Vertical	Rotational	Load Case Name

Number	(in)	(in)	(Radians)

1

2

3

4

5

Support Constraints

General

Support	Support

Number	Type	X Translation	Y Translation	Z Rotation

1	Pinned	Fixed	Fixed	Free

2	Roller	Free	Fixed	Free

3	Roller	Free	Fixed	Free

4	Roller	Free	Fixed	Free

5	Roller	Free	Fixed	Free

Elastic

Support	X Translation	Y Translation	Z Rotation	Override Computed

Number	(kip/ft)	(kip/ft)	(kip-in/rad)	Z Rotation

1

2

3

4

5

Member Alternative   Girder 1

Description: Member alternative generated using the design wizard

Description

Material Type:	Steel

Girder Type:	Plate

Member units:	US Customary

Girder property input method:	Schedule based

Left end X:	  (in)

Right end X:	  (in)

Additional Self Load:	  (kip/ft)

Additional Self Load %:	  (%)

Analysis Module

Analysis Method:	ASD

Analysis Module:

Analysis Module Component:

Properties:

Analysis Method:	LFD

Analysis Module:	AASHTO LFD

Analysis Module Component:

Properties:

Analysis Method:	LRFD

Analysis Module:	AASHTO LRFD

Analysis Module Component:

Properties:

Analysis Method:	LRFR

Analysis Module:

Analysis Module Component:

Properties:

Analysis Method:	Distribution Factors

Analysis Module:

Analysis Module Component:

Properties:

Default rating method:	LFD

Factors

Factor Override

LRFD:

LFD:

ASD Factors

Inventory	Operating

Structural steel

Concrete

PS Concrete Comp.

PS Concrete Tens.

PS Moment Cap.

Reinforcement

Bearing Stiffener

Stirrup

Timber	NA

Default Materials

Structural steel:	1936  to 1963

Deck concrete:	CLASS QC2

Deck reinforcement:	Grade 60

Welds:

Bolts:

Impact

Standard Impact Factor

Type:	Standard - AASHTO

LRFD Dynamic Load Allowance

Fatigue and fracture limit states:	15.0  (%)

All other limit states:	33.0  (%)

Live Load Distribution

Standard

D  i  s  t  r  i  b  u  t  i  o  n     F  a  c  t  o  r  (Wheels)

Lanes		Shear at

Loaded	Shear	Supports	Moment	Deflection

1 Lane	1.277	0.933	1.277	0.400

Multi-Lane	1.277	0.933	1.277	0.800

Girder Profile

Web

Begin	Depth	End

Depth	Vary	Depth	Thickness	Distance	Length	Material

(in)		(in)	(in)	(ft)	(ft)

53.0000	None	53.0000	0.3750	0.00	369.00	1936  to ...

Top Flange

Begin	End

Width	Width	Thickness	Distance	Length	Material

(in)	(in)	(in)	(ft)	(ft)

14.0000	14.0000	0.6875	0.00	59.75	1936  to ...

14.0000	14.0000	1.5000	59.75	30.25	1936  to ...

14.0000	14.0000	1.0000	90.00	76.50	1936  to ...

14.0000	14.0000	1.8750	166.50	36.00	1936  to ...

14.0000	14.0000	1.0000	202.50	76.50	1936  to ...

14.0000	14.0000	1.5000	279.00	30.25	1936  to ...

14.0000	14.0000	0.6875	309.25	59.75	1936  to ...

Bottom Flange

Begin	End

Width	Width	Thickness	Distance	Length	Material

(in)	(in)	(in)	(ft)	(ft)

14.0000	14.0000	0.6875	0.00	59.75	1936  to ...

14.0000	14.0000	1.5000	59.75	30.25	1936  to ...

14.0000	14.0000	1.0000	90.00	76.50	1936  to ...

14.0000	14.0000	1.8750	166.50	36.00	1936  to ...

14.0000	14.0000	1.0000	202.50	76.50	1936  to ...

14.0000	14.0000	1.5000	279.00	30.25	1936  to ...

14.0000	14.0000	0.6875	309.25	59.75	1936  to ...

Deck Profile

Deck Concrete

Total	Structural	Effective	Effective

Material	Distance	Length	Thickness	Thickness	Width (Std)	Width (LRFD)	n

(ft)	(ft)	(in)	(in)	(in)	(in)

CLASS QC2	0.00	369.00		7.5000	76.9920	76.9920	7.50...

Reinforcement

Start		Bar	Bar

Material	Distance	Length	Count	Size	Distance	Row

(ft)	(ft)			(in)

Grade 60	0.00	369.00	9.00	4	2.7500	Top of...

Grade 60	0.00	369.00	9.00	5	1.3125	Bottom...

Grade 60	54.00	45.00	8.00	6	1.3125	Bottom...

Grade 60	157.50	53.00	8.00	6	1.3125	Bottom...

Grade 60	271.00	45.00	8.00	6	1.3125	Bottom...

Shear Connectors

Start		Connector	Number	Number of	Transverse

Distance	Length	Name	per Row	Spaces	Spacing

(ft)	(ft)				(in)

0.00	0.00	SHEAR STU...	3	1	5.0000

1.50	366.00	SHEAR STU...	3	183	5.0000

367.50	1.50	SHEAR STU...	3	1	5.0000

Haunch Profile

Haunch Type:	Flange edges

Embedded flange:	TRUE

Distance	Length	Z1	Z2	Z3	Z4	Y1	Y2

(ft)	(ft)	(in)	(in)	(in)	(in)	(in)	(in)

0.00	369.00	0.0000	9.0000	0.0000	25.0000	3.0000	-0.6250

Bracing Ranges

Lateral Support

Distance	Length

(ft)	(ft)

0.00	369.00

Stiffener Ranges

Transverse Stiffener Ranges (Location)

Name	Distance	Number	Spacing

(ft)		(in)

INTERMEDIATE STIFF...	0.00	13	48.9100

INTERMEDIATE STIFF...	52.99	5	46.0000

INTERMEDIATE STIFF...	72.15	1	87.1500

INTERMEDIATE STIFF...	79.42	6	41.1500

INTERMEDIATE STIFF...	99.99	15	48.4000

INTERMEDIATE STIFF...	160.49	5	48.1500

INTERMEDIATE STIFF...	180.55	1	96.3000

INTERMEDIATE STIFF...	188.58	5	48.1500

INTERMEDIATE STIFF...	208.64	15	48.4000

INTERMEDIATE STIFF...	269.14	6	41.1500

INTERMEDIATE STIFF...	289.71	1	87.1500

INTERMEDIATE STIFF...	296.98	5	46.0000

INTERMEDIATE STIFF...	316.14	12	48.9100

Bearing Stiffener Locations

Support 1

Stiffener		Offset

Pair	Name	(in)

1	ABUTMENT STIFFENER	0.0000

2	ABUTMENT STIFFENER	0.0000

Support 2

Stiffener		Offset

Pair	Name	(in)

1	PIER BEARING STIFFENER	0.0000

Support 3

Stiffener		Offset

Pair	Name	(in)

1	PIER BEARING STIFFENER	0.0000

Support 4

Stiffener		Offset

Pair	Name	(in)

1	PIER BEARING STIFFENER	0.0000

Support 5

Stiffener		Offset

Pair	Name	(in)

1	ABUTMENT STIFFENER	0.0000

2	ABUTMENT STIFFENER	0.0000

Member   Girder 2

Link with: None

Description: Girder member generated using the design wizard

Existing:	Girder 2 - Member alternative generated using the design wizard

Current:	Girder 2 - Member alternative generated using the design wizard

Number of Spans:	4

Span	Span Length

Number	(ft)

1	76.000000

2	108.500000

3	108.500000

4	76.000000

Support	Frame Connection

1

2

3

4

5

Pedestrian load:	  (lb/ft)

Member Loads

Member Loads - Settlement

Support	Horizontal	Vertical	Rotational	Load Case Name

Number	(in)	(in)	(Radians)

1

2

3

4

5

Support Constraints

General

Support	Support

Number	Type	X Translation	Y Translation	Z Rotation

1	Pinned	Fixed	Fixed	Free

2	Roller	Free	Fixed	Free

3	Roller	Free	Fixed	Free

4	Roller	Free	Fixed	Free

5	Roller	Free	Fixed	Free

Elastic

Support	X Translation	Y Translation	Z Rotation	Override Computed

Number	(kip/ft)	(kip/ft)	(kip-in/rad)	Z Rotation

1

2

3

4

5

Member Alternative   Girder 2

Description: Member alternative generated using the design wizard

Description

Material Type:	Steel

Girder Type:	Plate

Member units:	US Customary

Girder property input method:	Schedule based

Left end X:	  (in)

Right end X:	  (in)

Additional Self Load:	  (kip/ft)

Additional Self Load %:	  (%)

Analysis Module

Analysis Method:	ASD

Analysis Module:

Analysis Module Component:

Properties:

Analysis Method:	LFD

Analysis Module:	AASHTO LFD

Analysis Module Component:

Properties:

Analysis Method:	LRFD

Analysis Module:	AASHTO LRFD

Analysis Module Component:

Properties:

Analysis Method:	LRFR

Analysis Module:

Analysis Module Component:

Properties:

Analysis Method:	Distribution Factors

Analysis Module:

Analysis Module Component:

Properties:

Default rating method:	LFD

Factors

Factor Override

LRFD:

LFD:

ASD Factors

Inventory	Operating

Structural steel

Concrete

PS Concrete Comp.

PS Concrete Tens.

PS Moment Cap.

Reinforcement

Bearing Stiffener

Stirrup

Timber	NA

Default Materials

Structural steel:	1936  to 1963

Deck concrete:	CLASS QC2

Deck reinforcement:	Grade 60

Welds:

Bolts:

Impact

Standard Impact Factor

Type:	Standard - AASHTO

LRFD Dynamic Load Allowance

Fatigue and fracture limit states:	15.0  (%)

All other limit states:	33.0  (%)

Live Load Distribution

Standard

D  i  s  t  r  i  b  u  t  i  o  n     F  a  c  t  o  r  (Wheels)

Lanes		Shear at

Loaded	Shear	Supports	Moment	Deflection

1 Lane	1.071	1.200	1.071	0.400

Multi-Lane	1.364	1.467	1.364	0.800

Girder Profile

Web

Begin	Depth	End

Depth	Vary	Depth	Thickness	Distance	Length	Material

(in)		(in)	(in)	(ft)	(ft)

53.0000	None	53.0000	0.3750	0.00	369.00	1936  to ...

Top Flange

Begin	End

Width	Width	Thickness	Distance	Length	Material

(in)	(in)	(in)	(ft)	(ft)

14.0000	14.0000	0.6875	0.00	59.75	1936  to ...

14.0000	14.0000	1.5000	59.75	30.25	1936  to ...

14.0000	14.0000	1.0000	90.00	76.50	1936  to ...

14.0000	14.0000	1.8750	166.50	36.00	1936  to ...

14.0000	14.0000	1.0000	202.50	76.50	1936  to ...

14.0000	14.0000	1.5000	279.00	30.25	1936  to ...

14.0000	14.0000	0.6875	309.25	59.75	1936  to ...

Bottom Flange

Begin	End

Width	Width	Thickness	Distance	Length	Material

(in)	(in)	(in)	(ft)	(ft)

14.0000	14.0000	0.6875	0.00	59.75	1936  to ...

14.0000	14.0000	1.5000	59.75	30.25	1936  to ...

14.0000	14.0000	1.0000	90.00	76.50	1936  to ...

14.0000	14.0000	1.8750	166.50	36.00	1936  to ...

14.0000	14.0000	1.0000	202.50	76.50	1936  to ...

14.0000	14.0000	1.5000	279.00	30.25	1936  to ...

14.0000	14.0000	0.6875	309.25	59.75	1936  to ...

Deck Profile

Deck Concrete

Total	Structural	Effective	Effective

Material	Distance	Length	Thickness	Thickness	Width (Std)	Width (LRFD)	n

(ft)	(ft)	(in)	(in)	(in)	(in)

CLASS QC2	0.00	369.00		7.5000	90.0000	90.0000	7.50...

Reinforcement

Start		Bar	Bar

Material	Distance	Length	Count	Size	Distance	Row

(ft)	(ft)			(in)

Grade 60	0.00	369.00	9.00	4	2.7500	Top of...

Grade 60	0.00	369.00	9.00	5	1.3125	Bottom...

Grade 60	53.00	45.00	8.00	6	1.3125	Bottom...

Grade 60	157.50	53.00	8.00	6	1.3125	Bottom...

Grade 60	270.00	45.00	8.00	6	1.3125	Bottom...

Shear Connectors

Start		Connector	Number	Number of	Transverse

Distance	Length	Name	per Row	Spaces	Spacing

(ft)	(ft)				(in)

0.00	0.00	SHEAR STU...	3	1	5.0000

1.50	366.00	SHEAR STU...	3	183	5.0000

367.50	1.50	SHEAR STU...	3	1	5.0000

Haunch Profile

Haunch Type:	Flange edges

Embedded flange:	TRUE

Distance	Length	Z1	Z2	Y1

(ft)	(ft)	(in)	(in)	(in)

0.00	369.00	0.0000	9.0000	3.0000

Bracing Ranges

Lateral Support

Distance	Length

(ft)	(ft)

0.00	369.00

Stiffener Ranges

Transverse Stiffener Ranges (Location)

Name	Distance	Number	Spacing

(ft)		(in)

INTERMEDIATE STIFF...	0.00	13	48.9100

INTERMEDIATE STIFF...	52.99	5	46.0000

INTERMEDIATE STIFF...	72.15	1	87.1500

INTERMEDIATE STIFF...	79.42	6	41.1500

INTERMEDIATE STIFF...	99.99	15	48.4000

INTERMEDIATE STIFF...	160.49	5	48.1500

INTERMEDIATE STIFF...	180.55	1	96.3000

INTERMEDIATE STIFF...	188.58	5	48.1500

INTERMEDIATE STIFF...	208.64	15	48.4000

INTERMEDIATE STIFF...	269.14	6	41.1500

INTERMEDIATE STIFF...	289.71	1	87.1500

INTERMEDIATE STIFF...	296.98	5	46.0000

INTERMEDIATE STIFF...	316.14	12	48.9100

Bearing Stiffener Locations

Support 1

Stiffener		Offset

Pair	Name	(in)

1	ABUTMENT STIFFENER	0.0000

2	ABUTMENT STIFFENER	0.0000

Support 2

Stiffener		Offset

Pair	Name	(in)

1	PIER BEARING STIFFENER	0.0000

Support 3

Stiffener		Offset

Pair	Name	(in)

1	PIER BEARING STIFFENER	0.0000

Support 4

Stiffener		Offset

Pair	Name	(in)

1	PIER BEARING STIFFENER	0.0000

Support 5

Stiffener		Offset

Pair	Name	(in)

1	ABUTMENT STIFFENER	0.0000

2	ABUTMENT STIFFENER	0.0000

Member   Member 3

Link with: Girder 2

Description: Girder member generated using the design wizard

Existing:

Current:

Number of Spans:	4

Span	Span Length

Number	(ft)

1	76.000000

2	108.500000

3	108.500000

4	76.000000

Support	Frame Connection

1

2

3

4

5

Pedestrian load:	  (lb/ft)

Member   Member 4

Link with: Girder 2

Description: Girder member generated using the design wizard

Existing:

Current:

Number of Spans:	4

Span	Span Length

Number	(ft)

1	76.000000

2	108.500000

3	108.500000

4	76.000000

Support	Frame Connection

1

2

3

4

5

Pedestrian load:	  (lb/ft)

Member   Member 5

Link with: Girder 1

Description: Girder member generated using the design wizard

Existing:

Current:

Number of Spans:	4

Span	Span Length

Number	(ft)

1	76.000000

2	108.500000

3	108.500000

4	76.000000

Support	Frame Connection

1

2

3

4

5

Pedestrian load:	  (lb/ft)

OUTPUT FILE

Bridge Name: MOT-70-0603
NBI Structure ID: 5704987
Bridge ID: 5704987

Analyzed By: BrR
Analyze Date: Friday, March 04, 2016 13:26:41
Analysis Engine: AASHTO LFR Engine Version 6.7.0.3001
Analysis Preference Setting: None

Report By: brr
Report Date: Friday, March 04, 2016 13:31:41

Structure Definition Name: BROOKVILLE-SALEM SPANS 1 THRU 4
Member Name: Girder 1
Member Alternative Name: Girder 1

Load Factor Rating Summary

|  |  | Rating |  | Capacity |  | Location |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Live Load |  | Factor | Controls | (Ton) | Span | (ft) | Percent | Impact | Lane |
| HS 20-44 | Inventory | 1.240 | Design Flexure - Steel | 44.63 | 2 | 108.50 | 100.0 | As Requested | As Requested |
| HS 20-44 | Operating | 2.070 | Design Flexure - Steel | 74.53 | 2 | 108.50 | 100.0 | As Requested | As Requested |
| NRL | Permit Opr. | 2.557 | Design Flexure - Steel | 102.30 | 1 | 59.75 | 78.6 | As Requested | As Requested |
| SU4 | Permit Opr. | 3.582 | Service - Steel | 96.71 | 4 | 45.60 | 60.0 | As Requested | As Requested |
| SU5 | Permit Opr. | 3.230 | Service - Steel | 100.14 | 4 | 45.60 | 60.0 | As Requested | As Requested |
| SU6 | Permit Opr. | 2.915 | Service - Steel | 101.28 | 4 | 45.60 | 60.0 | As Requested | As Requested |
| SU7 | Permit Opr. | 2.654 | Design Flexure - Steel | 102.84 | 1 | 59.75 | 78.6 | As Requested | As Requested |
| OH-2F1 | Legal Opr. | 6.073 | Service - Steel | 91.10 | 4 | 45.60 | 60.0 | As Requested | As Requested |
| OH-3F1 | Legal Opr. | 4.100 | Service - Steel | 94.30 | 4 | 45.60 | 60.0 | As Requested | As Requested |
| OH-4F1 | Legal Opr. | 3.603 | Service - Steel | 97.28 | 4 | 45.60 | 60.0 | As Requested | As Requested |
| OH-5C1 | Legal Opr. | 2.973 | Design Flexure - Steel | 118.92 | 1 | 59.75 | 78.6 | As Requested | As Requested |

Note:
"N/A" indicates not applicable
"**" indicates not available

Bridge Name: MOT-70-0603
NBI Structure ID: 5704987
Bridge ID: 5704987

Analyzed By: BrR
Analyze Date: Friday, March 04, 2016 13:26:41
Analysis Engine: AASHTO LFR Engine Version 6.7.0.3001
Analysis Preference Setting: None

Report By: brr
Report Date: Friday, March 04, 2016 13:31:41

Structure Definition Name: BROOKVILLE-SALEM SPANS 1 THRU 4
Member Name: Girder 2
Member Alternative Name: Girder 2

Load Factor Rating Summary

|  |  | Rating |  | Capacity |  | Location |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Live Load |  | Factor | Controls | (Ton) | Span | (ft) | Percent | Impact | Lane |
| HS 20-44 | Inventory | 1.138 | Design Flexure - Steel | 40.97 | 2 | 108.50 | 100.0 | As Requested | As Requested |
| HS 20-44 | Operating | 1.900 | Design Flexure - Steel | 68.41 | 2 | 108.50 | 100.0 | As Requested | As Requested |
| NRL | Permit Opr. | 2.385 | Design Flexure - Steel | 95.41 | 4 | 16.25 | 21.4 | As Requested | As Requested |
| SU4 | Permit Opr. | 3.356 | Service - Steel | 90.62 | 1 | 30.40 | 40.0 | As Requested | As Requested |
| SU5 | Permit Opr. | 3.027 | Service - Steel | 93.83 | 1 | 30.40 | 40.0 | As Requested | As Requested |
| SU6 | Permit Opr. | 2.731 | Service - Steel | 94.90 | 1 | 30.40 | 40.0 | As Requested | As Requested |
| SU7 | Permit Opr. | 2.475 | Design Flexure - Steel | 95.92 | 4 | 16.25 | 21.4 | As Requested | As Requested |
| OH-2F1 | Legal Opr. | 5.691 | Service - Steel | 85.36 | 1 | 30.40 | 40.0 | As Requested | As Requested |
| OH-3F1 | Legal Opr. | 3.841 | Service - Steel | 88.34 | 1 | 30.40 | 40.0 | As Requested | As Requested |
| OH-4F1 | Legal Opr. | 3.376 | Service - Steel | 91.16 | 1 | 30.40 | 40.0 | As Requested | As Requested |
| OH-5C1 | Legal Opr. | 2.774 | Design Flexure - Steel | 110.95 | 1 | 59.75 | 78.6 | As Requested | As Requested |

Note:
"N/A" indicates not applicable
"**" indicates not available

Figure 1: Brr Transverse Section

Figure 2: Brr Framing Plan