PID 114955, MOT-70/75-11.30/3.73L: QUANTITY CALCULATIONS

Calculated by:  Jun Xie, Date: 04/10/2024

Checked by:

Revised by:

Erosion Control

- Item 832E30000 Erosion Control

  - MOT-70-1130: 1,000 EACH (minimum quantity for erosion control is sufficient)

  - MOT-75-0373L: 1,000 EACH (minimum quantity for erosion control is sufficient)

  - Total: 2,000 EACH

Traffic Control

- Item 646E10010 Edge Line 6”

    - MOT-70-1130

      - [121’+10’ (5” beyond each end of the epoxy overlay limits)] = 131’/5280 = 0.025 mile. Round up to 0.03 mile.

      - White Edge Line: (MOT-70-1130) 0.03 mi. x 3 = 0.09 mi.

      - Yellow Edge Line: (MOT-70-1130) 0.03 mi. x 3 = 0.09 mi.

    - Total: 0.09 + 0.09 = 0.18 mile.

- Item 646E10110 Lane line 6”

- (121’+10’) = 131’/5280 = 0.025 mile. Round up to 0.03 mile.

- 0.03 mile x [(1 lane line) westbound + (2 lane lines) eastbound] = 0.09 mile.

- Item 646E20504 Dotted Line, 6"

- (121’+10’) x (1 dotted line) westbound = 131ft

Structures: MOT-70-1130

- Item Special - 519E12300 Patching Concrete Bridge Deck – Type B

- Area to be patched:

  - (ORD 57.95 SF) 58 SF + (ORD 8.15 SF) 9 SF = 67 SF/9 = 7.5 SQ YD. Round up to 8 SQ YD

- Item 858E10000 Thin Polymer Epoxy Overlay

  - Area overlay to be placed.

    - Westbound [120’-3”] limits of epoxy overlay x [(80’-7”) epoxy overlay width, transverse + 2 x 3” epoxy overlay at parapets, 3” above the deck surface] = 9751 SQ FT

    - Eastbound [120’-3”] limits of epoxy overlay x [(59’-7”) epoxy overlay width, transverse + 2 x 3” epoxy overlay at parapets, 3” above the deck surface] = 7226 SQ FT

    - Total 9751 SQ FT + 7226 SQ FT = 16977 SQ FT / 9 = 1887 SQ YD

Structures: MOT-75-0373L

- Item Special - 519E12300 Patching Concrete Bridge Deck – Type B

Area to be patched 4 FT x 4 FT = 16 SQ FT / 9 = 1.78 SQ YD. Round up to 2 SQ YD.

- Item 858E10000 Thin Polymer Epoxy Overlay

- Areas overlay to be repaired.

    - Area 1, 60’ longitudinal x 8’ transverse = 480 SQ FT

    - Area 2, 6’ x 6’ = 36 SQ FT

    - Total, 480 SQ FT + 36 SQ FT = 516 SQ FT / 9 = 57.34 SQ YD. Round up to 58 SQ YD.

Maintenance of Traffic

- Item 614E11110 Law Enforcement Officer with Patrol Car for Assistance

[Reference project 97105 MOT-75-3.73 utilized 120 - 53(change order: 150411-001) = 67 hours]

  - MOT-70-1130: 70 hours (4 phases)

  - MOT-75-0373 L: 20 hour (1 phase)

  - Total: 70 hours + 20 hours = 90 hours

  - (Construction feedback, stage 3 comments) Increase to 100 hours.

- Item 614E12420 Detour Signing (LS)

Incidentals

- Item 614E11000 - Maintaining Traffic (LS)

- Item 624E10000 - Mobilization (LS)

END OF CALCULATIONS