PID 116928, CLA/MOT-BH-FY26: QUANTITY CALCULATION

Calculated by:  Dan Grilliot, P.E., Date: 2/28/2025

Checked by: Lawton Gerlinger, P.E., Date: 4/4/2025

Revised by: Dan Grilliot, P.E., Date:  3/13/2026

Roadway

- Item 202-Pavement Removed (SY)

  - MOT-75-0614R:  Four of areas of pavement removal assumed for the concrete patching of the parapet

  - 1 ft. x 11.5 ft. = 11.5 sq. ft = 12 sq. ft.

  - 1 ft. x 9.2 ft. = 9.2 ft. = 10 sq. ft.

  - 1 ft. x 4.6 ft. = 4.6 ft. = 5 sq. ft.

  - 1 ft. x 2.3 ft. = 2.3 ft. = 3 sq. ft.

  - Total = 30 sq. ft. = 3.33 sq. yd = 4 sq. yd.

  - Total = 4 sq. yd.

Erosion Control

- Item 832-Erosion Control (EA)

  - Considered Maintenance Project:  11 bridges x $100/bridge = $1,100

Pavement

- Item 442-Asphalt Concrete Surface Course, 12.5mm, Type A (448), As Per Plan, PG70-22M (CY)

  - 4 sq. yd. x (3 in. x (1 yd./36 in.)) = 0.33 cu. yd. = round to 1 cu. yd.

Structure Repair (CLA-004-0667R)

- Item 519-Patching Concrete Structure, As Per Plan (SF)

  - Lt. Parapet = 48 sq. ft.

  - Rt. Parapet = 33 sq. ft.

  - Total = 81 sq. ft.

Structure Repair (CLA-040-1380)

- Item 519-Patching Concrete Structure, As Per Plan (SF)

  - Curb (median) = 17 sq. ft.

  - Curb (Lt. side) = 2 sq. ft.

  - Curb (Rt. side) = 11 sq. ft.

  - Rear Abutment = 18 sq. ft.

  - Fwd. Abutment = 25 sq. ft.

  - Pier #1 = 3 sq. ft.

  - Pier #2 = 4 sq. ft.

  - Total = 80 sq. ft.

Structure Repair (CLA-068-0667L)

- Item 202-Portions of Structure Removed, Over 20 Foot Span, As Per Plan (LS)

  - For partial deck edge removal.

  - Total = LUMP SUM

- Item 202-Bridge Railing Removed for Reuse (FT)

  - Lt. Deck Edge Railing Removed for Partial Deck Edge Replacement

  - Total = 25 ft.

- Item 509-Epoxy Coated Steel Reinforcement (LB)

  - Reinforcing Steel for Partial Deck Edge/Safety Curb Replacement

  - Total = 50 lbs

- Item 509-Concrete Reinforcement, Replacement of Existing Concrete Reinforcement, As Per Plan (LB)

  - Total = 50 lbs

- Item 511-Class QC2 Concrete, Superstructure (CY)

  - Deck edge/safety curb replacement = 2.9 sq. ft. x 18.75 ft = 54.375 cu. ft. = 2 cu. yd.

  - Total = 2 cu. yd.

- Item 517-Railing Misc.:  Type B Anchors (EACH)

  - 3 posts x 4 anchors/post = 12 each

  - Total = 12 each

- Item 519-Patching Concrete Structure, As Per Plan (SF)

  - Curb (Lt. side) = 10 sq. ft.

  - Curb (Rt. side) = 6 sq. ft.

  - Rear Abutment = 5 sq. ft.

  - Fwd. Abutment = 32 sq. ft.

  - Total = 53 sq. ft.

Structure Repair (CLA-068-0667R)

- Item 519-Patching Concrete Structure, As Per Plan (SF)

  - Curb (Lt. side) = 2 sq. ft.

  - Curb (Rt. side) = 7 sq. ft.

  - Rear Abutment = 10 sq. ft.

  - Fwd. Abutment = 35 sq. ft.

  - Total = 54 sq. ft.

Structure Repair (CLA-675-0061L)

- Item 519-Patching Concrete Structure, As Per Plan (SF)

  - Forward Abutment = 24 sq. ft.

  - Total = 24 sq. ft.

Structure Repair (MOT-035-1395L)

- Item 519-Patching Concrete Structure, As Per Plan (SF)

  - Lt. Parapet = 101 sq. ft.

  - Rt. Parapet = 76 sq. ft.

  - Total = 177 sq. ft.

Structure Repair (MOT-035-1395R)

- Item 519-Patching Concrete Structure, As Per Plan (SF)

  - Lt. Parapet = 56 sq. ft.

  - Rt. Parapet = 133 sq. ft.

  - Total = 189 sq. ft.

Structure Repair (MOT-070-1439)

- Item 519-Patching Concrete Structure, As Per Plan (SF)

  - Lt. Parapet = 86 sq. ft.

  - Rt. Parapet = 110 sq. ft.

  - Rt. Curb = 3 sq. ft.

  - Total = 199 sq. ft.

Structure Repair (MOT-075-0614R)

- Item 514-Field Painting, Misc.:  Zinc Rich Primer (SF)

  - Underside of Deck Areas = 40 sq. ft. + 40 sq. ft. + 10 sq. ft. + 5 sq. ft. + 5 sq. ft. + 5 sq. ft. = 105 sq. ft.

- Item 519-Patching Concrete Structure, As Per Plan (SF)

  - Lt. Parapet = 70 sq. ft.

  - Rt. Parapet = 50 sq. ft.

  - Pier #2 = 15 sq. ft.

  - Total = 135 sq. ft.

Structure Repair (MOT-075-0958)

- Item 519-Patching Concrete Structure, As Per Plan (SF)

  - Rear Abutment = 249 sq. ft.

  - Fwd. Abutment = 38 sq. ft.

  - Fwd. Abutment Estimated Behind the Beams = 69 sq. ft.

  - Total = 356 sq. ft.

- Item 519-Patching Concrete Bridge Deck – Type B

  - Quantity of 23 sq. yd. included based off 3/12/2026 site visit by D7 bridge inspectors.

Structure Repair (MOT-675-0055L)

- Item 519-Patching Concrete Structure, As Per Plan (SF)

  - Lt. Parapet = 17 sq. ft.

  - Rt. Parapet = 604 sq. ft.

  - Pier #1 = 2 sq. ft.

  - Pier #3 = 2 sq. ft.

  - Total = 625 sq. ft.

Maintenance of Traffic

- Item 614-Law Enforcement Officer with Patrol Car for Assistance (HOUR)

  - CLA-068-0667L = Estimated 48 hr. for setting traffic control zone with portable barrier for partial deck edge/safety curb replacement.

  - Total = 48 hr.

- Item 614-Work Zone Impact Attenuator, 24” Wide Hazards, (Unidirectional) (EACH)

  - CLA-068-0667L = 1 each

  - Total = 1 each

- Item 614-Detour Signing (LS)

  - Lump Sum (LS)

- Item 614-Barrier Reflector, Type 1 (Unidirectional) (EACH)

  - CLA-068-0667L = 320 ft/50 ft. = 7 spaces = 8 each

  - Total = 8 each

- Item 614-Object Marker, One Way (EACH)

  - Same as Item 614 Barrier Reflector, Type 1 (Unidirectional) = 8 each = total

- Item 614-Work Zone Edge Line, Class I, 4”. 740.06, Type I (MILE)

  - CLA-068-0667L = 0.20 mile

  - Total = 0.2 mile

- Item 622-Portable Barrier, Unanchored (FT)

  - CLA-068-0667L = 320 ft.

  - Total = 320 ft.

Incidentals

- Item 614-Maintaining Traffic (LS)

  - Lump Sum (LS)

- Item 624-Mobilization (LS)

  - Lump Sum (LS)

END OF CALCULATIONS