MOT CALCS. (MT-95.40)

Item 622 – Portable Barrier

Impact attenuator = 25’ (paid for separately)

Last full section of PB parallel to impact attenuator = 10’

9’ max gap from edge of paved shoulder to edge of PB + 2’ wide PB

Lane line to edge of paved shoulder = 12’ + 10’ = 22’

PB taper = 22’-11’ = 11’ x 19:1 PB flare rate = 209’

EB work zone length: 1079’

Tapered end section and buffer: 10’ + 10’ = 20’

TOTAL = 1,318’; round to 1,320’

Item 614 – Work Zone Edge Line

Taper: 12’ x 65:1 taper rate = 780’

Buffer (D): 645’

Impact attenuator and PB flare = 25’ + 10’ + 209’ = 244’

EB work zone length: 1079’

Tapered end section and buffer: 10’ + 10’ = 20’

TOTAL = 2,768’ / 5280 = 0.52 Miles

Both directions are greater than 0.5 miles  Work zone speed zone warranted

Work zone condition

EB: 13.52 + (125’ / 5280) = 13.54 - ((780’ + 645’ + 244’) / 5280) = 13.22;

13.73 + (110’ / 5280) = 13.75

SLM 13.75 – SLM 13.22 = 0.53 miles