EXISTING INFO.

US 127

- Station 16+60.58 in existing plans = Station 848+23.26 from survey

  - Survey station = station in existing plans + 831.6268 stations

- 2-10’ shoulders @ 0.04 cross slope

- 2-12’ lanes @0.016 cross slope until Sta. 843+72.62 (12+09.94 in existing plans)

  - Total pavement width = 44’

- Center lane starts at Sta. 843+72.62

  - Width varies from 0’ on each side of centerline @ Sta. 843+72.62 to 6’ on each side of centerline @ Sta. 847+35.45

  - Same cross slope as travel lanes

  - Total pavement width varies from 44’ to 56’ at same limits

- Pavement build-up:

  - 9.5” non-reinforced concrete pavement

  - 6” aggregate base, extends 6” beyond concrete pavement

  - 4” base pipe underdrain @ edge of concrete pavement (DND)

    - 18” below aggregate base

  - 6” shallow pipe underdrain at edge line

    - 18”-30” below aggregate base

- Guardrail face 2’ beyond edge of concrete pavement

- 4’ graded shoulder beyond edge of concrete pavement @ 0.08 cross slope

- 2’ ditch width with 4’ rounding

- 3.04% longitudinal slope

Ramp D

- 16’ lane

  - 0.083 max cross slope to the right until Sta. 1371+25.52

  - 0.074 max cross slope to the left for superelevation section

- 3’ concrete shoulder on the left

  - Cross slope varies 0.04 - 0.074

- 6’ concrete shoulder on the right

  - Cross slope varies 0.04 to 0.01

- Total pavement width = 25’

- Pavement build-up

  - 12.5” Portland cement concrete pavement

  - 6” aggregate base, extends 6” beyond concrete pavement

  - 6” shallow pipe underdrain at BL/edge line

    - 18”-30” below aggregate base

- Guardrail face 2’ beyond edge of concrete pavement

- 5’ graded shoulder beyond edge of concrete pavement @ 0.08 cross slope

- 2’ ditch width with 4’ rounding, 10’ wide ditch starts at Sta. 1375+50

- -2.66% longitudinal slope

PROPOSED INFO.

- Curb, Type 4-C, at existing edge of concrete pavement

- Aggregate base extends 12” beyond back of curb

  - 18” wide total

  - 6.5” thick for Ramp D

- 10:1 max graded shoulder slope (3’ wide)

- MGS Guardrail

  - Face is flush with face of curb

    - Can be offset a max of 6” behind face of curb

  - Slope break point 1’ behind guardrail

    - 3’ from edge of concrete pavement/face of curb

  - 2’ offset transition at ends

    - 25’ tangent from end of curb

    - 2-25’ flare arcs

- Try 3:1 slope, then try 2.75:1 if that doesn’t work

- Keep same ditch widths

- 2:1 backslope

- See drainage details and calculations for catch basins, tied concrete block mats, and curb type determinations

Ditch slope (from survey) = (714.9 – 713.9)/700’ = .00143

.4 = (62.4)(D)(.00143)

D = 4.5’ ------> water would have to be running at > 4.5’ deep to exceed the allowable shear stress limit for seed (659)