WAY-095-0.17

PID 109854

CENTERLINE DETERMINATION

The only right-of-way plans on this section of S.R. 095 is the Dedication Plat of Mohicanville-Wooster Road (1936).  This Dedication Plat shows a 60’ right-of-way, but no alignment. I first tried to use the Mohicanville-Wooster Road S.H. 77 Sec. T pt. & U pt. (1947) plans for the alignment. These plans show a centerline of right-of-way and right-of-way takes west of my project, and east of my project. The plans show an alignment between the two parts, but the alignment does not take any right-of-way nor does it fit the centerline of pavement. I believe this is just a centerline of construction not a centerline of right-of-way. I then found the original field book 27 pages 39-46 of the Muskingum Watershed (1936) which showed the alignment of the road and tied it to barns, residents, and property lines. I used these ties to establish the centerline of Right-of-way. The centerline of right-of-way does not match the property lines, but the local surveyors did not have the field books, so they just split pavement.

For stationing I used the county line established by Donald C. Baker “VV-158”.

Brett R. Tieben, P.S.