Surveyors Report for PID 113396-241-01.32

The alignment of this project was based upon WAY -241-1.738 (1999) and WAY-241-1.38 (1987).  No older plans exist for this area, the plan on file is just a dedication plat with all owner’s signatures and no alignment shown.  I did find the field notes for this section from 1931 at the Wayne County Engineers office in field book 577.   They set stakes, not pins, on the PI’s so no old monuments were likely to be found. This section was originally recorded as state route in 1825 in road book 1 of Wayne County Engineer.  I tried to fit the existing 2 R/W’s to these old field notes with no luck. If I disregarded all surveying work on these 2 projects I could come up with a decent rendition of the 1931 C/L, but this line would disagree with both existing projects R/W and C/L definitions.  I did not feel this served the purpose of establishing the R/W of this project at the expense of the other 2 existing jobs. The deviations are mostly around a foot up to at most 2 feet.  So, to define the R/W I projected the tangents of both jobs till they intersected and created a PI. The C/L monuments from 1.738 were used to project northwards, and I established the R/W of 1.38 from found section corner and distant C/L pins. I then tweaked it slightly to match the existing boundary lines established and monumented in the area.  This only moved this plan set 0.2’-0.3’ from where I originally calculated it then it matched property lines which were called and shown in takes.   The corner stone at the NW corner of NW quarter of section 21 paint township is a large 1.5’ square corner stone with large drill hole in center. It is leaning to the SW probably over a foot. Checking the subsequent surveys done in the last 20-30 years in this area everyone has used the current location of the stone for all surveys done in this area.  It matches up well with where it was located on the R/W plans for the 1.38 project also.  The Property line is clearly different form the road C/L of R/W definition. The property line for the parcels to the NW was retraced from the Baker, Gasbarre and Shamp surveys which basically match each other well. The property to the SE has not been surveyed in recent history and is labeled as needing surveyed on a 1948 survey of the parent parcels of the NW side of the road.

There are 3 Gas meters located on the SE side of 241 that I believe service the houses on the NW side of 241. Contacting the Consumer Gas coop the owner was informed they have no records of where service lines are installed and that is entirely the owner responsibility, ongoing efforts to get actual location of these lines ongoing. The lines are probably north of the area impacted by this project, engineer was notified.  One area has large amount of farm debris dumped in bottom of valley and will require substantial clean up, engineer notified of this area.

The stationing is established from the county line north matching destape values.

## Horizontal Alignment Review Report

Report Created:  Tuesday, June 14, 2022
Time:  2:56:13 PM

| Project: | Design |  |
| --- | --- | --- |
| Description: |  |  |
| File Name: | c:\users\jdarr\d0568381\113396_BR001.dgn |  |
| Last Revised: | 6/14/2022 14:55:53 |  |
|  | Note: All units in this report are in feet unless specified otherwise. |  |

| Alignment Name: |  | CLX_RW_241 |  |  |
| --- | --- | --- | --- | --- |
| Alignment Description: |  |  |  |  |
| Alignment Style: |  | Alignment\RW_X_Centerline |  |  |
|  |  | Station | Northing | Easting |
| Element: Linear |  |  |  |  |
| POT | ( ) | 49+35.40 R1 | 368759.5260 | 2183220.2299 |
| HPI | ( ) | 67+74.15 R1 | 369902.4775 | 2184660.6063 |
| Tangential Direction: |  | N51°34'03.66"E |  |  |
| Tangential Length: |  | 1838.76 |  |  |
| Element: Linear |  |  |  |  |
| HPI | ( ) | 67+74.15 R1 | 369902.4775 | 2184660.6063 |
| POT | ( ) | 77+00.00 R1 | 370484.1609 | 2185380.9123 |
| Tangential Direction: |  | N51°04'38.56"E |  |  |
| Tangential Length: |  | 925.85 |  |  |

## Horizontal Alignment Review Report

Report Created:  Wednesday, June 15, 2022
Time:  12:45:45 PM

| Project: | Design |  |
| --- | --- | --- |
| Description: |  |  |
| File Name: | c:\users\jdarr\d0568377\113396_BK001.dgn |  |
| Last Revised: | 6/15/2022 12:45:13 |  |
|  | Note: All units in this report are in feet unless specified otherwise. |  |

| Alignment Name: |  | CLX_RW_241 |  |  |
| --- | --- | --- | --- | --- |
| Alignment Description: |  |  |  |  |
| Alignment Style: |  | Alignment\RW_X_Centerline |  |  |
|  |  | Station | Northing | Easting |
| Element: Linear |  |  |  |  |
| POT | ( ) | 59+76.31 R1 | 369406.5449 | 2184035.6195 |
| HPI | ( ) | 67+74.15 R1 | 369902.4775 | 2184660.6063 |
| Tangential Direction: |  | N51°34'03.66"E |  |  |
| Tangential Length: |  | 797.85 |  |  |
| Element: Linear |  |  |  |  |
| HPI | ( ) | 67+74.15 R1 | 369902.4775 | 2184660.6063 |
| POT | ( ) | 77+00.00 R1 | 370484.1609 | 2185380.9123 |
| Tangential Direction: |  | N51°04'38.56"E |  |  |
| Tangential Length: |  | 925.85 |  |  |

## Station Offset Northing Easting Elevation Feature Report

Report Created:  Wednesday, June 15, 2022
Time:  12:26:39 PM

| Project: | Design |  |
| --- | --- | --- |
| Description: |  |  |
| Baseline Alignment: | CLX_RW_241 |  |
| File Name: | c:\users\jdarr\d0568381\113396_BR001.dgn |  |
| Last Revised: | 6/15/2022 12:21:30 |  |
| Input Grid Factor: |  | Note: All units in this report are in feet unless specified otherwise. |

| Point | Station | Offset | Northing | Easting | Elevation | Feature |
| --- | --- | --- | --- | --- | --- | --- |
| MN50 | 59+76.31 R1 | 0.00 | 369406.5449 | 2184035.6195 | 1087.5431 | MONBOX |
| MN51 | 63+86.36 R1 | 0.00 | 369661.4297 | 2184356.8319 | 1093.5945 | MONBOX |
| CLRW1 | 67+74.15 R1 | 0.00 | 369902.4775 | 2184660.6063 | 0.0000 | CALPT |
| CLRW2 | 77+00.00 R1 | -0.00 | 370484.1609 | 2185380.9123 | 0.0000 | CALPT |

## Station Offset Northing Easting Elevation Feature Report

Report Created:  Wednesday, June 15, 2022
Time:  7:27:04 AM

| Project: | Design |  |
| --- | --- | --- |
| Description: |  |  |
| Baseline Alignment: | CLX_RW_241 |  |
| File Name: | c:\users\jdarr\d0568381\113396_BR001.dgn |  |
| Last Revised: | 6/15/2022 07:06:33 |  |
| Input Grid Factor: |  | Note: All units in this report are in feet unless specified otherwise. |

| Point | Station | Offset | Northing | Easting | Elevation | Feature |
| --- | --- | --- | --- | --- | --- | --- |
| CP01 | 68+21.66 R1 | -23.54 | 369950.6382 | 2184682.7795 | 1120.2159 | IPINS |
| BM01 | 69+91.91 R1 | 23.94 | 370020.6581 | 2184845.0627 | 1118.9141 | SMSS |
| CP02 | 71+77.48 R1 | 14.52 | 370144.5801 | 2184983.5168 | 1116.0221 | IPINS |
| BM02 | 72+31.11 R1 | -29.71 | 370212.6803 | 2184997.4516 | 1114.9651 | SMSS |
| CP03 | 74+29.59 R1 | -20.01 | 370329.8401 | 2185157.9586 | 1115.2972 | IPINS |
| CP04 | 76+87.29 R1 | 13.72 | 370465.5007 | 2185379.6488 | 1117.4193 | IPINS |

## Station Offset Northing Easting Elevation Feature Report

Report Created:  Wednesday, June 15, 2022
Time:  7:22:34 AM

| Project: | Design |  |
| --- | --- | --- |
| Description: |  |  |
| Baseline Alignment: | CLX_RW_241 |  |
| File Name: | c:\users\jdarr\d0568381\113396_BR001.dgn |  |
| Last Revised: | 6/15/2022 07:06:33 |  |
| Input Grid Factor: |  | Note: All units in this report are in feet unless specified otherwise. |

| Point | Station | Offset | Northing | Easting | Elevation | Feature |
| --- | --- | --- | --- | --- | --- | --- |
| MN50 | 59+76.31 R1 | 0.00 | 369406.5449 | 2184035.6195 | 1087.5431 | MONBOX |
| MN25 | 63+20.93 R1 | 30.19 | 369597.1123 | 2184324.3399 | 1092.3585 | IPID |
| MN23 | 63+69.91 R1 | -29.81 | 369674.5615 | 2184325.4176 | 1091.4123 | IPID |
| MN51 | 63+86.36 R1 | 0.00 | 369661.4297 | 2184356.8319 | 1093.5945 | MONBOX |
| MN22 | 64+99.46 R1 | -29.75 | 369755.0348 | 2184426.9379 | 1096.9914 | IPIN |
| MN27 | 71+21.57 R1 | -39.57 | 370151.5347 | 2184906.0333 | 1107.9342 | IPID |
| MN21 | 72+01.34 R1 | -40.84 | 370202.6411 | 2184967.2989 | 1107.6513 | IPID |
| MN28 | 72+41.03 R1 | -41.00 | 370227.7026 | 2184998.0824 | 1112.7862 | IPID |
| MN20 | 73+87.88 R1 | -42.62 | 370321.2236 | 2185111.3078 | 1108.7397 | IPIN |
| MN32 | 84+13.61 R1 | -715.67 | 370974.7299 | 2185901.9940 | 1166.1640 | IPID |
| MN30 | 85+02.44 R1 | -819.97 | 371119.4957 | 2185899.2687 | 1167.4914 | CSF |
| MN33 | 83+42.98 R1 | 644.04 | 370859.4718 | 2185904.2943 | 1163.0903 | IPIN |

## Station Offset Northing Easting Elevation Feature Report

Report Created:  Wednesday, June 15, 2022
Time:  8:02:14 AM

| Project: | Design |  |
| --- | --- | --- |
| Description: |  |  |
| Baseline Alignment: | CLX_RW_241 |  |
| File Name: | c:\users\jdarr\d0568381\113396_BR001.dgn |  |
| Last Revised: | 6/15/2022 08:00:34 |  |
| Input Grid Factor: |  | Note: All units in this report are in feet unless specified otherwise. |

| Point | Station | Offset | Northing | Easting | Elevation | Feature |
| --- | --- | --- | --- | --- | --- | --- |
| CALCPT6 | 64+76.81 R1 | 0.24 | 369717.4656 | 2184427.8338 |  | Point\Control and Monuments\Scratch\CALCPT |
| CALCPT5 | 70+95.79 R1 | -9.28 | 370111.7742 | 2184905.0132 |  | Point\Control and Monuments\Scratch\CALCPT |
| CALCPT4 | 71+75.38 R1 | -10.20 | 370162.4941 | 2184966.3488 |  | Point\Control and Monuments\Scratch\CALCPT |
| CALCPT3 | 72+15.18 R1 | -10.66 | 370187.8605 | 2184997.0270 |  | Point\Control and Monuments\Scratch\CALCPT |
| CALCPT2 | 73+80.16 R1 | -12.58 | 370293.0060 | 2185124.1768 |  | Point\Control and Monuments\Scratch\CALCPT |
| CALCPT1 | 83+88.45 R1 | -688.79 | 370933.5281 | 2185902.9327 |  | Point\Control and Monuments\Scratch\CALCPT |
| MN30 | 85+02.44 R1 | -819.97 | 371119.4957 | 2185899.2687 | 1167.4914 | CSF |

## Station Offset Elevation Feature Report

Report Created:  Wednesday, June 15, 2022
Time:  8:24:22 AM

| Project: | Design |  |
| --- | --- | --- |
| Description: |  |  |
| Baseline (Active) Alignment: | CLX_RW_241 |  |
| File Name: | c:\users\jdarr\d0568381\113396_BR001.dgn |  |
| Last Revised: | 6/15/2022 08:09:58 |  |
| Input Grid Factor: |  | Note: All units in this report are in feet unless specified otherwise. |

| Point | Description | Station | Offset | Elevation | Feature |
| --- | --- | --- | --- | --- | --- |
| RWLT1 |  | 67+73.89 R1 | -30.00 | 0.0000 | CALPT |
| RWRT1 |  | 71+57.05 R1 | 30.00 | 0.0000 | CALPT |
| RWRT2 |  | 72+30.00 R1 | 65.00 | 0.0000 | CALPT |
| RWLT2 |  | 72+31.64 R1 | -30.00 | 0.0000 | CALPT |
| RWLT3 |  | 73+00.00 R1 | -50.00 | 0.0000 | CALPT |
| RWRT3 |  | 74+25.00 R1 | 65.00 | 0.0000 | CALPT |
| RWRT4 |  | 75+00.00 R1 | 47.00 | 0.0000 | CALPT |
| RWLT4 |  | 76+00.00 R1 | -50.00 | 0.0000 | CALPT |
| RWRT5 |  | 76+50.00 R1 | 40.00 | 0.0000 | CALPT |
| RWLT5 |  | 77+00.00 R1 | -30.00 | 0.0000 | CALPT |
| RWRT6 |  | 77+00.00 R1 | 30.00 | 0.0000 | CALPT |