| Input from Hydraulic Model |  |  |
| --- | --- | --- |
|  | 2% AEP (50 year) | 1% AEP (100 year) |
| Average Depth Upstream (ft) | 9.36 | 9.62 |
| Average Velocity in Contracted Section (ft/sec) | 5.57 | 6.18 |
| Manning’s n | 0.03 | 0.03 |
| Depth prior to Scour in Contracted Section (ft) | 8.52 | 8.66 |
| Duration of flow (hours) | 24 | 24 |

FUL-109-16.68 PID 102931 Contraction Scour Calculation

| NCHRP Contraction Scour by Layer Using D50 equivalent |  |  |
| --- | --- | --- |
| Boring # | 2% AEP Scour Depth (ft) | 1% AEP Scour Depth (ft) |
| B-001-0-22 | 3 | 3 |
| B-002-0-22 | 2.66 | 3.09 |

Soil Input

| Boring | Layer # | ODOT (Modified AASHTO)/USCS Classification | Elevations (ft) | Thickness (ft) | Sampled D50 (mm) | Scour Critical Shear Stress (psf) | Scour Critical Shear Stress (Pa) = D 50 equivalent (mm) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| B-001-0-22 | 1 | A-4a Sandy Lean Clay (CL) | 739.2 | 1.5 | 0.063 | 0.027 | 1.2902 |
|  | 2 | A-6a Sandy Lean Clay (CL) | 737.7 | 1.5 | 0.017 | 0.126 | 6.0248 |
|  | 3 | A-6a Lean Clay with Sand (CL) | 736.2 | 1.5 | 0.006 | 0.680 | 32.5698 |
|  | 4 | A-6a Lean Clay with Sand (CL) | 734.7 | 1.5 | 0.007 | 0.861 | 41.2267 |
|  | 5 | A-6a Lean Clay with Sand (CL) | 733.2 | 1.5 | 0.006 | 0.632 | 30.2808 |
|  | 6 | A-6a Lean Clay with Gravel (CL) | 731.7 | 1.5 | 0.01 | 0.751 | 35.9779 |

| Boring | Layer # | ODOT (Modified AASHTO)/USCS Classification | Elevations (ft) | Thickness (ft) | Sampled D50 (mm) | Scour Critical Shear Stress (psf) | Scour Critical Shear Stress (Pa) = D 50 equivalent (mm) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| B-002-0-22 | 1 | A-6a Lean Clay with Sand (CL) | 735.8 | 1.5 | 0.006 | 0.237 | 11.3472 |
|  | 2 | A-6a Lean Clay with Sand | 734.3 | 1.5 | 0.006 | 0.261 | 12.5088 |
|  | 3 | A-6a Lean Clay with Sand (CL) | 732.8 | 1.5 | 0.006 | 0.330 | 15.7962 |
|  | 4 | A-6a Lean Clay with Sand (CL) | 731.3 | 1.5 | 0.008 | 0.429 | 20.5609 |
|  | 5 | A-6a Lean Clay with Sand (CL) | 729.8 | 1.5 | 0.008 | 0.407 | 19.5034 |
|  | 6 | A-6a Lean Clay with Sand (CL) | 728.3 | 1.5 | 0.006 | 0.430 | 20.5773 |

Cohesive Time-Rate of Scour Calculation

B-001-0-22 Soil Boring

2% AEP

Use critical shear stress = 0.027 psf

Layer thickness (1.5 feet) scours in 5.9 hours. Remaining time for scour check = 24 hours – 5.9 hours = 18.1 hours. Check Layer 2.

Use average velocity of 4.38 fps

Use critical shear stress = 0.13 psf

Depth prior to scour: 8.52 +1.5 = 10.02 ft.

Layer 2 scours in 17.8 hours. Remaining time for scour check: 24 – 5.9 – 17.8 = 0.3. Check Layer 3.

Use average velocity of 3.7 fps

Use critical shear stress = 0.68 psf

Depth prior to scour: 10.02 + 1.5 = 11.52 ft.

Layer 3 does not scour in remaining time. Predicted scour depth: 3 feet.

B-001-0-22 Soil Boring

1% AEP

Use critical shear stress = 0.027 psf

Layer thickness (1.5 feet) scours in 4.55 hours. Remaining time for scour check = 24 hours – 4.55 hours = 19.45 hours. Check Layer 2.

Use average velocity of 4.86 fps

Use critical shear stress of 0.13 psf

Depth prior to scour: 8.66 + 1.5 = 10.16 ft.

Layer 2 scours in 11.3 hours. Remaining time for scour check: 24 – 4.55 – 11.3 = 8.15. Check Layer 3.

Use average velocity of 4.09 fps

Use critical shear stress of 0.68 psf

Depth prior to scour: 10.16 + 1.5 = 11.66 ft.

Layer 3 does not scour in remaining time. Therefore, predicted scour depth: 3 feet.

B-002-0-22 Soil Boring

2% AEP

Use Critical Shear Stress = 0.24 psf

Layer thickness (1.5 feet) scours in 8 hours. Remaining time for scour check = 24 hours – 8 hours = 16 hours. Check Layer 2.

Use average velocity 4.38 fps

Use critical shear stress = 0.26 psf

Depth prior to scour: 8.52 ft. + 1.5 ft. = 10.02 ft.

Predicted scour depth in remaining time: 1.16 ft. Therefore, scour depth is 1.5’ + 1.16’ = 2.66’

B-002-0-22 Soil Boring

1% AEP

Layer thickness (1.5 feet) scours in 5.5 hours. Remaining time for scour check = 24 hours – 5.5 hours = 18.5 hours. Check Layer 2.

Use average velocity of 4.86 fps

Use critical shear stress = 0.26 psf

Depth prior to scour = 8.66 ft. + 1.5 ft. = 10.16 ft.

Layer 2 scours in 17.5 hours. 1 hour remains. Check Layer 3.

Use average velocity of 4.09 fps

Use critical shear stress = 0.33 psf

Depth prior to scour = 10.16’ + 1.5’ = 11.66’

In remaining time, Layer 3 scours 0.09’. Therefore, scour depth is 1.5’ + 1.5’ + 0.09’ = 3.09’