| Input from Hydraulic Model |  |  |
| --- | --- | --- |
|  | 2% AEP (50 year) | 1% AEP (100 year) |
| Unit Discharge Upstream in Main Channel (cfs/ft) | 18.77 | 19.56 |
| Unit Discharge in Constricted Channel (cfs/ft) | 32.10 | 36.24 |
| Upstream Flow Depth (ft) | 9.36 | 9.62 |
| Flow Depth prior to Scour at Abutment (Initial depth) (ft) | 8.52 | 8.66 |
| Local Velocity (fps) | 5.44 | 6.01 |

FUL-109-16.98 PID 102931 Local Abutment Scour Calculation

| NCHRP Abutment Scour by Layer Using D50 equivalent |  |  |
| --- | --- | --- |
| Boring # | 2% AEP Scour Depth (ft) | 1% AEP Scour Depth (ft) |
| B-001-0-22 | 1.5 | 1.57 |
| B-002-0-22 | 0 | 0 |

Soil Input

| Boring | Layer # | ODOT (Modified AASHTO)/USCS Classification | Elevations (ft) | Thickness (ft) | Sampled D50 (mm) | Scour Critical Shear Stress (psf) | Scour Critical Shear Stress (Pa) = D 50 equivalent (mm) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| B-001-0-22 | 1 | A-4a Sandy Lean Clay (CL) | 739.2 | 1.5 | 0.063 | 0.027 | 1.2902 |
|  | 2 | A-6a Sandy Lean Clay (CL) | 737.7 | 1.5 | 0.017 | 0.126 | 6.0248 |
|  | 3 | A-6a Lean Clay with Sand (CL) | 736.2 | 1.5 | 0.006 | 0.680 | 32.5698 |
|  | 4 | A-6a Lean Clay with Sand (CL) | 734.7 | 1.5 | 0.007 | 0.861 | 41.2267 |
|  | 5 | A-6a Lean Clay with Sand (CL) | 733.2 | 1.5 | 0.006 | 0.632 | 30.2808 |
|  | 6 | A-6a Lean Clay with Gravel (CL) | 731.7 | 1.5 | 0.01 | 0.751 | 35.9779 |

| Boring | Layer # | ODOT (Modified AASHTO)/USCS Classification | Elevations (ft) | Thickness (ft) | Sampled D50 (mm) | Scour Critical Shear Stress (psf) | Scour Critical Shear Stress (Pa) = D 50 equivalent (mm) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| B-002-0-22 | 1 | A-6a Lean Clay with Sand (CL) | 735.8 | 1.5 | 0.006 | 0.237 | 11.3472 |
|  | 2 | A-6a Lean Clay with Sand | 734.3 | 1.5 | 0.006 | 0.261 | 12.5088 |
|  | 3 | A-6a Lean Clay with Sand (CL) | 732.8 | 1.5 | 0.006 | 0.330 | 15.7962 |
|  | 4 | A-6a Lean Clay with Sand (CL) | 731.3 | 1.5 | 0.008 | 0.429 | 20.5609 |
|  | 5 | A-6a Lean Clay with Sand (CL) | 729.8 | 1.5 | 0.008 | 0.407 | 19.5034 |
|  | 6 | A-6a Lean Clay with Sand (CL) | 728.3 | 1.5 | 0.006 | 0.430 | 20.5773 |

Boring B-001-0-22, 2% AEP, Layer 1

Layer 1 thickness: 1.5 ft.

Use D50 equivalent = 1.2902 mm

Scour hole depth (6.44 ft.) is greater than layer thickness. So layer 1 completely scours.

Boring B-001-0-22, 2% AEP, Layer 2

Layer 2 thickness: 1.5 ft.

Use D50 equivalent = 6.0248 mm

Flow depth prior to scour: 8.52 + 1.5 = 10.02 ft.

Scour is negative, which indicates that there is no scour in layer 2. Therefore, 2% AEP scour depth for Boring B-001-0-22 is 1.5’.

Boring B-001-0-22, 1% AEP, Layer 1

Layer 1 thickness: 1.5 ft.

Use D50 equivalent = 1.2902 mm

Scour hole depth (7.22 ft.) is greater than layer thickness. So layer 1 completely scours.

Boring B-001-0-22, 1% AEP, Layer 2

Layer 2 thickness: 1.5 ft.

Use D50 equivalent = 6.0248 mm

Flow depth prior to scour: 8.66 + 1.5 = 10.16 ft.

Scour hole depth is 0.07 ft. Predicted scour depth is equal to 1.5 ft. + 0.07 ft. = 1.57 ft.

Boring B-002-0-22, 2% AEP, Layer 1

Layer 1 thickness: 1.5 ft.

Use D50 equivalent = 11.347200 mm

Scour is negative, which indicates that there is no scour in Layer 1. Therefore, 2% AEP scour depth for Boring B-002-0-22 is 0’.

Boring B-002-0-22, 1% AEP, Layer 1

Layer 1 thickness: 1.5 ft.

Use D50 equivalent = 11.347200 mm

Scour is negative, which indicates that there is no scour in layer 1. Therefore, 1% AEP scour depth for Boring B-002-0-22 is 0’.