# HAM-264-10.42 – Design Analysis

PID 25349

Existing 36” storm pipe (SMU and MSD) design analysis:

- No additional water added to 36” SMU and MSD system that runs under existing bridge. Areas 1, 2, 3, 4, and 5 on drainage map all drain to existing headwall under bridge in the existing conditions.

- Catch basins redirect water slightly on how it enters the system, but does not add water to the system in total.

- Connecting existing 36” SMU line to 36” MSD line with manhole and pipe extension. There is approximately 22’ of open channel between the two pipes in the existing condition.

- Catch basins will be replaced on existing bridge in place and connect to existing pipes.  No additional or redirected water to the system.