HAS-22-5.45, PID 114190

Wall Design/Notes

7/20/2022

Limits – The failure in the road has progressed and has lengthened slightly to the east since this project was originally scoped. I measured new limits based off the surveyed boreholes. The wall will begin at sta. 251+22 and end near 253+22 depending on the shaft spacing (200’±).

Existing wall – there’s a driven pipe pile wall at the toe of slope which I’ll ignore for this analysis, assuming it doesn’t provide any support of the slope.

Geotech Exploration – we have two borings drilled in the eastbound lane in July of 2021. The borings are similar, with soft material around 12-15’ deep and bedrock 47’ (B-001) and 41’ (B-002).

Design section – I’ll use the existing cross section of sta. 252+50 which is slightly deeper fill and bedrock averaging the depth between the two boreholes.

Stability Analysis (Gstabl)

I modeled “soft” layer a little thicker than what’s shown in the borings, but I think it’s going to be necessary to get a low F.S. This area matches up with the original ground line before fill. My initial stability analysis gives F.S. = 1.078. Dropping the “Soft A-7-6” layer down slightly (75/20 down to 58/19), I get F.S. = 1.001. Failure surface: x= 22.702, y=111.868, R=85.116.

Downslope Analysis

Unadjusted with no downslope drop, F.S. = 1.127. Dropping the slope slightly (3’), leaving a 5’ cantilever, gives a F.S. = 1.456. This is assuming the pushed out toe doesn’t get removed or excavated, which I’m comfortable with since it’s within our R/W and there’s no major stream that would erode it out. Since dropping it 3’ gave such a huge jump in F.S., I’m going to assume that a 2’ drop (4’ cantilever) will give the 1.30 F.S. goal (if it’s linear, it would be 1.346).

Based on the slope of the shoulder (wall would need 2’ cantilever from existing ground line) and the anticipated 2’ drop in the lower slope, I’m going to rule out the option of using spaced shafts. We need a wall which is fully lagged, either a soldier pile with precast lagging or plug pile wall. With bedrock so deep, I anticipate a precast lagging wall will be more economical by spacing out the shafts more (5’ or 6’), but I’ll run a few options and see what we have.

UAslope Analysis

20.5’ offset (for soldier pile and concrete lagged wall)

| C/C Spacing (ft) | Load (lbs) |
| --- | --- |
| 4.0 | 13,174 |
| 4.5 | 13,925 |
| 5.0 | 14,745 |
| 6.0 | 15,407 |

21.5’ offset (for plug pile wall)

| C/C Spacing (ft) | Load (lbs) |
| --- | --- |
| 4.0 | 13,404 |
| 4.5 | 14,192 |
| 5.0 | 15,053 |
| 6.0 | 15,796 |

Lpile Analysis

Drilled Shaft, solider pile, and precast concrete lagging wall

| Offset | King Diam. | Spacing | Rock Socket | Beam | M Cap. | V Cap. | Defl (in) | Estimate (assuming 200’) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 20.5’ | 24” | 5.0’ | 8’ | HP-12x53 | 585.5% | 1607.6% | 0.45” |  |
| 20.5’ | 24” | 6.0’ | 8’ | HP-12x53 | 518.2% | 1431.7% | 0.52” | $232,720.13 |

Plug pile wall

| Offset | King Diam. | Plug Diam. | Spacing | Rock Socket | Beam | M Cap. | V Cap. | Defl (in) | Estimate (assuming 200’) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 21.5’ | 24” | 30” | 4.0’ | 8’ | W-16x40 | 806.7% | 2159.4% | 0.20” | $248,049.26 |
| 21.5’ | 24” | 36” | 4.5’ | 8’ | W-16x40 | 804.6% | 2167.9% | 0.23” | $226,838.26 |

The plug wall with 36” plugs comes out on top. I need to confirm the amount of forming necessary based on existing cross sections.

Final Design:

| Wall type | Plug Pile |
| --- | --- |
| Wall Length | 202.5’ (sta. 251+22 to sta. 253+24.50) |
| King Diameter | 24” |
| Shaft Spacing | 40.5’ |
| King Offset | 22.75’ |
| Beam Size | W-16x40 |
| Plugs | 36” diameter, 10’ deep, 21.5’ offset |
| Rock Socket | 8’ minimum |
| Beam Length | Varies, I’ll draft top-of-rock once wall profile is available |