UTILITIES

LISTED BELOW ARE ALL UTILITIES LOCATED WITHIN THE PROJECT CONSTRUCTION LIMITS TOGETHER WITH THEIR RESPECTIVE OWNERS.

(APPLIES ONLY to SLM 5.90-11.50)

AMERICAN ELECTRIC POWER CO.  (TRANSMISSION)

700 MORRISON ROAD,

GAHANNA, OHIO 43230

ATTN: MIKE CARR

614-552-1893

mdcarr@aep.com

tl_publicprojects@aep.com

AMERICAN ELECTRIC POWER CO. (DISTRIBUTION)

777 HOPEWELL DRIVE

HEATH, OHIO 43056

ATTN: PAUL PAXTON

740-348-5322

ptpaxton@aep.com

AT&T OHIO

160 NORTH SIXTH STREET

ZANESVILLE, OHIO 43701

ATTN: BARRETT TAMASOVICH

740-454-3552

BT2178@att.com

BURR OAK REGINAL WATER DISTRIBUTION

23554 JENKINS DR.

GLOUSTER, OHIO 45732

ATTN: MICHAEL ELLIOTT

740-767-2558

elliott@buroakwater.org

BRIGHTSPEED (FORMERLY LUMEN)

2025 AKRON RD.

WOOSTER, OHIO 44691

ATTN: JEFF SCHOONOVER

740-263-2819

330-262-1128

Jeffery.L.schoonover@brightspeed.com

Relocations@Brightspeed.com

COLUMBIA GAS OF OHIO

2429 LINDEN AVENUE,

ZANESVILLE OHIO 43701

ATTN: JARED FAULKNER

740-255-1318

jfaulkner@nisource.com

UTILITIES (CONT’D)

TRANSCANADA (COLUMBIA GAS TRANSMISSION)

CROSSINGS & ENCROACHMENT APPLICATIONS

700 LOUISIANA STREET

SUITE 700

HOUSTON, TEXAS 77002

800-562-8931

Us_crossings@tcenergy.com

Us_crossings@transcanada.com

NRIEGEL@NISOURCE.COM

RICHARD_STREET@TRSNSCANADA.COM

FORAKER GAS

420 S. STATE ST,

NEW LEXINGTON, OHIO 43764

740-342-2852

forakergas@yahoo.com

NATIONAL GAS AND OIL COOPERATIVE

120 O’NEIL DRIVE

HEBRON, OHIO 43025

ATTN: WILL POLING

740-641-8751

wpoling@theenergycoop.com

NELSONVILLE CABLE TV

1120 EAST CANAL STREET

NELSONVILLE, OHIO 45764

ATTN: HUNTER EDWARDS

740-753-2686

62765@nelsonvilletv.com

SPECTRUM CABLE TV

ATTN: RICK IVES

740-404-2894

Rick.Ives@charter.com

UTILITY PIPELINE LTD

4455 WALNUT ROAD

BUCKEYE LAKE, OHIO 43008

ATTN: JOSH MCLEAN

DISTRICT MANAGER

724-321-2960

jmclean@utilitypipelineltd.com

SOUTHERN PERRY COUNTY WATER DISTRICT

2075 STATE ROUTE 13

CROOKSVILLE, OHIO 43731

P.O. BOX 335

CORNING, OHIO 43730

ATTN: JONATHAN PLANT

740-347-9030

Spcwd43730@yahoo.com

# CONTINGENCY QUANTITIES

THE CONTRACTOR SHALL NOT ORDER MATERIALS OR PERFORM WORK FOR

ITEM(S) DESIGNATED BY PLAN NOTE TO BE USED "AS DIRECTED BY THE ENGINEER" UNLESS AUTHORIZED BY THE ENGINEER.  THE ACTUAL WORK LOCATIONS AND QUANTITIES USED FOR SUCH ITEMS SHALL BE INCORPORATED INTO THE FINAL CHANGE ORDER GOVERNING COMPLETION OF THIS PROJECT.

# PAVEMENT MARKINGS

ALL LONG LINE AND AUXILIARY PAVEMENT MARKINGS (STOP LINES, CROSSWALK LINES, CHANNELIZING LINES, ETC.) SHOWN IN THE PLANS ARE TAKEN FROM EXISTING LOCATIONS. THE CONTRACTOR SHALL DOCUMENT ALL PAVEMENT MARKING LOCATIONS THAT WILL BE REMOVED/ OBLITERATED DURING THIS PROJECT AND PLACE MARKINGS AT THE LOCATION OF THE EXISTING MARKINGS, UNLESS SHOWN OR STATED DIFFERENTLY IN THE PLANS AND/OR DIRECTED OTHERWISE BY THE ENGINEER.

CENTER LINE MARKINGS SHALL BE PLACED PER THE NO PASSING ZONE LOGS FOUND ON THE WEBSITE BELOW. ANY DISCREPANCIES BETWEEN THE EXISTING MARKINGS ON THE PAVEMENT AND THE LOGS SHALL BE BROUGHT TO THE ATTENTION OF THE ENGINEER PRIOR TO PLACEMENT.

https://www.transportation.ohio.gov/wps/portal/gov/odot/programs/traffic-regulations/no-passing-zones

THE CONTRACTOR SHALL NOTIFY THE PROJECT ENGINEER A MINIMUM

OF 24 HOURS PRIOR TO APPLYING PAVEMENT MARKING MATERIALS ON

ANY ROUTES SO THAT ODOT PERSONNEL MAY BE PRESENT DURING

PAVEMENT MARKING OPERATIONS.

PER CMS 641.04, THE CONTRACTOR SHALL PROVIDE ODOT PERSONNEL A COPY OF THE DLS SHORT REPORT AT THE END OF EVERY WORKDAY THROUGH CLOUD BASED REPORTING. THE CONTRACTOR SHALL NOT RECEIVE PAYMENT FOR ANY WORK DONE WITHOUT NOTIFICATION AS STATED ABOVE OR IF DSL SHORT REPORTS ARE NOT PROVIDED DAILY. SEPARATE DLS REPORTS SHALL BE PROVIDED FOR EVERY RUN OF LONG LINE MARKING PLACED. COMBINED DLS REPORTS (I.E. CENTER LINE AND EDGE LINE) WILL NOT BE ACCEPTED OR PAID BY THE DEPARTMENT.

PAVEMENT PLANING, ASPHALT CONCRETE, BY DEPTH

DEPTH OF PLANING SHALL BE AS SHOWN ON THE PAVEMENT AND SHOULDER DATA TABLES. PLANING SHALL BE THE FULL WIDTH OF THE EXISTING PAVEMENT, INCLUDING PAVED SHOULDERS. THE ROADWAY SHALL BE PLANED SUCH THAT POSITIVE DRAINAGE IS CREATED FROM THE CENTER LINE TO THE EDGE OF PAVEMENT IN TANGENT SECTIONS AND SHALL FOLLOW EXISTING SUPERELEVATIONS WHERE APPLICABLE. ALL REQUIREMENTS OF ITEM 254

SHALL APPLY.

IF DURING PLANING OPERATIONS EXCESSIVE RIDGES OR OTHER IRREGULARITIES ARE FOUND, PLANING DEPTH ADJUSTMENTS SHALL BE

MADE UP TO 3/8 INCH, AS DIRECTED BY THE ENGINEER. PAYMENT SHALL

BE INCLUDED IN THE UNIT PRICE BID PER CMS 254.07.

ITEM 203, EXCAVATION, AS PER PLAN

THIS ITEM INCLUDES THE REMOVAL OF EXISTING COMPOSITE PAVEMENT CONTAINING ASPHALT CONCRETE, BRICK, CONCRETE, AND MACADAM BASE

AS SHOWN IN THE TYPICAL SECTIONS.

PAYMENT SHALL BE MADE AT THE UNIT PRICE BID AND INCLUDE ALL LABOR, EQUIPMENT, MATERIALS, AND ANY INCIDENTALS NECESSARY TO EXCAVATE

ALL MATERIALS.

ITEM 209, LINEAR GRADING

IN ORDER TO PROVIDE POSITIVE DRAINAGE FROM THE ROADWAY SURFACE

TO THE SHOULDER BREAK, THE EXISTING ROADWAY SHOULDERS SHALL BE GRADED AND SHAPED USING A GRADER OF ADEQUATE SIZE TO PERFORM

THE WORK TO THE SATISFACTION OF THE ENGINEER.

ALL LINEAR GRADING WORK SHALL BE COMPLETED PRIOR TO PLACEMENT

OF THE ASPHALT SURFACE COURSE. NO LINEAR GRADING SHALL OCCUR

INSIDE VILLAGE LIMITS (SLM 11.50-12.57).

ALL EXCESS MATERIAL REMAINING AROUND GUARDRAIL AND OTHER AREAS AFTER THE GRADER WORK IS COMPLETED SHALL BE REMOVED AND DISPOSED

OF OFF-SITE BY THE CONTRACTOR.  ALL EQUIPMENT, LABOR, OR INCIDENTALS REQUIRED TO COMPLETE THIS ITEM SHALL BE INCLUDED FOR PAYMENT IN

THE UNIT PRICE BID FOR ITEM 209, LINEAR GRADING.

ITEM 209, LINEAR GRADING

LOCATION 1A:  9.56 MILE

LOCATION 1B:  11.20 MILE

ITEM 209, PREPARING SUBGRADE FOR SHOULDER PAVING, AS PER PLAN

BEFORE PLACEMENT OF THE SURFACE COURSE, THE CONTRACTOR SHALL

EXCAVATE AN AREA 10 INCHES WIDE OUTSIDE THE EXISTING PAVED SHOULDER

TO PROVIDE A LEVEL SURFACE FREE OF VEGETATION FOR CONSTRUCTION

OF THE SAFETY EDGE PER STANDARD DRAWING BP-3.2.

THIS ITEM SHALL ALSO BE USED IN MILL/FILL SECTIONS TO PROVIDE AN AREA

10 INCHES WIDE TO PLACE ITEM 617, COMPACTED AGGREGATE, AS PER PLAN.

ALL EXCAVATED MATERIAL SHALL BE REMOVED AND DISPOSED OFF-SITE BY

THE CONTRACTOR PRIOR TO PLACEMENT OF THE SURFACE COURSE AND

SAFETY EDGE.

ALL EQUIPMENT, LABOR, AND INCIDENTALS REQUIRED TO PERFORM

EXCAVATION OF SHOULDER SHALL BE INCLUDED FOR PAYMENT IN THE UNIT

PRICE BID FOR ITEM 209, PREPARING SUBGRADE FOR SHOULDER PAVING,

AS PER PLAN.

ITEM 253, PAVEMENT REPAIR

AN ESTIMATED QUANTITY FOR PAVEMENT REPAIR HAS BEEN INCLUDED IN

THE PLAN TO BE USED AS DIRECTED BY THE ENGINEER. REPAIRS SHALL

TAKE PLACE PRIOR TO ANY PLANING OPERATIONS.

THE INTENT OF THIS OPERATION IS TO REPAIR THOSE AREAS OF PAVEMENT

WHICH HAVE COMPLETELY FAILED (PUMPING OF SUB-BASE MATERIAL) AND

NOT TO CORRECT SURFACE IRREGULARITIES. DEPTH OF EXCAVATION SHALL

BE 7". THE MINIMUM WIDTH SHALL BE 4 FT. AFTER EXCAVATION HAS BEEN COMPLETED, THE FACE OF THE REPAIR SHALL BE COATED WITH 407 TACK COAT. REPLACEMENT MATERIAL WILL BE 7" OF ITEM 301 ASPHALT CONCRETE BASE, PG64-22 (PLACED, COMPACTED, AND TACKED IN TWO LIFTS).

REPAIR QUANTITIES MAY BE USED ON THE MAINLINE PAVEMENT OR ON

PAVED SHOULDERS. ALL EXCAVATION, MATERIALS, LABOR, EQUIPMENT,

TOOLS, TRAFFIC CONTROL AND INCIDENTALS NEEDED TO COMPLETE THE

WORK DESCRIBED ABOVE SHALL BE PAID FOR UNDER ITEM 253, PAVEMENT REPAIR.

ITEM 253, PAVEMENT REPAIR

LOCATION 1A:   500 CU.YD.

# ITEM 407, NON-TRACKING TACK COAT

THE RATE OF APPLICATION OF THE ITEM 407, NON-TRACKING TACK COAT

SHALL BE PER CMS TABLE 407.06-1 AND SUBJECT TO ADJUSTMENT AS DIRECTED BY THE ENGINEER.  PLAN QUANTITIES INDICATE AN AVERAGE APPLICATION

RATE OF 0.08 GAL/SY FOR TACK COAT UNDER THE SURFACE COURSE AND 0.05 GAL/SY UNDER INTERMEDIATE COURSE. (FOR ESTIMATING PURPOSES ONLY).

# ITEM 408, PRIME COAT, AS PER PLAN

THE CONTRACTOR SHALL APPLY ONE COAT OF MC-70 (AS PER CMS 702) AT A RATE OF 0.40 GAL/SY TO THE COMPLETED AGGREGATE SHOULDER. TO REDUCE AGGREGATE LOSS, THE PRIME COAT SHALL BE APPLIED WITHIN SEVEN (7) DAYS AFTER PLACEMENT OF THE AGGREGATE SHOULDER OR LIQUATED DAMAGES PER CMS 108.07 WILL BE ASSESSED. THE CONTRACTOR SHALL PROVIDE A SHIELD TO PREVENT THE SPRAYING OR DRIFTING OF LIQUID BITUMINOUS MATERIAL ONTO THE EDGE OF PAVEMENT OR EDGE LINE. THE ATTENTION OF THE CONTRACTOR IS DIRECTED TO 107.10 OF THE SPECIFICATIONS.

# ITEM 516, 2" DEEP JOINT SEALER, AS PER PLAN (A)

THE CONTRACTOR SHALL PLACE A 1" X 2" DEEP BEAD OF JOINT SEALER (AS

PER 705.04) AT THE LOCATIONS SHOWN IN PLANS. THE CONTRACTOR SHALL

SAW CUT A CHANNEL FOR THE JOINT SEALER. THE COST FOR SAW CUTTING

THE CHANNEL FOR THE JOINT SEALER SHALL BE INCLUDED FOR PAYMENT

WITH ITEM 516, 2" DEEP JOINT SEALER, AS PER PLAN (A).

# ITEM 617, COMPACTED AGGREGATE, AS PER PLAN

ALL AGGREGATE SHALL BE 100% CRUSHED LIMESTONE OR RECYCLED

ASPHALT CONCRETE PAVEMENT (RAP) MEETING THE REQUIREMENTS

OF 703.18.

ALL AREAS SHALL BE LOOSENED AND FREE OF VEGETATION PER 617.04

PRIOR TO PLACEMENT OF COMPACTED AGGREGATE. AGGREGATE

SHOULDERS SHALL BE SLOPED TO PROVIDE POSITIVE DRAINAGE AWAY

FROM THE ROADWAY. AGGREGATE SHOULDERS MAY BE REDUCED TO

ONE (1) FOOT WIDE WHERE NECESSARY AND MAY BE OMITTED ON

SLOPES STEEPER THAN 6:1 AT THE APPROVAL OF THE ENGINEER.

IN AREAS WHERE TOPSOIL IS ENCOUNTERED, THE WIDTH OF THE

COMPACTED AGGREGATE SHALL BE REDUCED TO THE WIDTH OF THE

EXISTING BERM. IF NO EXISTING BERM EXISTS THAN THE COMPACTED AGGREGATE SHALL BE

NON-PERFORMED.

SHOULDER PREPARATION SHALL BE INCLUDED FOR PAYMENT IN THE

UNIT PRICE BID FOR ITEM 617, COMPACTED AGGREGATE, AS PER PLAN.

# ITEM 621, RAISED PAVEMENT MARKER REMOVED

AN ESTIMATED QUANTITY HAS BEEN INCLUDED IN THE PLANS TO REMOVE

RAISED PAVEMENT MARKERS FOR DISPOSAL BY THE CONTRACTOR. RPM

REMOVAL SHALL NOT OCCUR SOONER THAN 10 DAYS PRIOR TO RESURFACING

OF THE ROADWAY. ALL RPM'S REMOVED SHALL BECOME THE PROPERTY OF

THE CONTRACTOR.

ENVIRONMENTAL NOTES

THE FOLLOWING RESTRICTION SHALL APPLY TO ALL structures in the project AREA THAT involve work over waterways:

All work is prohibited to occur below the OHWM that flows

under each structure, and no material may enter any stream during construction.

# RESIDENTIAL AND COMMERCIAL DRIVES

AN ESTIMATED QUANTITY OF ITEM 441, ASPHALT CONCRETE, HAS BEEN INCLUDED IN THE PLAN TO BE USED AS DIRECTED BY THE ENGINEER TO PAVE APPROACH AREAS TO EXISTING DRIVEWAYS. PAVING SHALL EXTEND AN AVERAGE OF 4' INTO THE DRIVEWAY (MEASURED FROM THE EDGE OF PAVEMENT OR PAVED SHOULDER IF PRESENT). THE ENGINEER MAY EXTEND PAVING DISTANCE FOR ASPHALT DRIVEWAYS IN ORDER TO PROVIDE A SMOOTH TRANSITION AND/OR ELIMINATE SHORT DISTANCES OF UNDESIRABLE PROFILE. ABRUPT CHANGES IN DRIVEWAY PROFILE ARE NOT PERMITTED.

GRAVEL DRIVES SHALL ALSO BE PAVED AS DESCRIBED ABOVE. FIELD DRIVES AND OIL WELL DRIVES SHALL NOT BE PAVED.

IF AN EXISTING APRON CANNOT BE PAVED OVER (FOR EXAMPLE, BROKEN INTO SMALL PIECES) AS DETERMINED BY THE ENGINEER, IT SHALL BE REMOVED BEFORE BEING PAVED BACK 4' INTO THE DRIVEWAY. ALL GRADING, PRIME OR TACK COAT, MATERIALS, LABOR, EQUIPMENT TOOLS AND INCIDENTALS NECESSARY TO COMPLETE THE DRIVES SHALL BE INCLUDED IN THE UNIT PRICE BID FOR THE ITEMS LISTED BELOW.

BUTT JOINTS AT THE END OF ALL DRIVEWAYS SHALL BE 1.50” IN DEPTH TO ACCOMMODATE THE SURFACE COURSE.  NO WORK SHALL BE PERFORMED ON DRIVEWAYS LOCATED IN CURB SECTIONS UNLESS OTHERWISE DIRECTED BY THE ENGINEER.

THE FOLLOWING QUANTITIES HAVE BEEN CARRIED TO THE LOCATION SUB-SUMMARIES FOR THE ABOVE-DESCRIBED PURPOSE:

# ITEM 254, PAVEMENT PLANING, ASPHALT CONCRETE, 1.50”

# LOCATION 1A:  560 SQ.YD.

# LOCATION 1B:  390 SQ.YD.

# ITEM 441, ASPHALT CONCRETE SURFACE COURSE, TYPE 1, (446),

# PG70-22M

# LOCATION 1A:  24 CU.YD.

# LOCATION 1B:  17 CU.YD.

MAILBOX TURN OUTS

AN ESTIMATED QUANTITY OF ITEM 441, ASPHALT CONCRETE, HAS BEEN

INCLUDED IN THE PLAN TO BE USED AS DIRECTED BY THE ENGINEER TO

PAVE MAILBOX TURN OUTS. TURN OUTS SHALL BE PAVED AS SHOWN IN STANDARD DRAWING BP-4.1. THE ENGINEER MAY MODIFY TO MEET

EXISTING CONDITIONS IF NECESSARY.

ALL GRADING, MATERIALS, LABOR, EQUIPMENT, AND INCIDENTALS

NECESSARY TO COMPLETE THE MAILBOX, TURN OUTS SHALL BE INCLUDED

IN THE UNIT PRICE BID FOR THE ITEMS LISTED BELOW.

# ITEM 254, PAVEMENT PLANING, ASPHALT CONCRETE, 1.50”

# LOCATION 1A:  300 SQ.YD.

# LOCATION 1B:  330 SQ.YD.

# ITEM 441, ASPHALT CONCRETE INTERMEDIATE COURSE, TYPE 2, (446)

# LOCATION 1B:   16 CU.YD.

# ITEM 441, ASPHALT CONCRETE SURFACE COURSE, TYPE 1, (446), PG70-22M

# LOCATION 1A:   13 CU.YD.

# LOCATION 1B:   14 CU.YD.

ITEM 611, CATCH BASIN ADJUSTED TO GRADE

ITEM 611, MANHOLE ADJUSTED TO GRADE

ITEM 638, VALVE BOX ADJUSTED TO GRADE

THESE ITEMS SHALL BE USED TO ADJUST CATCH BASINS, MANHOLES, AND WATER VALVE BOXES TO GRADE LOCATED THROUGHOUT THE PROJECT

LIMITS AS DESCRIBED BELOW:

EXISTING CONCRETE COLLARS SHOULD ONLY BE ADJUSTED IF BROKEN, DAMANGED, OR MISALIGNED AS DIRECTED BY THE ENGINEER. ALL ADJUSTMENTS SHALL BE AGREED ON BY THE PROJECT ENGINEER AND VILLAGE OF NEW LEXINGTON BEFORE WORK MAY BEGIN.

ANY GAS VALVE BOXES AND TELEPHONE COMPANY MANHOLES ON

THIS PROJECT SHALL NOT BE DISTURBED.

ALL MATERIALS, LABOR, EQUIPMENT, TOOLS, AND INCIDENTALS NECESSARY

TO COMPLETE THE WORK AS DESCRIBED ABOVE AND SHALL BE INCLUDED

FOR PAYMENT WITH THE ITEMS LISTED BELOW.

ITEM 611, CATCH BASIN ADJUSTED TO GRADE

LOCATION 1A:  8 EACH

ITEM 611, MANHOLE ADJUSTED TO GRADE

LOCATION 1A:  8 EACH

ITEM 638, VALVE BOX ADJUSTED TO GRADE

LOCATION 1A:  2 EACH

ITEM 632, DETECTOR LOOP, AS PER PLAN

ALL STOP LINE INDUCTANCE DETECTOR LOOPS SHALL BE THE POWER

HEAD CONFIGURATION PER TC-82.10. THE MINIMUM LENGTH SHALL

BE 20’. THE STOP LINE DETECTOR LOOPS SHALL NOT BE WIRED TO ANY

OTHER LOOPS AND SHALL HAVE ITS OWN DETECTOR CHANNEL. ALL STOP

LINE DETECTION SHALL BE TESTED FOR A BICYCLE TARGET.

ALL DILEMMA ZONE INDUCTANCE DETECTOR LOOPS SHALL BE ANGULAR

DESIGN DETECTION (ADD) PER TC-82.10. ALL DILEMMA DETECTION

ZONES SHALL BE TESTED FOR A MOTORCYCLE TARGET.

SEE SHEET 18 FOR LOCATIONS

ALL DETECTOR LOOPS SHALL BE CUT INTO THE PROPOSED INTERMEDIATE

COURSE AT A DEPTH OF 4" FROM THE PROPOSED SURFACE ELEVATION.

THE CONTRACTOR SHALL TEST ALL LEAD-IN CABLES PRIOR TO MAKING THE

FINAL SPLICE.

ALL MATERIALS, LABOR, TOOLS, EQUIPMENT, TRAFFIC CONTROL AND INCIDENTALS NECESSARY TO PERFORM THE WORK DESCRIBED ABOVE SHALL

BE INCLUDED IN THE UNIT PRICE BID FOR ITEM 632, DETECTOR LOOP, AS

PER PLAN. THE FOLLOWING QUANTITY HAS BEEN CARRIED TO THE LOCATION

SUB-SUMMARIES TO BE USED AS DIRECTED BY THE ENGINEER.

s.r. 93 @ s.r. 13/37: 2 powerhead

item 632, detector loop, as per plan

location 1a:  2 each

ITEM 874, LONGITUDINAL JOINT PREPARATION

THE FOLLOWING QUANTITY IS BEING CARRIED TO THE LOCATION SUB-

SUMMARIES FOR LONGITUDINAL JOINT PREPARATION OF THE CENTER LINE

JOINT PER SUPPLEMENTAL SPECIFICATION 874 AND STANDARD DRAWING

BP-3.1.

CL JOINT = (3.90-5.90)   = 2.00 mile

(5.90-11.50) = 5.60 mile

ITEM 874, LONGITUDINAL JOINT PREPARATION

Location 1A:  2.00 miLE

Location 1B:  5.60 miLE

ITEM SPECIAL, REINFORCED MESH FOR TRANSVERSE AND/OR LONGITUDINAL

JOINTS AND CRACKS

THIS ITEM SHALL BE USED TO REINFORCE THE LONGITUDINAL WIDENING JOINT ALONG S.R. 93 FROM SLM 5.90 TO 11.50 IN BOTH THE NORTHBOUND AND SOUTHBOUND DIRECTIONS. PLACE REINFORCING MESH ON PLANED SURFACE, 5.0' WIDE CENTERED OVER LONGITUDINAL JOINT. THIS WORK SHALL BE PERFORMED ON ALL WIDENING JOINTS THROUGHOUT THE SECTION, INCLUDING RADIUS RETURNS. REINFORCING MATERIAL SHALL BE GLASGRID CG100 OR EQUIVALENT AND SHALL BE PLACED ACCORDING TO MANUFACTURER'S SPECIFICATIONS AND THIS NOTE.

the ASPHALT concrete INTERMEDIATE COURSE SHALL BE PLACED ON

the MESH BEFORE OPENING TO TRAFFIC.

ALL MATERIALS, LABOR, EQUIPMENT, TOOLS, TRAFFIC CONTROL AND INCIDENTALS NEEDED TO COMPLETE THE WORK DESCRIBED ABOVE SHALL

BE INCLUDED FOR PAYMENT IN THE UNIT PRICE BID FOR ITEM SPECIAL, REINFORCED MESH FOR TRANSVERSE AND/OR LONGITUDINAL JOINTS AND CRACKS

GUARDRAIL WORK

ALL MATERIALS EXCAVATED FOR POST HOLES AND/OR CONCRETE ANCHORS SHALL BE REMOVED AND DISPOSED OF IN ACCORDANCE WITH CMS 503.07 WITH THE AREA RESTORED TO A NEAT CONDITION SATISFACTORY TO THE ENGINEER.

ALL HOLES REMAINING AFTER REMOVAL OF GUARDRAIL POSTS SHALL BE

FILLED WITH GRANULAR MATERIAL MEETING CMS 203.03R.  FILL MATERIAL CONTAINING SOD SHALL NOT BE USED.  ALL FILL MATERIAL SHALL BE

APPROVED BY THE ENGINEER.  MATERIAL PLACED IN THE HOLES SHALL BE THOROUGHLY COMPACTED AND LEVELED OFF AS DIRECTED BY THE ENGINEER.

CONCRETE POST ANCHORS SHALL BE REMOVED A MINIMUM OF 1’ BELOW EXISTING GRADE AND NOT CONFLICT WITH ANY PROPOSED WORK.

THE LOCATION OF GUARDRAIL RUNS SPECIFIED IN THE PLANS ARE SUBJECT TO ADJUSTMENTS TO ASSURE THAT THE PLANNED INSTALLATION WILL AFFORD MAXIMUM PROTECTION FOR TRAFFIC AND REDUCE THE RISK OF FUTURE

EROSION ISSUES.

PAYMENT FOR THE ABOVE WORK SHALL BE INCIDENTAL TO THE APPLICABLE GUARDRAIL OR ANCHOR ASSEMBLY ITEM.

# ITEM 202, GUARDRAIL REMOVED, AS PER PLAN

# THIS ITEM SHALL CONSIST OF REMOVING EXISTING GUARDRAIL IN

# ACCORDANCE WITH CMS EXCEPT THE EXISTING GUARDRAIL PANELS AND

# POSTS SHALL BE CAREFULLY DISMANTLED AND STORED TO BE USED AS

# EROSION PROTECTION AS STATED IN ITEM 606 GUARDRAIL, MISC.:

# SHOULDER SUPPORT BENCHING PLAN NOTE.

# EXISTING PANELS AND/OR POSTS THAT ARE DAMAGED OR DEEMED UNNECCESSARY BY THE ENGINEER SHALL BE DISPOSED OF ACCORDING TO

# CMS 202.02. HAULING OF GUARDRAIL PANELS AND POSTS FROM ONE

# LOCATION TO WHERE THEY ARE NEEDED SHALL BE INCLUDED IN THE UNIT

# BID PRICE. ALL OTHER HARDWARE NOT BEING USED (I.E. BLOCKOUT, BOLTS, WASHERS, ETC.) SHALL BE DISPOSED OFF-SITE AT THE EXPENSE OF THE CONTRACTOR.

# PAYMENT SHALL BE AT THE UNIT BID PRICE AND INCLUDE ALL LABOR,

# HAULING, MATERIALS, EQUIPMENT, AND INCIDENTALS NECESSARY TO

# COMPLETE THE WORK AS DESCRIBED ABOVE.

# ITEM 606 GUARDRAIL, MISC.:  SHOULDER SUPPORT BENCHING

THIS ITEM SHALL CONSIST OF INSTALLING PREVIOUSLY REMOVED GUARDRAIL PANELS AND POSTS (ITEM 202, GUARDRAIL REMOVED, AS PER PLAN) BY

# PLACING THEM TWO (2) FEET BEHIND NEWLY INSTALLED ITEM 606 GUARDRAIL, TYPE MGS WITH LONG POSTS (ITEMIZED SEPARATELY) TO PROVIDE PROPER BENCHING AND RETAIN GRADING LIMITS BEHIND GUARDRAIL.

AFTER INSTALLATION OF EXISTING PANEL AND POST, GEOTEXTILE FABRIC SHALL

BE PLACED AT A WIDTH OF 6.25’ (OVERLAPPING EXISTING RAIL) AND BACKFILLED WITH RECYCLED ASPHALT CONCRETE PAVEMENT (RAP) MEETING THE REQUIREMENTS OF 703.18, AS SHOWN IN DETAIL ON SHEET 27.

# ADDITIONAL PANELS MAY BE NECESSARY TO ACHIEVE A MAXIMUM 2:1 SLOPE,

# AS DIRECTED BY THE ENGINEER.

# PAYMENT SHALL BE AT THE UNIT BID PRICE AND INCLUDE ALL LABOR,

# HAULING, MATERIALS, EQUIPMENT, AND INCIDENTALS NECESSARY TO

# COMPLETE THE WORK AS DESCRIBED ABOVE.

ITEM 606, ANCHOR ASSEMBLY, MGS TYPE B (NCHRP 350)

ITEM 606, ANCHOR ASSEMBLY, MGS TYPE E (NCHRP 350 OR MASH 2016)

THIS ITEM SHALL CONSIST OF FURNISHING AND INSTALLING ANY OF THE

GUARDRAIL END TERMINALS FOR TYPE MGS GUARDRAIL AS LISTED ON ROADWAY ENGINEERING'S WEB PAGE UNDER ROADSIDE SAFETY DEVICES FOR APPROVED GUARDRAIL END TREATMENTS:

https://www.transportation.ohio.gov/wps/portal/gov/odot/working/engineering/roadway/manuals-standards/roadway-approved-products

INSTALLATION SHALL BE AT THE LOCATIONS SPECIFIED IN THE PLANS, IN ACCORDANCE WITH THE MANUFACTURER'S SPECIFICATIONS. THE FACE OF THE IMPACT HEAD SHALL BE COVERED WITH A SHEET OF TYPE G REFLECTIVE SHEETING, PER CMS 730.19.

REFER TO THE MANUFACTURER'S INSTRUCTIONS REGARDING THE INSTALLATION OF, AND THE GRADING AROUND THE FOUNDATION TUBES AND GROUND STRUT. THE TOP OF ANY FOUNDATION TUBE SHOULD BE LESS THAN 4 INCHES ABOVE THE GROUND. THE PLACEMENT OF THE FOUNDATION TUBES SHOULD BE AN APPROPRIATE DEPTH BELOW THE LEVEL LINE IN ORDER TO MAINTAIN THE FINISHED GUARDRAIL HEIGHT OF 31 INCHES FROM THE EDGE OF THE SHOULDER.

ON-SITE GRADING IS REQUIRED IF THE TOP OF THE FOUNDATION TUBES OR TOP OF THE GROUND STRUT DOES PROJECT MORE THAN 4 INCHES ABOVE THE GROUND LINE.

PAYMENT FOR THE ABOVE WORK SHALL BE MADE AT THE UNIT PRICE BID FOR ITEM 606, ANCHOR ASSEMBLY, MGS, BY TYPE, EACH, AND SHALL INCLUDE ALL LABOR, TOOLS, EQUIPMENT AND MATERIALS NECESSARY TO CONSTRUCT A COMPLETE AND FUNCTIONAL ANCHOR ASSEMBLY SYSTEM, INCLUDING ALL RELATED TRANSITIONS, REFLECTIVE SHEETING, HARDWARE, GRADING, EMBANKMENT AND EXCAVATION NOT SEPARATELY SPECIFIED, AS REQUIRED

BY THE MANUFACTURER.

THE GRADING LAYOUT SHOWN BELOW SHALL BE USED IN ADDITION TO STANDARD DRAWING MGS-5.2 (TYPE B):

THE GRADING LAYOUT SHOWN BELOW SHALL BE USED IN ADDITION TO STANDARD DRAWING MGS-5.3 (TYPE E):

ITEM 623, CONSTRUCTION LAYOUT STAKES AND SURVEYING, AS PER PLAN

IN ADDITION TO THE REQUIREMENTS OF ITEM 623 CONSTRUCTION LAYOUT STAKES AND SURVEYING, THE CONTRACTOR SHALL PROVIDE THE FOLLOWING INFORMATION TO THE DEPARTMENT:

THE CONTRACTOR SHALL PROVIDE AS-BUILT DATA FOR THE SPECIFIED COMPLETED CONSTRUCTION ITEMS IN OHIO STATE PLANE COORDINATES (GRID).  THE CONSTRUCTION ITEMS SHALL BE LOCATED AS PER THE SURVEY FEATURE CODE LIST FOUND ON THE OHIO DEPARTMENT OF TRANSPORTATION OFFICE OF CADD & MAPPING SERVICES WEBSITE. AN EMAIL CONTAINING A COMMA DELIMITED ASCII FILE AND A SURVEYOR'S CERTIFICATION SHALL BE DELIVERED TO Cody.Gierhart@dot.ohio.gov WITH THE PROJECT ENGINEER AND AREA ENGINEER COPIED ON THE EMAIL. AFTER ALL INFORMATION HAS BEEN COLLECTED. THE ASCII FILE SHALL INCLUDE A HEADER CONTAINING NAME OF SURVEYOR, DATE(S) OF COLLECTION, HORIZONTAL DATUM (I.E. NAD83 (2011), OHIO STATE PLANE COORDINATE SYSTEM NORTH OR SOUTH), VERTICAL DATUM (I.E. NAVD 88, GEOID12A) AND METHOD OF COLLECTION (I.E. OHIO VRS, GPS RTK, TOTAL STATION, ETC.) AND BE IN A TABLE FORM AS FOLLOWS:

POINT NUMBER, NORTHING, EASTING, ELEVATION, FEATURE CODE, DESCRIPTION

BELOW IS A LIST OF THE ITEMS THE CONTRACTOR IS REQUIRED TO PROVIDE

FOR THE PROJECT:

- GUARDRAIL

- SIGNS

THE ABOVE ITEMS SHALL BE COLLECTED USING SURVEY GRADE EQUIPMENT MEETING THE REQUIREMENTS OF SECTION 400 IN THE OHIO DEPARTMENT OF TRANSPORTATION SURVEY & MAPPING SPECIFICATIONS MANUAL.

THE CONTRACTOR SHALL NOT DISTURB ANY RIGHT-OF-WAY MONUMENTATION AS SHOWN IN THE CENTERLINE PLAT. ANY DISTURBED

PINS SHALL BE RESET AT THE CONTRACTOR’S EXPENSE.

ALL COST ASSOCIATED WITH OBTAINING THE INFORMATION LISTED ABOVE SHALL BE INCLUDED IN THE LUMP SUM BID FOR ITEM 623 CONSTRUCTION LAYOUT STAKES AND SURVEYING, AS PER PLAN.

CADD FILES

CADD FILES SHOWING EXISTING SURVEY AND CENTER LINE ALIGNMENT/ STATIONING CAN BE FOUND AT THE FTP LOCATION BELOW

https://www.dot.state.oh.us/Divisions/ContractAdmin/Contracts/Pages/designfiles.aspx

THE PROPOSED CENTER LINE OF CONSTRUCTION IS BASED ON THE EXISTING CENTER LINE MARKING OBSERVED IN THE FIELD, AT APPROVAL OF THE ENGINEER.