UTILITY NOTE

PIC-22-17.12 PID 113673

July 29, 2022

- Bidders are advised that the following utility facilities shall remain in place and in service during the construction of the project as set out below.

- All station locations listed below are approximate unless otherwise stated.

- Stations are based on the Centerline of Construction for U.S. 22.

ACD.Net

Shirley Yohnka

517-999-3240

YOHNKA.SHIRLEY@ACD.NET

Per communications with Shirley Yohnka, ACD.Net aerial fiber is located on the east side of the at-grade Norfolk Southern railroad tracks, east of the construction limits. No relocations required.

AEP Ohio (Distribution)

Clarke Saunders

28831 State Route 7

Reedsville, OH 45772

740-985-3054

CMSaunders@AEP.com

AEP Ohio (Transmission)

Michael D. Carr

8600 Smiths Mill Road

New Albany, OH 43054

380-205-5072

TL_PublicProjects@AEP.com

AEP has distribution lines on the south side of the project limits and large transmission lines going over the project east of U.S. 23 and running north-south. Clearance beneath the overhead transmission is expected to be large enough to not be an issue, but the Contractor shall use caution and be aware of the overhead transmission lines during construction and observe proper clearances. No relocations required.

City of Circleville – Water & Sewer

Josh Anderson

802 S. Pickaway Street

Circleville, OH 43113

740-477-6764

Janderson@circlevilleoh.gov

Waterline is primarily located on the south side of U.S. 22 and northeast of the Island Road / U.S. 22 intersection. Based on subsurface utility engineering, the waterline should be deep enough to maintain the minimum cover requirements relative to the proposed construction. No relocations are required. Contractor to exercise caution as the facility will remain in place for the duration of the project.

Charter Communications (Spectrum)

Joseph Bailey

740-648-3099 (Desk) / 740-253-0174 (Cell)

Joseph.Bailey@charter.com

Charter has facilities located on two poles: one pole in the northwest corner of the Cargill lot (Sta. 122+80, 134’ RT) and one pole approximately 100’ north of U.S. 22 on the east side of Island Road (Sta. 125+15, 161’ LT). These poles are outside of the project. No relocations required.

Columbia Gas

Hudson Park

843 Piatt Street

Chillicothe, OH 45601

740-637-9378

hpark@nisource.com

Columbia Gas has gas lines running along the south side of U.S. 22, with crossings under U.S. 22 near Sta. 123+40 that continues to the north under the existing slip ramp as well as another crossing near Sta. 126+25. The gas line crossing at Sta. 123+40 is an 8” coated steel high pressure main. On the high side of the U.S. 22 pavement (left of Centerline of Construction U.S. 22), the clearance between the bottom of the proposed pavement buildup and the top of the existing 8” gas line is as little as 1.6’. Although the minimum cover is expected to be maintained between the existing gas lines and the proposed construction, the Contractor shall exercise caution near gas mains, especially the shallow areas as mentioned in this note.  Hand digging is suggested where possible.  No relocations required.

Frontier (Verizon)

Travis Brannon

740-835-6825

Travis.l.brannon@ftr.com

Frontier has a crossing under U.S. 22 near Sta. 113+75 as well as copper/fiber lines on the north side of U.S. 22. The fiber and copper run parallel from a manhole just west of the CSX railroad bridge and crossing under the on-ramp/off-ramp to/from U.S. 23 (loop ramp). The copper and fiber then run through two manholes west of the U.S. 23 bridge (Sta. 118+50, 62’ LT and Sta. 118+49, 44’ LT) and continue eastward just inside of the existing curb line beneath the pavement to another manhole located at Sta. 121+25, 57’ LT. East of the U.S. 23 bridge, the copper and fiber lines begin to separate and continue towards the northeast. There is also an existing manhole in the infield created by the slip ramp onto U.S. 23 north.

There is potential for conflict where the copper and fiber are running beneath the pavement between the manholes Sta. 118+49, 44’ LT and Sta. 121+25, 57’ LT as there will be approximately 2’ of excavation for the proposed pavement work. Based on measurements taken at the manholes mentioned in the previous sentence, the estimated depth of the fiber/copper is at least 3.5’ deep. Therefore, it is estimated that there will be 1.5’ of cover between the fiber/copper lines and the bottom of the proposed aggregate base for the pavement.

Although there is not expected to be a conflict during construction, the Contractor shall exercise caution to avoid damaging the copper and fiber lines. No relocations required.

Horizon

Zach Goebbel

68 E. Main Street

Chillicothe, OH, 45601

740-993-9793

Zach.goebbel@horizonconnects.com

Horizon is located on the south side of U.S. 22 and crosses beneath the southbound U.S. 23 on-ramp (for eastbound U.S. 22 traffic) and under Mound Street before continuing south on Mound Street. The existing depths range from 7 feet to 13 feet within the project area. No relocations required.

Mox Networks

Dave Allison

2040 Mariposa Avenue

El Segundo, CA 90245

740-794-1678

Cwa4324pres@gmail.com

Mox is located on the east side of the CSX railroad tracks and runs north-south parallel with the tracks with a crossing under U.S. 22 near Sta. 115+50. The existing depths range from 4.5 feet to 7 feet within the project area. No relocations required.

T-Mobile

Steven Hughes

3007 Tucumcari Drive

Sardinia, OH 45171

513-459-5796 / 814-553-2300 (Cell)

Steven.Hughes1@T-Mobile.com

There is a T-Mobile line on the west side of the CSX railroad going under U.S. 22 at Sta. 114+14. No conflicts are anticipated.  Contractor shall especially exercise caution near Sta. 114+20, 15’ LT for the full depth shoulder construction, as the T-Mobile line passes beneath at near that location. No relocations required.