August 14, 2025

Attn: James Stafa

RE: PID 121076 - Survey Task

Project Identification:

PIC-159-0.00, PID #121076

Resurfacing of SR 159 from the Ross County Line (SLM 0.00) to the Fairfield County Line (SLM 9.39).

Services Requested:

Full-topographic survey

Overview:

See photos and maps starting on page 3 of this document for detailed locations of the proposed survey limits. Please provide data in the Pickaway County coordinate system.

- Location 1: SLM 3.998, Address 27784

- Location 2: SLM 6.068, Address 26530

- Location 3: SLM 6.143, Address 26480

Please collect data to the level of detail needed for In-House Design to proceed onto Detailed Design and Right-of-Way Plan preparation if required. This project will be done in ProjectWise and Open Roads Designer (ORD). All files to be in the format of the latest ODOT supported edition.

Detailed Description:

- Provide Horizontal Control Reference Points (minimum of 3)

- Provide Vertical Control Benchmarks (minimum of 3)

- Survey data collection to include:

  - Data necessary for R/W plan development and acquisition (CL R/W, parcels within and adjacent to limits, etc.)

  - Underground utilities, including pedestal locations

  - Overhead utility heights/elevations where lines cross SR 95

  - Existing signage

  - Trees, including type and diameter

  - Invert/flow line, size, and type of drainage items including but not limited to culverts, storm sewers, catch basins, underdrains and drive pipes.

  - Monument boxes

  - Topographic survey of Centerline of Road, Edge of pavement, Edge of shoulder, any shoulder breaks and ditch lines.

  - Driveways and side street break lines and elevation shots to the L/A or RW.

- Survey department to Contact OUPS prior to data collection to flag/mark utilities.

Existing Information

- Existing Construction Plans are available:

Deliverables

Survey Task

  - Survey Basemap *.DGN file (2-D file, Features plotted @ 20 Scale).

  - The ORD files should contain appropriate breaklines at all applicable features such as; Utility Locations, Channels, Culvert Invert Elevations, Drives, etc…

  - Microsoft Excel Spreadsheet containing the following:

- A table with all Horizontal and Vertical Control Points including: Pt ID, Northing, Easting, Elevation, Feature Code, and Detailed Location Description.

- Horizontal and Vertical Datum utilized.

- If needed, provide grid to ground conversion factor and point to be used for conversion.

- Clearly indicate which points may be used for construction plan benchmark/reference mark use (a minimum of three must be provided).

  - Provide supplemental copy of field sketches of reference point locations.

  - Provide a Comma Separated Value (CSV - Text file) of the raw survey points containing all information recorded with each point (sometimes some description information is lost in post-processing)

  - Provide the Completed “Survey Parameter” Note (Note G105) from the L&D Volume 3, Appendix B.  See link:

http://tas.transportation.ohio.gov/CADD/VOL3/VOL3_APPENDIX_B_2020_07.pdf

ODOT Project Contacts

Katie Montoya, PM, 740-833-8236

Completion Date

Survey Task – Field work complete and data delivered by 9/15/25.

Location 1: SLM 3.998, Address 27784

  - 1 Tree and stump to be removed – Approximately 36” Dia. (tree within yellow box, survey limits will extend outside of this box).

Location 2: SLM 6.068, Address 26530

  - 2 Trees and stumps to be removed – Approximately 40” Dia. (trees within yellow box, survey limits will extend outside of this box).

Location 3: SLM 6.143, Address 26480

  - 5 Trees and stumps to be removed – Approximately 36”-40” Dia. (trees within yellow box, survey limits will extend outside of this box).