PIK-220-0564 OPUS CP03(B)

IRON PIN & CAP SET

PID: 115772

FILE: 87101451.DAT OP1653561117041

2005   NOTE:  The IGS precise and IGS rapid orbits were not available

2005   at processing time.  The IGS ultra-rapid orbit was/will be used to

2005   process the data.

2005

NGS OPUS-RS SOLUTION REPORT

========================

All computed coordinate accuracies are listed as 1-sigma RMS values.

For additional information: https://gcc02.safelinks.protection.outlook.com/?url=https%3A%2F%2Fwww.ngs.noaa.gov%2FOPUS%2Fabout.jsp%23accuracy&amp;data=05%7C01%7Cjason.robinson%40dot.ohio.gov%7C38db3fa4afe44026e22908da3f03637d%7C50f8fcc494d84f0784eb36ed57c7c8a2%7C0%7C0%7C637891581052899648%7CUnknown%7CTWFpbGZsb3d8eyJWIjoiMC4wLjAwMDAiLCJQIjoiV2luMzIiLCJBTiI6Ik1haWwiLCJXVCI6Mn0%3D%7C3000%7C%7C%7C&amp;sdata=nCY9t10iLJSXnQz9gqb9%2BroS8ZKtzGe3HG%2Bpm41YiPA%3D&amp;reserved=0

USER: jason.robinson@dot.ohio.gov              DATE: May 26, 2022

RINEX FILE: 8710145q.22o                             TIME: 10:34:11 UTC

SOFTWARE: rsgps  1.38 RS292.prl 1.99.3            START: 2022/05/25 16:22:00

EPHEMERIS: igu22113.eph [ultra-rapid]               STOP: 2022/05/25 17:02:15

NAV FILE: brdc1450.22n                         OBS USED:  3258 /  3474   :  94%

ANT NAME: TRMR8           NONE              QUALITY IND.   4.25/ 24.27

ARP HEIGHT: 2.000                         NORMALIZED RMS:        0.355

REF FRAME: NAD_83(2011)(EPOCH:2010.0000)              ITRF2014 (EPOCH:2022.39643)

X:       601461.953(m)   0.005(m)            601461.018(m)   0.005(m)

Y:     -4918044.007(m)   0.023(m)          -4918042.590(m)   0.023(m)

Z:      4003204.037(m)   0.013(m)           4003203.958(m)   0.013(m)

LAT:   39  7 30.69941      0.007(m)        39  7 30.72850      0.007(m)

E LON:  276 58 20.91803      0.005(m)       276 58 20.88656      0.005(m)

W LON:   83  1 39.08197      0.005(m)        83  1 39.11344      0.005(m)

EL HGT:          151.022(m)   0.026(m)               149.793(m)   0.026(m)

ORTHO HGT:  	605.17’        184.457(m)   0.031(m) [NAVD88 (Computed using GEOID18)]

UTM COORDINATES    STATE PLANE COORDINATES

UTM (Zone 17)         SPC (3402 OH S)

Northing (Y) [meters]     4332626.937           125044.574	Grid: 410,250.4065	Ground: 410,282.2501

Easting (X)  [meters]      324735.529           554385.958            1,818,847.9300                      1,818,989.1090

Convergence  [degrees]    -1.27972778          -0.33472222

Point Scale                0.99997824           0.99994608

Combined Factor            0.99995455           0.99992239	P.A.F. = 1.00007762

US NATIONAL GRID DESIGNATOR: 17SLD2473632627(NAD 83)

BASE STATIONS USED

PID       DESIGNATION                        LATITUDE    LONGITUDE DISTANCE(m)

DI1856 OHHO HOCKING COUNTY CORS ARP        N393207.276 W0822637.876   67870.4

DF4048 GALP GALLIPOLIS CORS ARP            N385039.148 W0821640.091   72062.1

DI1876 OHAD ADAMS COUNTY CORS ARP          N384709.347 W0833322.647   59324.9

DR6981 OHFY FAYETTE COUNTY 2 CORS ARP      N393133.473 W0832834.395   58965.1

AI4945 STKR STOCKER OHIO UNIV CORS ARP     N391933.824 W0820625.629   82547.8

DH9001 WVHU MARSHALL UNIV-HUN CORS ARP     N382522.617 W0822528.918   93924.4

DF4050 GRTN GEORGETOWN CORS ARP            N385134.330 W0835244.711   79453.9

AJ7184 COLB COLUMBUS CORS ARP              N395735.112 W0830244.747   92672.6

DI1858 OHLI LICKING COUNTY CORS ARP        N395709.138 W0822451.031  105915.6

NEAREST NGS PUBLISHED CONTROL POINT

JY1899      PETE AZ MK                     N390751.815 W0830020.134    2005.0

This position and the above vector components were computed without any knowledge by the National Geodetic Survey regarding the equipment or field operating procedures used.