LEAP Bridge Input for Prestressed Concrete Box Beam B 27x48 Design

State Route 523 over little Muddy Creek

Units

| cft = ft3 | Cyd = yd3 | sft = ft2 | si = in2 | plf = lb/linear ft | klf = kip/linear ft | conc= 150 pcf |
| --- | --- | --- | --- | --- | --- | --- |

Legend

Structural Data

| Limits = 68 ft | Limit of structure between abutment bearings |
| --- | --- |
| nSpans = 1 | Number of Spans |
| nPiers = 0 | Number of Piers |
| nBeams= 7 | Number of Beams |
| WBridge = 28 ft | Width of Bridge |
| LBrg.Offset.Abut = ________ | Distance from CL Abut to CL Brg |
| LAbut.Offset = 1in | Length from Abutment CL to Bearing CL |
| SpaBeams = 0in | Beam Spacing |
| Skew = 30.0 deg | Skew |
| THaunch = 0in | Haunch Thickness |

Beam Geometry

See ODOT Standard Drawing PSBD-2-07 (Beam CB 27-48)

| wt.flange = | Top Flange Width |
| --- | --- |
| ht.flange = | Top Flange Heigh |
| hbeam = | Beam Height |
| tweb.interior.beam = | Web Width |
| tweb.exterior.beam = |  |
| wb.flange = |  |
| hb.flange = |  |
| wtType.4.Mod = ______ plf |  |
| AType.4.Mod = | Cross Sectional Area of Beam |

Dead Load Intensity of Superstructure Railing

See ODOT Standard TST-2-21 for Railing Details

Per page 15/15 in TST-2-21, the unit weight of the bridge railing is 80lbs/ft based on an 8-ft post spacing.

The weight of each end transition from the railing limit to the centerline of splice is 750 lbs.

80lbs/ft was placed in the Permanent Load Wizard for each side of the bridge.

Dead Load Intensity of Superstructure Future Wearing Surface (FWS)

fws = 0.06 ksf

This was found in the ODOT BDM on pages 3-183, 3-184, and 10-3.

Dead Load Superstructure Diaphragms

Diaphragm dimensions and calculations are based on ODOT Standard Drawing PSBD-02-07.

Volume of void areas were included as part of overall diaphragm weight for a conservative approach and to simplify calculations.

Beam 1 is the southern most beam.  Beat 7 is the northern most beam.

Diaphragms were numbered from west to east.

2 intermediate diaphragms were used as required per PSBD 2-07 for spans between 50ft and 75ft, outlined on page 2/4.

Measurement for ‘Y’ was taken from page 1 to be 3’3” for a 27” tall beam.

Beam 1 (fascia)

Diaphragm 1:

- Load of square section: 5.31ft x 4ft x(27in/12 depth of beam) =47.79cft.  47.79cft x 150pcf = 7168.6lbs = 7.2 kips

- Load of triangular section: 2.31ft x 4ft x (27in/12) x 0.5 = 10.395cft.  10.395 x 150 = 1559.25lbs = 1.56 kips

- Total Load: 7.2kips + 1.56 kips = 8.76 kips

- Distance from left bearing line: Use center of square section = 3.31ft

Diaphragm 2

- Load: 4ft x 1ft x 2.25 ft = 9cft.  9 cft x 150pcf = 1350lbs = 1.35 kips

- Distance from left bearing line: 23.9ft

Diaphragm 3

- Load: 4ft x 1ft x 2.25 ft = 9cft.  9 cft x 150pcf = 1350lbs = 1.35 kips

- Distance from left bearing line: 44.85ft

Diaphragm 4

- Load of square section: 3.25ft x 4ft x 2.25ft = 29.25 cft. 29.25 cft x 150 pcf = 4387.5 lbs = 4.39kips

- Load of triangular section: 2.309ft x 4ft x 2.25ft x 0.5 = 10.39cft.  10.39cft x 150pcf = 1558.5lb = 1.56 kips

- Total Load: 4.39 kips + 1.56 kips = 5.95 kips

- Distance from left bearing line: Use center of square section = Assume center of square section.  66.06ft

Beam 2 (interior)

Diaphragm 1:

- Load of square section: 5.31ft x 4ft x 2.25ft = 47.79 cft. 47.79 x 150 = 7168.5lbs = 7.2 kips

- Load of triangular section: 2.31ft x 4ft x (27in/12) x 0.5 = 10.395cft.  10.395 x 150 = 1559.25lbs = 1.56 kips

- Total Load: 7.2kips + 1.56 kips = 8.76 kips

- Distance from left bearing line: Use center of square section = 3.31ft

Diaphragm 2

- Load: 3.31ft x 4ft x 2.25 ft = 29.79cft.  29.79 cft x 150pcf = 4468.5lbs = 4.5 kips

- Distance from left bearing line: 22.74ft

Diaphragm 3

- Load: 3.31ft x 4ft x 2.25 ft = 29.79cft.  29.79 cft x 150pcf = 4468.5lbs = 4.5 kips

- Distance from left bearing line: 43.7ft

Diaphragm 4

- Load of square section: 5.31ft x 4ft x 2.25ft = 47.79 cft. 47.79 x 150 = 7168.5lbs = 7.2 kips

- Load of triangular section: 2.31ft x 4ft x (27in/12) x 0.5 = 10.395cft.  10.395 x 150 = 1559.25lbs = 1.56 kips

- Total Load: 7.2kips + 1.56 kips = 8.76 kips

- Distance from left bearing line: Use center of square section = 65ft

Beam 3 (interior)

Diaphragm 1:

- Load of square section: 5.31ft x 4ft x 2.25ft = 47.79 cft. 47.79 x 150 = 7168.5lbs = 7.2 kips

- Load of triangular section: 2.31ft x 4ft x (27in/12) x 0.5 = 10.395cft.  10.395 x 150 = 1559.25lbs = 1.56 kips

- Total Load: 7.2kips + 1.56 kips = 8.76 kips

- Distance from left bearing line: Use center of square section = 3.31ft

Diaphragm 2

- Load: 3.31ft x 4ft x 2.25 ft = 29.79cft.  29.79 cft x 150pcf = 4468.5lbs = 4.5 kips

- Distance from left bearing line: 22.74ft

Diaphragm 3

- Load: 3.31ft x 4ft x 2.25 ft = 29.79cft.  29.79 cft x 150pcf = 4468.5lbs = 4.5 kips

- Distance from left bearing line: 43.7ft

Diaphragm 4

- Load of square section: 5.31ft x 4ft x 2.25ft = 47.79 cft. 47.79 x 150 = 7168.5lbs = 7.2 kips

- Load of triangular section: 2.31ft x 4ft x (27in/12) x 0.5 = 10.395cft.  10.395 x 150 = 1559.25lbs = 1.56 kips

- Total Load: 7.2kips + 1.56 kips = 8.76 kips

- Distance from left bearing line: Use center of square section = 65ft

Beam 4 (interior)

Diaphragm 1:

- Load of square section: 5.31ft x 4ft x 2.25ft = 47.79 cft. 47.79 x 150 = 7168.5lbs = 7.2 kips

- Load of triangular section: 2.31ft x 4ft x (27in/12) x 0.5 = 10.395cft.  10.395 x 150 = 1559.25lbs = 1.56 kips

- Total Load: 7.2kips + 1.56 kips = 8.76 kips

- Distance from left bearing line: Use center of square section = 3.31ft

Diaphragm 2

- Load: 3.31ft x 4ft x 2.25 ft = 29.79cft.  29.79 cft x 150pcf = 4468.5lbs = 4.5 kips

- Distance from left bearing line: 22.74ft

Diaphragm 3

- Load: 3.31ft x 4ft x 2.25 ft = 29.79cft.  29.79 cft x 150pcf = 4468.5lbs = 4.5 kips

- Distance from left bearing line: 43.7ft

Diaphragm 4

- Load of square section: 5.31ft x 4ft x 2.25ft = 47.79 cft. 47.79 x 150 = 7168.5lbs = 7.2 kips

- Load of triangular section: 2.31ft x 4ft x (27in/12) x 0.5 = 10.395cft.  10.395 x 150 = 1559.25lbs = 1.56 kips

- Total Load: 7.2kips + 1.56 kips = 8.76 kips

- Distance from left bearing line: Use center of square section = 65ft

Beam 5 (interior)

Diaphragm 1:

- Load of square section: 5.31ft x 4ft x 2.25ft = 47.79 cft. 47.79 x 150 = 7168.5lbs = 7.2 kips

- Load of triangular section: 2.31ft x 4ft x (27in/12) x 0.5 = 10.395cft.  10.395 x 150 = 1559.25lbs = 1.56 kips

- Total Load: 7.2kips + 1.56 kips = 8.76 kips

- Distance from left bearing line: Use center of square section = 3.31ft

Diaphragm 2

- Load: 3.31ft x 4ft x 2.25 ft = 29.79cft.  29.79 cft x 150pcf = 4468.5lbs = 4.5 kips

- Distance from left bearing line: 22.74ft

Diaphragm 3

- Load: 3.31ft x 4ft x 2.25 ft = 29.79cft.  29.79 cft x 150pcf = 4468.5lbs = 4.5 kips

- Distance from left bearing line: 43.7ft

Diaphragm 4

- Load of square section: 5.31ft x 4ft x 2.25ft = 47.79 cft. 47.79 x 150 = 7168.5lbs = 7.2 kips

- Load of triangular section: 2.31ft x 4ft x (27in/12) x 0.5 = 10.395cft.  10.395 x 150 = 1559.25lbs = 1.56 kips

- Total Load: 7.2kips + 1.56 kips = 8.76 kips

- Distance from left bearing line: Use center of square section = 65ft

Beam 6 (interior)

Diaphragm 1:

- Load of square section: 5.31ft x 4ft x 2.25ft = 47.79 cft. 47.79 x 150 = 7168.5lbs = 7.2 kips

- Load of triangular section: 2.31ft x 4ft x (27in/12) x 0.5 = 10.395cft.  10.395 x 150 = 1559.25lbs = 1.56 kips

- Total Load: 7.2kips + 1.56 kips = 8.76 kips

- Distance from left bearing line: Use center of square section = 3.31ft

Diaphragm 2

- Load: 3.31ft x 4ft x 2.25 ft = 29.79cft.  29.79 cft x 150pcf = 4468.5lbs = 4.5 kips

- Distance from left bearing line: 22.74ft

Diaphragm 3

- Load: 3.31ft x 4ft x 2.25 ft = 29.79cft.  29.79 cft x 150pcf = 4468.5lbs = 4.5 kips

- Distance from left bearing line: 43.7ft

Diaphragm 4

- Load of square section: 5.31ft x 4ft x 2.25ft = 47.79 cft. 47.79 x 150 = 7168.5lbs = 7.2 kips

- Load of triangular section: 2.31ft x 4ft x (27in/12) x 0.5 = 10.395cft.  10.395 x 150 = 1559.25lbs = 1.56 kips

- Total Load: 7.2kips + 1.56 kips = 8.76 kips

- Distance from left bearing line: Use center of square section = 65ft

Beam 7 (fascia)

Diaphragm 1:

- Load of square section: 3.25ft x 4ft x 2.25ft = 29.25 cft. 29.25 cft x 150 pcf = 4387.5 lbs = 4.39kips

- Load of triangular section: 2.309ft x 4ft x 2.25ft x 0.5 = 10.39cft.  10.39cft x 150pcf = 1558.5lb = 1.56 kips

- Total Load: 4.39 kips + 1.56 kips = 5.95 kips

- Distance from left bearing line: Use center of square section = Assume center of square section.  3.31ft

Diaphragm 2

- Load: 4ft x 1ft x 2.25 ft = 9cft.  9 cft x 150pcf = 1350lbs = 1.35 kips

- Distance from left bearing line: 23.9ft

Diaphragm 3

- Load: 4ft x 1ft x 2.25 ft = 9cft.  9 cft x 150pcf = 1350lbs = 1.35 kips

- Distance from left bearing line: 44.85ft

Diaphragm 4

- Load of square section: 5.31ft x 4ft x(27in/12 depth of beam) =47.79cft.  47.79cft x 150pcf = 7168.6lbs = 7.2 kips

- Load of triangular section: 2.31ft x 4ft x (27in/12) x 0.5 = 10.395cft.  10.395 x 150 = 1559.25lbs = 1.56 kips

- Total Load: 7.2kips + 1.56 kips = 8.76 kips

- Distance from left bearing line: Use center of square section = 66.06ft

Dead Load Intensity from Self Weight of Beam

Cross Sectional Area of B17 x 48 beam = 713.8 in2 = 4.956sft 4.956sft x 150lb/cft =743.4lbs = 0.743kip/ft

These are shown as a line loads for each individual beam in LEAP.

THIS WAS REMOVED FROM THE DEAD LOAD list in LEAP

NEED TO VERIFY THIS LOAD IS ALREADY CALCULATED AND DOES NOT NEED TO BE INCLUDED AS A SEPARATE DEAD LOAD.  Check the tutorial example again.

Weight of composite concrete deck

Total Weight of Concrete = Weight of Composite Concrete Wearing Surface = 0.5 (6in thick) x 1ft wide 1 1ft wide = 0.5 cft x 150lb/cft = 75lb/cft =

OR

Total weight = 0.5ft x 28ft x 69.34ft x 150lb/cft = 145614lb / (28ft x 69.34ft) = 75lb/sft = 0.075kip/sft

This is modeled as a composite load in LEAP

Need to add the weight of rebar as well?

THIS WAS REMOVED FROM THE DEAD LOAD list in LEAP

NEED TO VERIFY THIS LOAD IS ALREADY CALCULATED AND DOES NOT NEED TO BE INCLUDED AS A SEPARATE DEAD LOAD.  Check the tutorial example again.