ROADWAY

CLEARING AND GRUBBING, AS PER PLAN

| ITEM | EXTENSION | GRAND TOTAL | UNIT |
| --- | --- | --- | --- |
| 201 | 11001 | LS | LS |

CALCULATIONS

Approximate tied block matting dimensions taken from field measurements are listed below:

NW Quadrant: 30’ x 6’ = 180sft

SW Quadrant: 30’ x 6’ = 180sft

NE Quadrant: 30’ x 6’ = 180sft

SE Quadrant: 30’ x 6’ = 180sft

Grand total = 180sft x 4 = 720sft / 9 = 80syd

NOTES

Includes removal of Type 2 Tied Block Mat in all four quadrants.  This is called out on the bridge site plan sheet.

COST ESTIMATE

$5,000

COST ESTIMATE EXPLANATION

This is a mall job with not much clearing and grubbing.  Even with the Tied Block Mat removal, it doesn’t justify a higher lump sum price.  Bids for this item from PID 102934 range from $1,500 to $4,300.

DIAGRAMS

None

PAVEMENT REMOVED

| ITEM | EXTENSION | GRAND TOTAL | UNIT |
| --- | --- | --- | --- |
| 202 | 23000 | 657 | SY |

CALCULATIONS

West Approach Existing Pavement and Shoulder Widths

| Station | Total Lane Width | North Shoulder Width | South Shoulder Width |
| --- | --- | --- | --- |
| 159+50.0 | 20.5’ | 1.5’ paved + 1.5’ gravel = 3.0’ | 5’ paved + 0’ gravel = 5’ |
| 159+79.10 | 20.5’ | 1.5’ paved + 2.5’ gravel = 4.0’ | 5’ paved + 0’ gravel = 5’ |
| 160+29.10 | 20.5’ | 2’ paved + 6’ gravel = 8’ | 4’ paved + 5’ gravel = 9’ |
| 160+62.17 | 20.5’ | 5.5’ paved + 5’ gravel = 10.5’ | 4’ paved + 4’ gravel = 8’ |

Roadway widths measured on 3/9/2023

Stationing measured along centerline of road.

Total Lane Width was measured from the outside edge of the white edge line on the north side to the outside edge of the white edge line on the south side.

Gravel shoulder at stations 160+29.10 and 160+62.17 extends beyond the existing guardrail posts.

- Station 159+50.00 to 159+79.10

Length = 29.10ft

Pavement Width = 1.5’ + 20.5’ + 5’ = 27'

29.10ft x 27ft = 785.7sft

785.7 / 9 = 87.3syd = 88syd

- Station 159+79.10 to 160+29.10

Length = 50ft

Pavement Width = 1.5’ + 20.5’ + 5’ = 27'

50ft x 27ft = 1350sft

1350sft / 9 = 150syd

- Station 160+29.10 to 160+54.10

Length = 25ft

Pavement Width = 2’ + 20.5’ + 4’ = 26.5'

25ft x 26.5ft = 662.5sft

662.5sft / 9 = 73.61syd = 74syd

- Station 160+54.10 to 160+70.24

This segment encompasses the remainder of the pavement up to the approach slab.  Dimensions are calculated using a triangular area.

Length = 16.14ft

Pavement Width = 4’+20.5+5.5 = 30ft

16.14ft x 30ft x 0.5  = 242.1sft

242.1sft / 9 = 26.9syd = 27syd

West Approach total = 88syd + 150syd + 74syd + 27syd = 339syd

East Approach Existing Pavement and Shoulder Widths

| Station | Total Lane Width | North Shoulder Width | South Shoulder Width |
| --- | --- | --- | --- |
| 161+81.15 | 19.5’ | 6’ paved + 4’ gravel = 10’ | 3.5’ paved + 4’ gravel = 7.5’ |
| 162+14.22 | 19.0’ | 1’ paved + 9’ gravel = 10’ | 1’ paved + 6’ gravel = 7’ |
| 162+64.22 | 19.0’ | 1’ paved + 6’ gravel = 7’ | 1’ paved + 3’ gravel = 4’ |
| 163+00.0 | 19.0’ | 1’ paved + 5’ gravel = 6’ | 1’ paved + 3’ gravel = 3’ |

Roadway widths measured on 3/9/2023

Stationing measured along centerline of road.

Total Lane Width was measured from the outside edge of the white edge line on the north side to the outside edge of the white edge line on the south side.

Gravel shoulder at stations 161+81.61 and 162+14.68 extends beyond the existing guardrail posts.

- Station 161+73.08 to 161+89.22

This segment encompasses the remainder of the pavement up to the approach slab.  Dimensions are calculated using a triangular area.

Length = 16.16ft

Pavement Width = 3.5ft + 19ft + 6ft = 28.5ft

16.14ft x 28.5ft x 0.5 = 230.28sft

229.99 / 9 = 25.55syd = 26syd

- Station 161+89.22 to 162+14.22

Length = 25ft

Pavement Width = 1ft + 19ft + 1ft

25ft x 21ft = 525sft

525sft / 9 = 58.33syd = 59syd

- Station 162+14.22 to 162+64.22

Length = 50ft

Pavement Width = 1ft + 19ft + 1ft = 21ft'

50ft x 21ft = 1050sft

1050sft / 9 = 116.6syd = 117syd

- Station 162+64.22 to 163+00.0

Length = 35.78ft

Pavement Width = 6ft + 19.5ft + 3.5ft = 29ft

35.78ft x 29ft = 1037.62sft

1037.62sft / 9 = 115.29syd = 116syd

East Approach Total = 26syd + 59syd + 117syd +116syd = 318syd

Grand Total = 339syd + 318syd = 657syd

NOTES

Per 202.05 in CM&S, this item pays for removal of all HMA in the approach roadway.  It does not pay for removal of any of the roadway aggregate base, or any of the HMA wearing course on top of the bridge deck or approach slabs.

In notes from the January 2023 meeting between the District 2 Design and Construction groups, Construction has asked that no less that 100ft of HMA pavement be removed as part of a bridge rehabilitation project.  112.27ft is being removed from the west approach, and 121.39ft is being removed from the east approach.

All pavement dimensions taken from field measurements performed on 3/9/2023.

COST ESTIMATE

$19,601.28

COST ESTIMATE EXPLANATION

Unit price from estimator used.

DIAGRAMS

Inner most white lines represent lanes of travel limits.

Outer white lines represent paved shoulder limits.

Outer brown lines represent gravel shoulder limits.

This drawing can be found in structures basemap for this project.

PAVEMENT REMOVED

| ITEM | EXTENSION | GRAND TOTAL | UNIT |
| --- | --- | --- | --- |
| 202 | 23000 | 150 | SY |

CALCULATIONS

Quantity from similar project used, PID 102934.

NOTES

This pay item is for full-depth pavement removal in areas of the detour route that need to be repaired.

COST ESTIMATE

$12,000

COST ESTIMATE EXPLANATION

Unit price from Estimator was increased based on an educated guess due to the smaller quantity of this work.

DIAGRAMS

None

GUARDRAIL REMOVED, AS PER PLAN

| ITEM | EXTENSION | GRAND TOTAL | UNIT |
| --- | --- | --- | --- |
| 202 | 38001 | 160 | FT |

CALCULATIONS

NE QUADRANT TOTAL LENGTH = 40ft

SE QUADRANT TOTAL LENGTH = 40ft

NW QUADRANT TOTAL LENGTH = 40ft

SW QUADRANT TOTAL LENGTH = 40ft

Total = 40ft x 4 = 160

NOTES

Sandusky County Garage has requested that guardrail and bridge railing be salvaged and brough to their facility by the contractor.  A note regarding this, including contact information and an address is shown on the plans.

The quantity above does not include removal of the Bridge Terminal Assembly or the MGS Anchor Assembly.  This is the length of the guardrail between these two items.

The guardrail length for each quadrant was measured from the center of the post at the first joint past the anchor, to the anchor assembly, to the center of the post at the first joint with a wood post for the Bridge Terminal Assembly.

COST ESTIMATE

$566.40

COST ESTIMATE EXPLANATION

Unit price from Estimator for non as per plan pay item was used.

DIAGRAMS

None

ANCHOR ASSEMBLY REMOVED, TYPE E, AS PER PLAN

| ITEM | EXTENSION | GRAND TOTAL | UNIT |
| --- | --- | --- | --- |
| 202 | 42011 | 4 | EACH |

CALCULATIONS

Anchor assemblies are approximately 15ft long from the end piece to the first joint going toward the bridge.  They include the end piece, two posts, and one 13’-6” piece of guardrail.

NOTES

Sandusky County Garage has requested that guardrail and bridge railing be salvaged and brough to their facility by the contractor.  A note regarding this, including contact information and an address is shown on the plans.

Verified in plans from PID 84079 (2017 rehabilitation project in SR 523) that Type E guardrail endings were installed at SAN-523-0307 bridge.

COST ESTIMATE

$819.94

COST ESTIMATE EXPLANATION

Unit price from Estimator for non as per plan pay item was used.

DIAGRAMS

None

BRIDGE TERMINAL ASSEMBLY REMOVED, AS PER PLAN

| ITEM | EXTENSION | GRAND TOTAL | UNIT |
| --- | --- | --- | --- |
| 202 | 47001 | 4 | EACH |

CALCULATIONS

Length of the Bridge Terminal Assembly is approximately 26ft and has wooden posts and blockouts.

NOTES

Sandusky County Garage has requested that guardrail and bridge railing be salvaged and brough to their facility by the contractor.  A note regarding this, including contact information and an address is shown on the plans.

COST ESTIMATE

$467.74

COST ESTIMATE EXPLANATION

Unit price from Estimator for non as per plan pay item was used.

DIAGRAMS

None

EXCAVATION

| ITEM | EXTENSION | GRAND TOTAL | UNIT |
| --- | --- | --- | --- |
| 203 | 10000 | 161 | CY |

CALCULATIONS

Total roadway cross section = 28’ of HMA (2ea 10ft lanes and 2ea 4ft shoulders) and 2ft gravel shoulders on each side.

Total width = 28’ + 2’+2’ = 32’

Aggregate base subgrade should extend 6” beyond limits of shoulder on each side.

This yields 32’ + 6” + 6” = 33’

Total width of excavation is 33’

West Approach

- Station 159+50.00 to 160+54.10

Length = 104.10ft

Width = 33’

Thickness of existing aggregate base = 5.5”

Total excavation volume = 104.10ft x 33ft x (5.5”/12) = 1574.51

- Station 160+54.10 to 160+70.24

This segment encompasses excavation in the triangular shaped area adjacent to the approach slab.

Length of short side of triangular area = 16.14ft

Length of long side of triangle = 33’

Thickness of existing aggregate base = 5.5”

Area = 16.14ft x 33ft x 0.5 =266.31sft

Total excavation volume = 266.31sft x (5.5”/12) = 122.05cft

- Station 160+62.17 to 160+87.17

This segment encompasses the area beneath the approach slab.

Approach slab width = 28’

Approach slab length = 25’

Assume gravel shoulder on each side is 2’ wide with additional 6” of agg base beyond 2’ limit of the shoulder.

Total width = 28’ + 2.5’ + 2.5’ = 33’

Assume 6” existing agg base under approach slabs.

Total excavation volume = 33’ x 25’ x (6”/12) of agg base = 412.5cft

West Approach total = 1574.51cft + 122.05cft + 412.5cft = 2109.06cft / 27 = 78.11cyd = 79cyd

East Approach

- Station 161+56.15 to 161+81.15

Approach slab width = 28’

Approach slab length = 25’

Assume gravel shoulder on each side is 2’ wide with additional 6” of agg base beyond 2’ limit of the shoulder.

Total width = 28’ + 2.5’ + 2.5’ = 33’

Assume 6” existing agg base under approach slabs.

Total excavation volume = 33’ x 25’ x (6”/12) of agg base = 412.5cft

- Station 161+73.08 to 161+89.22

This segment encompasses excavation in the triangular shaped area adjacent to the approach slab.

Length of short side of triangular area = 16.14ft

Length of long side of triangle = 33’

Thickness of existing aggregate base = 5.5”

Area = 16.14ft x 33ft x 0.5 =266.31sft

Total excavation volume = 266.31sft x (5.5”/12) = 122.05cft

- Station 161+89.22 to 163+00.0

Length = 110.78ft

Width = 33’

Thickness of existing aggregate base = 5.5”

Total excavation volume = 110.78ft x 33ft x (5.5”/12) = 1675.54

East Approach total excavation = 412.5cft + 122.05cft + 1675.54cft = 2210.09cft / 27 = 81.85cyd = 82cyd

Grand Total = 79cyd + 82cyd = 161cyd

- Determination if additional material needs to be excavated below existing top of subgrade elevation.

The final deck surface elevation will rise due to the installation of a 6” composite concrete deck.  This will affect the elevation of the existing approach pavement, requiring it to be raised.

Existing elevation of west edge of approach slab = Approximately 577.82 (elevation taken from survey basemap)

Existing pavement section:

Surface course = 4in HMA

Base course = 6in HMA

Aggregate Base = 5.5”

Total depth = 15.5” = 1.29’

Existing elevation of top of subgrade at west edge of approach slab = 577.82 – 1.29’ = 576.53

Proposed elevation of west edge of approach slab = 578.35

Proposed pavement section:

Surface course = 4in HMA

Base course = 6in HMA

Aggregate Base = 6”

Total depth = 16” = 1.33’

Proposed elevation of top of subgrade at west edge of approach slab = 578.35 – 1.33’ = 577.02

Difference between proposed and existing top of subgrade elevation = 577.02 – 576.53 = 0.49’ = 0.5’

The existing subgrade elevation will need to be built up by approximately 0.5ft at the west end of the approach slab, and tapered off going to the west.  The quantity for this will be accounted for with the embankment pay item.

Assume that this will be the same for the east approach as well.

However, this determines that excavation does not need to take place below the elevation of the existing top of subgrade.

- Determination if additional material needs to be excavated below existing top of subgrade elevation below the approach slabs.

Existing elevation of approach slab at bridge reference line (Station 160+62.27) = 578.31 (elevation taken from survey basemap)

Existing approach slab thickness = 15” = 1.25’

Existing aggregate base thickness beneath approach slab = 6”

Existing elevation of top of subgrade beneath approach slab at bridge reference line = 578.31 – 1.25’ – (6”/12) = 576.56

Proposed elevation of approach slab at bridge reference line (Station 160+62.27) = 578.85

Proposed approach slab thickness = 15” = 1.25’

Proposed aggregate base thickness beneath approach slab = 6”

Proposed elevation of top of subgrade beneath approach slab at bridge reference line = 578.85 – 1.25’ – (6”/12) = 577.1

Difference between proposed and existing top of subgrade elevation beneath approach slab at bridge reference line = 577.1 – 576.56 = 0.46’ = Approximately 0.5’

The existing subgrade elevation will need to be built up by approximately 0.5ft beneath the approach slab at the bridge reference line.  The quantity for this will be accounted for with the embankment pay item.

Assume that this will be the same for the east approach slab.

However, this determines that excavation does not need to take place below the elevation of the existing top of subgrade.

NOTES

This item pays for removal of existing aggregate base under the roadway HMA, as well as excavation under the shoulder area to the extent needed to install a new aggregate base for the shoulder and HMA.  It also includes excavation of aggregate based beneath both approach slabs.

The plan is to install 20’ft of pavement for the travelled lanes, 4ft paved shoulders, and 2ft gravel shoulders for all approach pavement replaced with the bridge superstructure replacement.

The idea behind this is to improve the area by adding additional pavement width while work is going on in the area for this project.

The width of pavement replacement should meet or exceed the existing widths and can be shifted as needed to match the large existing shoulder.

Installing 28’ of new HMA on the approaches will match the width of the approach slabs and the bridge deck.

The minimum 4ft paved shoulder will be up to the face of the guardrail that will run straight off the bridge railing and into each quadrant.

COST ESTIMATE

$4,306.22

COST ESTIMATE EXPLANATION

Unit price from estimator used.

DIAGRAMS

EMBANKMENT

| ITEM | EXTENSION | GRAND TOTAL | UNIT |
| --- | --- | --- | --- |
| 203 | 20000 | 89 | CY |

CALCULATIONS

West Approach

| West Approach Existing Elevations |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Stations |  |  |  |  |  |  |  |  |
|  | 159+50.00 | 159+57.48 | 159+82.19 | 160+7.12 | 160+30.69 | 160+57.46 | 160+62.17 | 160+87.17 |
| North Edge | 576.36 | 576.36 | 576.43 | 576.84 | 577.28 | 577.69 | 577.49 | 578.03 |
| Centerline | 576.48 | 576.48 | 576.58 | 576.89 | 577.38 | 577.73 | 577.82 | 578.36 |
| South Edge | 576.13 | 576.13 | 576.38 | 576.71 | 577.08 | 577.67 | 577.5 | 578.04 |

Existing elevations were taken from the survey basemap.

Existing elevation difference between rear abutment and end of new HMA pavement = 1.87’ (578.36-576.48).

Total horizontal distance= 160+87.17 – 159+50.00 = 137.17’

Rate of change = 1.87’ vertical / 137.17’ horizontal = 0.0136’ vertical change per foot horizontal

| West Approach Proposed Elevations |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Stations |  |  |  |  |  |  |  |  |
|  | 159+50.00 | 159+57.48 | 159+82.19 | 160+7.12 | 160+30.69 | 160+57.46 | 160+62.17 | 160+87.17 |
| North Edge | 576.36 | 576.40 | 576.82 | 577.25 | 577.66 | 578.12 | 578.20 | 578.63 |
| Centerline | 576.48 | 576.62 | 577.04 | 577.47 | 577.88 | 578.34 | 578.42 | 578.85 |
| South Edge | 576.13 | 576.40 | 576.82 | 577.25 | 577.88 | 577.12 | 578.20 | 578.63 |

Proposed elevations calculated by starting with the proposed deck surface elevation at the rear abutment, then after determining the amount of vertical elevation difference per horizontal foot between the rear abutment and the end of the new HMA, and determining the amount of vertical rise per station based on that rate.

Proposed elevation difference at rear abutment to the end of new HMA pavement = 2.37' (578.85-576.48).

Total horizontal distance= 160+87.17 – 159+50.00 = 137.17’

Rate of change = 2.37’ vertical / 137.17’ horizontal = 0.172ft vertical change per foot horizontal

Embankment Need on West Approach

Picture a triangle with a long side length of 137.17’ and a short side average length of 0.50’ (2.37’-1.87’)

Area of triangle = 137.17’ x 0.50’ x 0.5 = 34.29sft

Multiply the triangle area by average of 33’ width of roadway excavation = 34.29’ x 33’ = 1131.57 / 27 = 41.91cyd = 42cyd

| East Approach Existing Elevations |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Stations |  |  |  |  |  |  |
|  | 161+56.15 | 161+81.15 | 162+00.00 | 162+26.01 | 162+51.08 | 162+78.67 | 163+00.00 |
| North Edge | 579.26 | 577.97 | 577.89 | 577.50 | 577.29 | 577.07 | 576.99 |
| Centerline | 578.47 | 578.21 | 578.08 | 577.56 | 577.33 | 577.20 | 577.14 |
| South Edge | 578.39 | 578.13 | 577.96 | 577.50 | 577.25 | 577.15 | 577.04 |

East Approach

Existing elevations were taken from the survey basemap

Existing elevation difference between forward abutment and end of new HMA pavement = 1.33’ (578.47-577.14)

Total horizontal distance = 163+00.00 – 161+56.15 = 143.85’

Rate of change = 1.33’ vertical / 143.85’ horizontal = 0.0092 vertical change per foot horizontal

| East Approach Proposed Elevations |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Stations |  |  |  |  |  |  |
|  | 161+56.15 | 161+81.15 | 162+00.00 | 162+26.01 | 162+51.08 | 162+78.67 | 163+00.00 |
| North Edge | 578.78 | 578.46 | 578.21 | 577.88 | 577.56 | 577.20 | 576.99 |
| Centerline | 579.00 | 578.68 | 578.43 | 578.10 | 577.78 | 577.42 | 577.14 |
| South Edge | 578.78 | 578.46 | 578.21 | 577.88 | 577.56 | 577.20 | 577.04 |

Proposed elevations calculated by starting with the proposed deck surface elevation at the rear abutment, then after determining the amount of vertical elevation difference per horizontal foot between the rear abutment and the end of the new HMA, and determining the amount of vertical rise per station based on that rate.

Proposed elevation difference between the forward abutment and the end of new HMA pavement = 1.86’ (579.00 – 577.14).

Total horizontal distance = 163+00.00 – 161+56.15 = 143.85’

Rate of change = 1.86’ vertical / 143.85’ horizontal = 0.129 vertical change per foot horizontal.

Embankment Needed on East Approach

Picture a triangle with a long side length of 143.85’ and a short side average length of 0.53’ (1.86’-1.33’)

Area of triangle = 143.85’ x 0.53’ x 0.5 = 38.12sft

Multiply the triangle area by average of 33’ width of roadway excavation = 38.12sft x 33ft = 1257.96cft / 27 = 46.59cyd = 47cyd

Grand total embankment quantity = 42cyd + 47cyd = 89cyd

NOTES

This pay item is used to add material on top of the existing subgrade in order to increase the elevation under the area of new HMA and the bridge approach slab in order to meet the new elevation of the new composite 6in bridge deck.

Additional calculations are shown under the Excavation pay item if needed.

Spreadsheet titled ‘Approach Pavement Elevations – 107718’ shows additional details.

COST ESTIMATE

$2,342.36

COST ESTIMATE EXPLANATION

Unit price from estimator used.

DIAGRAMS

None.

SUBGRADE COMPACTION

| ITEM | EXTENSION | GRAND TOTAL | UNIT |
| --- | --- | --- | --- |
| 204 | 10000 | 1031 | SY |

CALCULATIONS

Quantity calculated using area dimensions from excavation pay item.

West Approach

- Station 159+50 to 160+54.1

Length = 104.1ft

Excavation Width = 33ft

Total Surface Area for compaction = 104.1ft x 33ft = 3435.3sft

- Station 160+54.1 to 160+70.24 (triangular area adjacent to approach slab)

Long side of triangle = 33ft (excavation width)

Short side of triangle = 16.14ft (160+70.24-160+54.1)

Total Surface Area = 33ft x 16.14ft x 0.5 = 266.31sft

- Station 160+62.17 to 160+87.17 (stationing along centerline)

Surface area underneath approach slab

Approach slab width = 28’ + 2.5ft shoulder excavation on each side for shoulders = 33ft

Length = 25ft

Total Surface Area for compaction= 33ft x 25ft = 825sft

West Approach Total Surface Area for compaction = 3435.3sft + 266.31sft + 825sft = 4526.61 / 9 = 502.95syd = 503syd

East Approach

- Station 161+56.15 to 160+81.15 (stationing along centerline)

Surface area underneath approach slab

Approach slab width = 28’ + 2.5ft shoulder excavation on each side for shoulders = 33ft

Length = 25ft

Total Surface Area for compaction= 33ft x 25ft = 825sft

- Station 161+73.08 to 161+89.22 (triangular area adjacent to approach slab)

Long side of triangle = 33ft (excavation width)

Short side of triangle = 16.14ft (160+70.24-160+54.1)

Total Surface Area = 33ft x 16.14ft x 0.5 = 266.31

- Station 161+89.22 to 163+00.0

Length = 110.78ft

Excavation Width = 33ft

Total Surface Area for compaction = 110.78ft x 33ft = 3655.74sft

East approach total surface area for compaction = 825sft + 266.31sft + 3655.74sft = 4747.05sft / 9 = 527.45syd = 528syd

Grand Total = 503syd + 528syd = 1031syd

NOTES

Includes compaction of subgrade where excavation under roadway and approach slabs was completed.

COST ESTIMATE

$5,013.61

COST ESTIMATE EXPLANATION

Unit price from estimator used.

DIAGRAMS

See diagram included with Excavation pay item notes, if needed.

RESHAPING UNDER GUARDRAIL

| ITEM | EXTENSION | GRAND TOTAL | UNIT |
| --- | --- | --- | --- |
| 209 | 15000 | 8 | STA |

CALCULATIONS

Calculations for footage measurements below are shown under the MGS guardrail pay item.

Total Footage NW Quadrant: 93’-8 5/8” = 1 station

Total Footage SW Quadrant: 182’-2 5/8”= 2 stations

Total Footage NE Quadrant: 182’-2 5/8”= 2 stations

Total Footage SE Quadrant: 93’-8 5/8” = 1 station

Total = 8 stations.

NOTES

Grading in area where new guardrail will be installed.  Assume a 5ft swath.

COST ESTIMATE

$2,000.00

COST ESTIMATE EXPLANATION

Estimator would not give a unit price until the quantity reached 8 stations.  Unit price at this point was $33.33 per station.  I increased it to $250 per station.

DIAGRAMS

None

LINEAR GRADING

| ITEM | EXTENSION | GRAND TOTAL | UNIT |
| --- | --- | --- | --- |
| 209 | 60200 | 8 | STA |

CALCULATIONS

Calculations for footage measurements below are shown under the MGS guardrail pay item.

Total Footage NW Quadrant: 93’-8 5/8” = 1 station

Total Footage SW Quadrant: 182’-2 5/8”= 2 stations

Total Footage NE Quadrant: 182’-2 5/8”= 2 stations

Total Footage SE Quadrant: 93’-8 5/8” = 1 station

Add 2 stations for other miscellaneous work = 8 stations

NOTES

Grading to remove berm along edge of pavement for entire length of all guardrail components installed in each quadrant, plus other miscellaneous grading.

COST ESTIMATE

$1,915.37

COST ESTIMATE EXPLANATION

Unit price from Estimator used.

DIAGRAMS

None

TIED CONCRETE BLOCK MAT WITH TYPE 2 UNDERLAYMENT

| ITEM | EXTENSION | GRAND TOTAL | UNIT |
| --- | --- | --- | --- |
| 601 | 21060 | 80 | SY |

CALCULATIONS

Approximate tied block matting dimensions taken from field measurements are listed below:

NW Quadrant: 30’ x 6’ = 180sft

SW Quadrant: 30’ x 6’ = 180sft

NE Quadrant: 30’ x 6’ = 180sft

SE Quadrant: 30’ x 6’ = 180sft

Grand total = 180sft x 4 = 720sft / 9 = 80syd

NOTES

This pay item is for the replacement of the existing tied concrete block mat that will be removed.  Removal will be paid for under Clearing and Grubbing, As Per Plan.  Dimensions taken from survey basemap.

COST ESTIMATE

$6,519.94

COST ESTIMATE EXPLANATION

Unit price from Estimator used.

DIAGRAMS

None

GUARDRAIL, TYPE MGS

| ITEM | EXTENSION | GRAND TOTAL | UNIT |
| --- | --- | --- | --- |
| 606 | 15050 | 275 | FT |

CALCULATIONS

Guardrail length of need calculations

Design ADT = 700

Design Speed = 60mph

Design classification: Rural Minor Collector

Use Figure 600-1 (600-1.pdf (ohio.gov)) to determine clear zone width.

Using the chart in Figure 600-1, with 60mph design speed and less than 750 ADT.

Figure 600-1 gives guidance on what to use for a clear zone if the foreslope is 6:1 or flatter, or between 6:1 and 4:1.  It does not give any guidance about what to do if the foreslope is greater than 4:1.

SW Quadrant

Using points on the survey basemap, at roughly 117ft from the abutment, the foreslope grade is around 0.33, which is greater than 4:1 (0.25).  At roughly 167 from the abutment, the foreslope grade is between 0.35-0.72.  Therefore, using 22ft as the clear zone for this quadrant.

Using the equation from Figure 602-1 (602-1.pdf (ohio.gov)), X = Length of need

X = ?

LH= 22ft

L2= 4ft from white edgeline to face of guardrail

LR= 200ft from Figure 602-1 for a Design speed of 60mph and less than 1000vpd

a/b = 0

X = (22+0-4)/(0+(22/200)) = 18/ 0.11 = 163.63 ft…. Use 164ft

Bridge Terminal Assembly, Type TST-2 (Standard drawing MGS-3.3) will be used in all four quadrans of the bridge.  The entire length of the TST-2 can be counted as contributing to the length of need.  Per standard drawing MGS 3.3, 22’-10 1/8” of the Bridge Terminal Assembly can be counted as contributing toward the length of need.

On the Roadway Approved Products List website (Roadway Approved Products List | Ohio Department of Transportation), two approved Type E guardrail end treatments are listed.

The shop drawing for the MSKT end treatment specifies that 34’-4 1/2” can be counted as contributing toward the length of need.

The shop drawing for the SoftStop end treatment specifies that 31’-3” can be counted as contributing toward the length of need.

Since either product could be installed by the contractor, use the shorter one to determine how much MGS Guardrail is needed.

Total length of MGS guardrail = Length of Need – Bridge Terminal Assembly Contribution – Type E Anchor Assembly Contribution

Total Length of MGS Guardrail = 164ft – 22’-10 1/8” – 31’-3”= 109’-10 7/8” of guardrail needed.

Using 12’6” as the standard length = 109’-10 7/8” / 12’-6” = 8.79 pieces = 9 pieces = 112.5ft

Grand Total Length for SW Quadrant = BTA + MGS Guardrail + Anchor Assembly Total Length (Using MSKT)

= 22’-10 1/8” + 112’-6” + 46’-10 ½” = 182’-2 5/8”

NW Quadrant:

Using points on the survey basemap, at roughly 65ft from the abutment, the foreslope grade is around 0.34 which is greater than 4:1 (0.25).  At roughly 134ft from the abutment, the foreslope grade is around 0.33.  Therefore, using 22ft as the clear zone for this quadrant.

The 22ft clear zone is measured starting at the centerline of the road going north.  A westbound vehicle would have to cross through the north 10ft lane and 4ft shoulder before hitting the guardrail.

Using the equation from Figure 602-1 (602-1.pdf (ohio.gov)), X = Length of need

X = ?

LH= 22ft

L2= 14ft from the centerline to face of guardrail

LR= 200ft from Figure 602-1 for a Design speed of 60mph and less than 1000vpd

a/b = 0

X = (22+0-14)/(0+(22/200)) = 8/ 0.11 = 72.72 ft…. Use 73ft

Total Length of MGS Guardrail Needed = 73 – 22’- 10 1/8” – 31’-3”= 18’-10 7/8” … This requires 2ea 12’6” pieces = 25ft total.

Grand Total Length for SW Quadrant = BTA + MGS Guardrail + Anchor Assembly Total Length (Using MSKT)

= 22’-10 1/8” + 25’ + 46’-10 ½” = 93’-8 5/8”

NE Quadrant

Using points on the survey basemap, at roughly 128ft from the abutment, the foreslope grade is around 0.23 which is greater than 6:1 (0.16).  At roughly 162ft from the abutment, the foreslope grade is around 0.18.  Therefore, using 22ft as the clear zone for this quadrant.

Using the equation from Figure 602-1 (602-1.pdf (ohio.gov)), X = Length of need

X = ?

LH= 22ft

L2= 4ft from white edgeline to face of guardrail

LR= 200ft from Figure 602-1 for a Design speed of 60mph and less than 1000vpd

a/b = 0

X = (22+0-4)/(0+(22/200)) = 18/ 0.11 = 163.63 ft…. Use 164ft

Bridge Terminal Assembly, Type TST-2 (Standard drawing MGS-3.3) will be used in all four quadrans of the bridge.  The entire length of the TST-2 can be counted as contributing to the length of need.  Per standard drawing MGS 3.3, 22’-10 1/8” of the Bridge Terminal Assembly can be counted as contributing toward the length of need.

On the Roadway Approved Products List website (Roadway Approved Products List | Ohio Department of Transportation), two approved Type E guardrail end treatments are listed.

The shop drawing for the MSKT end treatment specifies that 34’-4 1/2” can be counted as contributing toward the length of need.

The shop drawing for the SoftStop end treatment specifies that 31’-3” can be counted as contributing toward the length of need.

Since either product could be installed by the contractor, use the shorter one to determine how much MGS Guardrail is needed.

Total length of MGS guardrail = Length of Need – Bridge Terminal Assembly Contribution – Type E Anchor Assembly Contribution

Total Length of MGS Guardrail = 164ft – 22’-10 1/8” – 31’-3”= 109’-10 7/8” of guardrail needed.

Using 12’6” as the standard length = 109’-10 7/8” / 12’-6” = 8.79 pieces = 9 pieces = 112.5ft

Grand Total Length for SW Quadrant = BTA + MGS Guardrail + Anchor Assembly Total Length (Using MSKT)

= 22’-10 1/8” + 112’-6” + 46’-10 ½” = 182’-2 5/8”

SE Quadrant

Using points on the survey basemap, at roughly 28ft from the abutment, the foreslope grade is around 0.86 which is greater than 4:1 (0.25).  At roughly 55ft from the abutment, the foreslope grade is around 0.77.  Therefore, using 22ft as the clear zone for this quadrant.

The 22ft clear zone is measured starting at the centerline of the road going south.  An eastbound vehicle would have to cross through the north 10ft lane and 4ft shoulder before hitting the guardrail.

Using the equation from Figure 602-1 (602-1.pdf (ohio.gov)), X = Length of need

X = ?

LH= 22ft

L2= 14ft from the centerline to face of guardrail

LR= 200ft from Figure 602-1 for a Design speed of 60mph and less than 1000vpd

a/b = 0

X = (22+0-14)/(0+(22/200)) = 8/ 0.11 = 72.72 ft…. Use 73ft

Total Length of MGS Guardrail Needed = 73 – 22’- 10 1/8” – 31’-3”= 18’-10 7/8” … This requires 2ea 12’6” pieces = 25ft total.

Grand Total Length for SW Quadrant = BTA + MGS Guardrail + Anchor Assembly Total Length (Using MSKT)

= 22’-10 1/8” + 25’ + 46’-10 ½” = 93’-8 5/8”

Grand Total Guardrail Needed = 25ft + 25ft +112.5ft + 112.5ft = 275ft

NOTES

The guardrail paid for this with item is the portion of that goes between Bridge Terminal Assembly, Type TST-2 and the Type E Anchor Assembly.

COST ESTIMATE

$6,944.48

COST ESTIMATE EXPLANATION

Unit price from Estimator used.

DIAGRAMS

None

ANCHOR ASSEMBLY, MGS TYPE E

| ITEM | EXTENSION | GRAND TOTAL | UNIT |
| --- | --- | --- | --- |
| 606 | 26150 | 4 | EACH |

CALCULATIONS

One per quadrant.

NOTES

Specifications and shop drawings for the two approved Type E end treatments are found on the Roadway Approved Products website (Roadway Approved Products List | Ohio Department of Transportation).  Used the one named MSKT for guardrail length calculations.

Shop drawings specifying how much of the anchor assembly can be counted as contributing to the length of need are found on the website above as well.

COST ESTIMATE

$11,482.00

COST ESTIMATE EXPLANATION

Unit price from Estimator used.

DIAGRAMS

None

MGS BRIDGE TERMINAL ASSEMBLY, TYPE TST-2

| ITEM | EXTENSION | GRAND TOTAL | UNIT |
| --- | --- | --- | --- |
| 606 | 34600 | 4 | EACH |

CALCULATIONS

One per quadrant x 4 quadrants = 4ea total

NOTES

This is a newer specification.

Estimator did not have a unit price available.

This item was not found in the 2019-2023 ODOT Bid Data Item Search as having been used in any project either.

The entire length of the BTA can be counted as contributing toward to the length of need.  After reviewing standard drawing MGS 3.3, this length was determined to be 22’-10 1/8.

COST ESTIMATE

$8,000.00

COST ESTIMATE EXPLANATION

Unit price for Bridge Terminal Assembly, Type TST = $1,806 each in Estimator.

Estimate $2000 each based on this.

DIAGRAMS

None

SIGNING, MISC.

| ITEM | EXTENSION | GRAND TOTAL | UNIT |
| --- | --- | --- | --- |
| 630 | 95000 | LS | LS |

CALCULATIONS

None

NOTES

This pay item includes removal of ground mounted bridge identification sign, salvaging the sign, and reinstalling it on a new ground mounted post in the same location.  This was consolidated into one pay item, as opposed to having multiple pay items for the removal and replacement of the sign.

COST ESTIMATE

$1,500.00

COST ESTIMATE EXPLANATION

Educated guess.

DIAGRAMS

None

EROSION CONTROL

SEEDINGING AND MULCHING

| ITEM | EXTENSION | GRAND TOTAL | UNIT |
| --- | --- | --- | --- |
| 659 | 10000 | 566 | SY |

CALCULATIONS

Area under guardrail = 8 STA x 5ft wide = 800ft x 5ft = 4000sft

Area around abutments

Abutment width is roughly 49ft.

28ft of this width will have the bridge superstructure on it.

Assume the remainder (49ft – 28ft = 21ft) needs to be reseeded, starting and the abutment and going out 15ft.

21ft x 15ft = 315sft x 2 abutments = 630sft

Total = 4000sft + 630sft = 4630sft

Add an additional 10% for other areas that may need it = 5093sft

5093sft / 9 = 565.88 syd = 566syd

NOTES

This pay item is for reseeding under new guardrail, around abutment, and extra for any additional area that needs it.  Assume 5ft swath under guardrail.

COST ESTIMATE

$2,514.70

COST ESTIMATE EXPLANATION

Unit price from Estimator used.

DIAGRAMS

None

COMMERCIAL FERTILIZER

| ITEM | EXTENSION | GRAND TOTAL | UNIT |
| --- | --- | --- | --- |
| 659 | 20000 | 0.06 | TON |

CALCULATIONS

Total square footage of estimated area to be reseeded = 5093sft

5093sft / 1000sft = 5.093

5.093 x 20lbs = 101.86lbs = 102lbs

102lbs / 2000lbs = 0.051 ton = 0.06 ton

NOTES

This pay item is for application of fertilizer to all areas that were seeded.  It was estimated as dry fertilizer at 20lbs per 1000sft, as outlined in spec book.

COST ESTIMATE

$64.84

COST ESTIMATE EXPLANATION

Unit price from Estimator used.

DIAGRAMS

None

EROSION CONTROL

| ITEM | EXTENSION | GRAND TOTAL | UNIT |
| --- | --- | --- | --- |
| 832 | 30000 | 2000 | EACH |

CALCULATIONS

Estimated based on amount from similar project, PID 102934.

NOTES

This pay item is for all erosion control and will be paid based on what is used by the contractor.

COST ESTIMATE

$2,000

COST ESTIMATE EXPLANATION

Estimated based on amount from similar project, PID 102934.

DIAGRAMS

None

PAVEMENT

PAVEMENT PLANING, ASHPALT CONCRETE, 1.5”

| ITEM | EXTENSION | GRAND TOTAL | UNIT |
| --- | --- | --- | --- |
| 254 | 10000 | 200 | SY |

CALCULATIONS

Used quantity from similar project, PID 102934.

NOTES

This pay item is for milling HMA in areas of the detour route that need to be repaired.  Supplemental description added specifying 1.5” of HMA removal.

COST ESTIMATE

$5,322.73

COST ESTIMATE EXPLANATION

Unit price from Estimator used.

DIAGRAMS

None

ASPHALT CONCRETE BASE, PG64-22, (449)

| ITEM | EXTENSION | GRAND TOTAL | UNIT |
| --- | --- | --- | --- |
| 301 | 56000 | 125 | CY |

CALCULATIONS

The HMA base thickness is 6” for both the east and west approaches.

West Approach

- Station 159+50.00 to 160+54.10

Installing surface course of 28ft of HMA pavement (2ea 10ft lanes and 2ea 4ft paved shoulders) and 2ft gravel shoulders on each side

HMA base = 6” wider than surface course pavement

= 28’ + 6” + 6” = 29’

Length = 160+54.10-159+50.0 = 104.1ft

Total Volume = 104.1ft x 29ft x (6”/12) thick = 1509.45cft

- Station 160+54.10 to 160+70.24

This is the triangular area adjacent to the approach slab

Long side of triangle = 29ft

Short side = 16.14ft (160+70.24-160+54.1)

Area = 29ft x 16.14ft x 0.5 = 234.04sft

Total Volume = 234.04sft x (6”/12) thick = 117.01cft

West approach total = 1509.45cft + 117.01cft = 1626.46cft / 27 = 60.23cyd = 61cyd

East Approach

- Station 161+73.08 to 161+89.22

This is the triangular area adjacent to the approach slab

Long side of triangle = 29ft

Short side = 16.14ft (160+70.24-160+54.1)

Area = 29ft x 16.14ft x 0.5 = 234.04sft

Total Volume = 234.04sft x (6”/12) thick = 117.01cft

- Station 161+89.22 to 163+00.00

Installing surface course of 28ft of HMA pavement (2ea 10ft lanes and 2ea 4ft gravel shoulders) and 2ft gravel shoulders on each side

HMA base = 6” wider than surface course pavement

= 28’ + 6” + 6” = 29’

Length = 163+00 – 161+89.22 = 110.78

Total Volume = 110.78ft x 29ft x (6”/12) thick = 1606.31cft / 27 = 59.49cyd = 60cyd

East Approach Total = 117.01cft +1606.31cft = 1723.32cft / 27  63.82cyd = 64cyd

Grand Total HMA Base = 61cyd + 64cyd = 125cyd

NOTES

This pay item is for furnishing material, installing, and compacting a 6” HMA base course for the roadway approaches.  It does not include quantities for any HMA base course used for pavement repairs.

Asphalt Concrete Base, (449), PG 64-22 was selected based on Section 410.1 of the ODOT Pavement Design Manual, that outlines what pavements to use in the transition area at a structure.

COST ESTIMATE

$36,243.78

COST ESTIMATE EXPLANATION

Unit price from Estimator used.

DIAGRAMS

ASPHALT CONCRETE BASE, PG64-22, (449)

| ITEM | EXTENSION | GRAND TOTAL | UNIT |
| --- | --- | --- | --- |
| 301 | 56000 | 25 | CY |

CALCULATIONS

Quantity from similar project used, PID 102934.

NOTES

This pay item is for furnishing material, installing, and compacting a 6” HMA base course as needed for pavement repairs on the detour route.  It has a higher unit price than the HMA base course for the roadway approach paving.

Asphalt Concrete Base, (449), PG 64-22 was selected based on Section 410.1 of the ODOT Pavement Design Manual, that outlines what pavements to use in the transition area at a structure.

COST ESTIMATE

$10,000.00

COST ESTIMATE EXPLANATION

Educated guess based on quantity.

DIAGRAMS

None

AGGREGATE BASE

| ITEM | EXTENSION | GRAND TOTAL | UNIT |
| --- | --- | --- | --- |
| 304 | 20000 | 173 | CY |

CALCULATIONS

West Approach

- Station 159+50.00 to 160+54.10

Total width of pavement and gravel shoulders = 28’ + 2’ + 2’ = 32’

Extend aggregate base 6” beyond extents of gravel shoulders on each side for a total width of 32’+6” +6” = 33’

Length = 160+54.10-159+50.0 = 104.1ft

Thickness of aggregate base = 6”

Total volume = 104.1ft x 33ft x (6”/12) = 1727.55cft

- Station 160+54.10 to 160+70.24

This is the triangular area adjacent to the approach slab

Long side of triangle = 33ft

Short side = 16.14ft (160+70.24-160+54.1)

Area = 33ft x 16.14ft x 0.5 = 266.31sft

Total Volume = 266.31sft x (6”/12) thick = 133.15cft

- Station 160+62.17 to 160+87.17 (stationing along centerline)

Area beneath approach slab.

Approach slab width = 28’ + 2.5ft shoulder excavation on each side for shoulders = 33ft

Length = 25ft

Total Volume = 33ft x 25ft x (6”/12) = 412.5cft

West Approach Total = 1727.55cft + 133.15cft + 412.5cft = 2273.2cft / 27 = 84.19cyd = 85cyd

East Approach

- Station 161+56.15 to 161+81.15 (stationing along centerline)

Area beneath approach slab.

Approach slab width = 28’ + 2.5ft shoulder excavation on each side for shoulders = 33ft

Length = 25ft

Total Volume = 33ft x 25ft x (6”/12) = 412.5cft

- Station 161+73.08 to 161+89.22

This is the triangular area adjacent to the approach slab

Long side of triangle = 33ft

Short side = 16.14ft (161+89.22-161+73.08)

Area = 33ft x 16.14ft x 0.5 = 266.31sft

Total Volume = 266.31sft x (6”/12) thick = 133.15cft

- Station 161+89.22 to 163+00.0

Total width of pavement and gravel shoulders = 28’ + 2’ + 2’ = 32’

Extend aggregate base 6” beyond extents of gravel shoulders on each side for a total width of 32’+6” +6” = 33’

Length = 163+00 – 161+89.22 = 110.78ft

Thickness of aggregate base = 6”

Total volume = 110.78ft x 33ft x (6”/12) = 1827.87cft

East Approach Total = 412.5cft + 133.15cft + 1827.87cft = 2373.52cft / 27 = 87.90cyd = 88cyd

Grand Total = 85cyd + 88cyd = 173cyd

NOTES

This pay item is for furnishing, installing, and compacting a 6” aggregate base on top of the prepared subgrade.

COST ESTIMATE

$12,978.56

COST ESTIMATE EXPLANATION

Unit price from Estimator used.

DIAGRAMS

None

NON-TRACKING TACK COAT

| ITEM | EXTENSION | GRAND TOTAL | UNIT |
| --- | --- | --- | --- |
| 407 | 20000 | 88 | GAL |

CALCULATIONS

West Approach

Total Surface Area of Base Course

- Station 159+50.00 to 160+54.10

Length = 160+54.10-159+50.0 = 104.1ft

Width = 29ft

Area = 104.1ft x 29ft = 3018.9sft

- Station 160+54.10 to 160+70.24

This is the triangular area adjacent to the approach slab

Long side of triangle = 29ft

Short side = 16.14ft (160+70.24-160+54.1)

Area = 29ft x 16.14ft x 0.5 = 234.03sft

Total base course surface area = 3018.9+234.03 = 3252.83sft

Total surface area of surface course

- Station 159+50.00 to 160+54.10

Length = 160+54.10-159+50.0 = 104.1ft

Width = 28ft

Area = 104.1ft x 28ft = 2914.8sft

- Station 160+54.10 to 160+70.24

This is the triangular area adjacent to the approach slab

Long side of triangle = 28ft

Short side = 16.14ft (160+70.24-160+54.1)

Area = 28ft x 16.14ft x 0.5 = 225.96sft

Total surface course surface area = 2914.8sft + 225.96sft = 3140.76sft

East Approach

Total surface area of base course

- Station 161+73.08 to 161+89.22

This is the triangular area adjacent to the approach slab

Long side of triangle = 29ft

Short side = 16.14ft (160+70.24-160+54.1)

Area = 29ft x 16.14ft x 0.5 = 234.03sft

- Station 161+89.22 to 163+00

Length = 163+00 – 161+89.22 = 110.78ft

Width = 29ft

Area = 110.78ft x 29ft = 3212.62sft

Total base course surface area = 234.03sft + 3212.62sft = 3446.65

Total surface area of surface course

- Station 161+73.08 to 161+89.22

This is the triangular area adjacent to the approach slab

Long side of triangle = 28ft

Short side = 16.14ft (160+70.24-160+54.1)

Area = 28ft x 16.14ft x 0.5 = 225.96sft

- Station 161+89.22 to 163+00

Length = 163+00 – 161+89.22 = 110.78ft

Width = 28ft

Area = 110.78ft x 28ft = 3101.84sft

Total surface course surface area = 225.96sft + 3101.84sft = 3327.8sft

Grand Total = 3252.83sft + 3140.76sft + 3446.65sft + 3327.8sft = 13168.04sft / 9 = 1463.11syd = 1464syd

Application rate for tack coat on new asphalt = 0.05-0.06 gal/syd.  Use 0.06gal/syd

0.06gal/syd x 1464 syd = 87.84 gallons = 88 gallons

NOTES

This pay item is for the application of a non-tracking tack coat between the HMA base course and the first lift of the surface course, and between the first and second lift of surface course HMA.  It does not include any quantity for pavement repairs on the detour route.

COST ESTIMATE

$311.46

COST ESTIMATE EXPLANATION

Unit price from Estimator used.

DIAGRAMS

None

NON-TRACKING TACK COAT

| ITEM | EXTENSION | GRAND TOTAL | UNIT |
| --- | --- | --- | --- |
| 407 | 20000 | 40 | GAL |

CALCULATIONS

Quantity from similar project use, PID 102934

NOTES

This pay item is for the application of a non-tracking tack coat where needed for pavement repairs on the detour route.  It has a larger unit price than the tack coat for the roadway approach paving.

COST ESTIMATE

$160.00

COST ESTIMATE EXPLANATION

Educated guess based on quantity.

DIAGRAMS

None

ASPHALT CONCRETE SURFACE COURSE, TYPE 1, (446), PG64-22

| ITEM | EXTENSION | GRAND TOTAL | UNIT |
| --- | --- | --- | --- |
| 441 | 70000 | 81 | CY |

CALCULATIONS

Width of HMA surface course needs to be 6" less than width of HMA base course.

Total surface course thickness 2ea lifts of 2" = 4" total

The HMA base thickness is 6” for both the east and west approaches.

West Approach

- Station 159+50.00 to 160+54.10

Total width of surface course HMA pavement is 28ft (2ea 10ft lanes and 2ea 4ft paved shoulders)

Length = 160+54.10-159+50.0 = 104.1ft

Total Volume = 104.1ft x 28ft x (4”/12) thick = 971.6cft

- Station 160+54.10 to 160+70.24

This is the triangular area adjacent to the approach slab

Long side of triangle = 28ft

Short side = 16.14ft (160+70.24-160+54.1)

Area = 28ft x 16.14ft x 0.5 = 225.96sft

Total Volume = 225.96sft x (4”/12) thick = 75.32cft

West approach total = 971.6cft + 75.32cft = 1046.92cft / 27 = 38.77cyd = 39cyd

East Approach

- Station 161+73.08 to 161+89.22

This is the triangular area adjacent to the approach slab

Long side of triangle = 28ftZShort side = 16.14ft (160+70.24-160+54.1)

Area = 28ft x 16.14ft x 0.5 = 225.96sft

Total Volume = 225.96sft x (4”/12) thick = 75.32cft

- Station 161+89.22 to 163+00.00

Total width of surface course HMA pavement is 28ft (2ea 10ft lanes and 2ea 4ft paved shoulders)

Length = 163+00 – 161+89.22 = 110.78

Total Volume = 110.78ft x 28ft x (4”/12) thick = 1033.94cft

East Approach Total = 75.32cft + 1033.94cft = 1109.26cft / 27  41.08cyd = 42cyd

Grand Total HMA Base = 39cyd + 42cyd = 81cyd

NOTES

This pay item is for furnishing material, installing, and compacting a 4” HMA surface course in two lifts of 2” each, for the roadway approaches.  It does not include quantities for any HMA surface course used for pavement repairs.

Asphalt Concrete Surface Course, Type 1, (449), PG 64-22 was selected based on Section 410.1 of the ODOT Pavement Design Manual, that outlines what pavements to use in the transition area at a structure.

NEED TO INCLUDE PLAN NOTE AS OUTLINED IN SECTION 410.1.

COST ESTIMATE

$27,014.85

COST ESTIMATE EXPLANATION

Unit price from Estimator used.

DIAGRAMS

ASPHALT CONCRETE SURFACE COURSE, TYPE 1, (449), PG64-22

| ITEM | EXTENSION | GRAND TOTAL | UNIT |
| --- | --- | --- | --- |
| 441 | 70000 | 20 | CY |

CALCULATIONS

Quantity from similar project used, PID 102934

NOTES

This pay item is for furnishing material, installing, and compacting a 4” HMA surface course in two lifts of 2” each, as needed for repairs to the detour route.  It has a higher unit price than the HMA surface course for the roadway approaches.

Asphalt Concrete Surface Course, Type 1, (449), PG 64-22 was selected based on Section 410.1 of the ODOT Pavement Design Manual, that outlines what pavements to use in the transition area at a structure.

NEED TO INCLUDE PLAN NOTE AS OUTLINED IN SECTION 410.1

COST ESTIMATE

$8,300.00

COST ESTIMATE EXPLANATION

Educated guess based on quantity.

DIAGRAMS

None

COMPACTED AGGREGATE

| ITEM | EXTENSION | GRAND TOTAL | UNIT |
| --- | --- | --- | --- |
| 617 | 10100 | 35 | CY |

CALCULATIONS

Shoulder gravel goes on top of aggregate base layer to the same height as the top of the second HMA surface course.

Total HMA = 6” base + 4” surface = 10”

Therefore, aggregate shoulder needs to be 10” thick.

West Approach

North Shoulder

Station 159+50.00 to 160+95.24

Length = 145.24ft

Width = 2ft

Depth = 10”

Total Volume = 145.24ft x 2ft x (10”/12) = 242.06cft

South Shoulder

Station 159+50.00 to 160+79.10

Length = 129.1ft

Width = 2ft

Depth = 10”

Total Volume = 129.1ft x 2ft x (10”/12) = 215.16cft

West approach total = 242.06cft + 215.16cft = 457.22cft / 27 = 16.93cyd = 17cyd

East Approach

North Shoulder

Station 161+64.22 to 163+00.0

Length = 135.78ft

Width = 2ft

Depth = 10”

Total Volume = 135.78ft x 2ft x (10”/12) = 226.3cft

South Shoulder

Station 161+48.08 to 163+00

Length = 151.92ft

Width = 2ft

Depth = 10”

Total Volume = 151.92ft x 2ft x (10”/12) = 253.2cft

West approach total = 226.3cft + 253.2cft = 479.5cft / 27 = 17.75cyd = 18cyd

Grand Total = 17cyd + 18cyd = 35cyd

NOTES

This pay item is for furnishing, installing, and compacting an aggregate shoulder.

COST ESTIMATE

$3,917.36

COST ESTIMATE EXPLANATION

Unit price from Estimator used.

DIAGRAMS

None

TRAFFIC CONTROL

RAISED PAVEMENT MARKER REMOVED

| ITEM | EXTENSION | GRAND TOTAL | UNIT |
| --- | --- | --- | --- |
| 621 | 54000 | 4 | EA |

CALCULATIONS

4 counted during field visit between Station 159+50 and Station 163+00

NOTES

This pay item is to pay for removal of reflectors embedded in the pavement before milling or pavement removal.  They will be replaced by ODOT crews or ODOT contractor under district-wide contract.

COST ESTIMATE

$83.08

COST ESTIMATE EXPLANATION

Unit price from Estimator used.

DIAGRAMS

None

BARRIER REFLECTOR, TYPE 5

| ITEM | EXTENSION | GRAND TOTAL | UNIT |
| --- | --- | --- | --- |
| 626 | 00116 | 14 | EA |

CALCULATIONS

Grand Total Length of guardrail components for each quadrant = BTA + MGS Guardrail + Anchor Assembly Total Length (Using MSKT)

BTA = 22’-10-1/8”

Anchor Assembly Total Length (assume MSKT TYPE E)= 53’- 1 ½”

Per CMS&S Item 626, spacing may vary from 65ft to 125ft for tangents.  It also states to place the first reflector to be visible to oncoming traffic at the buffer end section of the guardrail.

SW Quadrant

Length of required MGS Guardrail = 112’-6”

Total = = 22’-10 1/8” + 112’-6” + 53-1 1/2” = 188’-5 5/8” = 188.46’

Per CMS&S Item 626, spacing may vary from 65ft to 125ft for tangents.  It also states to place the first reflector to be visible to oncoming traffic at the buffer end section of the guardrail.

188.46’ / 65 = 2.89

Use 4 total: 1 at start of bridge railing, 1 at end buffer device, and two spaced in between.  This results in a spacing of 62.82ft

NW Quadrant

Length of required MGS Guardrail = 25

Total = = 22’-10 1/8” + 25’ + 53-1 1/2” = 100’-11 5/8” = 100.96’

Per CMS&S Item 626, spacing may vary from 65ft to 125ft for tangents.  It also states to place the first reflector to be visible to oncoming traffic at the buffer end section of the guardrail.

100.96’ / 65 = 2.89

Use 3 total: 1 at start of bridge railing, 1 at end buffer device, and one spaced in between.  This results in a spacing of 50.48ft

SE Quadrant

Length of required MGS Guardrail = 25

Total = = 22’-10 1/8” + 25’ + 53-1 1/2” = 100’-11 5/8” = 100.96’

Per CMS&S Item 626, spacing may vary from 65ft to 125ft for tangents.  It also states to place the first reflector to be visible to oncoming traffic at the buffer end section of the guardrail.

100.96’ / 65 = 2.89

Use 3 total: 1 at start of bridge railing, 1 at end buffer device, and one spaced in between.  This results in a spacing of 50.48ft

NE Quadrant

Length of required MGS Guardrail = 112’-6”

Total = = 22’-10 1/8” + 112’-6” + 53-1 1/2” = 188’-5 5/8” = 188.46’

Per CMS&S Item 626, spacing may vary from 65ft to 125ft for tangents.  It also states to place the first reflector to be visible to oncoming traffic at the buffer end section of the guardrail.

188.46’ / 65 = 2.89

Use 4 total: 1 at start of bridge railing, 1 at end buffer device, and two spaced in between.  This results in a spacing of 62.82ft

Grand Total = 4 + 3 + 4 + 3 = 14

NOTES

This item is to install reflectors along the approach guardrail in all four quadrants of the bridge.

COST ESTIMATE

$137.21

COST ESTIMATE EXPLANATION

Unit price from Estimator used.

DIAGRAMS

None

STRUCTURE OVER 20 FOOT SPAN (SAN-523-0307)

PORTIONS OF STRUCTURE REMOVED, OER 20 FOOT SPAN, AS PER PLAN

| ITEM | EXTENSION | GRAND TOTAL | UNIT |
| --- | --- | --- | --- |
| 202 | 11203 | $35,000 | EA |

CALCULATIONS

Abutment Removal Calculations

- NW Wingwall Removal = 18"deep x 31.5in high x 97" wide = 55000in3=31.8cft = 1.17cyd

- NE Wingwall Removal = 18"deep x 31"high x 104in = 58032in3 = 33.58cft = 1.24cyd

- SW Wingwall Removal = 18"deep x 30.5in high x 92" wide = 50508in3 = 29.22cft = 1.01cyd

- SE Wingwall Removal = 18"deep x 32in high x 84in wide = 48384in3 = 28cft = 1.03cyd

- Area under beam seat and wingwalls

Accounting for 3/16”/ft slope going toward the crown.

Calculated with a rectangular and triangular area

Rectangular area

Use 48’-4 ¼” from old plans for length = 48.35’

Depth of cut = 1ft deep

Width = 3ft (this was verified in the field)

Total volume = 3’ x 1’ x 48.35’ = 145.05cft x 2 abutments = 290.1cft / 27 = 10.74cyd = 11cyd

Triangular area

Divide the abutment in half = 48.35’ / 2 = 24.17’

24.17’ x (3/16”)/foot slope = 4.53”

Area = 24.17’ x (4.53/12) x 0.5 = 4.56sft

4.56sft x 3ft wide = 13.68sft x 4 (2 halves of each abutments) = 54.74cft = 2.02cyd = 2cyd

- Approach slab seat

Dimensions taken from old plans.

Length = (28’ / cos30) = 32.33’

Height = 6”

Depth = 11”

Total Volume of removal = 32.33’ x (6”/12) x (11”/12) = 14.81cft x 2 abutments = 29.63cft / 27 =1.09cyd = 2cyd

Grand Total = 1.17+1.24+1.01+1.03+11+2+2 = 19.45cyd = 20cyd

NOTES

This pay items includes removal or portions of each abutment, 7ea concrete box beams, and existing bridge railing.

Locations that will be paid for as Portions of Structure Removed are called out in the plans.  The bridge railing will be salvaged for the Sandusky County Garage.  This will be called out on the plans.

Calculations for removal elevations of abutment are found in Survey Information Spreadsheet.

Abutment removal elevations are shown in page 10 of plans.

COST ESTIMATE

$35,000.00

COST ESTIMATE EXPLANATION

The ODOT Bid Data Item Search spreadsheet was filtered to show this bid item from projects in District 2 in 2021 and 2022 with the following categories: bridge, interchange, and major reconstruction.

The interchange projects seemed the most reasonable and a baseline price of $25,000 lump sum was determined.

Next, the bid tabs from a similar project, PID 102934, were reviewed.  The average bid for this project was $30,000.  It was determined that this was a better estimate than the original $25,000.

Roughly 15% was added to the $30,000 lump sum amount to arrive at $35,000 for this estimate.

DIAGRAMS

None

APPROACH SLAB REMOVED

| ITEM | EXTENSION | GRAND TOTAL | UNIT |
| --- | --- | --- | --- |
| 202 | 22900 | 156 | SY |

CALCULATIONS

Existing approach slab shown on original plans from 1983 as being 25’ x 28’.

The 1983 plans reference standard AS-1-81 which indicates the slab should be 15” thick.

Existing approach slab area = 700sft x 2 slabs = 1400sft / 9 = 155.5syd = 156syd

NOTES

These quantities were calculated based on the approach slab dimensions from the original plans.

However, there is a joint perpendicular to the bridge that could be the end of the approach slab, making the dimensions for the approach slab removal larger than what is calculated.

This will not be able to be determined until the HMA wearing surface on top of the approach slab is removed.

The purpose of this note is to advise that this may be the case.

COST ESTIMATE

$10,730.63

COST ESTIMATE EXPLANATION

Unit price from Estimator used.

DIAGRAMS

None

WEARING COURSE REMOVED

| ITEM | EXTENSION | GRAND TOTAL | UNIT |
| --- | --- | --- | --- |
| 202 | 23500 | 372 | SY |

CALCULATIONS

The bridge deck and approach slabs currently has an HMA wearing course.

Bridge deck surface area = 1941sft = 215.66syd = 216syd

Approach slabs = 700sft = 77.77syd = 78syd x 2 approach slabs = 156syd

Total = 216 + 156 = 372syd

NOTES

This pay item is for removal of the HMA wearing course on top of the box beams and approach slabs.

Wearing course is approximately 4” thick HMA on top of box beams based on field measurements.

COST ESTIMATE

$4,498.25

COST ESTIMATE EXPLANATION

Unit price from Estimator used.

DIAGRAMS

None

COFFERDAMS AND EXCAVATION BRACING

| ITEM | EXTENSION | GRAND TOTAL | UNIT |
| --- | --- | --- | --- |
| 503 | 11100 | 1 | LS |

CALCULATIONS

Per spec book, sheet piling for cofferdams needs to constructed to an elevation of 3ft above OHWM

OHWM = 573.4

Tip elevation of cofferdam = 573.4’ +3’ = 576.4.

Use 50ft of length across the front of each abutment.

Assume that sheet piling needs to go to a depth of 10ft below OHWM.  10ft + 3.0ft = 13.ft.  13.0ft x 50ft across front of abutment = 650sft.  Sides = 5ft out from abutment x 13.0ft =65sft

Total amount needed = (650sft x 2 abutments) + (65sft x 4 sides) = 1300 + 260 = 1560sft

NOTES

This item pays for installation of sheeting piling in front of each abutment in order to dewater the area where the abutment will be cut down and repoured.  The sheet piling will be removed.

Location and tip elevation of sheet piling will be shown on bridge site plan sheet.

COST ESTIMATE

$25,000

COST ESTIMATE EXPLANATION

The ODOT Bid Data Item Search spreadsheet was filtered to show this bid item from projects in in 2021 and 2022.

The average bid was $40,696 and the average awarded bid was $30,895.

Bids were all over the board for this item.

Using $25,000 lump sump because this is generally an expensive item of work, but it is being performed on a relatively uncomplicated project.

DIAGRAMS

None

UNCLASSIFIED EXCAVATION, AS PER PLAN

| ITEM | EXTENSION | GRAND TOTAL | UNIT |
| --- | --- | --- | --- |
| 503 | 21301 | 1 | LS |

CALCULATIONS

Excavation of area behind abutment/ backwall

Length of abutment = 48.354ft

Per the plans, 2ft out from back face of abutment is to be excavated to a depth of 1ft below the abutment cut line elevation.

Abutment cut line elevation = 574.55 (rear abutment), and 574.69 (forward abutment)

1ft below cut line elevation = 573.55.

Approximate center of pavement elevation above abutment = 579.87.  Subtract 15" for existing approach slab thickness and the 6” aggregate base below the approach slab to get an elevation 578.12

578.12 - 573.55 = 4.57ft = 4.6ft

4.6ft deep x 2ft out from fact of abutment x 48.354 = 444.82 = 445cft

Assume 1:1 slope starting at 2ft from backwall until reaching elevation under approach slab = roughly 4.6x4.6x.5x48.35 = 511.54= 512cft

Total = (445cft + 512cft ) x 2 abutments = 1914cft /27 = 70.88 = 71cyd

Excavation of rock channel protection

Assume top of existing rip rap is roughly 1ft below beam seat.

Existing beam seat elevation (rear abutment) = 575.87

Existing rock channel protection elevation = 574.87

Approximately 2ft below the cut line needs to be clear and 5ft out from the abutment face for installation of sheet piling.

Cut line elevation (rear abutment) = 574.55

2ft below cut line elevation = 574.55 - 2ft = 572.55

Difference between rock channel protection elevation and elevation 2ft below cut line = 574.87-572.55 = 2.32ft…Use 3ft

3ft x 5ft (out from face of abutment) x 48.354ft long abutment = 725.31cft x 2 abutments = 1450.62cft / 27 = 53.72cyd = 54

Grand total unclassified excavation = 71cyd + 54cyd = 125cyd

This quantity was calculated to be input into Estimator to get a ballpark idea of what it would cost of paid for by the cubic yard, as explained below.

NOTES

This pay item is for excavation of material from behind the abutment/ backwall area, as well as for excavation of the rock channel protection in front of the abutment.

Assume excavation depth of 1ft below cut line behind the abutment will be adequate to maneuver a saw to cut the portion of abutment to be removed, as well as form and pour this area with new concrete.

Material to fill the triangular excavation area past the 2ft limit out from the back of the abutment is presumed to be included with this pay item.

ODOT construction group prefers to have this item as a lump sum as opposed to a defined quantity, because it can end in disputes if additional material needs to be excavated.

COST ESTIMATE

$10,000

COST ESTIMATE EXPLANATION

Input 125cyd into Estimator and it showed a price of roughly $8,100.

Average bid for similar project, PID 102934, was $118/cyd for only 36cyd of work.

Using $10,000 because it falls between the two of estimated prices outlined above.

DIAGRAMS

None

EPOXY COATED STEEL REINFORCEMENT

| ITEM | EXTENSION | GRAND TOTAL | UNIT |
| --- | --- | --- | --- |
| 509 | 10000 | 16326 | LB |

CALCULATIONS

Abutment = 2884lbs

Superstructure = 13442lbs

Total = 16326

Abutments

| A501 | 2ea for each abutment that run longitudinally through bottom of new beam seat area below A801. Lapped as necessary. |
| --- | --- |
| A502 | 7ea for NW wingwall. All 4 wingwalls have different lengths. |
| A503 | 7ea for SW wingwall. All 4 wingwalls have different lengths. |
| A504 | 7ea for NE wingwall. All 4 wingwalls have different lengths. |
| A505 | 7ea for SE wingwall. All 4 wingwalls have different lengths. |
| A601 | Under each beam seat doweled in to existing abutment concrete. 2ea per spacing to create U-shape. |
| A602 | Same as A601 but in wingwall sections and taller. 2ea per spacing. |
| A801 | 3ea for each abutment that run longitudinally through new top of new beam seat area above A501. |

January2023!

Dimension calculations

A501

Use table 304-4 in ODOT BDM.

2” clear cover and bar size 5 = 29” lap distance = 2’=5”

Total distance across abutment = 48’-4 ¼”

Subtract 4” cover from the vertical face of each side of the abutment

= (48’-4 ¼”)-4”-4” = 47’ 8 ¼”

Divide by 2 = 23’-10 1/8”

Add half the lap distance (29”/2) = 14.5” = 1’ 2 ½”

Total Distance for each A501 bar = 23’-10 1/8” + 1’-2 ½” = 25’ 5/8”  round up to 25’-1”

Total per abutment = 1 row of s spaces, with each space containing 2 bars lapped together = 4 total per abutment

8 total for the entire project.

A801

Use table 304-4 in ODOT BDM.

2” clear cover and bar size 8 = 57” lap distance = 4’=9”

Total distance across abutment = 48’-4 ¼”

Subtract 4” cover from the vertical face of each side of the abutment

= (48’-4 ¼”)-4”-4” = 47’ 8 ¼”

Divide by 2 = 23’-10 1/8”

Add half the lap distance (57”/2) = 28.5” = 2’ 4 ½”

Total Distance for each A801 bar = 23’-10 1/8” + 2’-4 ½” = 26’-2 5/8”  round up to 26’-3”

Total per abutment = 1 row of 3 spaces, with each space containing 2 bars lapped together = 6 total per abutment

12 total for the entire project.

****ONE ADDITIONAL ROW A801 BAR WAS ADDED BASED ON FIELD MEASURED WIDTH OF 3’-0” OF ABUTMENT.  TOTAL IS NOT 16 FOR THE ENTIRE PROJECT.

A601

Total cross-sectional width of abutment = 2’3” *****DIMENSIONS IN REBAR SCHEDULE IN PLANS WERE UPDATED TO REFLECT NEW, FIELD MEASURED ABUTMENT WIDTH OF 3’-0”.  THESE CALCULATIONS WERE NOTE UPDATED.

Subtract 2” each side for cover = 1’-11” for “A” dimension

For “B” dimension, take removal elevation for rear abutment of 574.55 and new beam seat elevation at middle (crown) of abutment of 575.87.  The difference is  1.32’ = roughly 1’ 4”.  Add 1’ dowel depth = 2’4”

A + B = 1’11” + 2’4” = 4’-3”

21 spaces at 1’6” = 22ea x 2 = 44 per abutment = 88 total for the project.

A602

Total cross-sectional width of abutment = 2’3” *****DIMENSIONS IN REBAR SCHEDULE IN PLANS WERE UPDATED TO REFLECT NEW, FIELD MEASURED ABUTMENT WIDTH OF 3’-0”.  THESE CALCULATIONS WERE NOTE UPDATED.

Subtract 2” each side for cover = 1’-11” for “A” dimension

For “B” dimension, take removal elevation for rear abutment of 574.55 and new elevation at outside edge of rear wingwall of 578.10.  The difference is  3.55’ = roughly 3’ 7”.  Add 1’ dowel depth = 4’-7”

A + B = 1’11” + 4’7” = 6’-6”

Number will vary per quadrant because of different wingwall lengths.

NW Quadrant

NW Wingwall Length = 97”

Subtract 8” for total cover (4” from outside edge and 4” from inside edge) = 97”-8”=89” = 7’-5”

Space at 12” or less (original plans show vertical rebar in wingwalls spaced at 12”)

89” / 12” = 7.416 spaces – round up to 8 spaces

89” / 8 = 11.125” = 11 1/8”

NW Wingwall = 8 spaces at 11 1/8” with 9 total bars for both the front and back face of the abutment = 18 total bars

SW Quadrant

SW Wingwall Length = 92”

Subtract 8” for total cover (4” from outside edge and 4” from inside edge) = 92”-8”=84” = 7’-0”

Space at 12” or less

84” / 12” = 7 spaces

84” / 7 = 12” spacing

NW Wingwall = 7 spaces at 12” with 8 total bars for both the front and back face of the abutment = 16 total bars

NE Quadrant

NE Wingwall Length = 104”

Subtract 8” for total cover (4” from outside edge and 4” from inside edge) = 104”-8”=96” = 8’-0”

Space at 12” or less

96” / 12” = 8 spaces

96” / 8 = 12” spacing

NE Wingwall = 8 spaces at 12” with 9 total bars for both the front and back face of the abutment = 18 total bars

SE Quadrant

SE Wingwall Length = 84”

Subtract 8” for total cover (4” from outside edge and 4” from inside edge) = 84”-8”=76” = 6’-4”

Space at 12” or less

76” / 12” = 6.33 spaces  round up to 7 spaces.

76” / 7 = 10.85” spacing = roughly 10 7/8”

SE Wingwall = 7 spaces at 10 7/8” with 8 total bars for both the front and back face of the abutment = 16 total bars

A502

NW Wingwall Length = 97”

Subtract 4” for total cover (2” from outside edge and 2” from inside edge) = 97”-4”=93” = 7’-9”

****ONE ADDITIONAL BAR WAS ADDED WHEN ABUTMENT DIMENSION WAS EXPANDED FROM 2’-3” TO 3’-0” BASED ON FIELD MEASUREMENTS.  TOP ROW OF REBAR WAS EXPANDED FROM 3 BARS TO 4 BARS.

A503

SW Wingwall Length = 92”

Subtract 4” for total cover (2” from outside edge and 2” from inside edge) = 92”-4”=88” = 7’-4”

****ONE ADDITIONAL BAR WAS ADDED WHEN ABUTMENT DIMENSION WAS EXPANDED FROM 2’-3” TO 3’-0” BASED ON FIELD MEASUREMENTS.  TOP ROW OF REBAR WAS EXPANDED FROM 3 BARS TO 4 BARS.

A504

NE Wingwall Length = 104”

Subtract 4” for total cover (2” from outside edge and 2” from inside edge) = 104”-4”=100” = 8’-4”

****ONE ADDITIONAL BAR WAS ADDED WHEN ABUTMENT DIMENSION WAS EXPANDED FROM 2’-3” TO 3’-0” BASED ON FIELD MEASUREMENTS.  TOP ROW OF REBAR WAS EXPANDED FROM 3 BARS TO 4 BARS.

A505

SE Wingwall Length = 84”

Subtract 4” for total cover (2” from outside edge and 2” from inside edge) = 84”-4”=80” = 6’-8”

****ONE ADDITIONAL BAR WAS ADDED WHEN ABUTMENT DIMENSION WAS EXPANDED FROM 2’-3” TO 3’-0” BASED ON FIELD MEASUREMENTS.  TOP ROW OF REBAR WAS EXPANDED FROM 3 BARS TO 4 BARS.

A507

These are the horizontal U-shaped pieces on the outside and inside edges of the wingwall.

A and C dimension = 1’-0”

B dimension = wingwall width of 2’3 – 4” (2” each side for cover) = 1’11”

Number per quadrant = 8 per quadrant = 16 per abutment = 32 total for the project.

Distance Between First S601 and First S602

Rear Abutment

Total Width of Abutment = 48’-4 ¼”

Subtract 4” from each outside edge for cover on the S602 bars = -8”

Subtract 7’-0” total spacing for S602 bars in SW quadrant (Rear, RT)

Subtract 7’-5” total spacing for S602 bars in NW quadrant (Rear, LT)

Subtract 31’-6” (1’-6” x 21 spaces) for S601 bars

48’-4 ¼” – (8”) – (7’-0”)-(7’-5”) – (31-6”) = 1’-9 ¼”

1’-9 ¼” / 2 = 21 ¼” / 2 = 10 5/8”

Accounting for 4” of cover from inside edge of wingwall to S602 bar, the S601 bar would be placed  6 5/8” from the line where the inside edge of the wingwall meets the beam seat.

Verify that 10 5/8” is adequate spacing between S601 and S602 bars.  Can move the first S601 in toward the  center of the abutment a little bit more if needed as well.

Verify where the last S601 bar can be.  Does it need to be a certain distance form the edge of where the wingwall starts?

Forward Abutment

Total Width of Abutment = 48’-4 ¼”

Subtract 4” from each outside edge for cover on the S602 bars = -8”

Subtract 6’-4” total spacing for S602 bars in SE quadrant (Forward, RT)

Subtract 8’-0” total spacing for S602 bars in NE quadrant (Forward, LT)

Subtract 31’-6” (1’-6” x 21 spaces) for S601 bars

48’-4 ¼” – (8”) – (6’-4”)-(8’-0”) – (31-6”) = 1’-10 ¼”

1’-10 ¼” / 2 = 22 ¼” / 2 = 11 1/8”

Accounting for 4” of cover from inside edge of wingwall to S602 bar, the S601 bar would be placed 7 5/8” from the line where the inside edge of the wingwall meets the beam seat.

Verify that 5” is adequate spacing.  Can move the first S601 in toward the  center of the abutment a little bit more if needed as well.

Verify where the last S601 bar can be.  Does it need to be a certain distance form the edge of where the wingwall starts?

Superstructure

S501

Square Type 3 piece within diaphragm.  Spaced at 1’6”, same as S601 bars in abutment beam seat.

28’ width of diaphragm.

28’ / 1’6” spacing = 18.6 spaces. Round up to 19

28’-0” / 19 = 1.47’ = 1’- 5 ½”

19 spaces with 20 bars per diaphragm = 40 bars total for the project.

Diaphragm dimensions calculated below:

Width = 6” gap between beam end and approach slab + 6” distance that approach slab overhangs onto beam diaphragm.  Total = 12”

Height

Height of beam and deck = 27”+6” = 33”

Bottom of beam is the same elevation as the bottom of the diaphragm.

The ‘X’ dimension on the approach slab is 15”.  33”-15” = 18” = 1’6”

Allow 2” of cover for S501 on all 4 sides.  This gives the 12” -2” -2” = 8” for top and bottom, and 18” – 2” -2” = 14” for the sides

A = 8”

B = 14”

S601

4ea longitudinal pieces that go through diaphragm

4 per abutment = 8 for entire project.
28’-0” total diaphragm length

Subtract 2” each end for cover = 27’-8”

S602

l-shape type 1 piece that connects with S501 and S603 or S604, and secures the deck to the diaphragm.

1 piece at each S501 bar = 20 bars per diaphragm = 40 bars total

Dimensions

B = portion that laps with S603 or S604 = 2’-0”

A = Total distance from bottom of diaphragm to deck surface = 33”.  This segment will connect with the bottom of the S501 bar which is 2” from the top of the diaphragm, and it will bend 90 degree 2.5” beneath the deck surface.  33”-2”-2.5” = 28.5” = 2’-4 ½”  round up to 2’-5”

S603

Longitudinal deck reinforcement.  Shorter piece that is lapped with S604

Total Deck Concrete Length = 70’-4”

Subtract 3” from each end for cover = 69’-11”

Multiply by 1/3 = 23’- 3 2/3”  round up to 23’-4”

Lap Distance from BDM  Use Figure 304-4 and 2.5” Clear Cover Column.

Size 6 bar requires 34” of lap length = 2’10”

Add (2’-10”)/2 to 23’-4”.

1’-5” + 23’-4” = 24’-9”

Total number needed = 10 per side of deck = 20 total for project

S604

Longitudinal deck reinforcement.  Longer piece that is lapped with S603

Total Deck Concrete Length = 70’-4”

Subtract 3” from each end for cover = 69’-11”

Multiply by 2/3 = 46’- 7 1/3”  round up to 46’-8”

Lap Distance from BDM  Use Figure 304-4 and 2.5” Clear Cover Column.

Size 6 bar requires 34” of lap length = 2’10”

Add (2’-10”)/2 to 46’-8”.

1’-5” + 46’-8” = 48’-1”

Total number needed spaced at 1’-5 ½”= 10 per side of deck = 20 total for project

S605

Transverse deck reinforcement.  2 pieces of same length lapped together.

Total Deck Width = hypotenuse of 30 degree right triangle w/ 28’-0” long side.

= 32.33’ = 32’-4”

Subtract 3” cover from each edge = (32’-4”)-3”-3” = 31’-10”

Lap required for “6 bars w/ 2.5” or 3” of cover from Figure 304-4 in ODOT BDM = 34” = 2’-10”

Add (2’-10”)/2 to 31’-10”.

1’-5” + 31’-10” = 33’-3”

First bar spaced at 3” from edge of deck

70’-4”-3”-3” = 69’-10”

69’-10” / 9” spacing = 93.11 – round up to 94 spaces

69’-10” / 94 spaces = 8.91’ spacing = roughly 8’-11”

Total number of spaces = 94, with 95 total bar locations.

2 bars of same length per location = 95 x 2 = 190 total for project.

D801

Type 18 bar that connects deck to diaphragm.

Spaced at 9” across diaphragm

Diaphragm total length = 28’-0”

Subtract 2” from each end for cover

= 27’-8”

27’-8” = 332” / 9” spacing = 36.88 spaces  round up to 37 spaces

27’8” / 37 spaces = 8.97” = roughly 9” spacing

37 spaces with 38 bars total per side = 38 x 2 = 76 total for the project.

Dimensions

C = 1’

B = 1’

A = (1.414*X + .823ft)*sec(theta)      X = front dimension of approach slab.  Theta = skew angle

A = (1.414*(15/12)+.823)*sec(30) = 2.59*1.154 = 2.988’ = roughly 3’-0”

Total length = A + B + C = (3’-0”)+(1’-0”)+(1’-0”) = 5’-0”

NOTES

None.

COST ESTIMATE

$32,353.56

COST ESTIMATE EXPLANATION

Unit price from Estimator used.

DIAGRAMS

None

DOWEL HOLES WITH NONSHRINK, NONMETALLIC GROUT

| ITEM | EXTENSION | GRAND TOTAL | UNIT |
| --- | --- | --- | --- |
| 510 | 10000 | 156 | EA |

CALCULATIONS

| Bar # | Total Bars | Holes Per Bar | Total Dowel Holes |
| --- | --- | --- | --- |
| A501 | 8 | 0 | 0 |
| A502 | 7 | 0 | 0 |
| A503 | 7 | 0 | 0 |
| A504 | 7 | 0 | 0 |
| A505 | 7 | 0 | 0 |
| A507 | 32 | 0 | 0 |
| A601 | 88 | 1 | 88 |
| A602 | 68 | 1 | 68 |
| A801 | 12 | 0 | 0 |
| S501 | 40 | 0 | 0 |
| S601 | 8 | 0 | 0 |
| S602 | 40 | 0 | 0 |
| S603 | 20 | 0 | 0 |
| S604 | 20 | 0 | 0 |
| S605 | 190 | 0 | 0 |
| D801 | 76 | 0 | 0 |
| Total Dowel Holes = 156 |  |  |  |

NOTES

None.

COST ESTIMATE

$3,683.58

COST ESTIMATE EXPLANATION

Unit price from Estimator used.

DIAGRAMS

None

CLASS QC2 CONCRETE, SUPERSTRUCTURE, AS PER PLAN

| ITEM | EXTENSION | GRAND TOTAL | UNIT |
| --- | --- | --- | --- |
| 511 | 31610 | 45 | CY |

CALCULATIONS

Precast deck beam length from LEAP = 68.84’

Deck thickness at abutments (least amount of camber) = 7.63”

Deck thickness at center of beam (most camber and minimum concrete thickness) = 6”

Average thickness from abutment to center of beam = (7.63+6)/2 = 6.815”

Width of deck = 28’ +fit up

Fit up joint width = 0.5” per the BDM.  6 fit up joints total

Total width of deck = 28’ + (0.5x6) = 28’3”

Total volume of deck = 28.25’ x (6.815”/12) x 68.84’ = 1104.44cft / 27 = 40.9cyd = 41cyd

Semi-Integral diaphragm concrete volume

This goes on top of the beam seat area of the abutment and is what the approach slab will rest on.

Dimensions are:

Height = 18” = 1.5’

Width = 12”

Length = (28’/cos30) + fit up = 32.33’ + (.5” x 6) = 35.33’

Dimensions and other details are shown on pages 14 and 15 of the plans.

Dimension of diaphragm will be called out on the plans.

Volume = 1.5’ x 1’ x  32.33’ = 48.49cft x 2 abutments = 96.99cft / 27 = 3.59cyd = 4cyd

Total superstructure concrete = 41cyd + 4cyd = 45cyd

NOTES

This pay item is for the furnishing and placement of concrete for the 6” composite concrete deck.  Deck grooving is assumed to be part of this work and there is no pay item for deck grooving.

It also includes quantities for the semi-integral abutment diaphragm.

Calculations were performed using dimensions from the drawing in the bottom left corner of the deck plan sheet.

This pay item includes installation of the polystyrene placed in the horizontal joint between the diaphragm and the abutment.  The note as specified on page 3-94 of the BDM will be included in the plans.  This joint filled should extend all the way up to the elastomeric bearing pads, as shown in the plans.  This pay item is as per plan based on the note on page 3-94.

COST ESTIMATE

$46,751.40

COST ESTIMATE EXPLANATION

Unit price from Estimator used.

DIAGRAMS

None

CLASS QC1 CONCRETE, ABUTMENT

| ITEM | EXTENSION | GRAND TOTAL | UNIT |
| --- | --- | --- | --- |
| 511 | 45710 | 26 | CY |

CALCULATIONS

Beam Seat Area and Area Under Wingwalls

Installation of new concrete under the beam seat area and the area under the wingwalls will be completed as one pour.

Use abutment length shown in 1983 plans of 48’-4 ¼ inches, or 48.354’

Depth of abutment = 1983 plans show 27” inches, but a field measurement showed that they are actually 36”

Removal depth = 1ft below top of outside edge of middle section of abutment (illustrated in bottom drawing on page 11 of plans).

Assume the middle section of the abutment will be cut level all the way across.

For calculating the replacement concrete quantity, need the account for the fact that the abutment height increases at a rate of 3/16” per foot starting at the outside edge going toward the middle of the abutment to create a crown.

For each half of the abutment, the height will rise approximately 4.6” from the outside edge of the abutment.

For each half of the abutment (24' 2 1/8" = 24.177) = 24.177' X 3/16" = 4.533" = 4.6".

Calculate the volume of the abutment using a rectangular section and a triangular section.

Rectangular Section

48.354’ x 3’ x 1’ = 145.06 cft

Triangular section (from edge to center of abutment)

(48.354’ x (4.6”/12) x 0.5) x 3’ = 27.80cft x 2 sides per abutment = 55.60cft

Total for one abutment = 145.06cft + 55.60cft = 200.66cft x 2 abutments = 401.32cft / 27 = 14.86cyd = 15cyd

Wingwalls

Wingwalls will be competed in 4 separate pours after the beam seat area has been completed.

They are currently only 18” creating somewhat of shelf on top of the beam seat, however with this rehabilitation project, they will have the same  depth (36”) as the beam seat area below them.  This was done to better accommodate a rebar configuration.

Wingwall dimensions come from field measurements.

Wingwall Average height = 31.25in

NW Wingwall Replacement = 36"deep x 31.25in high x 97" wide = 63.15cft /27 = 2.33cyd = 3cyd

NE Wingwall Replacement = 36"deep x 31.25"high x 104in = 67.70cft /27 = 2.50cyd

SW Wingwall Replacement = 36"deep x 31.25in high x 92" wide = 59.89cft /27 = 2.21cyd = 3cyd

SE Wingwall Replacement = 36"deep x 31.25in high x 84in wide = 54.68cft / 27= 2.02cyd = 2cyd

Total = 3 + 2.5 + 3 + 2 =  10.5cyd

Grand Total = 15cyd + 10.5cyd = 25.5cyd = 26cyd

NOTES

This pay item is for the furnishing and placement of concrete in the beam seat area, for the semi-integral diaphragm, and for the wingwalls.

COST ESTIMATE

$27,934.81

COST ESTIMATE EXPLANATION

Unit price from Estimator used.

DIAGRAMS

None

SEALING OF CONCRETE SURFACES, NON-EPOXY

| ITEM | EXTENSION | GRAND TOTAL | UNIT |
| --- | --- | --- | --- |
| 512 | 10050 | 93 | SY |

CALCULATIONS

Deck and Beam Fascia Areas

The sealer will be applied to the outside edge of the 6” composite concrete deck, the entire outside edge of the concrete box beam (27”) and 6” onto the underside of the box beam.

6" deck + 27" beam + 6" underneath deck = 39" = 3.25ft.   Length of precast = 68.84ft

68.84ft x 3.25ft = 223.73sft x 2 fascia areas = 447.46sft…Use 448ft = 49.71syd….Use 50syd.

Abutments

The sealant will cover the new concrete from the top of the beam seat area to 3” below the construction joint.

Sealant coverage will extend from the top of the beam seat area to 3” below the straight construction joint.

Dimensions of rectangular area

Height of Coverage Area = 1’ of new concrete + 3” joint overlap = 1’3” = 1.25’

Abutment Length: Use abutment width from old plans of 48' 4 1/4" = 48.354'.

48.354’ x 1.25’ = 60.44sft x 2 abutments = 120.885sft / 9 = 13.43syd = 14syd

Assume coverage of entire triangular portion that creates the crown.

Length of long side = (48.354’/2) x (4.6”/12) x 0.5 = 4.633sft x 2 areas per abutment = 9.26sft x 2 abutments = 18.53sft / 9 = 2.05syd = 2syd

Wingwalls

Coverage of new wingwall concrete will include the entire front face, entire side face, entire top face, and as much of the back face as can be covered before it meets the embankment.  It was assumed that 1ft of vertical coverage would be attained on each wingwall before reaching the embankment material for the purposes of calculating this quantity.

NW Wingwall

Front Face = 31.25" x 97" = 21.05sft        Side Face = 36"x31.25" = 7.81sft    Top Face = 36"x 97"= 24.25sft         Back Face = 12"x97"= 8.1sft   Total = 61.21sft = 6.8syd

SW Wingwall

Front Face = 31.25" x 92" = 20.0sft        Side Face = 36"x31.25" = 7.81sft    Top Face = 36"x 92"= 23sft         Back Face = 12"x92"= 7.6sft   Total = 58.4sft = 6.48syd

NE Wingwall

Front Face = 31.25" x 104" = 22.6sft        Side Face = 36"x31.25" = 7.81sft    Top Face = 36"x 104"= 26sft         Back Face = 12"x104"= 8.6sft   Total = 65.01sft = 7.22syd

SE Wingwall

Front Face = 31.25" x 84" = 18.2sft        Side Face = 36"x31.25" = 7.81sft    Top Face = 36"x 84"= 21sft         Back Face = 12"x84"= 7.0sft   Total = 54.01sft = 6syd

Wingwall Total = 26.5syd = 27.0syd

Grand Total = 50syd +14syd + 2syd + 27syd = 93syd

NOTES

This pay item is for the application of concrete sealer to the deck and beam fascia area, the new concrete on the abutments, and the new concrete on the wingwalls.

Application locations are called out on pages 12 and 14 of the plans.

COST ESTIMATE

$1,315.73

COST ESTIMATE EXPLANATION

Unit price from Estimator used.

DIAGRAMS

See pages 12 and 14 in the plans.

PRESTRESSED CONCRETE COMPOSITE BOX BEAM BRIDGE MEMBERS, LEVEL 1, CB27-48

| ITEM | EXTENSION | GRAND TOTAL | UNIT |
| --- | --- | --- | --- |
| 515 | 12070 | 7 | EA |

CALCULATIONS

Existing bridge has 7ea 27x48 non-composite concrete box beams.

These will be replaced with 7ea 27x48 composite concrete box beams.

NOTES

None

COST ESTIMATE

$119,000

COST ESTIMATE EXPLANATION

The ODOT Bid Data Item Search spreadsheet was filtered to show this bid item from projects in 2021 and 2022 with 4-11 composite box beams ranging in size from CB21-48 to CB33-48.

The average bid for these projects was $21,079 per beam, and the average awarded bid was $17,965 per beam.

For a similar project, PID 102934, the average bid was $11,300 per beam

$17,000 per beam was selected based on these information above and accounting for inflation over the past few years since PID 102934 took place.

DIAGRAMS

None.

1” PREFORMED EXPANSION JOINT FILLER

| ITEM | EXTENSION | GRAND TOTAL | UNIT |
| --- | --- | --- | --- |
| 516 | 13600 | 40 | SF |

CALCULATIONS

Between Wingwalls and Beams

Wingwall width accounting for skew = 3ft / cos30 = 3.46ft… use 3.5ft

Depth = 27" beam + 6" deck = 33" = 2.75ft

2.75ft x 3.5ft = 9.625sft x 4 quadrants = 38.5sft….use 40sft.

NOTES

This pay item is for the furnishing and installation of expansion joint filler between the wingwalls and box beams in all four quadrants.

The expansion joint filler between the semi-integral abutment diaphragm and the top of the bearing seat shall be included with the Class QC1 Concrete, Abutment pay item and be accompanied by the appropriate plan note, as shown on page 3-94 of the BDM, Note 2 of figure 306-8.

COST ESTIMATE

$720.04

COST ESTIMATE EXPLANATION

Unit price from Estimator used.

DIAGRAMS

None.

SEMI-INTEGRAL ABUTMENT EXPANSION JOINT SEAL

| ITEM | EXTENSION | GRAND TOTAL | UNIT |
| --- | --- | --- | --- |
| 516 | 14020 | 66 | FT |

CALCULATIONS

Length of semi-integral abutment diaphragm with skew= 28.25’/cos30 = 32.62’

32.62’ x 2 abutments = 65.24’ = 66’

NOTES

This material is installed on the vertical back face of the abutment to cover the horizontal joint between the semi-integral diaphragm and the abutment.

This is shown in Figure 306-8 on page 3-94 of the BDM.

The piece should have a vertical dimension across the back of the abutment.

This piece is called out a neoprene sheeting on the plans but should be renamed for clarity.

COST ESTIMATE

$1,1873.72

COST ESTIMATE EXPLANATION

Unit price from Estimator used.

DIAGRAMS

See Figure 306-8 on page 3-94 of the BDM.

JOINT SEALER, 705.4

| ITEM | EXTENSION | GRAND TOTAL | UNIT |
| --- | --- | --- | --- |
| 516 | 31000 | 66 | FT |

CALCULATIONS

Length of joint at bridge deck approach slab (bridge reference line) = (28.25’/cos30) = 32.62’ x 2 abutments = 65.24ft = 66ft

NOTES

This pay item is for the joint sealer installed between the bridge deck and the approach slab at the reference line of the bridge.

It can be paid for as part of the approach slab installation pay item per the standard drawing, however it is added as a separate pay item so that the contractor will not forget about it.

COST ESTIMATE

$1,527.06

COST ESTIMATE EXPLANATION

Unit price from Estimator used.

DIAGRAMS

None.

ELASTOMERIC BEARING WITH INTERNAL LAMINATES ONLY (NEOPRENE)

| ITEM | EXTENSION | GRAND TOTAL | UNIT |
| --- | --- | --- | --- |
| 516 | 43100 | 28 | EA |

CALCULATIONS

2ea per side of beam = 4 total per beam.

7 total beams x 4 = 28 bearings.

NOTES

None

COST ESTIMATE

$3,808.44

COST ESTIMATE EXPLANATION

Unit price from Estimator used.

DIAGRAMS

None.

RAILING (THREE STEEL TUBE BRIDGE RAILING)

| ITEM | EXTENSION | GRAND TOTAL | UNIT |
| --- | --- | --- | --- |
| 517 | 70100 | 140 | FT |

VERIFY RAILING SPACING AND STATION OF FIRST POST ON AND OFF THE BRIDGE. ACCOUNT FOR NEW WIDTH OF WINGWALL.

CALCULATIONS

Total span length = 69.34’ = 70’

70’ x 2 sides – 140ft

NOTES

This is a new item in 2022.  There were only 6 bids found in the ODOT Bid Data Item Search spreadsheet for 2022.

The two closest in quantity for this project were $590/ft and $522/ft.

Average of average bid price was $568/ft and average of awarded bids was $424/ft

Originally had $615/ft to account for inflation, but dropped to $450/ft after discussing with Dave.

It is probably a high estimate.

COST ESTIMATE

$63,000

COST ESTIMATE EXPLANATION

$450 used based on information outlined above.

DIAGRAMS

None.

POROUS BACKFILL WITH GEOTEXTILE FABRIC

| ITEM | EXTENSION | GRAND TOTAL | UNIT |
| --- | --- | --- | --- |
| 518 | 21200 | 30 | CY |

CALCULATIONS

Total abutment length = 48.354ft

This material will extend 2ft out from back face of abutment.

Depth of excavation will be 1’ below cut line elevation that needs to be filled in.

Cut line elevation = 574.55 (rear abutment) and 574.68 (forward abutment).

1ft below cut line elevation = 573.55.

Approximate center of pavement elevation above abutment = 579.87.  Subtract 15” for concrete approach slab, 6” for aggregate base, and roughly 6” for embankment to build up subgrade elevation = 577.62.

577.62- 573.55 = 4.07ft

4.07ft deep x 2ft out from abutment x 48.354 long = 393.6cft x 2 abutments = 787.20cft / 27 = 29.15cyd = 30cyd

NOTES

None

COST ESTIMATE

$2,955.16

COST ESTIMATE EXPLANATION

Unit price from Estimator used.

DIAGRAMS

None.

SPECIAL – STEEL DRIP STRIP

| ITEM | EXTENSION | GRAND TOTAL | UNIT |
| --- | --- | --- | --- |
| 518 | 22300 | 140 | FT |

CALCULATIONS

Span length = 69.34ft … use 70ft x 2 sides – 140FT

NOTES

None.

COST ESTIMATE

$1,559.20

COST ESTIMATE EXPLANATION

Unit price from Estimator used.

DIAGRAMS

None.

6” PERFORATED CORRUGATED PLASTIC PIPE

| ITEM | EXTENSION | GRAND TOTAL | UNIT |
| --- | --- | --- | --- |
| 518 | 40000 | 100 | FT |

CALCULATIONS

Length of abutment = 48.354ft…use 50ft x 2 sides = 100ft of pipe needed.

NOTES

Pipe installed at bottom of excavation along back face of abutment for dewatering.

COST ESTIMATE

$923.77

COST ESTIMATE EXPLANATION

Unit price from Estimator used.

DIAGRAMS

None.

6” NON-PERFORATED CORRUGATED PLASTIC PIPE

| ITEM | EXTENSION | GRAND TOTAL | UNIT |
| --- | --- | --- | --- |
| 518 | 40012 | 24 | FT |

CALCULATIONS

6ft per quadrant needed based on field measurements.

6ft x 4 = 24ft

NOTES

This pipe is installed as the out for the perforated plastic pipe behind the abutments.

COST ESTIMATE

$428.09

COST ESTIMATE EXPLANATION

Unit price from Estimator used.

DIAGRAMS

None.

REINFORCED CONCRETE APPROACH SLABS (T=15”)

| ITEM | EXTENSION | GRAND TOTAL | UNIT |
| --- | --- | --- | --- |
| 526 | 25000 | 156 | FT |

CALCULATIONS

Approach slab dimensions = 28ft x 25ft = 700sft x 2 slabs = 1400sft / 9 = 155.5syd = 156syd

NOTES

Grooving of surface included with this pay item and will not be paid separately.

See standard drawing for additional information.

COST ESTIMATE

$42,247.19

COST ESTIMATE EXPLANATION

Unit price from Estimator used.

DIAGRAMS

None.

TYPE A INSTALLATION

| ITEM | EXTENSION | GRAND TOTAL | UNIT |
| --- | --- | --- | --- |
| 526 | 90010 | 66 | FT |

CALCULATIONS

Skewed distance along end of bridge = 28.25’/cos30 = 32.62’ x 2 sides = 65.24’ = 66’

NOTES

Installation of sleeper slab on both sides of bridge.

COST ESTIMATE

$10,537.23

COST ESTIMATE EXPLANATION

Unit price from Estimator used.

DIAGRAMS

None.

CRUSHED AGGREGATE SLOPE PROTECTION

| ITEM | EXTENSION | GRAND TOTAL | UNIT |
| --- | --- | --- | --- |
| 601 | 20010 | 8 | CY |

CALCULATIONS

Using quantity of 2cyd per quadrant from similar project, PID 102934, for a total of 8cyd.

NOTES

Used to protect slope where drain pipe behind abutment outlets, as well as wrap around the front of the abutment.

COST ESTIMATE

$1,426.11

COST ESTIMATE EXPLANATION

Unit price from Estimator used.

DIAGRAMS

None.

ROCK CHANNEL PROTECTION, TYPE C WITH FILTER

| ITEM | EXTENSION | GRAND TOTAL | UNIT |
| --- | --- | --- | --- |
| 601 | 32200 | 34 | CY |

CALCULATIONS

Assume top of existing rock slope protection is roughly 1ft below the beam seat.

Existing beam seat elevation (rear abutment) = 575.87

Existing rock slope protection elevation = 575.87 – 1’ = 574.87

Elevation of replaced rock slope protection needs to be 1ft lower than existing in order to better accommodate inspection of the abutment and bearing area.

Proposed rock slope protection elevation = 574.87 -1’ – 573.87

Rock slope protection needs to extend 5ft out from the abutment face at a 2:1 slope.

Extending out 5ft at a 2:1 slope gives a 2.5ft depth of material.  Use 3ft for calculations.

Picture a triangle having sides with dimensions of 3ft and 5ft.

Area = 3ft x 5ft x 0.5 = 7.5sft

Use 60ft for the length across the abutment to add coverage along the outside edges

7.5sft x 60ft = 450cft x 2 abutments = 900cft / 27 = 33.33cyd = 34cyd

NOTES

Type C with filter was specified on original plans from 1983.

COST ESTIMATE

$4,772.29

COST ESTIMATE EXPLANATION

Unit price from Estimator used.

DIAGRAMS

None.

INCIDENTALS

MAINTAINING TRAFFIC

| ITEM | EXTENSION | GRAND TOTAL | UNIT |
| --- | --- | --- | --- |
| 614 | 11000 | 1 | LS |

CALCULATIONS

None

NOTES

None

COST ESTIMATE

$5,000

COST ESTIMATE EXPLANATION

The ODOT Bid Data Item Search spreadsheet was filtered to show this bid item from all culvert and bridge projects in 2021 and 2022 in District 2.

The average bid was $10,292 and the average awarded big was $9,076

Traffic control for this project will involve a full road closure on a relatively low-volume route and should be straight forward and simple.

Decided to use a lump sum amount of $5,000 for that reason.

DIAGRAMS

None.

CONSTRUCTION LAYOUT STAKES AND SURVEYING

| ITEM | EXTENSION | GRAND TOTAL | UNIT |
| --- | --- | --- | --- |
| 623 | 10000 | 1 | LS |

CALCULATIONS

None

NOTES

None

COST ESTIMATE

$7,500

COST ESTIMATE EXPLANATION

The ODOT Bid Data Item Search spreadsheet was filtered to show this bid item from all culvert and bridge projects in 2021 and 2022 in District 2.

The average bid was $10,391 and the average awarded big was $9,076

Decided to use $7,500 because it is a smaller job.

DIAGRAMS

None.

MOBILIZATION

| ITEM | EXTENSION | GRAND TOTAL | UNIT |
| --- | --- | --- | --- |
| 624 | 10000 | 1 | LS |

CALCULATIONS

None

NOTES

None

COST ESTIMATE

$20,000

COST ESTIMATE EXPLANATION

Using Table 642.02-1 on page 561 in ODOT CM&S, a project with a total contract amount between $500,000 and $1,000,000 should use a mobilization lump sum amount of $20,000.

DIAGRAMS

None.

Survey Information

Survey points along centerline of road taken from survey basemap.

Points labeled with the Feature Definition as ‘Bridge Deck’ located along the abutments were assumed to the bridge reference line.

All of these points for each abutment were connected with a red line.  The station where the red line crossed the purple stationing line was used as the station for the reference line.

All other stations were determined from these two points.

Those locations are circled in green below.

For the west reference line, the station was determined to be 160+87.17.

For the east reference line, the station was determined to be 161+56.15.

Stationing for points along the approaches is shown in the diagram below.