SCI-335-1170 OPUS SV2(B)

IRON PIN & CAP SET

PID: 107346

FILE: 87100841.DAT OP1616756206823

2005   NOTE:  The IGS precise and IGS rapid orbits were not available

2005   at processing time.  The IGS ultra-rapid orbit was/will be used to

2005   process the data.

2005

NGS OPUS-RS SOLUTION REPORT

========================

All computed coordinate accuracies are listed as 1-sigma RMS values.

For additional information: https://gcc01.safelinks.protection.outlook.com/?url=https%3A%2F%2Fwww.ngs.noaa.gov%2FOPUS%2Fabout.jsp%23accuracy&amp;data=04%7C01%7Cjason.robinson%40dot.ohio.gov%7Cf55d08d874ca42e667d508d8f0465102%7C50f8fcc494d84f0784eb36ed57c7c8a2%7C0%7C0%7C637523532144050960%7CUnknown%7CTWFpbGZsb3d8eyJWIjoiMC4wLjAwMDAiLCJQIjoiV2luMzIiLCJBTiI6Ik1haWwiLCJXVCI6Mn0%3D%7C1000&amp;sdata=bAg463XfI4qFQ6vHl%2FwQMj4qjkmj%2FoeZ7fV1YcZ87MM%3D&amp;reserved=0

USER: jason.robinson@dot.ohio.gov             DATE: March 26, 2021

RINEX FILE: 8710084p.21o                            TIME: 10:59:39 UTC

SOFTWARE: rsgps  1.38 RS52.prl 1.99.3            START: 2021/03/25 15:47:45

EPHEMERIS: igu21504.eph [ultra-rapid]              STOP: 2021/03/25 16:20:30

NAV FILE: brdc0840.21n                        OBS USED:  2961 /  3150   :  94%

ANT NAME: TRMR8           NONE             QUALITY IND.  27.35/ 46.65

ARP HEIGHT: 2.000                         NORMALIZED RMS:        0.352

REF FRAME: NAD_83(2011)(EPOCH:2010.0000)              ITRF2014 (EPOCH:2021.22923)

X:       617098.428(m)   0.007(m)            617097.512(m)   0.007(m)

Y:     -4932470.073(m)   0.034(m)          -4932468.652(m)   0.034(m)

Z:      3983181.192(m)   0.026(m)           3983181.110(m)   0.026(m)

LAT:   38 53 34.77149      0.004(m)        38 53 34.80043      0.004(m)

E LON:  277  7 52.27910      0.005(m)       277  7 52.24872      0.005(m)

W LON:   82 52  7.72090      0.005(m)        82 52  7.75128      0.005(m)

EL HGT:          164.355(m)   0.043(m)               163.118(m)   0.043(m)

ORTHO HGT:	648.63’          197.704(m)   0.047(m) [NAVD88 (Computed using GEOID18)]

UTM COORDINATES    STATE PLANE COORDINATES

UTM (Zone 17)         SPC (3402 OH S)

Northing (Y) [meters]     4306561.795            99199.682	Grid: 325,457.6233	Ground: 325,474.9832

Easting (X)  [meters]      337926.736           568004.102             1,863,526.7910                    1,863,626.1920

Convergence  [degrees]    -1.17362222          -0.23401667

Point Scale                0.99992346           0.99997245

Combined Factor            0.99989768           0.99994666	C.S.F. USED FROM SV2(A) = 1.00005334

US NATIONAL GRID DESIGNATOR: 17SLD3792606561(NAD 83)

BASE STATIONS USED

PID       DESIGNATION                        LATITUDE    LONGITUDE DISTANCE(m)

DI1856 OHHO HOCKING COUNTY CORS ARP        N393207.276 W0822637.876   80206.2

DF4048 GALP GALLIPOLIS CORS ARP            N385039.148 W0821640.091   51577.7

DI1876 OHAD ADAMS COUNTY CORS ARP          N384709.347 W0833322.647   60861.6

DI1854 OHHI HIGHLAND COUNTY CORS ARP       N391200.738 W0833550.494   71703.1

DH9003 WVRA MARSHALL UNIV-RAV CORS ARP     N385628.863 W0814504.842   97064.2

DH9001 WVHU MARSHALL UNIV-HUN CORS ARP     N382522.617 W0822528.918   64939.4

DF4050 GRTN GEORGETOWN CORS ARP            N385134.330 W0835244.711   87749.1

AJ7184 COLB COLUMBUS CORS ARP              N395735.112 W0830244.747  119413.2

AJ7194 MCON MCCONNELSVILLE CORS ARP        N393939.031 W0814945.121  123751.1

NEAREST NGS PUBLISHED CONTROL POINT

HY1311      P 215                          N385315.000 W0825210.000     612.2

This position and the above vector components were computed without any knowledge by the National Geodetic Survey regarding the equipment or field operating procedures used.