Justification for footprint change of existing guardrail on SR-38 in Union County between Approx. SLM’s 3.79 – 4.04.

The existing barrier in this location were 2 separate runs of guardrail (west side of SR-38 only) with an approximate 250’ gap between them. The scope of the existing guardrail condition was noted as “decent” Type 5 with Type E anchor assemblies.

Investigating this location concluded that the protected hazard is the encroaching drop off into the Big Darby Creek (at normal water levels). The “Length of need” for the existing guardrail appears to be roughly based on minimum values of acceptable clear zone widths (per figure 600-1).

An evaluation was performed to verify the length of need and design/construct per the guidance from ODOT’s Location & Design Manual, Volume 1 – section 600, Roadway Design.

The following factors were considered for the barrier design:

- Minimum Clear zone width of 29’ (Figure 600-1)

  - 60 MPH Design Speed

  - ADT between 750-1,500

  - The clear zone width should be increased if a site investigation indicates that doing so would significantly lessen the potential for crashes. For example, if an obstruction exists just outside the required clear zone in an otherwise obstruction-free area, it should be considered for removal or protection. (600.2)

- Runout Length of 210’ (Figure 602-1)

  - 60 MPH Design Speed

  - ADT between 1,000-5,000

- Typical Barrier Minimum Clearance (Figure 603-2)

  - Need to deviate from 6’-3” standard post spacing to reduced post spacing to prohibit deflection paths into existing hazards – in this case Utility pole(s) are a concern.

- Site Considerations (Section 602)

  - 602.1.2 Length of Need on Tangent Alignments

    - Gaps of 300 feet or less between adjacent runs of guardrail should be closed.

  - 602.1.3 Length of Need on Curved Alignments

    - a vehicle leaving the roadway on the outside of a curve will generally follow a tangential runout path.

With all these factors considered, the following design features were incorporated:

- Barrier protection for the entire length of the encroaching drop off into the Big Darby Creek (running parallel to SR-38) equaling 997’.

  - There are areas within this range that does fall just outside Minimum Clear zone width of 29’ but the decision was to increase this (based on the site investigation) to significantly lessen the potential for crashes into water (not to mention closer encroachments during flooding conditions.

  - Constructed continuous guardrail to close the 250’ gap between the 2 runs.

  - Length of Need

  - South (trailing end) calculated for Tangent Alignments

  - North (leading end) calculated for Curved Alignments

  - Half post spacing used at existing utility poles