UTILITIES

THERE ARE NO UNDERGROUND UTILITIES SHOWN ON THIS PLAN. THE NATURE OF THE WORK REQUIRED BY THIS PROJECT WILL NOT AFFECT ANY KNOWN UNDERGROUND UTILITIES THAT EXIST UNDER, OR ADJACENT TO, THE WORK AREA.

NOTIFICATION OF TRAFFIC RESTRICTIONS

THROUGHOUT THE DURATION OF THE PROJECT, THE CONTRACTOR SHALL NOTIFY THE PROJECT ENGINEER IN WRITING OF ALL TRAFFIC RESTRICTIONS AND UPCOMING MAINTENANCE OF TRAFFIC CHANGES. THE CONTRACTOR SHALL ENSURE THE WRITTEN NOTIFICATION IS SUBMITTED IN A TIMELY MANNER TO ALLOW THE PROJECT ENGINEER TO MEET THE REQUIRED TIME FRAMES SET FORTH IN THE TABLE BELOW TO INFORM THE SPECIAL HAULING PERMITS SECTION (HAULING.PERMITS@DOT.OHIO.GOV) AND THE DISTRICT PUBLIC INFORMATION OFFICE (PIO). THIS NOTIFICATION SHALL BE RECEIVED BY THE PROJECT ENGINEER PRIOR TO THE PHYSICAL SETUP OF ANY APPLICABLE SIGNS OR MESSAGE BOARDS.

INFORMATION SHOULD INCLUDE, BUT IS NOT LIMITED TO, ALL CONSTRUCTION ACTIVITIES THAT IMPACT OR INTERFERE WITH TRAFFIC AND SHALL LIST THE SPECIFIC LOCATION, TYPE OF WORK, ROAD STATUS, DATE AND TIME OF RESTRICTION, DURATION OF RESTRICTION, NUMBER OF LANES MAINTAINED, NUMBER OF LANES CLOSED, MINIMUM VERTICAL CLEARANCE, MINIMUM WIDTH OF DRIVABLE PAVEMENT, DETOUR ROUTES, IF APPLICABLE, AND ANY OTHER INFORMATION REQUESTED BY THE PROJECT ENGINEER.

DISTRICT 02 PUBLIC INFORMATION OFFICER

D02.PUBLIC.INFORMATION@DOT.OHIO.GOV

DYLAN FOUKES, DISTRICT TRAFFIC ENGINEER

317 E POE RD

BOWLING GREEN, OH 43402

(419) 373-4303

DYLAN.FOUKES@DOT.OHIO.GOV

STATEWIDE TRAFFIC MANAGEMENT CENTER

PHONE: (614) 387-2438

STATEWIDETMC@DOT.OHIO.GOV

ODOT CO ITS

1606 W BROAD ST

COLUMBUS, OH 43223

614-387-4113

CEN.ITS.LAB@DOT.OHIO.GOV

NOTIFICATION OF TRAFFIC RESTRICTIONS TIME TABLE

| ITEM | DURATION OF CLOSURE | NOTICE DUE TO PERMITS & PIO |
| --- | --- | --- |
| RAMP & ROAD CLOSURES | ≥ 2 WEEKS | 21 CALENDAR DAYS |
|  | >12 HOURS TO <2 WEEKS | 14 CALENDAR DAYS |
|  | ≤ 12 HOURS | 4 BUSINESS DAYS |
| LANE CLOSURES & RESTRICTIONS | ≥ 2 WEEKS | 14 CALENDAR DAYS |
|  | < 2 WEEKS | 5 BUSINESS DAYS |
| START OF CONSTRUCTION & TRAFFIC PATTERN CHANGES | N/A | 14 CALENDAR DAYS PRIOR TO IMPLEMENTATION |

GENERAL COORDINATION AND COOPERATION BETWEEN CONTRACTORS

OTHER PROJECTS MAY BE CONSTRUCTED CONCURRENTLY WITH THIS PROJECT. IT IS IMPERATIVE THAT THE CONTRACTORS COOPERATE FULLY WITH EACH OTHER AS OUTLINED IN SECTION 105.08 OF THE CMS MANUAL. ALL MAINTENANCE OF TRAFFIC SHALL BE COORDINATED BETWEEN PROJECTS AND NOT CONFLICT WITH ONE ANOTHER.

PROJECTS IN THE VICINITY INCLUDE PIDS 92122, 99731, 105531, 106931, 108548 AND 116000.

MAINTAINING ITS DURING CONSTRUCTION

THE CONTRACTOR SHALL MAINTAIN ALL PREEXISTING OR NEWLY INSTALLED PERMANENT ITS/TRAFFIC DEVICES AND INFRASTRUCTURE DURING CONSTRUCTION ACCORDING TO ODOT SUPPLEMENTAL SPECIFICATION 809. DOWNTIME SHALL BE LIMITED TO 72 HOURS MAXIMUM AT EACH CCTV SITE.

ITEM 809, ITS DEVICE, MISC.: REMOVAL AND DELIVERY OF CCTV AND LOWERING UNIT

THE CONTRACTOR SHALL REMOVE THE EXISTING CCTV CAMERAS AND LOWERING UNITS SPECIFIED IN THE PLANS. THE CONTRACTOR SHALL COORDINATE A TIME TO DELIVER THE REMOVED ITEMS TO ODOT CENTRAL OFFICE ITS:

1606 W BROAD ST

COLUMBUS, OH 43223

614-387-4113

CEN.ITS.LAB@DOT.OHIO.GOV

ITEM 809, CCTV IP-CAMERA SYSTEM, QUAD MULTI-VIEW FIXED WITH PTZ

THE CONTRACTOR SHALL FURNISH AND INSTALL THIS ITEM ACCORDING TO ODOT SUPPLEMENTAL SPECIFICATION 809 AS WELL AS ANY STANDARD CONSTRUCTION DRAWINGS NOTED ON THE PLANS. THE CAMERA SHALL BE INSTALLED ON THE NEW LOWERING UNIT AND WIRED AND CONNECTED APPROPRIATELY IN THE EXISTING ITS CABINET TO BE FULLY FUNCTIONAL.

UPON INSTALLATION, THE CONTRACTOR SHALL INSTALL THE CAMERA AND CONNECT THE ETHERNET CABLE INTO THE ODOT NETWORK SWITCH PORT, AS INFORMED BY ODOT ITS. THE CONTRACTOR SHALL CALL THE ODOT ITS LAB TO VERIFY THE CAMERA CONNECTION AND VIEWS WHILE ON SITE INSTALLING THE CAMERA.  THE CONTRACTOR SHALL ADJUST THE CAMERA ORIENTATION OR LENSES AS NEEDED FOR OPTIMAL VIEWS.

THIS CAMERA ITEM HAS 4 FIXED CAMERAS AND ONE PAN/TILT/ZOOM CAMERA IN ONE HOUSING.  THE CONTRACTOR SHALL ORIENT THE FIXED CAMERAS SO ONE IS VIEWING EACH ROADWAY DIRECTION, TO BE APPROVED BY THE ODOT ITS ENGINEER.  THE FIXED CAMERA LENSES SHALL HAVE DIFFERENT LENS COMBINATIONS BASED ON THE LOCATION.   FOR EACH LOCATION, THE CONTRACTOR SHALL FURNISH 5 EXTRA LENSES (IN ADDITION TO THE DEFAULT LENSES) SO VARIOUS COMBINATIONS OF LENS SIZES CAN BE UTILIZED AND THE BEST FINAL CONFIGURATION DETERMINED.  ALL LENSES SHALL BE CAPABLE OF 1080P RESOLUTION.  ODOT ANTICIPATES HALF OF THE LENSES BEING 16 MM AND THE OTHER HALF BEING A COMBINATION OF 6MM AND 12 MM LENSES.  THE CONTRACTOR SHALL DELIVER EXTRA UNUSED LENSES TO ODOT AT THE END OF THE PROJECT.

ITEM 809, CCTV IP-CAMERA SYSTEM, DOME TYPE

THE CONTRACTOR SHALL FURNISH AND INSTALL THIS ITEM ACCORDING TO ODOT SUPPLEMENTAL SPECIFICATION 809 AS WELL AS ANY STANDARD CONSTRUCTION DRAWINGS NOTED ON THE PLANS. THE CAMERA SHALL BE INSTALLED ON THE NEW LOWERING UNIT AND WIRED AND CONNECTED APPROPRIATELY IN THE EXISTING ITS CABINET TO BE FULLY FUNCTIONAL.

UPON INSTALLATION, THE CONTRACTOR SHALL INSTALL THE CAMERA AND CONNECT THE ETHERNET CABLE INTO THE ODOT NETWORK SWITCH PORT, AS INFORMED BY ODOT ITS.  THE CONTRACTOR SHALL CALL THE ODOT ITS LAB TO VERIFY THE CAMERA CONNECTION AND VIEWS WHILE ON SITE INSTALLING THE CAMERA.  THE CONTRACTOR SHALL ADJUST THE CAMERA ORIENTATION OR LENSES AS NEEDED FOR OPTIMAL VIEWS.

ITEM 809, CCTV LOWERING UNIT (CLS) (ALTERNATE 1)

THE CONTRACTOR SHALL FURNISH THE FOLLOWING LOWERING UNIT FOR THIS ALTERNATE BID ITEM: CLS CDP6-DDDN-XX-YY-OH.

THE CONTRACTOR SHALL FURNISH AND INSTALL THIS ITEM ACCORDING TO ODOT SUPPLEMENTAL SPECIFICATION 809 AS WELL AS ANY STANDARD CONSTRUCTION DRAWINGS NOTED ON THE PLANS. THE CONTRACTOR SHALL ORIENT THE CAMERA ARM ACCORDING TO SCD ITS-12.10 SO THE LOWERING DEVICE OPERATOR IS NOT LOWERING THE CAMERA DIRECTLY ABOVE THEM FOR SAFETY REASONS, AND SO THE CAMERA IS CAPABLE OF VIEWING ALL ROADWAY DIRECTIONS WITHOUT BEING OBSTRUCTED BY THE POLE.

THE CONTRACTOR SHALL PROVIDE THE APPROPRIATE LENGTH OF LOWERING UNIT CABLING TO REACH FROM THE LOWERING UNIT TO THE ITS CABINET WHILE PROVIDING A MINIMUM OF 25 FEET OF SLACK IN THE CCTV POLE.  THE LOWERING UNIT CABLING WILL HAVE THREE CAT6 ETHERNET CABLES WHICH SHALL BE CONTINUOUS BETWEEN THE LOWERING UNIT AND THE CABINET OR SPLICED STRAIGHT THROUGH (COLOR TO COLOR) ONTO THE LOWERING UNIT ETHERNET CABLES USING INSULATION DISPLACEMENT CONNECTORS AT THE TOP OF THE POLE JUST INSIDE THE POLE TOP ADAPTER. THE USE OF ETHERNET COUPLERS SHALL NOT BE ACCEPTABLE.

THE CONTRACTOR SHALL TEST EACH ETHERNET CABLE WITH A QUALIFIED TESTER FROM THE CABINET TO THE CAMERA THROUGH THE LOWERING UNIT AND PROVIDE RESULTS TO ODOT TO VALIDATE COMPLETE CONNECTIVITY.  THE CAMERA AND LOWERING UNIT CABLING SHALL BE WIRED AND CONNECTED APPROPRIATELY IN THE EXISTING ITS CABINET TO BE FULLY FUNCTIONAL.

MOST OF THE EXISTING CAMERA LOWERING UNITS HAVE WIRELESS ETHERNET BRIDGES (RADIOS) INSTALLED ON A 12" OR 24" PIPE NIPPLE INSTALLED IN BETWEEN THE CCTV AND THE LOWERING UNIT CLAMSHELL JUNCTION BOX. THE CONTRACTOR SHALL RELOCATE THE PIPE NIPPLE AND RADIOS TO THE NEW LOWERING UNIT AND MAINTAIN THE SAME RADIO ORIENTATION. THE RADIOS SHALL BE CONNECTED TO THE NEW ETHERNET CABLE WITH INSULATION DISPLACING CONNECTORS IN THE CLAMSHELL JUNCTION BOX. THIS WILL REQUIRE FIELD DRILLING A 3/8” HOLE IN THE BOTTOM OF THE CLAMSHELL JUNCTION BOX. THE NEW ETHERNET CABLE SHALL BE PATCHED IN TO THE EXISTING POE INJECTOR IN THE CABINET.  RADIOS MUST BE ONLINE FOR NETWORK CONNECTIVITY FOR CCTV INSTALLATION.

AN ODOT ITS REPRESENTATIVE MUST BE ON SITE FOR THE FIRST REPLACEMENT. REMOTE ASSISTANCE CAN BE USED FOR THE REMAINING SITES.

ITEM 809, CCTV LOWERING UNIT (MG2) (ALTERNATE 2)

THE CONTRACTOR SHALL FURNISH THE FOLLOWING LOWERING UNIT FOR THIS ALTERNATE BID ITEM: MG SQUARED CLDMG2-HYPIP6*3-XXX.

THE CONTRACTOR SHALL FURNISH AND INSTALL THIS ITEM ACCORDING TO ODOT SUPPLEMENTAL SPECIFICATION 809 AS WELL AS ANY STANDARD CONSTRUCTION DRAWINGS NOTED ON THE PLANS. THE CONTRACTOR SHALL ORIENT THE CAMERA ARM ACCORDING TO SCD ITS-12.10 SO THE LOWERING DEVICE OPERATOR IS NOT LOWERING THE CAMERA DIRECTLY ABOVE THEM FOR SAFETY REASONS, AND SO THE CAMERA IS CAPABLE OF VIEWING ALL ROADWAY DIRECTIONS WITHOUT BEING OBSTRUCTED BY THE POLE.

THE CONTRACTOR SHALL PROVIDE THE APPROPRIATE LENGTH OF LOWERING UNIT CABLING TO REACH FROM THE LOWERING UNIT TO THE ITS CABINET WHILE PROVIDING A MINIMUM OF 25 FEET OF SLACK IN THE CCTV POLE.  THE LOWERING UNIT CABLING WILL HAVE THREE CAT6 ETHERNET CABLES WHICH SHALL BE CONTINUOUS BETWEEN THE LOWERING UNIT AND THE CABINET OR SPLICED STRAIGHT THROUGH (COLOR TO COLOR) ONTO THE LOWERING UNIT ETHERNET CABLES USING INSULATION DISPLACEMENT CONNECTORS AT THE TOP OF THE POLE JUST INSIDE THE POLE TOP ADAPTER. THE USE OF ETHERNET COUPLERS SHALL NOT BE ACCEPTABLE.

THE CONTRACTOR SHALL TEST EACH ETHERNET CABLE WITH A QUALIFIED TESTER FROM THE CABINET TO THE CAMERA THROUGH THE LOWERING UNIT AND PROVIDE RESULTS TO ODOT TO VALIDATE COMPLETE CONNECTIVITY.  THE CAMERA AND LOWERING UNIT CABLING SHALL BE WIRED AND CONNECTED APPROPRIATELY IN THE EXISTING ITS CABINET TO BE FULLY FUNCTIONAL.

MOST OF THE EXISTING CAMERA LOWERING UNITS HAVE WIRELESS ETHERNET BRIDGES (RADIOS) INSTALLED ON A 12" OR 24" PIPE NIPPLE INSTALLED IN BETWEEN THE CCTV AND THE LOWERING UNIT CLAMSHELL JUNCTION BOX. THE CONTRACTOR SHALL RELOCATE THE BOTTOM HALF OF THE CLAMSHELL JUNCTION BOX WITH PIPE NIPPLE AND RADIOS TO THE NEW LOWERING UNIT AND MAINTAIN THE SAME RADIO ORIENTATION. THE RADIOS SHALL BE CONNECTED TO THE NEW ETHERNET CABLE WITH INSULATION DISPLACING CONNECTORS IN THE CLAMSHELL JUNCTION BOX.  THE NEW ETHERNET CABLE SHALL BE PATCHED IN TO THE EXISTING POE INJECTOR IN THE CABINET.  RADIOS MUST BE ONLINE FOR NETWORK CONNECTIVITY FOR CCTV INSTALLATION.

AN ODOT ITS REPRESENTATIVE MUST BE ON SITE FOR THE FIRST REPLACEMENT.  REMOTE ASSISTANCE CAN BE USED FOR THE REMAINING SITES.