PID 108099, D07-BH-FY23(A): QUANTITY CALCULATIONS

Calculated by: Parker Brown, Date: 4/19/22

Checked by:

Revised by: Parker Brown, Date: 6/7/23

Roadway

- Item 201 Clearing and grubbing

  - SHE-47-1054 : 330sf

  - SHE-47-1833 : 347sf x 2 + 180sf x 2 = 1054sf

  - Total = 1384sf

- Item 202 Fence Removed for Reuse

  - DAR-36-1228 L/R: 10ft

- Item 203 Excavation

  - SHE-47-1833 :

    - Rear = 35sf x 40ft = 1400cf = 52cy

    - Fwd = 7sf x 40ft = 280cf = 11cy

    - Total = 63cy

  - SHE-75-1149L/R :

    - Left Rear = 53sf x 26ft = 1378cf = 52cy

    - Right Rear = 53sf x 20ft = 1060cf = 40cy

    - Total = 92cy

  - Total = 155cy

- Item 607 Fence Rebuilt, Type 47

  - DAR-36-1228 L/R: 10ft

Erosion Control

- Item 601 Dumped Rock Fill, Type C

  - SHE-47-1054 : 16ft x 34.5ft x 2ft = 1104cf

  - SHE-47-1833 : (40ft x 21.5 x 2) x 2 + (5ft x 47ft x 2ft) x 2 = 4380cf

  - SHE-75 : (26ft x 23ft x 2ft) + (13.167ft x 10ft x 2ft) + (16.5ft x 23ft x 2ft) = 2118cf

  - Total = 1104cf + 4380cf + 2118cf = 7602cf = 282cy

    - Note: A depth of 2’ was used for Type C rock

- Item 659E10000 Seeding and Mulching

  - DAR-127 (Entire Area): 516sf

  - SHE-47-1054 (North Bank Only): 1515sf

  - SHE-47-1833 (Entire Area): 1500sf + 1900sf = 3400sf

  - SHE-75 (North Bank only): 1180sf + 1020sf = 2200sf

  - Total = 7631sf / 9 = 848sy

- Item 659E00570 Seeding and Mulching for Wildlife

  - DAR-36 (entire area): 578sf +1154sf + 1180sf + 2730sf = 5642sf

  - SHE-47-1054 (South Bank Only): 580sf

  - SHE-75 (South Bank Only): 1530sf + 1395sf = 2925sf

  - Total = 9147sf / 9 = 1017sy

- Item 659E14000 Repair Seeding and Mulching

  - 5% of total (Sum of both seeding and mulching items) = 94sy

- Item 659E20000 Commercial Fertilizer

  - 30lbs/1000sf : 16778sf x (30lbs/1000sf) = 503.34lbs / (2000lbs/ton) = 0.2546tons = 0.26tons

- Item 659E35000 Water

  - 2 applications of 0.0027M.Gal/1sy : 2(1865sy x (0.0027M.Gal/1sy)) = 11 M.Gal

- Item 832E30000 Erosion Control

  - 1000 each for each location

  - Total = 7 x 1000 = 7000

Structure: DAR-36-1228L

- Item 503 Cofferdams and Excavation Bracing

  - Lump Sum (LS)

- Item 503 Unclassified Excavation

  - Lump Sum (LS)

- Special item 507 Pile Encasement

  - See sheet 6/6 = 17ft x 18 piles = 306ft

- Item 509E25000 Uncoated Reinforcing Steel

  - Abutment Refacing = 565lb

- Item 510 Dowel holes with nonshrink, nonmetallic grout

  - Abutment Refacing (sheet 5/6) = 64 total

- Item 516 1/2” Preformed Expansion Joint Filler

  - Rear Abutment = 46.24ft x 0.5ft = 23.12sf

  - Fwd Abutment = 44.03ft x 0.5ft = 22.03sf

  - Total = 45.15sf = 46sf

- Special Item 530 Structures, Misc.: Abutment Refacing with Galvanic Anode Protection

  - Rear Abutment = 124sf

  - Fwd Abutment = 133sf

  - Total = 257sf

Structure: DAR-36-1228R

- Item 503 Cofferdams and Excavation Bracing

  - Lump Sum (LS)

- Item 503 Unclassified Excavation

  - Lump Sum (LS)

- Special item 507 Pile Encasement

  - See sheet 6/6 = 17ft x 18 piles = 306ft

- Item 509E25000 Uncoated Reinforcing Steel

  - Abutment Refacing = 524lb

- Item 510 Dowel holes with nonshrink, nonmetallic grout

  - Abutment Refacing (sheet 5/6) = 64 total

- Item 516 1/2” Preformed Expansion Joint Filler

  - Rear Abutment = 45.03ft x 0.5ft = 22.52sf

  - Fwd Abutment = 45.02ft x 0.5ft = 22.51sf

  - Total = 45.03sf = 46sf

- Special Item 530 Structures, Misc.: Abutment Refacing with Galvanic Anode Protection

  - Rear Abutment = 102 SF

  - Fwd Abutment = 101 SF

  - Total = 203 SF

Structure: DAR-127-0137

- Item 202 Portions of Structure removed, Over 20-foot span, as per plan

  - Lump Sum (LS)

- Item 503 Cofferdams and Excavation Bracing

  - Lump Sum (LS)

- Item 503 Unclassified Excavation

  - Lump Sum (LS)

- Special item 507 Pile Encasement

  - See sheet 3/3 = 14.25ft x 1 piles = 14.25ft

- Item 507 Piling, Misc.: Pile Encasement Banding

  - Lump Sum (LS)

Structure: SHE-47-1054

- Item 844 Concrete Patching with Galvanic Anode Protection, As Per Plan

  - Rear Abutment = 20sf

Structure: SHE-47-1833

- Item 202 Portions of Structure removed, Over 20-foot span, as per plan

  - Lump Sum (LS)

- Item 503 Unclassified excavation

  - 1cy behind the two wingwall rebuilds = 2cy

- Item 509E25000 Uncoated Reinforcing Steel

  - Abutment Refacing (sheet 3/3) = 1200lb

- Item 510 Dowel holes with nonshrink, nonmetallic grout

  - Abutment Refacing (sheet 5/6) = 168 total

- Item 511 Class QC1 Concrete, abutment

  - Rear abutment = 8.97sf (area from ORD) x 1.5ft = 13.46cf

  - Rear abutment = 9.04sf (area from ORD) x 1.5ft = 13.56cf

  - Total = 27.02cf = 1.0007cy = 2cy

- Item 516 1/2” Preformed Expansion Joint Filler

  - Rear Abutment = 42.05ft x 0.5ft = 21.02sf

  - Fwd Abutment = 41.02ft x 0.5ft = 20.51sf

  - Total = 41.53 = 42sf

- Special Item 530 Structures, Misc.: Abutment Refacing with Galvanic Anode Protection

  - Rear Abutment = 276.86sf (Area from ORD) =284sf

  - Fwd Abutment = 245.19sf (Area from ORD) =252sf

  - Total = 536sf

- Item 844 concrete patching

  - Abutments = 5sf

Structure: SHE-75-1149L

- Item 503 Cofferdams and Excavation Bracing

  - Lump Sum (LS)

- Item 503 Unclassified Excavation

  - Lump Sum (LS)

- Special item 519 Composite Fiber Wrap

  - See sheet 2/2 = 673sf

Structure: SHE-75-1149R

- Excavation Bracing

  - Lump Sum (LS)

- Item 503 Unclassified Excavation

  - Lump Sum (LS)

- Special item 519 Composite Fiber Wrap

  - See sheet 2/2 = 673sf

Maintenance of Traffic

- LEO

  - SHE-75-1149L/R: 20HOURS PER NKH

Incidentals

- Item 614-Maintaining Traffic

  - Lump Sum (LS)

- Item 619-Field Office, Type A

  - 5 months

- Item 623-construction Layout stakes and Surveying

  - Lump Sum (LS)

- Item 624-Mobilization

  - Lump Sum (LS)

END OF CALCULATIONS