PID 110152, D07 BP FY26: QUANTITY CALCULATIONS

Calculated by: Jun Xie, Date: 03/20/25

Checked by:

Revised by: Jun Xie, Date: 06/05/25

Plan Splits:

- 01/IMS/47

  - CLA-675-0079

  - MOT-075-0259

  - MOT-675-0296

- 02/NHS/47

  - CLA-068-0773R

Erosion Control

- Item 659E10000 SEEDING AND MULCHING

(01/IMS/47)

  - CLA-675-0079

    - 2 (areas along each outside shoulder) x 20’(twice SS832 minimum width of 10’) x 175’(length) = 7000sf

  - MOT-075-0259

    - 1 (area in the median) x 20’(twice SS832 minimum width of 10’) x 175’(length) = 3500sf

  - MOT-675-0296

    - 1 (area in the median) x 20’(twice SS832 minimum width of 10’) x 175’(length) = 3500sf

  - Total: 7000sf + 3500sf + 3500sf = 14000sf = 1556sy

Grand Total: 1556sy

- Item 659E14000 REPAIR SEEDING AND MULCHING

(01/IMS/47)

- CLA-675-0779 / MOT-075-0259 / MOT-675-0296:

    - 1556sy x 5% = 78sy

Grand Total: 78sy

- Item 659E20000 COMMERCIAL FERTILIZER

(01/IMS/47)

- CLA-675-0779 / MOT-075-0259 / MOT-675-0296:

    - 1556sy / (7410 sy/ton) = 0.21 TON

Grand Total: 0.21TON

- Item 659E35000 WATER

(01/IMS/47)

- CLA-675-0779 / MOT-075-0259 / MOT-675-0296:

    - 2 applications x 1556sy x 0.0027MGal/sy = 8.4 MGal

Grand Total: 9 MGal

- Item 832E30000 EROSION CONTROL

(01/IMS/47)

- CLA-675-0779 / MOT-075-0259 / MOT-675-0296:

[SS 832.05.B.6] Construction entrance size: 10’(width) x 175’(length) x 0.5’(thickness, MTP suggests) = 875 cf = 32.4cy

[SS 832 Appendix F] Construction entrance fixed price: $104/cy x 32.4cy = $3,370

    - CLA-675-0779: Assume one construction entrance along each outside shoulder: 2 x $3370 = $6,740

    - MOT-075-0259: Assume one construction entrance in the median: $3,370

    - MOT-675-0296: Assume one construction entrance in the median: $3,370

    - Total Construction Entrance Cost: $6740 + $3370 + $3370 = $ 13,480

    - Erosion Control QTY: 13,480 EA. Adopt 20,000 EA

Grand Total: 20,000 EA

Structure: CLA-068-0773R

(02/NHS/47)

- Item 514E00050 SURFACE PREPARATION OF EXISTING STRUCTURAL STEEL

  - CLA-068-0773R:

    - Beam Painting:

      - Beam length per beam line: 48.96’ + 65.62’ + 52.50’ + 2 x 0.5’ = 168.08’

        - (assume 0.5’ additional beam end length from abutment centerline)

      - Beam perimeter to paint: (12”-0.65”) x 2 + (36.01” – 1.02”) x 2 + 12” = 104.68” = 8.73’

      - Beam painting area = 168.08’ x 8.73’ x 4 (beam lines) = 5869.36 ft2

    - Field Splice:

      - 2.375’ x [(0.4375”+0.5625”)/12]’ x 2 (sides of beam) x 2 (splices) x 4 (beam lines) = 3.17 ft2

    - Cross Frame:

      - Cross frame length total per location:

        - 8.75’ + 9.20’ + 9.20’ = 27.15’

      - Cross frame angle perimeter to paint:

        - 3” x 4 = 12” = 1’

      - # of intermediate cross frame locations: 44

      - Cross frame painting area = 27.15’ x 1’ x 44 = 1194.6 ft2

    - End Frame:

      - Angles:

        - End frame angle length per unit:

          - 4.44’ x 2 + 3.36’ x 2 + 10.03’ = 25.63’

        - End frame angle perimeter to paint:

          - 4” x 4 = 16” = 1.34’

        - End frame angle area to paint:

          - 25.63’ x 1.34’ x 3 (units) x 2 (abutments)= 206.1 ft2

      - Plates:

        - End frame plate area to paint:

          - 6” x 10” x 2 (faces) x 3 (plates) x 3 (units) x 2 (abutments) = 2160 in2  = 15 ft2

      - Total end frame paint area = 206.1 ft2 +15 ft2 = 221.1 ft2

    - Bearings:

      - 3 ft2 x 4 (bearings) x 4 (beam lines) = 48 ft2

    - Scuppers:

      - Scupper length: 4’-8” = 4.67’

      - Scupper perimeter to paint:

        - π x 6” = 18.85” = 1.57’

      - Total scuppers paint area:

        - 1.57’ x 4.67’ x 6 (# of scuppers total) = 44 ft2

    - Total painting quantities:

      - 5869.36 ft2 + 3.17 ft2 +1194.60 ft2 + 221.10 ft2 + 48 ft2 + 44 ft2 = 7380.23 ft2. Round up to 7381 ft2

- Item 514E00056 FIELD PAINTING OF EXISTING STRUCTURAL STEEL, PRIME COAT

  - CLA-068-0773R:

    - Total painting quantities:

      - 7381 ft2

- Item 514E00060 FIELD PAINTING STRUCTURAL STEEL, INTERMEDIATE COAT

  - CLA-068-0773R:

    - Total painting quantities:

      - 7381 ft2

- Item 514E00066 FIELD PAINTING STRUCTURAL STEEL, FINISH COAT

  - CLA-068-0773R:

    - Total painting quantities:

      - 7381 ft2

- Item 514E00504 GRINDING FINS, TEARS, SLIVERS ON EXISTING STRUCTURAL STEEL

  - CLA-068-0773R:

Per Ohio DOT BDM 2020 section 404.1.11, estimating 1-minute MNHR for each linear foot of beam/girder to be coated.

    - Beam length per beam line: 48.96’ + 65.62’ + 52.50’ + 2 x 0.5’ = 168.08’

      - (assume 0.5’ additional beam end length from abutment centerline)

    - Total beam length:

      - 168.08’ x 4 (beam lines) = 672.32’

    - Total Man Hour:

      - 672.32’ x (1 min/ft) x (1 hr/60 min) = 11.2 hours.

      - Round up to 12 hours.

- Item 514E10000 FINAL INSPECTION REPAIR

  - CLA-068-0773R:

Per CMS 514.21, final inspection repair (1 location per 300 ft beam line and 2.5% of all cross assemblies)

    - 672.32ft / 300ft + 0.025 x [ 44 (# of intermediate cross frame assemblies) + 6 (# of end cross frame assemblies) = 3.49 each

    - Round up to 4 each.

Structure: CLA-675-0079

(01/IMS/47)

- Item 514E00050 SURFACE PREPARATION OF EXISTING STRUCTURAL STEEL

  - CLA-675-0079:

    - Beam Painting:

      - Beam length per beam line: 60’ + 85.75’ + 85.75’ + 68.75’ + 2 x 0.5’ = 301.25’

        - (assume 0.5’ additional beam end length from abutment centerline)

      - Beam perimeter to paint: (16.51” - 0.8”) x 2 + (36.08” - 1.35”) x 2 + 16.51” = 117.39” = 9.79’

      - Beam painting area = 301.25’ x 9.79’ x 5 (beam lines) = 14746.19 ft2

    - Field Splice:

      - 5’ x [(0.5625”+0.6875”)/12]’ x 2 (sides of beam) x 3 (splices) x 5 (beam lines) = 15.63 ft2

    - Cross Frame:

      - Cross frame length total per location:

        - 9.625’ + 10.036’ x 2 = 29.696’

      - Cross frame angle perimeter to paint:

        - 3” x 4 = 12” = 1’

      - # of intermediate cross frame locations: 80

      - Cross frame painting area = 29.696’ x 1’ x 80 = 2375.64 ft2

    - End Frame:

      - Angles:

        - End frame angle length per unit:

          - 4.35’ x 2 + 3.34’ x 2 + 9.625’ = 25’

        - End frame angle perimeter to paint:

          - 4” x 4 = 16” = 1.34’

        - End frame angle area to paint:

          - 25’ x 1.34’ x 4 (units) x 2 (abutments)= 268 ft2

      - Plates:

        - End frame plate area to paint:

          - 6” x 10” x 2 (faces) x 3 (plates) x 4 (units) x 2 (abutments) = 2880 in2 = 20 ft2

      - Total end frame paint area = 268 ft2 +20 ft2 = 288 ft2

    - Bearings:

      - 3 ft2 x 5 (bearings) x 5 (beam lines) = 75 ft2

    - Scuppers:

      - Scupper length: 4’

      - Scupper perimeter to paint:

        - π x 6” = 18.85” = 1.57’

      - Total scuppers paint area:

        - 1.57’ x 4’ x 12 (# of scuppers total) = 75.36 ft2

    - Total painting quantities:

      - 14746.19 ft2 + 15.63 ft2 + 2375.64 ft2 + 288 ft2 + 75 ft2 + 75.36 ft2 = 17575.82 ft2. Round up to 17576 ft2

- Item 514E00056 FIELD PAINTING OF EXISTING STRUCTURAL STEEL, PRIME COAT

- CLA-675-0079:

- Total painting quantities:

      - 17576 ft2

- Item 514E00060 FIELD PAINTING STRUCTURAL STEEL, INTERMEDIATE COAT

- CLA-675-0079:

- Total painting quantities:

      - 17576 ft2

- Item 514E00066 FIELD PAINTING STRUCTURAL STEEL, FINISH COAT

- CLA-675-0079:

- Total painting quantities:

      - 17576 ft2

- Item 514E00504 GRINDING FINS, TEARS, SLIVERS ON EXISTING STRUCTURAL STEEL

- CLA-675-0079:

Per Ohio DOT BDM 2020 section 404.1.11, estimating 1-minute MNHR for each linear foot of beam/girder to be coated.

- Beam length per beam line: 60’ + 85.75’ + 85.75’ + 68.75’ + 2 x 0.5’ = 301.25’

      - (assume 0.5’ additional beam end length from abutment centerline)

- Total beam length:

      - 301.25’ x 5 (beam lines) = 1506.25’

- Total Man Hour:

      - 1506.25’ x (1 min/ft) x (1 hr/60 min) = 25.11 hours.

      - Round up to 26 hours.

- Item 514E10000 FINAL INSPECTION REPAIR

- CLA-675-0079:

Per CMS 514.21, final inspection repair (1 location per 300 ft beam line and 2.5% of all cross assemblies)

- 1506.25ft / 300ft + 0.025 x [ 80 (# of intermediate cross frame assemblies) + 8 (# of end cross frame assemblies) = 7.22 each

- Round up to 8 each.

Structure: MOT-075-0259

(01/IMS/47)

- Item 514E00051 SURFACE PREPARATION OF EXISTING STRUCTURAL STEEL, AS PER PLAN

- MOT-075-0259:

    - Pier 3 cap steel box interior wall painting:

      - Box length to be painted: (6’-3”) + (5’-2 15/16”) = 11.5’

      - Interior perimeter to paint: 3’ x 2 + (7’-8”) x 2 = 21.34’

      - Steel box interior wall painting area = 11.5’ x 21.34’ = 245.41 ft2

    - End diaphragm:

      - Diaphragm: [3’ x (7’-8”) - (3’-4”) x (1’-4”)] x 2 faces = 37.12 ft2

    - Intermediate diaphragm:

      - Diaphragm: [3’ x (7’-8”) – (3’-6”) x (1’-4”)] x 4 faces = 73.34 ft2

      - Stiffener: 8” x (7’-8”) x 16 faces = 81.78 ft2

      - Total: 73.34 ft2 + 81.78 ft2 = 155.12 ft2

    - Cover plate (Interior surface):

      - (1’-10”) x (3’- 8 1/2”) = 6.80 ft2

    - Total painting quantities:

      - 245.4 ft2 + 37.07 ft2 + 155.12 ft2 + 6.8 ft2 = 444.39 ft2. Round up to 445 ft2

- Item 514E00057 FIELD PAINTING OF EXISTING STRUCTURAL STEEL, PRIME COAT, AS PER PLAN

- MOT-075-0259:

- Total painting quantities:

      - 445 ft2

- Item 514E00061 FIELD PAINTING STRUCTURAL STEEL, INTERMEDIATE COAT, AS PER PLAN

- MOT-075-0259:

- Total painting quantities:

      - 445 ft2

- Item 514E00067 FIELD PAINTING STRUCTURAL STEEL, FINISH COAT, AS PER PLAN

- MOT-075-0259:

- Total painting quantities:

      - 445 ft2

- Item 514E00504 GRINDING FINS, TEARS, SLIVERS ON EXISTING STRUCTURAL STEEL

- MOT-075-0259:

Per Ohio DOT BDM 2020 section 404.1.11, estimating 1-minute MNHR for each linear foot of beam/girder to be coated.

- Total paint box length: 11.5’

- Total Man Hour:

      - 11.5’ x (1 min/ft) x (1 hr/60 min) = 0.192 hours.

      - Round up to 1 hours.

- Item 514E10000 FINAL INSPECTION REPAIR

- MOT-075-0259:

Per CMS 514.21, final inspection repair (1 location per 300 ft beam line and 2.5% of all cross assemblies)

- 11.5ft / 300ft + 0.025 x [6 (# of diaphragms)]= 0.189 each

- Round up to 1 each.

- Item 516E25000 NYLON REINFORCED NEOPRENE SHEETING

- 6.5065 ft2. Round up to 7 ft2 for each access door.

- Total: 7 SF x 2 = 14 ft2 for both doors.

Structure: MOT-675-0296

(01/IMS/47)

- Item 512E10100 SEALING OF CONCRETE SURFACES (EPOXY-URETHANE)

  - MOT-675-0296:

    - Perimeter: 5.5”+ 12.2”+ 19.1”+12”+ 19”+2”+15.5”+2”+10”+13.67” = 110.97” = 9.25’

    - Length: (bridge limits)282.52’ x 2 = 565.04’

    - Total area: (110.97/12)’ x 565.04’ = 5225.21 ft2 = 580.6 yd2 = 581 yd2

- Item 512E74000 REMOVAL OF EXISTING COATINGS FROM CONCRETE SURFACES

- MOT-675-0296:

- Perimeter: 5.5”+ 12.2”+ 19.1”+12”+ 19”+2”+15.5”+2”+10”+13.67” = 110.97” = 9.25’

- Removal length: 142’+27’+6’+11”+44’+133’ = 363’

- Total area: (110.97/12)’ x 363’ = 3356.85 ft2 = 373 yd2

- Item 514E00050 SURFACE PREPARATION OF EXISTING STRUCTURAL STEEL

- MOT-675-0296:

- Beam Painting:

      - Beam length per beam line: 57’ x 2 + 82’ x 2 + 2 x 0.5’ = 279’

        - (assume 0.5’ additional beam end length from abutment centerline)

      - Beam perimeter to paint: (15.805”-0.775”) x 2 + (33.93”-1.275”) x 2 + 15.805” = 111.175” = 9.27’

      - Beam painting area = 279’ x 9.27’ x 6 (beam lines) = 15508.92 ft2

- Field Splice:

      - 4.7083’ x [(0.5625”+0.6875”)/12]’ x 2 (sides of beam) x 3 (splices) x 6 (beam lines) = 17.66 ft2

- Cross Frame:

      - Cross frame length total per location:

        - 8.5’ + 8.9067’ x 2 = 26.3135’

      - Cross frame angle perimeter to paint:

        - 3” x 4 = 12” = 1’

      - # of intermediate cross frame locations: 100

      - Cross frame painting area = 26.3135’ x 1’ x 100 = 2631.35 ft2

- End Frame:

      - Standard End Frame:

        - Angles:

          - End frame angle length per unit:

            - 3.944’ x 2 + 3.088’ x 2 + 8.5’ = 22.564’

          - End frame angle perimeter to paint:

            - 4” x 4 = 16” = 1.34’

          - End frame angle area to paint:

            - 22.564’ x 1.34’ x 4 (units) x 2 (abutments)= 241.89 ft2

        - Plates:

          - End frame plate area to paint:

            - 6” x 10” x 2 (faces) x 3 (plates) x 4 (units) x 2 (abutments) = 2880 in2 = 20 ft2

        - Total standard end frame paint area = 241.89 ft2 +20 ft2 = 261.89 ft2

      - Special End Frame:

        - Angles:

          - End frame angle length per unit:

            - 3.62’ x 4 + 2.93’ x 2 + 8.5’ = 28.84’

          - End frame angle perimeter to paint:

            - 4” x 4 = 16” = 1.34’

          - End frame angle area to paint:

            - 28.84’ x 1.34’ x 1 (units) x 2 (abutments)= 77.29 ft2

        - Plates:

          - End frame plate area to paint:

            - 6” x 10” x 2 (faces) x 5 (plates) x 1 (unit) x 2 (abutments) = 1200 in2 = 8.33 ft2

        - Total special end frame paint area = 77.29 ft2 + 8.33 ft2 = 85.62 ft2

      - Total end frame painting area = 261.89 ft2  + 85.62 ft2 = 347.51 ft2

- Bearings:

      - 3 ft2 x 5 (bearings) x 6 (beam lines) = 90 ft2

- Scuppers:

      - Scupper length: 3’-8”

      - Scupper perimeter to paint:

        - π x 6” = 18.85” = 1.57’

      - Total scuppers paint area:

        - 1.57’ x 3.67’ x 12 (# of scuppers total) = 69.12 ft2

- Total painting quantities:

      - 15508.92 ft2 + 17.66 ft2 + 2631.35 ft2 + 347.51 ft2 + 90 ft2 + 69.12 ft2 = 18664.56 ft2. Round up to 18665 ft2

- Item 514E00056 FIELD PAINTING OF EXISTING STRUCTURAL STEEL, PRIME COAT

- MOT-675-0296:

- Total painting quantities:

      - 18665 ft2

- Item 514E00060 FIELD PAINTING STRUCTURAL STEEL, INTERMEDIATE COAT

- MOT-675-0296:

- Total painting quantities:

      - 18665 ft2

- Item 514E00066 FIELD PAINTING STRUCTURAL STEEL, FINISH COAT

- MOT-675-0296:

- Total painting quantities:

      - 18665 ft2

- Increase estimated bid price for the two-color finish coat.

- Item 514E00504 GRINDING FINS, TEARS, SLIVERS ON EXISTING STRUCTURAL STEEL

- MOT-675-0296:

Per Ohio DOT BDM 2020 (07/19/2024) section 404.1.11, estimating 1-minute MNHR for each linear foot of beam/girder to be coated.

- Beam length per beam line: 57’ x 2 + 82’ x 2 + 2 x 0.5’ = 279’

      - (assume 0.5’ additional beam end length from abutment centerline)

- Total beam length:

      - 279’ x 6 (beam lines) = 1674’

- Total Man Hour:

      - 1674’ x (1 min/ft) x (1 hr/60 min) = 27.9 hours.

      - Round up to 28 hours.

- Item 514E10000 FINAL INSPECTION REPAIR

- MOT-675-0296:

Per CMS 514.21, final inspection repair (1 location per 300 ft beam line and 2.5% of all cross assemblies)

- 1674ft / 300ft + 0.025 x [ 100 (# of intermediate cross frame assemblies) + 10 (# of end cross frame assemblies) = 8.33 each

- Round up to 9 each.

Maintenance of Traffic

- Item 614E11110 LAW ENFORCEMENT OFFICER WITH PATROL CAR FOR ASSISTANCE

- 400 Hours

- Item 614E12380 WORK ZONE IMPACT ATTENUATOR, 24" WIDE HAZARDS, (UNIDIRECTIONAL)

- Based on MTP suggested construction entrance quantities

    - CLA-675-0779: Assume one construction entrance along each outside shoulder: 2 entrance x 2 attenuators = 4 EA

    - MOT-675-0296: Assume one construction entrance in the median: 1 entrance x 2 attenuators = 2 EA

  - Total: 6 EACH

- Item 614E12420 DETOUR SIGNING

- LS

- Item 614E13310 BARRIER REFLECTOR, TYPE 1 (ONE WAY)

[MT-101.70] 50’ max intervals.

- Total = (1 reflector / 50’ portable barrier) x 1300’ (total PB length) = 26 EA

- Item 614E13350 OBJECT MARKER, ONE WAY

[MT-101.70] 50’ max intervals.

- Total = (1 marker / 50’ portable barrier) x 1300’ (total PB length) = 26 EA

- Item 622E41100 PORTABLE BARRIER, UNANCHORED

- CLA-675-0779:

    - SB outside shoulder closure: 420’

    - NB outside shoulder closure: 420’

    - Total: 840’

- MOT-675-0296:

    - SB inside shoulder closure: 420’

    - Total: 420’

  - Grand total: 840’ + 420’ = 1260’. Round up to 1300’

Incidentals

- Item 614E11000 MAINTAINING TRAFFIC

- LS

- Item 624E10000 MOBILIZATION

- LS

END OF CALCULATIONS