PID 114958, D07-BH-FY25(C): QUANTITY CALCULATIONS

Calculated by: Jun Xie, Date: 5/31/24

Checked by:

Revised by:

Plan Splits:

- 01/STR/13

  - CHP-245-1302

  - CHP-296-0021

  - CHP-296-0122

  - CHP-296-0212

- 02/S<2/13

  - CLA-40-0530

Roadway 01

(01/STR/13)

- Item 202E23500 WEARING COURSE REMOVED

- CHP-245-1302:

    - On Bridge: (32’ x 86’) = 2752sf / 9= 306sy

    - Off Bridge: (ORD: STA. 681+70 to STA. 683+20) 4146.61sf / 9 = 461sy;

    - Off Bridge: (ORD: STA. 684+06 to STA. 685+56) 4050.45sf / 9 = 451sy.

    - Total = 1218sy

  - CHP-296-0021:

    - On Bridge: (ORD) 4765.27sf / 9= 530sy

    - Off Bridge: (ORD) 4505.11sf + 6189.00sf = 10694.11sf / 9 = 1189sy

    - Total = 1719sy

  - CHP-296-0122:

    - On Bridge: (ORD) 3338.64sf / 9= 371sy

    - Off Bridge: (ORD) 4753.06sf + 4713.64sf = 9466.7sf / 9 = 1052sy

    - Total = 1423sy

  - CHP-296-0212:

    - On Bridge: (ORD) 3572.64sf / 9= 397sy

    - Off Bridge: (ORD) 4246.62sf + 4221.55sf = 8468.17sf / 9 = 941sy

    - Total = 1338sy

  - Total: 5698sy

- Item 209E15000 RESHAPING UNDER GUARDRAIL

  - CHP-245-1302:

    - (ORD: STA. 681+70 to STA. 683+20) 160’ + 39’ = 199’ = 2 STA.

    - (ORD: STA. 684+06 to STA. 685+56) 166’ + 172’ = 338’ = 4 STA.

    - Total = 6 STA.

  - CHP-296-0021:

    - Off Bridge: (ORD: STA. 09+75.00 to STA. 10+98.34) 119.14’ + 80.84’ = 200’ = 2 STA.

    - Off Bridge: (ORD: STA. 12+23.64 to STA. 14+25.00) 222.00’ + 190.11’ = 413’ = 5 STA.

    - Total = 7 STA.

  - CHP-296-0122:

    - Off Bridge: (ORD: STA. 62+63.13 to STA. 64+25.63) 165.70’ + 164.60’ = 331’ = 4 STA.

    - Off Bridge: (ORD: STA. 65+18.37 to STA. 66+80.87) 155.34’ + 155.76’ = 312’ = 4 STA.

    - Total = 8 STA.

  - CHP-296-0212:

    - Off Bridge: (ORD: STA. 110+80.38 to STA. 112+17.88) 152.90’ + 132.11’ = 285’ = 3 STA.

    - Off Bridge: (ORD: STA. 113+17.12 to STA. 114+54.62) 127.11’ + 147.90’ = 275’ = 3 STA.

    - Total = 6 STA.

  - Total: 27 STA.

Roadway 02

(02/S<2/13)

- Item 202E23500 WEARING COURSE REMOVED

  - CLA-40-0530:

    - On Bridge: 2679sf (from ORD) / 9= 298sy

    - Off Bridge: 2200sf + 2200.1sf (from ORD) = 4400.1sf / 9 = 489sy

    - Total = 787sy

Erosion Control 01

(01/STR/13)

- Item 601E28001 DUMPED ROCK FILL, TYPE D, AS PER PLAN

  - CHP-296-0122:

    - 2x 3’ x 3’ x 18”/12 = 27cf /27 = 1cy

  - CHP-296-0212:

    - 2x 3’ x 3’ x 18”/12 = 27cf /27 = 1cy

  - Total: 1cy + 1cy = 2cy

- Item 659E10000 SEEDING AND MULCHING

  - CHP-245-1302:

    - (ORD) 1606.23sf + 1550.01sf + 1658.82sf +1716.30sf = 6531.36sf = 726sy

  - CHP-296-0021:

    - (ORD) 1190.08sf + 807.98sf + 2206.30sf +1894.61sf = 6099sf = 678sy

  - CHP-296-0122:

    - (ORD) 1657.00sf + 1645.93sf + 1553.31sf +1557.55sf = 6413.79sf / 9 = 713sy

  - CHP-296-0212:

    - (ORD) 1528.93sf + 1321.08sf + 1271.08sf +1478.93sf = 5600sf / 9 = 623sy

  - Total: 726sy + 678sy + 713sy +623sy = 2740sy

- Item 659E14000 REPAIR SEEDING AND MULCHING

  - CHP-245-1302:

    - 726sy x 5% = 36sy

  - CHP-296-0021:

    - 678sy x 5% = 34sy

  - CHP-296-0122:

    - 713sy x 5% = 36sy

  - CHP-296-0212:

    - 623sy x 5% = 31sy

  - Total: 36sy + 34sy + 36sy + 31sy = 137sy

- Item 659E20000 COMMERCIAL FERTILIZER

  - CHP-245-1302:

    - 726sy / (7410 sy/ton) = 0.10 TON

  - CHP-296-0021:

    - 678sy / (7410 sy/ton) = 0.10 TON

  - CHP-296-0122:

    - 713sy / (7410 sy/ton) = 0.10 TON

  - CHP-296-0212:

    - 623sy / (7410 sy/ton) = 0.08 TON

  - Total: 0.10 TON + 0.10 TON + 0.10 TON + 0.08 TON = 0.38 TON

- Item 659E35000 WATER

  - CHP-245-1302:

    - 2 applications x 726sy x 0.0027MGal/sy = 4 MGal

  - CHP-296-0021:

    - 2 applications x 717sy x 0.0027MGal/sy = 4 MGal

  - CHP-296-0122:

    - 2 applications x 713sy x 0.0027MGal/sy = 4 MGal

  - CHP-296-0212:

    - 2 applications x 623sy x 0.0027MGal/sy = 3 MGal

  - Total: 4 MGal + 4 MGal + 4 MGal + 3 MGal = 15 MGal

- Item 832E30000 EROSION CONTROL

  - CHP-245-1302:

    - 1,000 EACH (minimum quantity for erosion control is sufficient)

  - CHP-296-0021:

    - 1,000 EACH (minimum quantity for erosion control is sufficient)

  - CHP-296-0122:

    - 1,000 EACH (minimum quantity for erosion control is sufficient)

  - CHP-296-0212:

    - 1,000 EACH (minimum quantity for erosion control is sufficient)

  - Total: 4,000 EACH

Erosion Control 02

(02/S<2/13)

- Item 659E10000 SEEDING AND MULCHING

  - CLA-40-0530: (ORD) 550sf x 4 = 2200sf / 9 = 245sy

- Item 659E14000 REPAIR SEEDING AND MULCHING

- CLA-40-0530: 245sy x 5% = 12sy

- Item 659E20000 COMMERCIAL FERTILIZER

- CLA-40-0530: 245sy/ (7410 sy/ton) = 0.03 TON

- Item 659E35000 WATER

- CLA-40-0530:

    - 2 applications x 245sy x 0.0027MGal/sy = 1 MGal

- Item 832E30000 EROSION CONTROL

- CLA-40-0530:

    - 1,000 EACH (minimum quantity for erosion control is sufficient)

Pavement 01

(01/STR/13)

- Item 407E10000 TACK COAT

- CHP-245-1302:

  - Off Bridge: (ORD: STA. 681+70 to STA. 683+20) 461sy x 0.085gal/sy = 40gal;

  - Off Bridge: (ORD: STA. 684+06 to STA. 685+56) 451sy x 0.085gal/sy = 39gal

(Application Rate C&MS Table 407.06-1, 0.085gal/sy)

  - Total = 79gal

- CHP-296-0021:

    - Off Bridge: (ORD: STA. 09+75.00 to STA. 10+98.34) (4505.11/9)sy x 0.085gal/sy = 43gal;

    - Off Bridge: (ORD: STA. 12+23.64 to STA. 14+25.00) (6189.00/9)sy x 0.085gal/sy = 59gal

(Application Rate C&MS Table 407.06-1, 0.085gal/sy)

    - Total = 102gal

- CHP-296-0122:

  - Off Bridge: (ORD: STA. 62+63.13 to STA. 64+25.63) (4753.06/9)sy x 0.085gal/sy = 45gal;

  - Off Bridge: (ORD: STA. 65+18.37 to STA. 66+80.87) (4713.64/9)sy x 0.085gal/sy = 45gal

(Application Rate C&MS Table 407.06-1, 0.085gal/sy)

  - Total = 90gal

- CHP-296-0212:

  - Off Bridge: (ORD: STA. 110+80.38 to STA. 112+17.88) (4246.62/9)sy x 0.085gal/sy = 41gal;

  - Off Bridge: (ORD: STA. 113+17.12 to STA. 114+54.62) (4221.55/9)sy x 0.085gal/sy = 40gal

(Application Rate C&MS Table 407.06-1, 0.085gal/sy)

  - Total = 81gal

- Total: 79gal + 102gal + 90gal + 81gal = 352gal

- Item 441E70100 ASPHALT CONCRETE SURFACE COURSE, TYPE 1, (449), PG70-22M

- CHP-245-1302:

  - (ORD: STA. 681+70 to STA. 683+20) 4146.61sf x 1.5”/12 = 518.33cf = 20cy

  - (ORD: STA. 684+06 to STA. 685+56) 4050.45sf x 1.5”/12 = 506.37cf = 19cy

  - Total = 39cy

- CHP-296-0021:

  - Off Bridge: (ORD: STA. 08+98.34 to STA. 10+98.34) 4505.11sf x 1.5”/12 = 563.14cf = 21cy

  - Off Bridge: (ORD: STA. 12+23.64 to STA. 14+23.64) 6189.00sf x 1.5”/12 = 773.63cf = 29cy

  - Total = 50cy

- CHP-296-0122:

  - Off Bridge: (ORD: STA. 62+63.13 to STA. 64+25.63) 4753.06sf x 1.5”/12 = 594.14cf / 27 = 22cy

  - Off Bridge: (ORD: STA. 65+18.37 to STA. 66+80.87) 4713.64sf x 1.5”/12 = 589.21cf / 27 = 22cy

  - Total = 44cy

- CHP-296-0212:

  - Off Bridge: (ORD: STA. 110+80.38 to STA. 112+17.88) 4246.62sf x 1.5”/12 = 530.83cf / 27 = 20cy

  - Off Bridge: (ORD: STA. 113+17.12 to STA. 114+54.62) 4221.55sf x 1.5”/12 = 527.70cf / 27 = 20cy

  - Total = 40cy

- Total: 39gal + 50gal + 44gal + 40gal = 173cy

Pavement 02

(02/S<2/13)

- Item 407E10000 TACK COAT

- CLA-40-0530:

- On Bridge: 298sy x 0.085gal/sy = 26gal

- Off Bridge: 489sy x 0.085gal/sy = 42gal

(Application Rate C&MS Table 407.06-1, 0.085gal/sy)

- Total = 69gal

- Item 441E70100 ASPHALT CONCRETE SURFACE COURSE, TYPE 1, (449), PG70-22M

- CLA-40-0530:

  - On bridge: 2796sf x 3.5”/12 = 781.4cf / 27 = 31cy

  - Off bridge: 4400sf x 1.5” / 12 = 550cf / 27 = 21cy

  - Total = 52cy

Traffic Control 01

(01/STR/13)

- Item 621E54000 RAISED PAVEMENT MARKER REMOVED

  - CHP-245-1302: (Each) 3 raised pavement markers to remove.

  - CHP-296-0021: (Each) 11 raised pavement markers to remove.

  - CHP-296-0122: (Each) 6 raised pavement markers to remove.

  - CHP-296-0212: (Each) 5 raised pavement markers to remove.

  - Total: 3+11+6+5 = 25

- Item 646E10010 EDGE LINE, 6"

- CHP-245-1302:

  - 2 edge lines x 386' /5280 = 0.15mi

- CHP-296-0021:

  - (LT.) 455.95’ / 5280 = 0.09mi

  - (RT.) 401.62’/ 5280 = 0.08mi

  - Total: 0.09mi + 0.08mi = 0.17mi

- CHP-296-0122:

  - 2 ea. x 417.74’ (from ORD) = 835.48’ / 5280 = 0.16mi

- CHP-296-0212:

  - 2 ea. x 374.24’ (from ORD) = 748.48’ / 5280 = 0.15mi

- Total: 0.15 + 0.17 + 0.16 + 0.15 = 0.63mi

- Item 646E10200 CENTER LINE

- CHP-245-1302: (DASHED) 386’/5280 = 0.08mi

- CHP-296-0021: (DOUBLE SOLID) 428.36’/5280 = 0.09mi

- CHP-296-0122: (SOLID/DASHED) 417.74’ /5280 = 0.08mi

- CHP-296-0212: (DASHED) 374.24’ / 5280 = 0.08mi

- TOTAL: 0.08mi + 0.09mi + 0.08mi + 0.08mi = 0.33mi

Traffic Control 02

(02/S<2/13)

- Item 621E54000 RAISED PAVEMENT MARKER REMOVED

  - CLA-40-0530: (Each) 1 raised pavement marker to remove.

- Item 642E00104 EDGE LINE, 6", TYPE 1

  - 2 edge lines x 167' /5280 = 0.07mi

- Item 642E00300 CENTER LINE, TYPE 1

  - (DASHED) 51’/5280 = 0.01mi

  - (SOLID/DASHED) 117’/5280 = 0.03mi

  - Total: 0.01mi + 0.03mi = 0.04mi

Structure Repair 01: CHP-245-1302

(01/STR/13)

- Item 202E11203 PORTIONS OF STRUCTURE REMOVED, OVER 20 FOOT SPAN, AS PER PLAN

- Lump Sum

- (estimate reference only) QTY Estimation:

  - Deck edge removal: 86’ (Deck edge length) x 3.2269sf (cross section area) = 278cf = 11cy

  - Wingwall removal: 2 abuts. x (11.23sf + 11.23sf) x 1.25ft = 57cf / 27 = 3cy

  - Total: 14cy

- Item 202E38602 BRIDGE RAILING REMOVED FOR REUSE

  - Total wingwall remove volume: 2 edges x 86’ = 172ft

- Item 503E21300 UNCLASSIFIED EXCAVATION

  - Lump Sum

  - (estimate reference only) QTY Estimation:

    - 4 (wing walls) x 2’ (excavation width behind the wall) x 17.5sf (cross section area: 3.5’ width x 5’ height) = 140cf = 6cy

- Item 509E20001 CONCRETE REINFORCEMENT, REPLACEMENT OF EXISTING CONCRETE REINFORCEMENT, AS PER PLAN

  - Estimated to be 100 lbs.

- Item 509E25000 UNCOATED STEEL REINFORCEMENT

  - Deck edge replacement: 1850 lbs. + Wingwall rebuild: 127 lbs. = 1977 lbs.

- Item 510E10000 DOWEL HOLES WITH NONSHRINK, NONMETALLIC GROUT

  - 32 Dowel holes wing wall rebuild.

- Item 511E34410 CLASS QC2 CONCRETE, SUPERSTRUCTURE

  - Deck edge replacement vol. = 3.03sf x 86’ = 261cf / 27 = 10 cy

- Item 511E50210 CLASS QC1 CONCRETE, SUBSTRUCTURE

  - Wingwall rebuild volume: 2 abuts. x (11.23sf + 11.23sf) x 1.25ft = 57cf / 27 = 3cy

- Item 511E81300 CONCRETE, MISC.: EMBEDDED GALVANIC ANODE

  - One anode needed every 2 ft of deck edge replaced. 86'/2' = 43 anodes.

- Item 516E31010 2" DEEP JOINT SEALER

  - 2 x 32’ = 64’

- Item 517E75600 DEEP BEAM BRIDGE RETROFIT RAILING

  - 2 sides x 86’ = 172’

- Item 517E76302 RAILING, MISC.: TYPE B ANCHORS

  - 14 posts x 4 Anchors = 56 Anchors (EACH)

- Item 518E22300 SPECIAL - STEEL DRIP STRIP

  - 14 Posts on bridge per side x 2 sides x 1.5’ + 86’ x 2 sides = 214ft

- Item 848E10201 SUPERPLASTICIZED DENSE CONCRETE OVERLAY USING HYDRODEMOLITION, AS PER PLAN, 2¼”

  - (32’ x 86’) = 2752sf/9 = 306sy

- Item 848E20001 SURFACE PREPARATION USING HYDRODEMOLITION, AS PER PLAN

  - (32’-2.5’) x 86’ = 2537sf/9 = 282sy

- Item 848E30200 SUPERPLASTICIZED DENSE CONCRETE OVERLAY (VARIABLE THICKNESS), MATERIAL ONLY

  - Buried under asphalt. Assume 50% variable thickness area: 2752sf x 0.5 = 1269sf.

  - Use BDM 403.4.1 average depth of variable thickness = 2".

  - Var. Vol = 1269sf x 2"/12 = 212cf/27 = 9 cy

- Item 848E50000 HAND CHIPPING

  - Hand Chipping Area: 10% of deck area (D07 pref.): 0.10 x 282sy = 28sy

- Item 848E50100 TEST SLAB

  - Lump Sum

- Item 848E50320 EXISTING CONCRETE OVERLAY REMOVED, 1¼”

  - (32’ x 86’) = 2752sf/9 = 306sy

- Item 848E50340 REMOVAL OF DEBONDED OR DETERIORATED EXISTING VARIABLE THICKNESS CONCRETE OVERLAY

  - 50% Hand chipping area (per D7 overlay memo): 28sy x 0.50 = 14sy.

Structure Repair 02: CHP-296-0021

(01/STR/13)

- Item 509E20001 REINFORCING STEEL, REPLACEMENT OF EXISTING REINFORCING STEEL, AS PER PLAN

  - Estimate: 50 lbs.

- Item 516E31010 2" DEEP JOINT SEALER

  - 42.6’ + 45.4' = 88'

- Item 517E75600 DEEP BEAM BRIDGE RETROFIT RAILING

  - (124.8’ + 126.2’) (from ORD) = 251’

- Item 518E22300 SPECIAL - STEEL DRIP STRIP

  - 20 Posts on bridge per side x 1.5’ x 2 sides + 126.2’ + 124.8’ = 311’

- Item 848E10200 SUPERPLASTICIZED DENSE CONCRETE OVERLAY USING HYDRODEMOLITION, 3”

  - 4765.27sf (from ORD)/9= 530sy

- Item 848E20001 SURFACE PREPARATION USING HYDRODEMOLITION, AS PER PLAN

  - 4765.27sf (from ORD)/9= 530sy

- Item 848E30200 SUPERPLASTICIZED DENSE CONCRETE OVERLAY (VARIABL THICKNES), MATERIAL ONLY

  - Buried under asphalt. Assume 50% variable thickness area: 4765.27sf x 0.5 = 2382.64sf.

  - Use BDM 403.4.1 average depth of variable thickness = 2".

  - Var. Vol = 2382.64sf x 2"/12 = 398cf/27 = 15 cy

- Item 848E50000 HAND CHIPPING

  - Hand Chipping Area: 10% of deck area (D07 pref.): 0.10 x 530sy = 53sy

- Item 848E50320 EXISTING CONCRETE OVERLAY REMOVED, 1¼”

  - 4765.27sf (from ORD)/9= 530sy

- Item 848E50340 REMOVAL OF DEBONDED OR DETERIORATED EXISTING VARIABLE THICKNESS CONCRETE OVERLAY

  - 50% Hand chipping area (per D7 overlay memo): 53sy x 0.50 = 27sy.

Structure Repair 03: CHP-296-0122

(01/STR/13)

- Item 509E20001 REINFORCING STEEL, REPLACEMENT OF EXISTING REINFORCING STEEL, AS PER PLAN

  - Estimate: 50 lbs.

- Item 516E31010 2" DEEP JOINT SEALER

  - 2 x 41.6' = 84’

- Item 517E75600 DEEP BEAM BRIDGE RETROFIT RAILING

  - 2 sides x 92.74’ = 186’

- Item 518E22300 SPECIAL - STEEL DRIP STRIP

  - 15 Posts on bridge per side x 2 sides x 1.5’ + 92.8’ x 2 sides = 231ft

- Item 848E10200 SUPERPLASTICIZED DENSE CONCRETE OVERLAY USING HYDRODEMOLITION, 2¼”

  - 3338.64sf (from ORD) / 9 = 371sy

- Item 848E20001 SURFACE PREPARATION USING HYDRODEMOLITION, AS PER PLAN

  - 3338.64sf (from ORD) / 9 = 371sy

- Item 848E30200 SUPERPLASTICIZED DENSE CONCRETE OVERLAY (VARIABLE THICKNESS), MATERIAL ONLY

  - Buried under asphalt. Assume 50% variable thickness area: 3338.64sf x 0.5 = 1669.32sf / 9 = 186sy

  - Use BDM 403.4.1 average depth of variable thickness = 2".

  - Var. Vol = 1669.32sf x 2"/12 = 279cf/27 = 11cy

- Item 848E50000 HAND CHIPPING

  - Hand Chipping Area: 10% of deck area (D07 pref.): 0.10 x 371sy = 37sy

- Item 848E50320 EXISTING CONCRETE OVERLAY REMOVED, 1¼”

  - 3338.64sf (from ORD) / 9 = 371sy

- Item 848E50340 REMOVAL OF DEBONDED OR DETERIORATED EXISTING VARIABLE THICKNESS CONCRETE OVERLAY

  - 50% Hand chipping area (per D7 overlay memo for this item): 37sy x 0.50 = 19sy.

Structure Repair 04: CHP-296-0212

(01/STR/13)

- Item 509E20001 REINFORCING STEEL, REPLACEMENT OF EXISTING REINFORCING STEEL, AS PER PLAN

  - Estimate: 50 lbs.

- Item 516E31010 2" DEEP JOINT SEALER

  - 2 sides x 41.57' = 84’

- Item 517E75600 DEEP BEAM BRIDGE RETROFIT RAILING

  - 2 sides x 99.24’ = 199’

- Item 518E22300 SPECIAL - STEEL DRIP STRIP

  - 16 Posts on bridge per side x 2 sides x 1.5’ + 99.24’ x 2 sides = 247ft

- Item 848E10201 SUPERPLASTICIZED DENSE CONCRETE OVERLAY USING HYDRODEMOLITION, AS PER PLAN, 2¼”

  - 3572.64sf (from ORD) / 9 = 397sy

- Item 848E20001 SURFACE PREPARATION USING HYDRODEMOLITION, AS PER PLAN

  - 3572.64sf (from ORD) / 9 = 397sy

- Item 848E30200 SUPERPLASTICIZED DENSE CONCRETE OVERLAY (VARIABLE THICKNESS), MATERIAL ONLY

  - Buried under asphalt. Assume 50% variable thickness area: 3572.64sf x 0.5 = 1786.32sf / 9 = 199sy

  - Use BDM 403.4.1 average depth of variable thickness = 2".

  - Var. Vol = 1786.32sf x 2"/12 = 298cf/27 = 12cy

- Item 848E50000 HAND CHIPPING

  - Hand Chipping Area: 10% of deck area (D07 pref.): 0.10 x 397sy = 40sy

- Item 848E50320 EXISTING CONCRETE OVERLAY REMOVED, 1¼”

  - 3572.64sf (from ORD) / 9 = 397sy

- Item 848E50340 REMOVAL OF DEBONDED OR DETERIORATED EXISTING VARIABLE THICKNESS CONCRETE OVERLAY

  - 50% Hand chipping area (per D7 overlay memo for this item): 40sy x 0.50 = 20sy

Structure Repair 05: CLA-40-0530

(02/S<2/13)

- Item 202E11203 PORTIONS OF STRUCTURE REMOVED, OVER 20 FOOT SPAN, AS PER PLAN

  - Lump Sum

  - (estimate reference only) QTY Estimation:

    - 2 (expansion joint) x 44’ (bridge width) x 3.0418sf (cross section area) = 268cf = 10cy

- Item 509E25000 UNCOATED STEEL REINFORCEMENT

  - Expansion joint replacement: 1053 lbs.

- Item 510E10000 DOWEL HOLES WITH NONSHRINK, NONMETALLIC GROUT

  - Expansion joint replacement: 164 Dowel holes.

- Item 511E34444 CLASS QC2 CONCRETE, SUPERSTRUCTURE

  - (ORD) (1.80sf + 1.25sf + 1.23sf + 1.70sf) x 44’ = 264cf / 27 = 10cy

- Item 512E33011 TYPE 3 WATERPROOFING, AS PER PLAN

  - (ORD) (60.89’) x 44’ = 2856cf / 9 = 298sy

- Item 516E10500 STRUCTURAL EXPANSION JOINT INCLUDING ELASTOMERIC COMPRESSION SEAL

  - 2ea x 44' = 88’

- Item 516E31010 2" DEEP JOINT SEALER

  - 4 x 44' = 176'

- Item 518E22300 SPECIAL - STEEL DRIP STRIP

  - 11 Posts on bridge per side x 2 sides x 1.5’ + 62.5’ x 2 sides = 158’

Maintenance of Traffic 01

(01/STR/13)

- Item 614E12420 DETOUR SIGNING (LS)

Maintenance of Traffic 02

(02/S<2/13)

- Item 614E12420 DETOUR SIGNING (LS)

Incidentals 01

(01/STR/13)

- Item 614E11000 MAINTAINING TRAFFIC (LS)

- Item 619E16010 FIELD OFFICE, TYPE B

    - 1 MNTH

- Item 623E10000 CONSTRUCTION LAYOUT STAKES AND SURVEYING (LS)

- Item 624E10000 Mobilization (LS)

Incidentals 02

(02/S<2/13)

- Item 614E11000 MAINTAINING TRAFFIC (LS)

- Item 623E10000 CONSTRUCTION LAYOUT STAKES AND SURVEYING (LS)

- Item 624E10000 Mobilization (LS)

END OF CALCULATIONS