| Ver. Date | 11-2-2023 |  | PID |  |  | 113662 |
| --- | --- | --- | --- | --- | --- | --- |
| PARCEL |  |  |  | 3-T | 2 |  |
| DEL-315-5.52 |  |  |  |  |  |  |

TEMPORARY EASEMENT FOR THE PURPOSE OF PERFORMING THE WORK NECESSARY TO CONSTRUCT A TEMPORARY SEDIMENT POND AND PERFORM GRADING AND SEEDING FOR MONTHS FROM DATE OF ENTRY BY THE STATE OF OHIO, DEPARTMENT OF TRANSPORTATION

[Surveyor’s description of the premises follows]

Situated in the State of Ohio, Delaware County, Part of Liberty Township, Farm Lot 8, Township 3, Range 19, Section 1 and being part of a 1.500 acre tract, conveyed to Kemper Enterprises, LLC, an Ohio Limited Liability Company, as recorded in Volume 2053 and Page 2632 in the Delaware County Recorder’s Office. The below described parcel laying on the left side of the existing centerline of right of way for SR. 315 in project DEL-315-5.52 and the right side of the existing centerline of right of way for Hyatts Road as platted by 2LMN, Inc., and being more particularly described as follows:

COMMENCING at an iron pin set on the west proposed Right of Way for S.R. 315, said pin being 50.00 feet left of existing centerline of right of way for S.R. 315 at station 297+00.00 and being the TRUE POINT OF BEGINNING for the parcel herein described:

- Thence, along the west proposed Right of Way for S.R. 315, South 25 Degrees 55 Minutes 43 Seconds West a distance of 29.90 feet to an iron pin set, said pin being 50.00 feet left of the existing centerline of right of way for S.R. 315 at Station 296+70.10;

- Thence, along the west proposed right of way for S.R. 315, with a curve to the left, having a radius of 766.20 feet, a central angle of 01 degrees 36 minutes 29 seconds, and an arc length of 21.50 feet, said curve has a chord which bears South 25 degrees 07 minutes 29 seconds West and has a distance of 21.50 feet to a point, said point being 50.00 feet left of the existing centerline of right of way for S.R. 315 at Station 296+50.00;

- Thence, across grantor’s said 1.500 acre tract, North 41 Degrees 46 Minutes 47 Seconds West a distance of 143.36 feet to a point on the south proposed Right of Way of Hyatts Road, said pin being 65.00 feet right of the existing centerline of right of way for Hyatts Road at Station 5+75.00;

- Thence, along the south proposed Right of Way of Hyatts Road, South 86 Degrees 22 Minutes 45 Seconds East a distance of 105.00 feet to an iron pin set on west proposed Right of Way for S.R. 315, said pin being 65.00 feet right of the existing centerline of right of way for Hyatts Road at Station 6+80.00, and 85.20 feet left of the existing centerline of right of way for S.R. 315 at Station 297+42.84;

- Thence, along the west proposed Right of Way for S.R. 315, South 13 Degrees 29 Minutes 06 Seconds East a distance of 55.44 feet, to the TRUE POINT OF BEGINNING, containing 0.142 acres.

It is understood that the parcel of land described contains, 0.142 acres, more or less, including the present road occupies 0.000 acres, and is located in Delaware County Auditor’s Parcel Number 319-120-01-029-000.

All irons pins set are 3/4 inch x 30 inch rebar with a 2 inch aluminum cap stamped “ODOT R/W

P.S. 7798 Richard F. Mathias”.

Description prepared from an actual field survey prepared by 2LMN, Inc. under the supervision of Richard F. Mathias, P.S. # 7798, July, 2020.

Grantor claim title by Volume 2053 and Page 2632, as recorded in the Delaware County Recorder’s Office.

All bearings shown are for project use only and are from the Ohio State Plane Coordinate System, North Zone, NAD 83 (2011).  As established by GPS measurements in 2020.

_______________________________						________________

Richard F. Mathias, P.S.									Date

Professional Land Surveyor No. 7798