| Ver. Date | 8-11-2023 |  | PID |  |  | 113662 |
| --- | --- | --- | --- | --- | --- | --- |
| PARCEL |  |  |  | 5-T |  |  |
| DEL-315-5.52 |  |  |  |  |  |  |

TEMPORARY EASEMENT FOR THE PURPOSE OF PERFORMING THE WORK NECESSARY TO CONSTRUCT A DRIVE AND PERFORM GRADING AND SEEDING FOR MONTHS FROM DATE OF ENTRY BY THE STATE OF OHIO, DEPARTMENT OF TRANSPORTATION

[Surveyor’s description of the premises follows]

Situated in the State of Ohio, Delaware County, Part of Liberty Township, Farm Lot 8, Township 3, Range 19, Section 1 and being part of a 1.090 acre tract conveyed to Susan M. Davy, as recorded in Volume 1170 and Page 1170 in the Delaware County Recorder’s Office. The below described parcel laying on the left side of the existing centerline of right of way for SR. 315 in project DEL-315-5.52 as platted by 2LMN, Inc., and being more particularly described as follows:

COMMENCING at a point on the grantor’s east line and the west existing Standard Highway easement S.R. 315, said pin being 30.00 feet left of the existing centerline of right of way for S.R. 315 at station 294+07.17 and being the TRUE POINT OF BEGINNING for the parcel herein described:

- Thence, along the west existing Standard Highway easement S.R. 315, with a curve to the left, having a radius of 746.20 feet, a central angle of 00 degrees 44 minutes 05 seconds, and an arc length of 9.57 feet, said curve has a chord which bears South 04 degrees 31 minutes 38 seconds West and has a distance of 9.57 feet, to a point, said point being 30.00 feet left of the existing centerline of right of way for S.R. 315 at Station 293+97.99;

- Thence, along the grantor’s south line, South 85 Degrees 45 Minutes 12 Seconds West a distance of 15.16 feet to a point, said point being 45.00 feet left of the existing centerline of right of way for S.R. 315 at Station 293+95.91;

- Thence, across the grantor’s tract, North 00 Degrees 51 Minutes 48 Seconds West a distance of 96.55 feet to a point on the grantor’s east line, said point being 59.17 feet left of the existing centerline of right of way for S.R. 315 at Station 294+85.00;

- Thence, along the grantor’s east line, South 11 Degrees 24 Minutes 26 Seconds East a distance of 87.61 feet to the TRUE POINT OF BEGINNING, containing 0.019 acres.

It is understood that the parcel of land described contains, 0.019 acres, more or less, including the present road occupies 0.000 acres, more or less and is located in Delaware County Auditor’s Parcel Number 319-120-01-028-000.

All irons pins set are 3/4 inch x 30 inch rebar with a 2 inch aluminum cap stamped “ODOT R/W

P.S. 7798 Richard F. Mathias”.

Description prepared from an actual field survey prepared by 2LMN, Inc. under the supervision of Richard F. Mathias, P.S. # 7798, July, 2020.

Grantor claim title by Volume 1170 and Page 1170, as recorded in the Delaware County Recorder’s Office.

All bearings shown are for project use only and are from the Ohio State Plane Coordinate System, North Zone, NAD 83 (2011).  As established by GPS measurements in 2020.

_______________________________						________________

Richard F. Mathias, P.S.									Date

Professional Land Surveyor No. 7798